3GPP TSG-RAN WG2 Meeting #109-e	Draft R2-2001950

Electronic meeting, 24 Feb – 6 Mar 2020

Source:	CATT

Title:	[AT109e][621][POS] Open issues on broadcast assistance data

Agenda Item:	6.8.2.4

Document for:	Discussion and Decision

# Introduction

This contribution provides a summary of views and agreements on the three topics as below which were updated by Chairman at the end of this week.

- [AT109e][621][POS] Open issues on broadcast assistance data (CATT)

Status: Started

Scope: Discuss the following issues on broadcast assistance data:

-       Whether to have a separate positioning system information area ID or reuse the existing area ID;

-       Whether to support the UE including GNSS ID/SBAS ID in the on-demand SI request;

-       Whether to include segment number in NRPPa metadata.

Intended outcome: Summary of agreements on the identified issues.  Summary in R2-2001950.

Deadline:  Wednesday 2020-03-04 1300 CET

# Discussion

## Whether a separate positioning system information area ID or reuse the existing area ID

The previous email discussion in R2-2001241[1] shows that 12/16 companies prefer to have a separate System Area ID for posSI, and 4/16 companies prefer not a separate System Area ID. This contribution provides a summary of views and agreements on the separate System Area ID.

- Reasons to introduce a separate positioning system information area ID:

Validity areas of positioning assistance data are different from the current area scope of common SIBs. Furthermore, validity areas of different positioning assistance data are different too. For example, the validity area of GPS positioning assistance data is larger than the validity area of DL-TDOA. So we need to introduce a separate System Area ID set from LMF for broadcast positioning assistance data.

- How to define a separate area ID?

Here is an example which is based on Running CR 38.331 in R2-2002112[2]. The separate System Area ID is defined as posSI-AreaID-r16 in PosSI-SchedulingInfoList.

PosSI-SchedulingInfo-r16 ::=	SEQUENCE {

offsetToSI-Used-r16			ENUMERATED {true}		OPTIONAL,	-- Need R

posSI-Periodicity-r16		ENUMERATED {rf8, rf16, rf32, rf64, rf128, rf256, rf512},

posSIB-MappingInfo-r16		PosSIB-MappingInfo-r16,

posSI-AreaID-r16 BIT STRING (SIZE (24))                            OPTIONAL,   -- Need R // Editorial Note: This is FFS

...

}

Please refer to the Annex for the text proposal in detail.

Question1. Do companies agree the separate positioning system information area ID? Please provide also a brief justification for your answer.

| Company | Yes/No | Comments |
| --- | --- | --- |

## Whether to support the UE including GNSS ID/SBAS ID in the on-demand SI request

- Reasons to introduce GNSS ID/SBAS ID in the on-demand SI request:

More than one company suggest that the UE include GNSS id or SBAS id in on-demand SI request in R2-2002053[4].

Proposal 2	Beside pos SIB type, the UE may include GNSS id or SBAS id together in on-demand SI request for broadcast positioning assistance data.

For example, if the UE only wants GPS AD in on-demand SI request, the network now will send all stored assistance data with kinds of GNSS, e.g. GPS, SBAS, QZSS and so on to UE. This will waste broadcast on air resources considering GPS AD VS. all A-GNSS AD.

- How to define it?

Here is an example which is based on Running CR 38.331 in R2-2000876[3] how the GNSS ID/SBAS ID is defined in DedicatedSIBRequest.

------------------Text proposal 2------------------------------

- DedicatedSIBRequest

-- ASN1START

-- TAG-DEDICATEDSIBREQUEST-START

DedicatedSIBRequest-r16-IEs ::=    SEQUENCE {

onDemandSIB-RequestList-r16                SEQUENCE   {

-- FFS the size of requestedSIB-List-r16 depends by how many SIBs will be specified in Release 16.

requestedSIB-List-r16                 SEQUENCE (SIZE(1..FFS)) OF SIB-ReqInfo-r16, OPTIONAL,

requestedposSIB-List-r16				SEQUENCE (SIZE(1..maxSIB)) OF PosSIB-ReqInfo-16,				 OPTIONAL,

lateNonCriticalExtension              OCTET STRING      OPTIONAL,

nonCriticalExtension                  SEQUENCE {}       OPTIONAL

}

}

-- FFS: Wheter SIB9 is in the scope of the on-demand SIB framework need to be confirmed by the IIoT WI.

SIB-ReqInfo-r16 ::=                            ENUMERATED {sib9, spare7, spare6, spare5, spare4, spare3, spare2, spare1}

PosSIB-ReqInfo-16 ::=							SEQUENCE {

gnss-id-r16			GNSS-ID-r16						OPTIONAL,

sbas-id-r16			SBAS-ID-r16						OPTIONAL,

posSibType-r16		ENUMERATED {ffs,... },//editor notes: discussion in [AT109e][601]

}

------------------End of Text proposal 2------------------------------

Question2. Do companies agree to include GNSS ID/SBAS ID in the on-demand SI request message? Please provide also a brief justification for your answer.

| Company | Yes/No | Comments |
| --- | --- | --- |

## Whether to include segment number in NRPPa metadata

There is a proposal in R2-2002053[4] on Segment Number.

- Proposal 12	Segment number is added in NRPPa meta data.

The proponent company provides the below argumentation:

- For on demand connected mode procedure, if segment number is known to gNB, it will save radio resource as UE can request with which segment number is needed and gNB thus provides only that segment number instead of all the segments.

- In order to provide in sequence delivery of AD via SI scheduling, it is needed for the base station to be aware of segment number.

- For SI scheduling, it would benefit if UE receives the segments in an order. It would simplify arranging the segment numbers and processing in UE.

Question3. Do companies agree to include segment number in NRPPa metadata? Please provide also a brief justification for your answer.

| Company | Yes/No | Comments |
| --- | --- | --- |

# Conclusion

To be updated

# References

[1] R2-2001241	Summary of [108#88][NR/Rel-16] Remaining issues on broadcast assistance data	Ericsson	discussion	Rel-16	Late

[2] R2-2002112	 Running CR for the introduction of NR positioning 	Ericsson	draftCR	Rel-16	38.331	15.8.0	B	NR_pos-Core

[3] R2-2000876	Running CR on 38.331 for on-demand SIB(s) in CONNECTED	Ericsson	CR	Rel-16	38.331	15.8.0	1462	-	B	NR_unlic-Core, 5G_V2X_NRSL-Core, NR_IIOT-Core

[4] R2-2002053	Summary for Broadcast of Assistance Data	Ericsson		discussion	Rel-16	NR_pos-Core

# Annex

------------------ A separate area ID text proposal  ------------------------------

-- ASN1START

-- TAG-POSSI-SCHEDULINGINFOLIST-START

PosSI-SchedulingInfoList-r16 ::= SEQUENCE (SIZE (1..maxSI-Message)) OF PosSI-SchedulingInfo-r16

PosSI-SchedulingInfo-r16 ::=	SEQUENCE {

offsetToSI-Used-r16			ENUMERATED {true}		OPTIONAL,	-- Need R

posSI-Periodicity-r16		ENUMERATED {rf8, rf16, rf32, rf64, rf128, rf256, rf512},

posSIB-MappingInfo-r16		PosSIB-MappingInfo-r16,

posSI-AreaID-r16 BIT STRING (SIZE (24))                            OPTIONAL,   -- Need R // Editorial Note: This is FFS

...

}

PosSIB-MappingInfo-r16 ::= SEQUENCE (SIZE (1..maxSIB)) OF PosSIB-Type-r16

PosSIB-Type-r16 ::= SEQUENCE {

encrypted-r16		ENUMERATED { true }				OPTIONAL,		-- Need R

gnss-id-r16			GNSS-ID-r16						OPTIONAL,		-- Need R

sbas-id-r16			SBAS-ID-r16						OPTIONAL,		-- Need R

posSibType-r16		ENUMERATED { posSibType1-1, posSibType1-2, posSibType1-3, posSibType1-4, posSibType1-5, posSibType1-6,

posSibType1-7, posSibType1-8, posSibType2-1, posSibType2-2, posSibType2-3, posSibType2-4, posSibType2-5,

posSibType2-6, posSibType2-7, posSibType2-8,	posSibType2-9, posSibType2-10, posSibType2-11,

posSibType2-12, posSibType2-13, posSibType2-14, posSibType2-15, posSibType2-16, posSibType2-17,

posSibType2-18, posSibType2-19, posSibType2-20, posSibType2-21, posSibType2-22, posSibType2-23,

posSibType3-1, ffs,	... },

areaScope-r16      ENUMERATED {true}                                                OPTIONAL -- Need S

}

PosSI-SchedulingInfoList field descriptions

areaScope Indicate that the posSIB is area specific. If it is absent, it means the posSIB is only valid in the broadcast cell.

encrypted The presence of this field indicates that the pos-sib-type is encrypted as specified in TS 37.355 [XX].

gnss-id The presence of this field indicates that the positioning SIB type is for a specific GNSS. Indicates a specific GNSS (see also TS 37.355 [XX])

pos-sib-MappingInfo List of the posSIBs mapped to this SystemInformation message.

posSibType The positioning SIB type is defined in TS 37.355 [XX].

posSI-AreaID Indicates the Positioning system information area that the posSI belongs to, if any. Any SIB with areaScope within the SI is considered to belong to this posSI-AreaID.

posSi-Periodicity Periodicity of the SI-message in radio frames, such that rf8 denotes 8 radio frames, rf16 denotes 16 radio frames, and so on.

offsetToSI-Used This field, if present indicates that the SI messages in PosSchedulingInfoList are scheduled with an offset of 8 radio frames compared to SI messages in SchedulingInfoList. offsetToSI-Used may be present only if the shortest configured SI message periodicity for SI messages in SchedulingInfoList is 80ms.

sbas-ID The presence of this field indicates that the positioning SIB type is for a specific SBAS. Indicates a specific SBAS (see also TS 37.355 [XX]).

------------------ End of a separate area ID Text proposal ------------------------------