3GPP TSG-RAN WG2 #109-e	draft_R2-200xxxx

Online, 24 Feburary – 06 March 2020

Agenda item:	6.4.3.1

Source: 	LG Electronics Inc. (Rapporteur)

Title: 		[Offline Disc#704] Identified proposals to V2X MAC

Document for:	Discussion and decision

# Introduction

This is the trigger of offline discussion #704:

R2-2001962	Summary of contributions to MAC for 5G V2X with NR Sidelink	LG Electronics	discussion	Rel-16	5G_V2X_NRSL

- Proposal 1.1 (with the correction of TYPO), 2.2, 2.3, 2.4, 3.1 (with the removal of “zero”. FFS for “zero” case), 4, 7, 10, 11, (FFS for configured grant case) and 15.1 are agreed.

- Proposal 1.2, 2.1 (including the need of the scenario to be considered), 3.2, 3.3, 5.1, 5.2, 6.1, 6.2, 8, 9.1, 9.2, 12.1, 12.2, 13.1, 14.1, 14.2, 14.3, 14.4, 15.2 and 15.3 will be discussed in offline.

- [Offline Disc#704]: To discuss the proposals identified in the above for further offline discussion (LG, R2-2001968) (Comeback Thurs.)

The deadline of inputs to offline discussion: Wednesday, 26th of February by 23:00 p.m. CET

In this document, the numbers of issues in R2-2001962 are reused.

# Offline discussion

#### Issue 1: Handling of unknown, unforeseen and erroneous protocol data for sidelink unicast.d

RAN2#108 agreement:

The UE shall discard the MAC PDU subheaders containing reserved values and the corresponding MAC SDUs for SL-SCH reception, at least for broadcast and groupcast, as in LTE. FFS for unicast.

RAN2#109e agreement:

Clause 5.13 is modified at least for broadcast and groupcast as follows:

When a MAC entity receives a MAC PDU on SL-SCH containing a Reserved LCID value, the MAC entity shall: 1> discard the received subPDU.

The related proposals are available below:

| Company | Tdoc | Level 1 Proposals |
| --- | --- | --- |
| Vivo | R2-2000286 | - For sidelink unicast, groupcast and broadcast, a MAC entity shall discard the received MAC subPDU containing reserved LCID values on SL-SCH. |
| Lenovo, Motorola Mobility | R2-2001023 | - For sidelink unicast, when a MAC entity receives a MAC PDU on SL-SCH containing reserved values, the MAC entity shall: discard the received subPDU and any remaining subPDUs in the MAC PDU |
| Samsung | R2-2001339 | - Proposal 1: The UE can discard a received MAC subPDU on SL-SCH containing a reserved LCID value or an LCID value the UE does not support, and any remaining subPDU in the MAC PDU. - Proposal 2: The UE can discard a received MAC subPDU on SL-SCH containing an LCID value which is not configured. |
| OPPO | R2-2000195 | - If receives a MAC PDU on SL-SCH containing reserved value, the UE shall discard the received subPDU and any remaining subPDUs in the MAC PDU, for unicast. |
| CATT | R2-2000205 | - For SL unicast, the UE shall discard the MAC PDU subheaders containing reserved values and the corresponding MAC SDUs for SL-SCH reception. |
| Spreadtrum | R2-2000562 | - For unicast, The UE shall discard the MAC PDU subheaders containing reserved values, the corresponding MAC SDUs and any remaining subPDUs in the MAC PDU for SL-SCH reception. |
| ZTE | R2-2000259 | - It is suggested to follow NR V2X broadcast and groupcast behaviour, the UE shall discard the MAC PDU subheaders containing reserved values and the corresponding MAC SDUs for SL-SCH reception for unicast. |

For SL unicast, one UE will send the RRCReconfigurationSidelink message to configure sl-LogicalChannelIdentity to a peer UE. Thus, we wonder if UE need to check whether the LCID has been configured by a peer UE.

Observation 1: The RRCReconfigurationSidelink message is used for a UE to configure sl-LogicalChannelIdentity to a peer UE.

Question 1A:	How should UE handles the case when a MAC entity receives a MAC PDU on SL-SCH containing an LCID value which is not configured for a PC5 unicast link?

- Option A1: As in Uu, the UE discards a received MAC subPDU on SL-SCH containing an LCID value which is not configured.

- Option A2: As in broadcast and groupcast, when a MAC entity receives a MAC PDU on SL-SCH containing reserved values, the UE discards the MAC PDU subheaders containing reserved values and the corresponding MAC SDUs (i.e. only the received subPDU containing a reserved value).

- Option A3: If receives a MAC PDU on SL-SCH containing reserved value, the UE shall discard the received subPDU and any remaining subPDUs in the MAC PDU.

| Company | Preferred Option | Comment |
| --- | --- | --- |
| OPPO | A1 for LCID not configured A3 for reserved LCID and not supported | When a MAC entity receives a MAC PDU for the MAC entity's C-RNTI or CS-RNTI, or by the configured downlink assignment, containing a Reserved LCID value, or an LCID value the MAC Entity does not support, the MAC entity shall at least: 1> discard the received subPDU and any remaining subPDUs in the MAC PDU. When a MAC entity receives a MAC PDU for the MAC entity's C-RNTI or CS-RNTI, or by the configured downlink assignment, containing an LCID value which is not configured, the MAC entity shall at least: 1> discard the received subPDU. Similar to Uu spec cited above, we believe there are 3 cases here: 1 – for reserved LCID: UE does not understand the format of the MAC SDU/CE, i.e., it cannot know the length of the related MAC CE, due to the introduction of MAC CE of fixed/variable length, as we already have done for CSI report MAC CE. So the whole MAC PDU should be discarded 2 – for LCID not supported: same as 1 above. 3 – for LCID not configured: UE understand the format of the MAC SDU/CE, so it can just skip the subPDU while keep the other subPDUs. |
| Ericsson | Option A2 with comment | In our view, both OptionA1 or Option A2 could work at the end, we prefer OptionA2 since it gives unified behaviour among unicast/groupcast/broadcast. |
| Interdigital | Option A2 | We prefer a unified approach. |
| Nokia | A1 the not configured A3 for others | We agree to OPPOs observation which means that the UE behaviour may become undefined due to variable length MAC CE. The specification effort is not significant in this approach, but we mat have a more future proof and reliable solution. |
| Intel | A2 | Same view as Ericsson and Interdigital |
| Apple | A1 fro LCID not configured; A3 for others | Same view as OPPO and Nokia |
| Huawei | A1 for LCID not configured; A3 for others | Same view as OPPO |

Question 1B:	How should UE handles the case when a MAC entity receives a MAC PDU on SL-SCH containing a reserved LCID value for a PC5 unicast link? (Option 1 is not valid)

- Option B2: As in broadcast and groupcast, when a MAC entity receives a MAC PDU on SL-SCH containing reserved values, the UE discards the MAC PDU subheaders containing reserved values and the corresponding MAC SDUs (i.e. only the received subPDU containing a reserved value).

- Option B3: If receives a MAC PDU on SL-SCH containing reserved value, the UE shall discard the received subPDU and any remaining subPDUs in the MAC PDU.

| Company | Preferred Option | Comment |
| --- | --- | --- |
| OPPO | B2 for LCID not configured B3 for reserved LCID and not supported | When a MAC entity receives a MAC PDU for the MAC entity's C-RNTI or CS-RNTI, or by the configured downlink assignment, containing a Reserved LCID value, or an LCID value the MAC Entity does not support, the MAC entity shall at least: 1> discard the received subPDU and any remaining subPDUs in the MAC PDU. When a MAC entity receives a MAC PDU for the MAC entity's C-RNTI or CS-RNTI, or by the configured downlink assignment, containing an LCID value which is not configured, the MAC entity shall at least: 1> discard the received subPDU. Similar to Uu spec cited above, we believe there are 3 cases here: 1 – for reserved LCID: UE does not understand the format of the MAC SDU/CE, i.e., it cannot know the length of the related MAC CE, due to the introduction of MAC CE of fixed/variable length, as we already have done for CSI report MAC CE. So the whole MAC PDU should be discarded 2 – for LCID not supported: same as 1 above. 3 – for LCID not configured: UE understand the format of the MAC SDU/CE, so it can just skip the subPDU while keep the other subPDUs. |
| Ericsson | OptionB2 with comment | Same comment as in Q1A |
| Interdigital | Option B2 | Same comment as in Q1A. |
| Nokia | B2 for LCID not configured B2 for the others | Same comment as Q1A |
| Intel | B2 |  |
| Apple | B2 for LCID not configured; B3 for others |  |
| Huawei | B2 for LCID not configured; B3 for others |  |

#### Issue 2: SL operation under SL-incapable RAN or inter-RAT SL control

The related proposals are available below:

| Company | Tdoc | Level 1 Proposals |
| --- | --- | --- |
| Apple | R2-2000608 | - Proposal 1: When UE is under SL incapable RAN node, UE uses LTE V2X method for UL/SL prioritization, i.e., only emergency call and MSG1/MSG3 in RACH gets prioritized. |
| OPPO | R2-2000202 | - Proposal7: Confirmation MAC CE for NR controlled V2X sidelink transmission should not be introduced in either 36.321 or 38.321. |
| Intel | R2-2000458 | - Proposal 1: LTE MAC BSR enhancements for NR SL dynamic mode 1 resource allocation are not supported. - Proposal 2: NR MAC BSR enhancements for LTE SL dynamic mode 3 resource allocation are not supported. |

As indicated in R2-2000608, UE supports two communication links, i.e., SL and UL, but the RAN node does not support the SL configuration due to reasons like the RAN node is not upgraded yet. That is to say, UE can only rely on pre-configuration message to handle its SL communication. It should be noted that this is a practical scenario in the field where the vehicles capable of V2X communication actually operate without RAN involvement.

Especially, if no OAM can be assumed between NR V2X control function and NG-RAN, NR V2X control function should have no knowledge about the UL LCH configuration criteria used at NG-RAN. In another word, NR V2X control function is in no place to make such configuration on UL LCH priority threshold.

Scenario 2.1: RAN may not always provide SL configuration/function to UE e.g. when the RAN node is not upgraded yet in R2-2000608. Thus, how the MAC entity performs SL operation seems unclear in this scenario.

Question 2A:	Do you agree that Scenario 2.1 should be taken into account in REL-16 WI?

| Company | Yes/No | Comment |
| --- | --- | --- |
| OPPO | Yes | This is always possible, e.g., SL may be operated in a scenario where the RAN does not support SL, e.g., the “non-operator-managed” geographical area, as defined for LTE-V2X. |
| Ericsson | Yes |  |
| Interdigital | Yes |  |
| Nokia | Yes | The earlier we handle this scenario, the more relevant a solution will be. |
| Intel | Yes |  |
| Apple | Yes | The UE may operate in an area where the RAN (gNB) does not support SL, but still support normal Uu traffic |
| Huawei | No | Anyway, pre-configuration should always exists for the UE supporting V2X. When RAN cannot provide SL configuration to UE, it only means that the pre-configuration cannot be updated by the CN, but, for the UE supporting V2X, pre-configuration is still there. |

According to R2-2000608, several potential approaches as below could help to solve the dilemma:

- Option B1: Leave it to NW implementation

- Option B2: Using LTE V2X method for SL/UL prioritization, i.e., only emergency call and MSG1/MSG3 in RACH gets prioritized.

  - According to the running CR, it has been specified that if a threshold for UL or SL is not provided, the concerned transmission cannot be prioritized based on the threshold (e.g. sl-PrioritizationThres, ul-PrioritizationThres). Thus, only ‘non-threshold’ based SL/UL prioritization would work.

- Option B3: Introduce QoS flow level priority in pre-configuration, and leave it to UE to handle the logical channel priority to always prioritize the certain QoS flows.

Question 2B:	How should UE perform SL/UL prioritization in Scenario 2.1 (if yes in 2.1)?

| Company | Preferred Option | Comment |
| --- | --- | --- |
| OPPO | B2 + including SL threshold into the pre-configuration | As in LTE, besides what is described in B2, one additional tool is that sl-PrioritizationThres is also included in pre-configuration. |
| Ericsson | OptionB2 | It seems the most straightforward way |
| Interdigital | Option B2 | We prefer to maintain the LTE baseline. |
| Nokia | B2 | We again agree with OPPO that a pre-configured sl-PrioritizationThres will easily solve the issue raised by Apple of unknown prioritisation rules |
| Intel | B2 | Ok to follow LTE approach |
| Apple | B2 | Reuse LTE-V2X approach is a straight-forward way for this corner case |
| Huawei | Rely on pre-configuration | One possible solution is that these two thresholds (i.e. sl-PrioritizationThres, ul-PrioritizationThres) are anyway provided in pre-configuration for the UE supporting NR SL based on proper NW implementation. If the UE is under the RAN node which not supports V2X, the UE will use the thresholds configured by pre-configuration for UL/SL prioritization. |

#### Issue 3: Scheduling Request for the SL-CSI reporting

RAN2#109e agreement:

For mode1 if there is no configured SL-resource, a SL CQI/RI reporting MAC CE may trigger SR and be mapped to one SR configuration. FFS on zero SR configuration

The related proposals are available below:

| Company | Tdoc | Level 1 Proposals |
| --- | --- | --- |
| Huawei | R2-2000711 | - Proposal 1: Confirm the working assumption “For mode1 if there is no configured SL-resource, a SL CQI/RI reporting MAC CE may trigger SR and be mapped to zero or one SR configuration” as an agreement. - Proposal 2: For each destination which the UE needs to report Sidelink CSI, the gNB may configure an SR configuration ID associated with the Sidelink CSI reporting. When the SR is triggered by the Sidelink CSI reporting of a destination, the UE shall use the SR configuration that is indicated by the associated SR configuration ID to transmit the SR. - Proposal 9: The pending SR triggered by a Sidelink CSI reporting shall be cancelled, when the Sidelink CSI reporting itself is cancelled. |
| InterDigital | R2-2000547 | - Proposal 2: For mode1 if there is no configured SL-resource, a SL CQI/RI reporting MAC CE may trigger SR and be mapped to zero or multiple SR configurations. The maximum number of SR configurations is pending further discussion in RAN1. |
| OPPO | R2-2000195 | - Proposal 2 : Add cancellation condition for SR triggered by CSI report: All pending SR(s) triggered according to the CSI Report procedure (clause 5.x.1.7) shall be cancelled and each respective sr-ProhibitTimer shall be stopped when the SL grant(s) can accommodate all pending CSI report. - Proposal 3: RAN2 confirm SL CSI reporting cannot be mapped to zero SR configuration. Otherwise, at least no RACH procedure should be triggered. - Proposal 4 : All pending SR(s) triggered by either Sidelink BSR or Sidelink CSI report shall be cancelled when UE is reconfigured to autonomous resource selection mode. |
| LG | R2-2000237 | - All pending SR(s) triggered according to the SL-CSI reporting (clause 5.x.1.7) shall be cancelled when the SL grant(s) can accommodate all SL-CSI reporting(s) that have been triggered but not cancelled. |
| Samsung | R2-2000229 | - Proposal 2: The pending SR for SL-CSI reporting of a destination should be cancelled upon transmission of Sidelink CSI Reporting MAC CE for the destination for which SL-CSI report is triggered. - Proposal 3: RAN2 should discuss and agree on one of the following: - Upon triggering a SL-CSI report for a destination, if SL-CSI reporting is not mapped to any SR configuration, MAC entity can trigger a SL BSR. - SR configuration is always provdied for SL-CSI reporting. |
| Ericsson | R2-2000950 | - Proposal 6 Agree the work assumption on UE behavior when there is SL CSI report to transmit but no configured SL grant. - Proposal 7 SR triggered by SL CSI report MAC CE should not trigger a SL BSR. |

On top of the above agreement, RAN2 is requested to further discuss how gNB provides a UE with SR configuration associated with the Sidelink CSI reporting.

- Option A1: For one or more destinations which the UE needs to report Sidelink CSI, the gNB may configure an SR configuration ID associated with the Sidelink CSI reporting. When the SR is triggered by the Sidelink CSI reporting of a destination, the UE shall use the SR configuration that is indicated by the associated SR configuration ID to transmit the SR.

- Option A2: Regardless of which destination the UE need to report Sidelink CSI for, the gNB may configure an SR configuration ID associated with the Sidelink CSI reporting for unicast. When the SR is triggered by the Sidelink CSI reporting of any destination, the UE shall use the SR configuration that is indicated by the associated SR configuration ID to transmit the SR.

Question 3A:	How should gNB provide SR configuration associated with the Sidelink CSI reporting?

| Company | Preferred Option | Comment |
| --- | --- | --- |
| OPPO | A2 | Considering SL grant is not destination specific anyway, we do not understand the motivation to make SR to be destination specific. |
| Ericsson | Option A1 | Similar as data transmission, where for each LCH there could be a corresponding SR configuration, it could be beneficial to configure SR resources for CSI report in a per destination manner. As such more resources could be provided to UE if the UE currently maintains multiple unicast links. |
| Interdigital | Option A2 (with comments) | We don’t see any benefit to having a destination-specific SR, since (as we mentioned online) the grant is not destination specific. The SR will be triggered due to the UE having only MAC CE to be sent (no data in the buffers) and so the NW grant will be the same regardless of which destination triggered the SR. We think instead that multiple SR configurations should instead be used to handle the RAN1 requirement that the CSI reporting delay can be configured to a value between 3ms-20ms. |
| Nokia | A1 | Seems like a more efficient solution to be able to configure SR resources for CSI report in a per destination manner |
| Intel | A2 | Same view as OPPO |
| Apple | A2 | No need for destination-specific SR configuration |
| Huawei | Option A1 | Actually, we think for each destination which the UE needs to report Sidelink CSI, the gNB need configure an SR configuration ID associated with the Sidelink CSI reporting. If more than one DST in which CSI reporting is triggered, the UE might use different SR configurations to signal multiple SRs respectively and makes the gNB know that multiple SL grants should be provided respectively for the CSI reporting MAC CE transition of different DSTs. |

In addition, according to the current version of the MAC CR, all pending SR(s) triggered according to the Sidelink BSR procedure (clause 5.x.1.6) shall be cancelled and each respective sr-ProhibitTimer shall be stopped when the SL grant(s) can accommodate all pending data available for transmission in sidelink.

In the endorsed running CR:

All pending SR(s) triggered according to the Sidelink BSR procedure (clause 5.x.1.6) shall be cancelled and each respective sr-ProhibitTimer shall be stopped when the SL grant(s) can accommodate all pending data available for transmission in sidelink.

Editor’s Note: FFS when all pending SR(s) triggered by or the SL-CSI reporting shall be cancelled.

For sidelink, CSI reporting can additionally trigger SR. Thus, it seems logical to define how pending SRs triggered by SL-CSI reporting can be cancelled. We propose that all pending SR(s) triggered according to the SL-CSI reporting (clause 5.x.1.7) shall be cancelled when the SL grant(s) can accommodate all SL-CSI reporting(s) that have been triggered but not cancelled.

Proposal 3.3: RAN2 is suggested to discuss that

- Option B1: All pending SR(s) triggered according to the SL-CSI reporting shall be cancelled and each respective sr-ProhibitTimer shall be stopped when the SL grant(s) can accommodate all SL-CSI reporting(s) that have been triggered but not cancelled.

- Option B2: The pending SR triggered according to the SL-CSI reporting for a destination shall be cancelled and each respective sr-ProhibitTimer shall be stopped when the SL grant(s) can accommodate the SL-CSI reporting that have been triggered but not cancelled.

Question 3B:	How should all pending SR(s) triggered by or the SL-CSI reporting be cancelled.?

| Company | Preferred Option | Comment |
| --- | --- | --- |
| OPPO | B1 | B2 already exists in current running CR in section 5.x.1.7. But B1 is not covered yet. |
| Ericsson | Option B2 | Similar as commented in Q3A, it seems reasonable to treat the SL CSI report in a per destination manner, so only pending SR for a destination shall be cancelled if the relevant CSI report can be transmitted. |
| Interdigital | Option B1 | Consistent with our answer for 3A, we don’t think this needs to be done per destination. |
| Nokia | B2 |  |
| Intel | B1 | Following our comment in the question above |
| Apple | B1 | AS explained in Q3A |
| Huawei | Option B2 | As mentioned in Q3A, the gNB can configure an SR configuration ID associated with the Sidelink CSI reporting for each destination, therefore, it is nature that the pending SR(s) triggered according to the SL-CSI reporting for each destination shall be cancelled respectively. For option B1, if there are pending SRs for more than one destinations, all pending SRs can be cancelled only when SL grants (at least equal to the number of destination) accommodate all SL-CSI reporting, for such case, the cancelation of some SRs is delayed, since the corresponding CSI reporting has already transmitted. |

For SL-BSR in the running CR:

All triggered SL-BSRs shall be cancelled, and retx-BSR-Timer and periodic-BSR-Timer shall be stopped, when upper layers configure autonomous resource selection.

Proposal 3.4: Like cancellation of SL-BSR, all pending SR(s) triggered by either Sidelink BSR or Sidelink CSI report shall be cancelled when UE is reconfigured to autonomous resource selection mode.

Question 3C:	Do you agree that all pending SR(s) triggered by either Sidelink BSR or Sidelink CSI report can be cancelled when UE is reconfigured to autonomous resource selection mode?

| Company | Yes/No | Comment |
| --- | --- | --- |
| OPPO | Yes |  |
| Ericsson | yes |  |
| Interdigital | Yes |  |
| Nokia | Yes |  |
| Intel | Yes |  |
| Apple | Yes |  |
| Huawei | Yes |  |

#### Issue 5: The value of CSI-RS priority

RAN2#108 agreement:

CSI report MAC CE is prioritized between PC5-RRC/S and SL data LCHs in SL LCP.

If MAC PDU consists of one or more logical channels, the SCI indicates L1 priority set to the value of the highest priority of the logical channels. Sometimes, there may be a case that CSI Reporting is triggered but no SL data from a logical channel is available. In this case, UE includes only the CSI Reporting MAC CE in MAC PDU. However, how to set the value of the priority of the CSI Reporting MAC CE is unclear.

Observation 5: It is not decided which priority value is set for PSSCH transmission carrying only the CSI Reporting MAC CE in MAC PDU.

The related proposals are available below:

| Company | Tdoc | Level 1 Proposals |
| --- | --- | --- |
| OPPO | R2-2000200 | - Proposal2: to specify priority value for SL CSI report - Proposal2a: the prioritization between UL data/UL SR and SL CSI report is to follow NR rule between UL data and SL data |
| CATT | R2-2000205 | - Proposal 2: If Sidelink CSI Reporting MAC CE is included in the MAC PDU to be transmitted on the PSSCH scheduled by the SCI, the Layer 1 Priority indicated in SCI should be 1. |
| LG | R2-2000237 | - Apart from Sidelink LCP, the priority value of the CSI Reporting MAC CE is set to ‘1’ for transmission. |
| Sanechips | R2-2000260 | - Proposal 1: The fixed priority value for SL MAC CE shall be 2. |
| Spreadtrum | R2-2000562 | - Proposal 7: The priority of the CSI MAC CE for Destination selection should be equal to the priority indicated in the SCI which triggers the CSI reporting. |

In Sidelink LCP, the priority of the CSI Reporting MAC CE is higher than the priority of STCH and lower than the priority of SCCH. This priority order could be considered when UE sets the L1 priority in a SCI corresponding the CSI Reporting MAC CE.

According to RRC CR, the priority of SCCH is currently set to ‘1’ while a priority of STCH can be configured one of the value from 1 to 8. Since the CSI Reporting MAC CE is considered between SCCH and STCH in the priority order of LCP operation, it seems logical to set the priority value of the CSI Reporting MAC CE to ‘1’.

Alternatively, some companies proposed that the priority of the CSI MAC CE can be equal to the priority indicated in the SCI which triggers the CSI reporting.

Note that the SCI triggering the CSI reporting will always schedule a PSSCH transmission. Thus, the priority indicated in the SCI corresponds to the highest priority of the MAC PDU transmitted over the PSCCH transmission.

Proposal 5.1: The priority value of the CSI Reporting MAC CE can be set by using one of the following options:

- Option A1: The priority value of the CSI Reporting MAC CE is set to a fixed value.

- Option A2: The priority value of the CSI Reporting MAC CE is set to the priority indicated in the SCI which triggers the CSI reporting.

Question 5A:	How should the priority value of the CSI Reporting MAC CE be set for the PSCCH/PSSCH transmission carrying the MAC CE (apart from the agreed priority order of SL LCP)?

| Company | Preferred Option | Comment |
| --- | --- | --- |
| OPPO | A1 | To confirm what RAN2 has agreed at last meeting |
| Ericsson | Option A2 | As commented online, we are in favor with Option A2, which can help to distinguish the priority of different links/traffics and assign the associated priority to the corresponding CSI report. |
| Interdigital | Option A2 | With a fixed (priority 1) priority for CSI MAC CE, destinations which trigger CSI reporting but have no data and for which the peer UE’s data is low priority would be selected over destinations that have pending high priority data. We would be fine to revert the agreement from the previous meeting, seeing that this problem can be solved with minimal specification impact to the running MAC CR. |
| Nokia | A2 | As mentioned in the last meeting, we see an issue with a fixed value. Also, setting it to 1 anyway still contradict previous agreements about MAC CE priority to be between SCCH and STCH |
| Intel | A1 | We prefer to keep in line with the earlier agreement |
| Apple | A1 | SL CSI needs to be set with high priority no matter which traffic triggers the SL CSI reporting because it is used to maintain the health of the D2D link which all SL LCHs rely on. We perfere to stick to the previous agreement |
| Huawei | A1 or configured by network | In our understanding, the priority value can be fixed, or configured by network, which is more flexible for network implementation. |

Question 5B:	If Option 1 is used in 5A, which fixed value (e.g. ‘1’ same as SCCH) is set to the CSI Reporting MAC CE?

| Company | Preferred priority value (e.g. ‘1’) | Comment |
| --- | --- | --- |
| OPPO | 1 |  |
| Interdigital | 1 | If we decide not to revert the agreement, we think 1 can work. |
| Nokia | (1) | Only if we decide to go for A1 in Q5A |
| Intel | 1 |  |
| Apple | 1 |  |
| Huawei | 1 or the value configured by network |  |

Proposal 5.2: The prioritization between UL data/UL SR and SL CSI report is to follow NR rule between UL data and SL data according to the priority value of the SL CSI reporting.

Question 5C:	Do you agree the above proposal 5.2 (regardless of 5.1)?

| Company | Yes/No | Comment |
| --- | --- | --- |
| OPPO | Yes |  |
| Ericsson | Yes |  |
| Interdigital | Yes |  |
| Nokia | Yes |  |
| Intel | Yes |  |
| Apple | Yes |  |
| Huawei | Yes |  |

#### Issue 6: Selection of destination for SL CSI Reporting

In the endorsed running CR:

The MAC entity shall for each SCI corresponding to a new transmission:

1>	select a Destination associated to one of unicast, groupcast and broadcast, having the logical channel with the highest priority, among the logical channels that satisfy all the following conditions for the SL grant associated to the SCI:

2>	SL data is available for transmission; and

2>	SBj > 0, in case there is any logical channel having SBj > 0; and

2>	sl-configuredSLGrantType1Allowed, if configured, is set to true in case the SL grant is a Configured Grant Type 1.

NOTE:	If multiple Destinations have the logical channels satisfying all conditions above with the same highest priority, which Destination is selected among them is up to UE implementation.

As specified in MAC, the MAC entity selects a Destination having the logical channel with the highest priority, among the logical channels having SL data available for transmssion. Thus, the MAC entity does not consider SL CSI Reporting triggered for selection of a Destination. Then, if a Destination is selected, UE may send a SL-SCI reporting of the Destination, if triggered.

Observation 6.1: According to the running CR, if SL data is available, UE selects a Destination having the logical channel with the highest priority, among the logical channels having SL data available for transmssion, regardless of SL-CSI reporting. Then, UE send a SL-SCI reporting of the Destination, if triggered for the selected Destination.

Since the MAC entity only checks logical channels having data for selection a Destination, when no SL data is available for all destinations, the MAC entity cannot select any destination for a SL grant to transmit any triggered SL CSI reporting.

Observation 6.2: According to the running CR, if no SL data is available, the MAC entity cannot select any destination for a SL grant and so it cannot transmit a SL CSI reporting, if triggered for a Destination.

The related proposals are available below:

| Company | Tdoc | Level 1 Proposals |
| --- | --- | --- |
| Huawei | R2-2000711 | - Proposal 7: The Sidelink CSI reporting MAC CE should be considered, when the UE selects the Destination during the Sidelink LCP procedure. |
| LG | R2-2000237 | - If no SL data is available for one or more destinations for which a SL-CSI reporting has been triggered and not cancelled according to subclause 5.x.1.7, UE may select one of the destination(s) to transmit the SL-CSI reporting for the SL grant associated to the SCI. Which destination is selected for SL-CSI reporting is up to UE implementation. |
| Samsung | R2-2000229 | - Proposal 4: Priority of both MAC CE and LCH is considered in destination selection. - Proposal 5: If there are LCH(s) with Bj>0 among the LCHs having data available for transmission: - If SL CSI reporting MAC CE is available and has higher priority than the highest priority LCH having Bj>0 among the LCHs having data available for transmission, UE select destination L2 ID of SL MAC CE. Otherwise, UE selects the destination L2 ID with highest priority LCH having Bj>0. - Proposal 6: If there are no LCH(s) with Bj>0 among the LCHs having data available for transmission: - If SL CSI reporting MAC CE is available and has higher priority than the highest priority LCH among the LCHs having data available for transmission, UE select destination L2 ID of SL MAC CE. Otherwise, UE selects the destination L2 ID with highest priority LCH 0. |
| ASUSTeK | R2-2001596 | - Proposal 1: When performing destination selection in SL LCP, the UE shall take triggered CSI reporting into consideration. - Proposal 2: If proposal 1 is agreed, modify SL LCP using one of the options: - Option 1: for a SCI corresponding to a new transmission, a UE selects a Destination having the logical channel data or CSI reporting MAC CE available for transmission with the highest priority - Option 2: for a SCI corresponding to a new transmission, a UE selects a Destination among destinations with triggered CSI reporting first. If there are no destination with triggered CSI reporting, the UE selects among all destinations with SL data available for transmission. |

Proposal 6.1: RAN2 is suggested to discuss which option is used for SL CSI Reporting in SL LCP when there are logical channels satisfying all conditions to select a Destination:

- Option A1: As currently specified in the running CR, UE selects a Destination only having such logical channels, regardless of SL-CSI reporting.

- Optoin A2: A UE selects a Destination having such logical channels and/or CSI reporting with the highest priority.

- Option A3: A UE selects a Destination among destinations with triggered CSI reporting first. If there are no destination with triggered CSI reporting, the UE selects a Destination having such logical channels with the highest priority.

Question 6A:	Which option is used for SL CSI Reporting in SL LCP when there are logical channels satisfying all conditions to select a Destination?

| Company | Preferred option | Comment |
| --- | --- | --- |
| OPPO | A2 | If CSI report is not taken into account in LCP, one consequence could be SL grant triggered by SR triggered by CSI report could be eventually used to transmit a LCH with low priority but different destination. And we don’t see any special treatment is needed for CSI report either i.e. A3 is not a proper approach. |
| Ericsson | Option A2 | First of all, in our view CSI report should be considered during destination selection. Considering the fact that CSI report is not associated with any Bj value, one way is to compare the priority of the CSI report with the priority of LCHs. |
| Interdigital | Option A2 | Option A3 has the problem mentioned in our answer to question 5A. Option A1 does not seem feasible because we are anyways discussing a priority to be assigned to CSI MAC CE, so that priority should be used in the LCP procedure as we do with data. |
| Nokia | A2 |  |
| Intel | A2 | Once CSI report is assigned the fixed priority, we can use the same for LCP as well |
| Apple | A2 |  |
| Huawei | Option A2 | As per the description in sub-clause 5.x.1.4.1.3 of the 38.321 running CR, logical channels shall be prioritised in accordance with the following order (highest priority listed first): - data from SCCH ; - Sidelink CSI Reporting MAC CE ; - data from any STCH. Which means that the UE need to consider all logical channel priority (i.e. SCCH, CSI MAC CE and STCH), then select the logical channel with highest priority, and the corresponding destination for this logical channel is the selected destination. |

Proposal 6.2: RAN2 is suggested to discuss which option is used in SL LCP when there is no logical channel satisfying all conditions to select a Destination:

- Option B1: UE selects any destination having CSI reporting. Which destination is selected for SL-CSI reporting is up to UE implementation.

- Optoin B2: A UE selects a Destination having CSI reporting with the highest priority.

Question 6B:	Which option is used for SL CSI Reporting in SL LCP when there is no logical channel satisfying all conditions to select a Destination?

| Company | Preferred option | Comment |
| --- | --- | --- |
| OPPO | B1 | All CSI report has same priority, hence it should be up to UE’s implementation to select one randomly. |
| Ericsson | Option B1 or B2 | Both ways seem working to us. If CSI report is always associated with the same priority, then B1 and B2 are eventually the same. |
| Interdigital | B2 | We don’t see why we need to differentiate the two cases in Q6. The UE should select the destination having the highest priority LCH with data available or the highest priority MAC CE (assuming we go with non-fixed priority). In this case, the LCP procedure is unified for data and MAC CE. |
| Nokia | B2 |  |
| Intel | B2 |  |
| Apple | B1 | My understanding is the case in Q6B is a corner case. So, it is OK to left to UE implementation |
| Huawei | Option B1 | Similar as the destination selection in LTE, it can be left to UE implementation. |

#### Issue 8: ACK after checking Layer-2 IDs in MAC PDU

According to the current version of the MAC CR, when HARQ feedback is enabled by SCI, the MAC entity instructs the physical layer to generate acknowledgement(s) of the data in this TB. We think that it should be clear from the procedural text so that the UE should send HARQ ACK only after checking the Layer-2 IDs in the MAC header of the received MAC PDU because Layer-1 ID in SCI is not collision-free. On the other hand, if UE fails to decode the MAC PDU, UE cannot check the MAC header of the received MAC PDU and so send HARQ NACK to the SL-SCH transmission just after checking Layer-1 IDs in the corresponding SCI.

The related proposals are available below:

| Company | Tdoc | Level 1 Proposals |
| --- | --- | --- |
| LG | R2-2000237 | - Upon successful decoding a MAC PDU, the MAC entity instructs the physical layer to send HARQ ACK after checking the Layer-2 IDs in the MAC header of the received MAC PDU. |

Proposal 8: How should the MAC entity instruct the physical layer to send HARQ ACK?

- Option A1: Sending HARQ ACK after finally checking the Layer-2 IDs in the MAC header of the received MAC PDU.

- Option A2: Sending HARQ ACK after checking the Layer-1 IDs in the SCI of the received MAC PDU, regardless of a result of checking the Layer-2 IDs in the MAC header.

Question 8A:	How should the MAC entity instruct the physical layer to send HARQ ACK?

| Company | Preferred option | Comment |
| --- | --- | --- |
| OPPO | A2 | Although A1 may bring some benefit, it breaks the layered structure of PHY/MAC but introducing MAC header reading before ACK feedback – it may lead to PHY processing impact, and thus not preferred. |
| Ericsson | Option A2 | We don’t see the necessity that MAC layer must check the L2 ID before sending the ACK. As long as the SCI is decoded and the associated PSFCH is known, UE is able to send ACK. In addition, in case the decoding fails and the UE cannot read L2 ID, UE can still send NACK according to the info carried in SCI. We prefer a unified behaviour for ACK and NACK transmission. |
| Interdigital | Option A1 | We see some benefit in avoiding incorrect HARQ transmission due to collision of the L1 ID. |
| Nokia | Indifferent | We see advantages of A1, but can have similar concerns as Ericsson |
| Intel | A2 | Agree with OPPO |
| Apple | A2 | A1 adds the implementation complexity and requires PHY layer to wait for MAC layer to checking MAC header after successful L1 decoding. Both the PSSCH decoding and PSFCH preparation need some processing time, and Option A2 shortens the overall processing time. It is preferable from implementation point of view. |
| Huawei | Option A2 | When the Rx cannot decode data successfully, Rx UE will feedback NACK to the Tx UE, for such case, Rx UE only check the Layer-1 ID, since it cannot obtain the remaining L2-ID in the MAC subheader. To have unified design for ACK and NACK feedback, we propose that Rx UE send HARQ ACK/NACK after checking the Layer-1 ID. Moreover, for option A1, the UE can get the L2 ID only after performing the “disassembly and demultiplexing”, which require the processing time, therefore, when deciding the time gap between PSSCH and PSFCH, the UE’s Layer 2 processing capability should be taken into account, which is not considered in RAN1. |

#### Issue 9: Need for V field

In LTE, the V field was initially introduced to differentiate a different version of a SL-SCH header structure and then enhanced to identify different cast-types a later release.

The related proposals are available below:

| Company | Tdoc | Level 1 Proposals |
| --- | --- | --- |
| LG | R2-2000237 | - Proposal 9: The V field is supported in this release as a version number for forward compatibility, possibly without any indication to cast-type using the V field in REL-16. - Proposal 10: Confirm the structure of the SL-SCH MAC subheader V/R/R/R/R/SRC/DST in Figure 6.1.x-1. |
| Lenovo, Motorola Mobility | R2-2001023 | - Proposal 8: V field is needed in SL-SCH MAC subheader |
| Samsung | R2-2001337 | - Proposal 1: V field in SL-SCH subheader can convey cast type identifier. |
| CATT | R2-2000205 | - Proposal 6: Send LS to check whether SCI can indicate the cast type. - Proposal 7: If SCI cannot indicate the cast type, the V field should be kept in SL-SCH MAC subheader to indicate the SL cast type. |
| Spreadtrum | R2-2000562 | - Observation: In PHY layer, service cast type needs to be distinguished - Proposal 9: The V field is not needed. |
| ZTE | R2-2000259 | - Proposal 8: In NR V2X, V field should be used to indicate cast type of MAC PDU which is similar like LTE sidelink. |

Proposal 9.1: RAN2 is suggested to discuss which option is agreed:

- Option A1: No V field in a SL-SCH MAC subheader

- Option A2: The V field is supported in a SL-SCH MAC subheader.

Question 9A:	Do we need to support the V field in a SL-SCH MAC subheader?

| Company | Preferred option | Comment |
| --- | --- | --- |
| OPPO | LS confirmation to RAN1 is preferred | Since it is anyway related to whether the cast type should be carried in SCI or MAC header, it would be good to notify RAN1, e.g., ask for confirmation even if RAN2 tend to agree A1 or A2. |
| Ericsson | Option A1 | We don’t see the need of having V field to indicate cast type in MAC subheader. RX UE can understand the cast type from the DST ID. |
| Interdigital | Option A1 | Agree with Ericsson. |
| Nokia | A1 |  |
| Intel |  | We think we should first discuss the need for indicating cast type in SCI or MAC header. If latter, the V field can be used for that purpose |
| Apple | A2 | We do not agree RX UE can understand the cast type from Destinaton. The presence of V field can be kept for forward-compatibility |
| Huawei | Option A1 | Agree with Ericsson, RX UE can understand the cast type from the DST ID and SCI. |

If the V field is supported, the V field may mean:

- Option B1: The V field indicates a version number for forward compatibility.

- Option B2: The V field indicates cast-type.

Question 9B:	What should the V field indicate (if A2 is supported)?

| Company | Preferred option (B1, B2 or both) | Comment |
| --- | --- | --- |
| OPPO |  | We do not see the need for B1, since we can always leave it as R-bit. |
| Apple | B2 if SCI does not indicate Cast-type; B1 if SCI indicates cast-type |  |

Proposal 9.2: If the V field is supported, RAN2 confirms the structure of the SL-SCH MAC subheader V/R/R/R/R/SRC/DST in Figure 6.1.x-1.

Figure 6.1.x-1: SL-SCH MAC subheader (in CR to 38.321)

Question 9C:	Can we confirm the structure of the SL-SCH MAC subheader V/R/R/R/R/SRC/DST in Figure 6.1.x-1 (if A2 is supported)?

| Company | Yes/No | Comment |
| --- | --- | --- |
| OPPO |  | In case V field is agreed, we are fine with the format above. |
| Apple | Yes |  |

#### Issue 12: HARQ feedback for SL CSI Reporting

In RAN2#108, RAN2 discussed as follows:

Proposal 4     CSI report event shall be cancelled if the CSI report has been transmitted.

[ZTE]: How to handle CSI report retransmission if HARQ A/N is applied to CSI report? [OPPO]: Cancellation does not mean HARQ retransmission is not allowed. [Ericsson, ITL]: Agree with the intention, but no need to specify it in MAC specification.

- Agreed. CSI report is one-shot transmission.

However, it is not clear whether HARQ feedback can be transmitted to SL-CSI reporting for retransmission of the MAC PDU only carrying SL-CSI reporting. One shot transmission may not mean HARQ retransmission is not allowed.

The related proposals are available below:

| Company | Tdoc | Level 1 Proposals |
| --- | --- | --- |
| Vivo | R2-2000283 | - Proposal 1: RAN2 to decide whether HARQ feedback attribute of CSI reporting MAC CE is specified to a default value or configured by network. |

Proposal 12.1: RAN2 is suggested to discuss whether to support HARQ feedback to a MAC PDU only carrying CSI reporting MAC CE for retransmission of the MAC PDU.

Question 12A:	 Can HARQ feedback be provided to a MAC PDU only carrying CSI reporting MAC CE for retransmission of the MAC PDU?

| Company | Yes/No | Comment |
| --- | --- | --- |
| OPPO | Yes | Although no strong need to go for this option, there is no need to exclude this either. |
| Ericsson | No | For MAC PDU only carrying CSI report, we don’t think HARQ feedback is needed since CSI report is usually time critical, so it does not make sense to retransmit it after 10s of ms. |
| Interdigital | No | We don’t see a strong need to have CSI report to generate HARQ feedback, and it would avoid complex procedures associated with retransmission at the transmitter of the CSI report. |
| Nokia | No | The MAC CE would anyway be outdated |
| Intel | No | While we agree that it does not seem useful to have HARQ feedback for retransmission of the CSI reporting MAC CE, we wonder if this is in line with proposal 8? |
| Apple | Yes | No reason to constaint this. |
| Huawei | No | In Uu, there is no HARQ feedback for CSI reporting. Similar mechanism can be reused in SL, i.e. HARQ feedback for SL CSI reporting is also not supported. |

Proposal 12.2: If HARQ feedback is supported, the following issues can be discussed e.g. as Level 2 discussion:

- Whether to configure enabling or disabling HARQ feedback of CSI reporting MAC CE

- Whether such configuration is done by a default value or by the network.

Question 12B:	 Can ‘enabling or disabling HARQ feedback’ be configured for (re-)transmission of CSI reporting MAC CE?

| Company | Yes/No | Comment |
| --- | --- | --- |
| OPPO | No | It can be fully up to UE implementation, e.g., if the CSI report MAC CE is to be transmitted together with LCH requiring HARQ feedback, it can benefit from FB from UE, or if it is to be transmitted together with LCH requiring no HARQ feedback, it can also allow no FB from Rx-UE. |
| Apple | No |  |

Question 12C:	How can ‘enabling or disabling HARQ feedback’ be configured for (re-)transmission of CSI reporting MAC CE (e.g. configured by network, pre-configuration, default/specified configuration)?

| Company | Preferred way(s) | Comment |
| --- | --- | --- |

#### Issue 13: PSSCH duration in SL LCP

RAN2 previously sent a LS to ask RAN1:

- Whether flexible PSSCH length would be supported for NR V2X PC5 communication?

- If flexible PSSCH length is to be supported, how can this be configured?

RAN1 recently replied to RAN2 in R2-2000022:

Agreements: - For Rel-16, (normal CP) - Support 7, 8, 9,…, 14 symbols in a slot without SL-SSB for SL operation - Target reusing Uu DM-RS patterns for each of the symbol-length, with modifications as necessary - No other additional spec impact is expected for supporting 7, 8, …, 13 - # of DM-RS symbols - 2, 3, 4 - For a dedicated carrier, only 14-symbol is mandatory - There is a single (pre-)configured length of SL symbols in a slot without SL-SSB per SL BWP. - There is a single (pre-)configured starting symbol for SL in a slot without SL-SSB per SL BWP. Based on these agreements, PSSCH transmissions with different numbers of symbols in length are supported in NR V2X.

Accordingly, PSSCH transmissions with different numbers of symbols in length are supported in NR V2X. However, There are a single (pre-)configured length of SL symbols in a slot and a single (pre-)configured starting symbol for SL in a slot without SL-SSB per SL BWP. Since only a single carrier/BWP is supported in REL-16 NR V2X, UE will be configured only with a single number of symbols in length for PSSCH transmissions per SL BWP. Moreover, a SCS is configured per SL BWP.

Observation 13: UE is configured only with a single number of symbols in length for PSSCH transmissions and a single SCS value per SL BWP according to RAN1 agreements.

The related proposals are available below:

| Company | Tdoc | Level 1 Proposals |
| --- | --- | --- |
| Ericsson | R2-2000950 | - Proposal 1: RAN2 does not introduce a PSSCH duration based LCP restriction for NR SL. |
| Spreadtrum | R2-2000562 | - Proposal 5: The restriction of PSSCH duration need not to be considered in LCP procedure. |

Question 13A:	 Can we confirm that UE is configured only with a single number of symbols in length for PSSCH transmissions and a single SCS value per SL BWP as in RAN1 agreements?

| Company | Yes/No | Comment |
| --- | --- | --- |
| OPPO | Yes |  |
| Ericsson | Yes |  |
| Interdigital | Yes |  |
| Nokia | Yes |  |
| Intel | Yes |  |
| Apple | Yes |  |
| Huawei | Yes |  |

Proposal 13.1: RAN2 is suggested to discuss whether or not to introduce a PSSCH duration based LCP restriction for NR SL.

Question 13B:		Can we exclude a PSSCH duration based LCP restriction for NR SL in REL-16?

| Company | Yes/No | Comment |
| --- | --- | --- |
| OPPO | No | Since R16 is limited to a single carrier, there is no need for this considering the PSSCH duration is of a single length for a single carrier. |
| Ericsson | Yes | We should not consider PSSCH duration based LCP |
| Interdigital | No | Agree with OPPO. |
| Nokia | Yes | Not for Rel-16 |
| Intel | Yes | We can exclude this restriction for Rel-16. (BTW, there seems to be some disconnect between OPPO’s comment and the chosen option due to how the question is worded…) |
| Apple | Yes |  |
| Huawei | Yes | As per RAN1 agreements mentioned in Q13A and their agreement that the scheduling unit is a slot, which means that for a specific UE, the PSSCH duration is fixed for a given time, therefore, it is reasonable not introduce PSSCH duration based LCP restriction. |

#### Issue 14: Selection of a Destination for logical channels enabling or disabling HARQ feedback

RAN2#108 agreement:

The logical channel with disabling the HARQ feedback cannot be multiplexed with a logical channel which enabling the HARQ feedback.

In the running CR,

The MAC entity shall for each SCI corresponding to a new transmission:

1>	select a Destination associated to one of unicast, groupcast and broadcast, having the logical channel with the highest priority, among the logical channels that satisfy all the following conditions for the SL grant associated to the SCI:

2>	SL data is available for transmission; and

2>	SBj > 0, in case there is any logical channel having SBj > 0; and

2>	sl-configuredSLGrantType1Allowed, if configured, is set to true in case the SL grant is a Configured Grant Type 1.

NOTE:	If multiple Destinations have the logical channels satisfying all conditions above with the same highest priority, which Destination is selected among them is up to UE implementation.

1>	select the logical channels satisfying all the following conditions:

2>	SL data is available for transmission; and

2>	sl-configuredSLGrantType1Allowed, if configured, is set to true in case the SL grant is a Configured Grant Type 1.

…

A logical channel configured with sl-HARQ-FeedbackEnabled set to enabled and a logical channel configured with sl-HARQ-FeedbackEnabled set to disabled cannot be multiplexed into the same MAC PDU.

Editor’s Note: FFS how LCP will take HARQ A/N enabled/disabled into account, e.g. packet with HARQ enabled will be multiplexed only with packets with HARQ enabled.

The related proposals are available below:

| Company | Tdoc | Level 1 Proposals |
| --- | --- | --- |
| CATT | R2-2000211 | - Proposal 2: Take HARQ A/N enable/disabled into consideration in the procedure of logical channel selection. |
| ZTE | R2-2000259 | - Proposal1：if an SLRB has no HARQ enable/disable attribute, the associated logical channel can be multiplexed with either the logical channel enabling the HARQ feedback or the logical channel disabling the HARQ feedback. - Proposal2: During the procedure of Selection of logical channels, after the MAC entity selects the Destination of the logical channel with the highest priority, it shall further select the HARQ feedback enabled/disabled attribute of the logical channel which has the highest priority among the logical channels having HARQ enable/disable attribute and belonging to the selected destination. - Proposal3: The MAC entity shall only consider and select sidelink logical channels having the same Destination and HARQ feedback enabled/disabled attribute for MAC PDU(s) in SL LCP. |
| Vivo | R2-2000287 | - Proposal 1: The LCP considering HARQ feedback enabled/disabled configuration will be: - firstly the logical channel with highest priority among the logical channels that satisfying all of transmission conditions is selected and its HARQ feedback configuration can be the HARQ feedback attribute of this whole MAC PDU; - All of logical channels with the same HARQ feedback configuration as the above step can participate to the next LCP, i.e. logical channels with different HARQ feedback configuration from the whole MAC PDU can not be considered in this transmission. |
| Lenovo, Motorola Mobility, Deutsche Telekom, Fraunhofer HHI and Fraunhofer IIS, Continental Automotive GmbH discussion | R2-2001078 | - P1: The HF Enabled/ Disabled configuration of the highest priority LCH determining the destination will determine the HF Enabled/ Disabled for the entire TB. - P2: Select only those LCH of the selected destination having the same Feedback mode as determined for the entire TB. The LCP procedure is run on the resulting LCH procedure. - P3: If the “Feedback mode as determined for the entire TB” is “enabled”, SL feedback is requested in SCI from the receivers. - P6: If the “Feedback mode as determined for the entire TB” is “disabled”, blind retransmission(s) will be instructed to the Physical layer while submitting the corresponding TB. |
| Samsung | R2-2001338 | - Proposal 1: HARQ feedback enabled/disabled can be a condition for logical channel selection. |

Proposal 14.1: RAN2 is suggested to discuss whether the HF Enabled/ Disabled configuration of the highest priority LCH determining the destination will determine the HF Enabled/ Disabled for the entire TB.

Proposal 14.2: If the above proposal is agreed, the MAC entity shall only consider and select sidelink logical channels having the same Destination and HARQ feedback attribute for MAC PDU(s) in SL LCP according to the determined HARQ feedback attribute (i.e. either enabled or disabled).

Question 14A:		Do you agree the following procedure in SL LCP?

- If the highest priority logical channel of the destination selected in SL LCP is configured with ‘HARQ enabled’, UE selects only logical channels with ‘HARQ enabled’ for the entire TB; and

- If the highest priority logical channel of the destination selected in SL LCP is configured with ‘HARQ disabled’, UE selects only logical channels with ‘HARQ disabled’ for the entire TB.

| Company | Yes/No | Comment |
| --- | --- | --- |
| OPPO | Yes but missing some cases | If there is no PUCCH configured, and there is PSFCH configured, the above behaviour is OK. But if there is PUCCH configured, or there is no PSFCH configured, it is possible to limit the LCH selection to a single type. E.g., for no-PSFCH case, one can only select LCH requiring no HARQ FB. Both scenarios have to be considered by MAC layer during LCP. |
| Ericsson | see comment | We agree with the intention, on the other hand, note that following the current procedure, UE will first allocate resource to LCHs with Bj>0. So strictly speaking, if the first selected SL LCH (e.g. highest priority among SL LCHs with Bj>0) is configured with “HARQ enabled/disabled”, UE will further selects only LCHs with the same config. |
| Interdigital | Yes | We agree with the statements in the questions. For the scenarios mentioned by OPPO, if there are no PSFCH configured in the resource pool, this should be handled by pool selection (different topic). When no PUCCH is not configured, the UE should select HARQ disabled, since it cannot report HARQ feedback to the network. If PUCCH is configured, no restriction is required, and the UE can select based on priority (to avoid further unnecessary restriction). |
| Nokia | See comment | As Ericsson |
| Intel | Yes | We agree with the statements. In addition, we wonder whether it is possible that some LCH is not configured with the sl-HARQ-FeedbackEnabled and if PUCCH is not configured, whether that LCH can be included alongside LCH with HARQ FB disabled. |
| Apple | Yes | We agree with those two principles. |
| Huawei | Yes with comments | This only applies to Mode 2. For mode 1, if PSFCH is configured, the HARQ feedback attribute relies on whether there is PUCCH resource, if there is PUCCH resource, UE selects only logical channels with ‘HARQ enabled’; otherwise, UE selects only logical channels with ‘HARQ disabled’. |

Proposal 14.3: If the HARQ feedback attribute is “enabled”, SL feedback is requested in SCI from the receivers.

Proposal 14.4: If the HARQ feedback attribute is “disabled”, blind retransmission(s) can be instructed to the Physical layer while submitting the corresponding TB.

In the endorsed running CR:

3>	if a MAC PDU to transmit has been obtained:

4>	determines Sidelink tranmssion information of the TB for the source and destination pair of the MAC PDU as follows:

…

5> enable HARQ feedback, if sl-HARQ-FeedbackEnabled has been set to Enabled for the logical channel(s) in the MAC PDU;

…

4>	deliver the MAC PDU, the sideink grant and the Sidelink transmission information of the TB to the associated Sidelink process;

4>	instruct the associated Sidelink process to trigger a new transmission;

Question 14B:		Do we need to further update the above yellow text for Proposal 14.3 and 14.4?

| Company | Yes/No | Comment |
| --- | --- | --- |
| OPPO | No | The yellow text seems OK. |
| Ericsson | No | It seems enough to us. |
| Interdigital | No |  |
| Nokia | No |  |
| Intel | No |  |
| Apple | No |  |
| Huawei | Yes | Regarding P14.3 and P14.4, once the HARQ feedback attribute is determined, what MAC need to instruct PHY is HARQ enable/disable attribute which will be filled in SCI, therefore, we propose change the sentence yellow highlighted as follows: 5> set the HARQ feedback to the value of sl-HARQ-FeedbackEnabled of the logical channel(s) in the MAC PDU; |

#### Issue 15: Simultaneous operation of Sidelink Mode 1 and 2, particularly in case of exceptional pool

RAN2#109e agreement:

Simultaneous operation of NR Sidelink Mode 1 and 2 is not supported for a UE performing transmission of NR sidelink communication.

The related proposals are available below:

| Company | Tdoc | Level 1 Proposals |
| --- | --- | --- |
| LG | R2-2000237 | - Proposal 2: Simultaneous operation of Sidelink Mode 1 and 2 is not supported for a UE performing transmission of NR sidelink communication. |
| Lenovo, Motorola Mobility, Deutsche Telekom, Fraunhofer HHI and Fraunhofer IIS, Continental Automotive GmbH discussion | R2-2001078 | - P5: In case a retransmission needs to be made, the UE switches to Mode 2 based retransmission(s) if the initial transmission was based on Mode 1 resource allocation but PUCCH resources for feedback are not available (i.e. Not provided in the corresponding DCI). - P10: If the PUCCH resources are not available, the UE could autonomously switch to Mode 2 based Blind re-transmissions (the Tx UE does not solicit any HARQ feedback from the Rx UE(s)). |

It is currently specified in RRC CR when an exceptional pool is configured by a certain condition, UE may still have sidelink mode 1 resources given by Configured Grant Type 1. However, RAN2 did not discuss how simultaneous operation is expected to work in this case. Furthermore, it is unlikely to support simultaneous operation only at the corner case.

Observation 1: When an exceptional pool is configured by a certain condition, UE may still have sidelink mode 1 resources given by Configured Grant Type 1. It is not clear how UE performs sidelink transmission in this case because UE cannot perform simultaneous transmissions in different modes.

Accordingly, we propose to confirm the following change (in green) to subcluase 5.x.1.1 in 38.321 to avoid unexpected simultaneous configuration of sidelink mode 1 and 2 for a UE.

If the MAC entity has been configured by RRC to transmit using pool(s) of resources in a carrier as indicated in TS 38.331 [5] or TS 36.331 [xy] based on sensing or random selection[and not configured by RRC to transmit using neither SL-RNTI nor SLCS-RNTI], the MAC entity shall for each Sidelink process:

Proposal 15.2: Confirm the below change in 38.321 for REL-16.

If the MAC entity has been configured by RRC to transmit using pool(s) of resources in a carrier as indicated in TS 38.331 [5] or TS 36.331 [xy] based on sensing or random selection[and not configured by RRC to transmit using neither SL-RNTI nor SLCS-RNTI], the MAC entity shall for each Sidelink process:

Question 15A:		Can we confirm the above change to subclause 5.x.1.1. in 38.321, i.e. [and not configured by RRC to transmit using neither SL-RNTI nor SLCS-RNTI] for REL-16?

| Company | Yes/No | Comment |
| --- | --- | --- |
| OPPO | yes |  |
| Ericsson | See comment | We agree with the intention, but no strong view if it is necessary to explicit reflect it in the spec without specifying the UE behaviour in case of the unexpected configuration. Maybe a NOTE in the spec is enough. |
| Interdigital | No | We prefer a NOTE that specifically indicates that the UE can be in the situation given by Observation 1. |
| Nokia | No | Note should be enough |
| Intel | See Comment | We agree with Ericsson that a not should be sufficient |
| Apple | Do nothing | Mode 1 UE will not do sensing-based resource selection. If a UE switches to mode 2, then it disregard any CGs. So nothing needs to be specified. |
| Huawei | Yes |  |

It seems worth noting that according to the current running CRs, it is possible for UE to be configured with either both LTE mode 3 and NR mode 2 or both LTE mode 4 and NR mode 1.

Proposal 15.3: RAN2 is suggested to confirm that UE can be configured with either both LTE mode 3 and NR mode 2 or both LTE mode 4 and NR mode 1, i.e. mixed mode can be already supported for inter-RAT sidelink.

Question 15B:		Can a UE be configured with either both LTE mode 3 and NR mode 2 or both LTE mode 4 and NR mode 1?

| Company | Yes/No | Comment |
| --- | --- | --- |
| OPPO | Yes |  |
| Ericsson | Yes |  |
| Interdigital | Yes |  |
| Nokia | Yes |  |
| Intel | Yes |  |
| Apple | Yes |  |
| Huawei | Yes | As per the running CR, seems these cases mentioned above are supported. Moreover, we also need confirm there is no standard impact from RAN2 perspective. |

# Conclusion and recommendation