3GPP TSG RAN WG2#109-e                                                                             R2-2002234

Electronic meeting, 24th February - 6th March, 2020                Revision of R2-2002018

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.323 | CR | 0038 | rev | 2 | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 38.323 CR for NR V2X |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2020-02-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR is to capture the following agreements related to PDCP for the 5G_V2X_NRSL-Core WI: RAN2#104 agreements: - Sidelink packet duplication is supported in NR PDCP for NR sidelink broadcast, groupcast. FFS on unicast. - Timer based SDU/PDU discard function is supported in NR PDCP for NR sidelink broadcast, groupcast and unicast. RAN2#106 agreements: - Carrier reselection and PDCP duplication are not supported in Rel-16. RAN2#107 agreements: - For NR Sidelink unicast, the establishment and release of transmitting PDCP entity and receiving PDCP entity can be requested by upper layer. - For NR Sidelink groupcast and broadcast, the establishment and release of transmitting PDCP entity can be requested by upper layer. The establishment of the receiving PDCP entity for NR Sidelink groupcast upon reception of first UMD PDU from a Source Layer 2 ID and Destination Layer 2 ID pair for an LCID, and there is not yet a corresponding receiving PDCP entity and release up to UE implementation (i.e., follow LTE V2X Sidelink as baseline) - The SN of PDCP shall be maintained for all cast types. - The header compression and decompression can be supported for all cast types. - The following questions and understandings will be asked to SA3: - Support of AS ciphering and deciphering for data? - Support of AS integrity protection and integrity verification for SL unicast data? - Support of AS integrity protection and integrity verification for SL groupcast data? - RAN2 understanding of no support of AS integrity protection and integrity verification for SL broadcast data (to be confirmed by SA3) - Any concern to reuse 18bits Uu PDCP-SN-Size for SL? Note feasibility to reuse 12bits Uu PDCP-SN-Size for SL is FFS. Also note RAN2 assumes only 18bits is used for groupcast and broadcast cases. - The timer based SDU discard is supported for all cast types. - The PDCP reordering and in-order delivery is supported for all cast types. - PDCP out-of-order delivery can be supported for SL unicast types. FFS for groupcast and broadcast. - On top of Uu values, 3ms and 25ms should be added for discard timer in case of NW configuration and preconfiguration. - The Default discardtimer values shall be up to the UE implementation. - INTEGER (1..16383) (i.e. Full set of values as specified in NR Uu) is supported for maxCID in case of network configuration and preconfiguration. - 15 is used for default maxCID. - Profile0x0001, profile0x0002, etc. (i.e. Full set of values as specified in NR Uu) is used for NR Sidelink in case of network configuration and preconfiguration. - No fixed profile ID is needed for NR Sidelink default configuration for all cast types. - 0ms, 1ms, 2ms, …, 2750ms, 3000ms (i.e. Full set of values as specified in NR Uu) is used for t-Reordering in case of network configuration and preconfiguration. - T-Reordering value is up to the UE implementation for default configuration. - SDU Type field in the PDCP PDU format is needed. - IP and non-IP types field are needed. FFS for ARP and PC5 Signalling Protocol. - PDCP SN field is needed. - For unicast, PDCP control PDU and D/C field is necessary. FFS for the need of D/C field for groupcast and broadcast. - The initialized PDCP state variable, i.e., TX_NEXT, in the transmitting PDCP entity is set to 0 for all cast type. - The initialized PDCP state variable, i.e., RX_NEXT, in the receiving PDCP entity is set to 0 for unicast. The initialized PDCP state variable set, i.e., RX_NEXT, in the receiving PDCP entity follows the LTE rule for groupcast and broadcast. RAN2#108 agreements: - For SL groupcast and broadcast, PDCP out-of-order delivery is not supported. - ARP SDU type is not supported in NR sidelink. - PC5 Signaling protocol SDU type is not supported in NR sidelink. - The length of SDU type is 2bits. - For data PDU format , D/C field is not used for groupcast and broadcast. - The initial value of RX_DELIV in the receiving PDCP entity sets to 0 for unicast. The initial value of RX_DELIV in the receiving PDCP entity follows the LTE rule for groupcast and broadcast. - PDCP should support AS ciphering and integrity protection for SL data and PC5-RRC. - For SL groupcast and broadcast, only 18bits pdcp-SN-Size is used for the SL DRB. For SL unicast, 12bits and 18bits pdcp-SN-Size are used for the SL DRB. - For SL unicast, only 12bits pdcp-SN-Size is used for the SL SRB including SL SRBs which carrying PC5 Signalling and PC5 RRC message. FFS on how to handle initial PC5-S signaling for unicast link. RAN2#109-e agreements: - Capture which SLRB need confidentiality protection in TS 38.331. The specific functionalities for confidentiality protection (e.g. parameters needed, procedures, etc.) are captured in TS 38.323. - Capture which SLRB need integrity protection in TS 38.331. The specific functionalities for integrity protection (e.g. parameters needed, procedures, etc.) are captured in TS 38.323. - For SL DRBs of groupcast and broadcast, the MAC-I field is not present. - For SL DRBs of unicast, if the integrity protection is not configured, the MAC-I field is not present. - For the first PC5 Signalling, i.e., Direct Communication Request, the MAC-I field is not present. - Except for Direct Communication Request, the MAC-I field is always present in the PDCP format for other PC5 Signallings and SL RRC signallings. - The D/C field is not present and the corresponding bit is present as a Reserved bit in data PDU format for groupcast and broadcast. - Captured the following note in PDCP layer for NR sidelink communication: - NOTE: As the PC5-S message which activates the integrity protection function is itself integrity protected with the configuration included in this PC5-S message, this message needs first be decoded by upper layer before the integrity protection verification could be performed for the PDU in which the message was received. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For RAN2#107bis the following changes has been implemented: 2 References - Added a reference to 3GPP TS 23.287. 3.1 Definitions - Added the definition of NR sidelink communication. 3.2 Abbreviations - Added abbreviations for SLRB and STCH. 4.2.1 PDCP structure - Added STCH and SCCH among the logical channels that the PDCP sublayer is used for RBs mapped on. 4.2.2 PDCP entities - Figure 4.2.2-1 updated to include also sidelink. 5.1.1 PDCP entity establishment - Added PDCP entity establishment procedure for NR sidelink. 5.1.3 PDCP entity release - Added PDCP entity release procedure for NR sidelink. 5.2.3 Sidelink transmit operation - Added sidelink transmit operation. 5.2.4 Sidelink receive operation - Added sidelink receive operation. 5.7.2 Configuration of header compression - Added PDCP entities associated with SLRBs that can be configured to use header compression for IP SDUs. 5.7.3 Protocol parameters - Added sidelink that the parameter PROFILES is configured for. 5.8 Ciphering and deciphering - Added an editor’s note whether/how to perform ciphering and deciphering for NR SL depends on SA3. 5.9 Integrity protection and verification - Added an editor’s note whether/how to perform integrity protection and verification for NR SL depends on SA3. 6.2.2.4 Data PDU for SLRBs - Added Data PDU format for SLRBs. 6.3.X SDU Type - Added PDCP SDU type for sidelink. 7.1 State variables - Added SN size for PDCP Data PDUs. - Added RX_NEXT for sidelink. 7.3 Timers - Added t-Reordering for sidelink. For RAN2#109 the following changes has been implemented: 3.2 Abbreviations - Added abbreviations for SCCH. 5.1.3 PDCP entity release - Updated PDCP entity release procedure for NR sidelink. 5.8 Ciphering and deciphering - Added the ciphering protection procedure of NR sidelink communication. 5.9 Integrity protection and verification - Added the integrity protection procedure of NR sidelink communication. 6.3.2 PDCP SN - Added PDCP SN length for sidelink. 6.3.4 MAC-I - Added description of MAC-I field for sidelink. 6.3.X SDU Type - Updated PDCP SDU type for sidelink. 7.1 State variables - Updated RX_NEXT for sidelink. - Added RX_DELIV for sidelink |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the CR is not approved the NR V2X will not be specified in the PDCP 38.323 specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.1, 3.2, 4.2.1, 4.2.2, 5.1.1, 5.1.3, 5.2.3, 5.2.4, 5.7.2, 5.7.3, 5.8, 5.9, 6.2.2.4, 6.3.2, 6.3.4, 6.3.X, 7.1, 7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

3GPP TS 38.323 V15.6.0 (2019-06)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

NR;

Packet Data Convergence Protocol (PDCP) specification

(Release 15)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	6

2	References	6

3	Definitions and abbreviations	6

3.1	Definitions	6

3.2	Abbreviations	7

4	General	7

4.1	Introduction	7

4.2	Architecture	7

4.2.1	PDCP structure	7

4.2.2	PDCP entities	8

4.3	Services	9

4.3.1	Services provided to upper layers	9

4.3.2	Services expected from lower layers	9

4.4	Functions	10

5	Procedures	10

5.1	PDCP entity handling	10

5.1.1	PDCP entity establishment	10

5.1.2	PDCP entity re-establishment	10

5.1.3	PDCP entity release	11

5.1.4	PDCP entity suspend	12

5.2	Data transfer	12

5.2.1	Transmit operation	12

5.2.2	Receive operation	13

5.2.2.1	Actions when a PDCP Data PDU is received from lower layers	13

5.2.2.2	Actions when a t-Reordering expires	14

5.2.2.3	Actions when the value of t-Reordering is reconfigured	14

5.3	SDU discard	15

5.4	Status reporting	15

5.4.1	Transmit operation	15

5.4.2	Receive operation	15

5.5	Data recovery	15

5.6	Data volume calculation	16

5.7	Header compression and decompression	16

5.7.1	Supported header compression protocols and profiles	16

5.7.2	Configuration of header compression	17

5.7.3	Protocol parameters	17

5.7.4	Header compression	17

5.7.5	Header decompression	18

5.7.6	PDCP Control PDU for interspersed ROHC feedback	18

5.7.6.1	Transmit Operation	18

5.7.6.2	Receive Operation	18

5.8	Ciphering and deciphering	18

5.9	Integrity protection and verification	19

5.10	Handling of unknown, unforeseen, and erroneous protocol data	19

5.11	PDCP duplication	19

5.11.1	Activation/Deactivation of PDCP duplication	19

5.11.2	Duplicate PDU discard	20

6	Protocol data units, formats, and parameters	20

6.1	Protocol data units	20

6.1.1	Data PDU	20

6.1.2	Control PDU	20

6.2	Formats	20

6.2.1	General	20

6.2.2	Data PDU	20

6.2.2.1	Data PDU for SRBs	20

6.2.2.2	Data PDU for DRBs with 12 bits PDCP SN	21

6.2.2.3	Data PDU for DRBs with 18 bits PDCP SN	21

6.2.3	Control PDU	22

6.2.3.1	Control PDU for PDCP status report	22

6.2.3.2	Control PDU for interspersed ROHC feedback	22

6.3	Parameters	23

6.3.1	General	23

6.3.2	PDCP SN	23

6.3.3	Data	23

6.3.4	MAC-I	23

6.3.5	COUNT	23

6.3.6	R	24

6.3.7	D/C	24

6.3.8	PDU type	24

6.3.9	FMC	24

6.3.10	Bitmap	24

6.3.11	Interspersed ROHC feedback	25

7	State variables, constants, and timers	25

7.1	State variables	25

7.2	Constants	25

7.3	Timers	25

Annex A (informative):	Change history	26

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document provides the description of the Packet Data Convergence Protocol (PDCP).

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.300: "NG Radio Access Network; Overall description".

[3]	3GPP TS 38.331: "NR Radio Resource Control (RRC); Protocol Specification".

[4]	3GPP TS 38.321: "NR Medium Access Control (MAC) protocol specification".

[5]	3GPP TS 38.322: "NR Radio Link Control (RLC) protocol specification".

[6]	3GPP TS 33.501: "Security Architecture and Procedures for 5G System ".

[7]	IETF RFC 5795: "The RObust Header Compression (ROHC) Framework".

[8]	IETF RFC 3095: "RObust Header Compression (ROHC): Framework and four profiles: RTP, UDP, ESP and uncompressed".

[9]	IETF RFC 4815: "RObust Header Compression (ROHC): Corrections and Clarifications to RFC 3095".

[10]	IETF RFC 6846: "RObust Header Compression (ROHC): A Profile for TCP/IP (ROHC-TCP)".

[11]	IETF RFC 5225: "RObust Header Compression (ROHC) Version 2: Profiles for RTP, UDP, IP, ESP and UDP Lite".

[12]	3GPP TS 36.321: "Evolved Universal Terrestrial Radio Access (E-UTRA) Medium Access Control (MAC) protocol specification".

[xx]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[xx2]	3GPP TS 33.536: "Security Aspect of 3GPP Support for Advanced V2X Services".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

AM DRB: a data radio bearer which utilizes RLC AM.

Non-split bearer: a bearer whose radio protocols are located in either the MgNB or the SgNB to use MgNB or SgNB resource, respectively.

NR sidelink communication: AS functionality enabling at least V2X communication as defined in TS 23.287 [xx], between two or more nearby UEs, using NR technology but not traversing any network node.

PDCP data volume: the amount of data available for transmission in a PDCP entity.

Split bearer: in dual connectivity, a bearer whose radio protocols are located in both the MgNB and the SgNB to use both MgNB and SgNB resources.

UM DRB: a data radio bearer which utilizes RLC UM.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AM	Acknowledged Mode

CID	Context Identifier

DRB	Data Radio Bearer carrying user plane data

gNB	NR Node B

HFN	Hyper Frame Number

IETF	Internet Engineering Task Force

IP	Internet Protocol

MAC	Medium Access Control

MAC-I	Message Authentication Code for Integrity

PDCP	Packet Data Convergence Protocol

PDU	Protocol Data Unit

RB	Radio Bearer

RFC	Request For Comments

RLC	Radio Link Control

ROHC	RObust Header Compression

RRC	Radio Resource Control

RTP	Real Time Protocol

SAP	Service Access Point

SCCH	Sidelink Control Channel

SDU	Service Data Unit

SLRB	Sidelink Radio Bearer carrying NR sidelink communication

SN	Sequence Number

SRB	Signalling Radio Bearer carrying control plane data

STCH	Sidelink Traffic Channel

TCP	Transmission Control Protocol

UDP	User Datagram Protocol

UE	User Equipment

UM	Unacknowledged Mode

X-MAC	Computed MAC-I

# 4	General

## 4.1	Introduction

The present document describes the functionality of the PDCP.

## 4.2	Architecture

### 4.2.1	PDCP structure

Figure 4.2.1.1 represents one possible structure for the PDCP sublayer; it should not restrict implementation. The figure is based on the radio interface protocol architecture defined in TS 38.300 [2].

Figure 4.2.1-1: PDCP layer, structure view

The PDCP sublayer is configured by upper layers TS 38.331 [3]. The PDCP sublayer is used for RBs mapped on DCCH, DTCH, SCCH and STCH type of logical channels. The PDCP sublayer is not used for any other type of logical channels.

Each RB (except for SRB0) is associated with one PDCP entity. Each PDCP entity is associated with one, two, or four RLC entities depending on the RB characteristic (e.g uni-directional/bi-directional or split/non-split) or RLC mode:

-	For split bearers or for RBs configured with PDCP duplication, each PDCP entity is associated with two UM RLC entities (for same direction), four UM RLC entities (two for each direction), or two AM RLC entities (for same direction);

-	Otherwise, each PDCP entity is associated with one UM RLC entity, two UM RLC entities (one for each direction), or one AM RLC entity.

### 4.2.2	PDCP entities

The PDCP entities are located in the PDCP sublayer. Several PDCP entities may be defined for a UE. Each PDCP entity is carrying the data of one radio bearer.

A PDCP entity is associated either to the control plane or the user plane depending on which radio bearer it is carrying data for.

Figure 4.2.2.1 represents the functional view of the PDCP entity for the PDCP sublayer; it should not restrict implementation. The figure is based on the radio interface protocol architecture defined in TS 38.300 [2].

For split bearers, routing is performed in the transmitting PDCP entity.

Figure 4.2.2-1: PDCP layer, functional view

## 4.3	Services

### 4.3.1	Services provided to upper layers

The PDCP layer provides its services to the RRC or SDAP layers. The following services are provided by PDCP to upper layers:

-	transfer of user plane data;

-	transfer of control plane data;

-	header compression;

-	ciphering;

-	integrity protection.

The maximum supported size of a PDCP SDU is 9000 bytes. The maximum supported size of a PDCP Control PDU is 9000 bytes.

### 4.3.2	Services expected from lower layers

- A PDCP entity expects the following services from lower layers per RLC entity (for a detailed description see TS 38.322 [5]):

-	acknowledged data transfer service, including indication of successful delivery of PDCP PDUs;

-	unacknowledged data transfer service.

## 4.4	Functions

The PDCP layer supports the following functions:

-	transfer of data (user plane or control plane);

-	maintenance of PDCP SNs;

-	header compression and decompression using the ROHC protocol;

-	ciphering and deciphering;

-	integrity protection and integrity verification;

-	timer based SDU discard;

-	for split bearers, routing;

-	duplication;

-	reordering and in-order delivery;

-	out-of-order delivery;

-	duplicate discarding.

# 5	Procedures

## 5.1	PDCP entity handling

### 5.1.1	PDCP entity establishment

When upper layers request a PDCP entity establishment for a radio bearer or an SLRB; or for NR sidelink communication for groupcast and broadcast, when receiving the first PDCP PDU from a UM RLC entity, and there is not yet a corresponding PDCP entity for an SLRB, the UE shall:

-	establish a PDCP entity for the radio bearer or the SLRB;

-	set the state variables of the PDCP entity to initial values;

-	follow the procedures in clause 5.2.

### 5.1.2	PDCP entity re-establishment

When upper layers request a PDCP entity re-establishment, the UE shall additionally perform once the procedures described in this clause. After performing the procedures in this clause, the UE shall follow the procedures in clause 5.2.

When upper layers request a PDCP entity re-establishment, the transmitting PDCP entity shall:

-	for UM DRBs and AM DRBs, reset the header compression protocol for uplink and start with an IR state in U-mode (as defined in RFC 3095 [8] and RFC 4815 [9]) if drb-ContinueROHC is not configured in TS 38.331 [3];

-	for UM DRBs and SRBs, set TX_NEXT to the initial value;

-	for SRBs, discard all stored PDCP SDUs and PDCP PDUs;

-	apply the ciphering algorithm and key provided by upper layers during the PDCP entity re-establishment procedure;

-	apply the integrity protection algorithm and key provided by upper layers during the PDCP entity re-establishment procedure;

-	for UM DRBs, for each PDCP SDU already associated with a PDCP SN but for which a corresponding PDU has not previously been submitted to lower layers, and;

-	for suspended AM DRBs, from the first PDCP SDU for which the successful delivery of the corresponding PDCP Data PDU has not been confirmed by lower layers, for each PDCP SDU already associated with a PDCP SN:

-	consider the PDCP SDUs as received from upper layer;

-	perform transmission of the PDCP SDUs in ascending order of the COUNT value associated to the PDCP SDU prior to the PDCP re-establishment without restarting the discardTimer, as specified in clause 5.2.1;

-	for AM DRBs which were not suspended, from the first PDCP SDU for which the successful delivery of the corresponding PDCP Data PDU has not been confirmed by lower layers, perform retransmission or transmission of all the PDCP SDUs already associated with PDCP SNs in ascending order of the COUNT values associated to the PDCP SDU prior to the PDCP entity re-establishment as specified below:

-	perform header compression of the PDCP SDU as specified in the clause 5.7.4;

-	perform integrity protection and ciphering of the PDCP SDU using the COUNT value associated with this PDCP SDU as specified in the clause 5.9 and 5.8;

-	submit the resulting PDCP Data PDU to lower layer, as specified in clause 5.2.1.

When upper layers request a PDCP entity re-establishment, the receiving PDCP entity shall:

-	process the PDCP Data PDUs that are received from lower layers due to the re-establishment of the lower layers, as specified in the clause 5.2.2.1;

-	for SRBs, discard all stored PDCP SDUs and PDCP PDUs;

-	for SRBs and UM DRBs, if t-Reordering is running:

-	stop and reset t-Reordering;

-	for UM DRBs, deliver all stored PDCP SDUs to the upper layers in ascending order of associated COUNT values after performing header decompression;

-	for AM DRBs, perform header decompression for all stored PDCP SDUs if drb-ContinueROHC is not configured in TS 38.331 [3];

-	for UM DRBs and AM DRBs, reset the header compression protocol for downlink and start with NC state in U-mode (as defined in RFC 3095 [8] and RFC 4815 [9]) if drb-ContinueROHC is not configured in TS 38.331 [3];

-	for UM DRBs and SRBs, set RX_NEXT and RX_DELIV to the initial value;

-	apply the ciphering algorithm and key provided by upper layers during the PDCP entity re-establishment procedure;

-	apply the integrity protection algorithm and key provided by upper layers during the PDCP entity re-establishment procedure.

### 5.1.3	PDCP entity release

When upper layers request a PDCP entity release for a radio bearer or an SLRB, the UE shall:

-	discard all stored PDCP SDUs and PDCP PDUs in the transmitting PDCP entity;

-	for UM DRBs, AM DRBs and sidelink DRBs, deliver the PDCP SDUs stored in the receiving PDCP entity to upper layers in ascending order of associated COUNT values after performing header decompression, if not decompressed before;

-	release the PDCP entity for the radio bearer.

NOTE:	For NR sidelink communication for groupcast and broadcast, the receiving PDCP entity release for an SLRB is up to UE implementation.

### 5.1.4	PDCP entity suspend

When upper layers request a PDCP entity suspend, the transmitting PDCP entity shall:

-	set TX_NEXT to the initial value;

-	discard all stored PDCP PDUs;

When upper layers request a PDCP entity suspend, the receiving PDCP entity shall:

-	if t-Reordering is running:

-	stop and reset t-Reordering;

-	deliver all stored PDCP SDUs to the upper layers in ascending order of associated COUNT values after performing header decompression;

-	set RX_NEXT and RX_DELIV to the initial value.

## 5.2	Data transfer

### 5.2.1	Transmit operation

At reception of a PDCP SDU from upper layers, the transmitting PDCP entity shall:

-	start the discardTimer associated with this PDCP SDU (if configured).

For a PDCP SDU received from upper layers, the transmitting PDCP entity shall:

-	associate the COUNT value corresponding to TX_NEXT to this PDCP SDU;

NOTE 1:	Associating more than half of the PDCP SN space of contiguous PDCP SDUs with PDCP SNs, when e.g., the PDCP SDUs are discarded or transmitted without acknowledgement, may cause HFN desynchronization problem. How to prevent HFN desynchronization problem is left up to UE implementation.

-	perform header compression of the PDCP SDU as specified in the clause 5.7.4;

-	perform integrity protection, and ciphering using the TX_NEXT as specified in the clause 5.9 and 5.8, respectively;

-	set the PDCP SN of the PDCP Data PDU to TX_NEXT modulo 2[pdcp-SN-SizeUL];

-	increment TX_NEXT by one;

-	submit the resulting PDCP Data PDU to lower layer as specified below.

When submitting a PDCP PDU to lower layer, the transmitting PDCP entity shall:

-	if the transmitting PDCP entity is associated with one RLC entity:

-	submit the PDCP PDU to the associated RLC entity;

-	else, if the transmitting PDCP entity is associated with two RLC entities:

-	if the PDCP duplication is activated:

-	if the PDCP PDU is a PDCP Data PDU:

-	duplicate the PDCP Data PDU and submit the PDCP Data PDU to both associated RLC entities;

-	else:

-	submit the PDCP Control PDU to the primary RLC entity;

-	else:

-	if the two associated RLC entities belong to the different Cell Groups; and

-	if the total amount of PDCP data volume and RLC data volume pending for initial transmission (as specified in TS 38.322 [5]) in the two associated RLC entities is equal to or larger than ul-DataSplitThreshold:

-	submit the PDCP PDU to either the primary RLC entity or the secondary RLC entity;

-	else:

-	submit the PDCP PDU to the primary RLC entity.

NOTE 2:	If the transmitting PDCP entity is associated with two RLC entities, the UE should minimize the amount of PDCP PDUs submitted to lower layers before receiving request from lower layers and minimize the PDCP SN gap between PDCP PDUs submitted to two associated RLC entities to minimize PDCP reordering delay in the receiving PDCP entity.

### 5.2.2	Receive operation

#### 5.2.2.1	Actions when a PDCP Data PDU is received from lower layers

In this clause, following definitions are used:

-	HFN(State Variable): the HFN part (i.e. the number of most significant bits equal to HFN length) of the State Variable;

-	SN(State Variable): the SN part (i.e. the number of least significant bits equal to PDCP SN length) of the State Variable;

-	RCVD_SN: the PDCP SN of the received PDCP Data PDU, included in the PDU header;

-	RCVD_HFN: the HFN of the received PDCP Data PDU, calculated by the receiving PDCP entity;

-	RCVD_COUNT: the COUNT of the received PDCP Data PDU = [RCVD_HFN, RCVD_SN].

At reception of a PDCP Data PDU from lower layers, the receiving PDCP entity shall determine the COUNT value of the received PDCP Data PDU, i.e. RCVD_COUNT, as follows:

-	if RCVD_SN < SN(RX_DELIV) – Window_Size:

-	RCVD_HFN = HFN(RX_DELIV) + 1.

-	else if RCVD_SN >= SN(RX_DELIV) + Window_Size:

-	RCVD_HFN = HFN(RX_DELIV) – 1.

-	else:

-	RCVD_HFN = HFN(RX_DELIV);

-	RCVD_COUNT = [RCVD_HFN, RCVD_SN].

After determining the COUNT value of the received PDCP Data PDU = RCVD_COUNT, the receiving PDCP entity shall:

-	perform deciphering and integrity verification of the PDCP Data PDU using COUNT = RCVD_COUNT;

-	if integrity verification fails:

-	indicate the integrity verification failure to upper layer;

-	discard the PDCP Data PDU;

-	if RCVD_COUNT < RX_DELIV; or

-	if the PDCP Data PDU with COUNT = RCVD_COUNT has been received before:

-	discard the PDCP Data PDU;

If the received PDCP Data PDU with COUNT value = RCVD_COUNT is not discarded above, the receiving PDCP entity shall:

-	store the resulting PDCP SDU in the reception buffer;

-	if RCVD_COUNT >= RX_NEXT:

-	update RX_NEXT to RCVD_COUNT + 1.

-	if outOfOrderDelivery is configured:

-	deliver the resulting PDCP SDU to upper layers.

-	if RCVD_COUNT = RX_DELIV:

-	deliver to upper layers in ascending order of the associated COUNT value after performing header decompression, if not decompressed before;

-	all stored PDCP SDU(s) with consecutively associated COUNT value(s) starting from COUNT = RX_DELIV;

-	update RX_DELIV to the COUNT value of the first PDCP SDU which has not been delivered to upper layers, with COUNT value > RX_DELIV;

-	if t-Reordering is running, and if RX_DELIV >= RX_REORD:

-	stop and reset t-Reordering.

-	if t-Reordering is not running (includes the case when t-Reordering is stopped due to actions above), and RX_DELIV < RX_NEXT:

-	update RX_REORD to RX_NEXT;

-	start t-Reordering.

#### 5.2.2.2	Actions when a t-Reordering expires

When t-Reordering expires, the receiving PDCP entity shall:

-	deliver to upper layers in ascending order of the associated COUNT value after performing header decompression, if not decompressed before:

-	all stored PDCP SDU(s) with associated COUNT value(s) < RX_REORD;

-	all stored PDCP SDU(s) with consecutively associated COUNT value(s) starting from RX_REORD;

-	update RX_DELIV to the COUNT value of the first PDCP SDU which has not been delivered to upper layers, with COUNT value >= RX_REORD;

-	if RX_DELIV < RX_NEXT:

-	update RX_REORD to RX_NEXT;

-	start t-Reordering.

#### 5.2.2.3	Actions when the value of t-Reordering is reconfigured

When the value of the t-Reordering is reconfigured by upper layers while the t-Reordering is running, the receiving PDCP entity shall:

-	update RX_REORD to RX_NEXT;

-	stop and restart t-Reordering.

### 5.2.3	Sidelink transmit operation

For sidelink transmission of the SLRB, the UE shall follow the procedures in subclause 5.2.1 with following modification:

-	perform the header compression using ROHC as specified in subclause 5.7.4, if SDU Type is IP.

### 5.2.4	Sidelink receive operation

For sidelink reception of the SLRB, the UE shall follow the procedures in subclause 5.2.2 with following modification:

-	perform the header decompression using ROHC as specified in subclause 5.7.5, if SDU Type is IP.

## 5.3	SDU discard

When the discardTimer expires for a PDCP SDU, or the successful delivery of a PDCP SDU is confirmed by PDCP status report, the transmitting PDCP entity shall discard the PDCP SDU along with the corresponding PDCP Data PDU. If the corresponding PDCP Data PDU has already been submitted to lower layers, the discard is indicated to lower layers.

For SRBs, when upper layers request a PDCP SDU discard, the PDCP entity shall discard all stored PDCP SDUs and PDCP PDUs.

NOTE:	Discarding a PDCP SDU already associated with a PDCP SN causes a SN gap in the transmitted PDCP Data PDUs, which increases PDCP reordering delay in the receiving PDCP entity. It is up to UE implementation how to minimize SN gap after SDU discard.

## 5.4	Status reporting

### 5.4.1	Transmit operation

For AM DRBs configured by upper layers to send a PDCP status report in the uplink (statusReportRequired in TS 38.331 [3]), the receiving PDCP entity shall trigger a PDCP status report when:

-	upper layer requests a PDCP entity re-establishment;

-	upper layer requests a PDCP data recovery.

If a PDCP status report is triggered, the receiving PDCP entity shall:

-	compile a PDCP status report as indicated below by:

-	setting the FMC field to RX_DELIV;

-	if RX_DELIV < RX_NEXT:

-	allocating a Bitmap field of length in bits equal to the number of COUNTs from and not including the first missing PDCP SDU up to and including the last out-of-sequence PDCP SDUs, rounded up to the next multiple of 8, or up to and including a PDCP SDU for which the resulting PDCP Control PDU size is equal to 9000 bytes, whichever comes first;

-	setting in the bitmap field as '0' for all PDCP SDUs that have not been received, and optionally PDCP SDUs for which decompression have failed;

-	setting in the bitmap field as '1' for all PDCP SDUs that have been received;

-	submit the PDCP status report to lower layers as the first PDCP PDU for transmission via the transmitting PDCP entity as specified in clause 5.2.1.

### 5.4.2	Receive operation

For AM DRBs, when a PDCP status report is received in the downlink, the transmitting PDCP entity shall:

-	consider for each PDCP SDU, if any, with the bit in the bitmap set to '1', or with the associated COUNT value less than the value of FMC field as successfully delivered, and discard the PDCP SDU as specified in clause 5.3.

## 5.5	Data recovery

For AM DRBs, when upper layers request a PDCP data recovery for a radio bearer, the transmitting PDCP entity shall:

-	perform retransmission of all the PDCP Data PDUs previously submitted to re-established or released AM RLC entity in ascending order of the associated COUNT values for which the successful delivery has not been confirmed by lower layers, following the data submission procedure in clause 5.2.1.

After performing the above procedures, the transmitting PDCP entity shall follow the procedures in clause 5.2.1.

## 5.6	Data volume calculation

For the purpose of MAC buffer status reporting, the transmitting PDCP entity shall consider the following as PDCP data volume:

-	the PDCP SDUs for which no PDCP Data PDUs have been constructed;

-	the PDCP Data PDUs that have not been submitted to lower layers;

-	the PDCP Control PDUs;

-	for AM DRBs, the PDCP SDUs to be retransmitted according to clause 5.1.2;

-	for AM DRBs, the PDCP Data PDUs to be retransmitted according to clause 5.5.

If the transmitting PDCP entity is associated with two RLC entities, when indicating the PDCP data volume to a MAC entity for BSR triggering and Buffer Size calculation (as specified in TS 38.321 [4] and TS 36.321 [12]), the transmitting PDCP entity shall:

-	if the PDCP duplication is activated:

-	indicate the PDCP data volume to the MAC entity associated with the primary RLC entity;

-	indicate the PDCP data volume excluding the PDCP Control PDU to the MAC entity associated with the secondary RLC entity;

-	else:

-	if the two associated RLC entities belong to the different Cell Groups; and

-	if the total amount of PDCP data volume and RLC data volume pending for initial transmission (as specified in TS 38.322 [5]) in the two associated RLC entities is equal to or larger than ul-DataSplitThreshold:

-	indicate the PDCP data volume to both the MAC entity associated with the primary RLC entity and the MAC entity associated with the secondary RLC entity;

-	else:

-	indicate the PDCP data volume to the MAC entity associated with the primary RLC entity;

-	indicate the PDCP data volume as 0 to the MAC entity associated with the secondary RLC entity.

## 5.7	Header compression and decompression

### 5.7.1	Supported header compression protocols and profiles

The header compression protocol is based on the Robust Header Compression (ROHC) framework defined in RFC 5795 [7]. There are multiple header compression algorithms, called profiles, defined for the ROHC framework. Each profile is specific to the particular network layer, transport layer or upper layer protocol combination e.g. TCP/IP and RTP/UDP/IP.

The detailed definition of the ROHC channel is specified as part of the ROHC framework defined in RFC 5795 [7]. This includes how to multiplex different flows (header compressed or not) over the ROHC channel, as well as how to associate a specific IP flow with a specific context state during initialization of the compression algorithm for that flow.

The implementation of the functionality of the ROHC framework and of the functionality of the supported header compression profiles is not covered in this specification.

In this version of the specification the support of the following profiles is described:

Table 5.7.1-1: Supported header compression protocols and profiles

| Profile Identifier | Usage | Reference |
| --- | --- | --- |
| 0x0000 | No compression | RFC 5795 |
| 0x0001 | RTP/UDP/IP | RFC 3095, RFC 4815 |
| 0x0002 | UDP/IP | RFC 3095, RFC 4815 |
| 0x0003 | ESP/IP | RFC 3095, RFC 4815 |
| 0x0004 | IP | RFC 3843, RFC 4815 |
| 0x0006 | TCP/IP | RFC 6846 |
| 0x0101 | RTP/UDP/IP | RFC 5225 |
| 0x0102 | UDP/IP | RFC 5225 |
| 0x0103 | ESP/IP | RFC 5225 |
| 0x0104 | IP | RFC 5225 |

### 5.7.2	Configuration of header compression

PDCP entities associated with DRBs can be configured by upper layers TS 38.331 [3] to use header compression. Each PDCP entity carrying user plane data may be configured to use header compression. In this version of the specification, only the robust header compression protocol (ROHC) is supported. Every PDCP entity uses at most one ROHC compressor instance and at most one ROHC decompressor instance. PDCP entities associated with SLRBs can be configured to use ROHC for IP SDUs.

### 5.7.3	Protocol parameters

RFC 5795 [7] has configuration parameters that are mandatory and that must be configured by upper layers between compressor and decompressor peers ; these parameters define the ROHC channel. The ROHC channel is a unidirectional channel, i.e. if rohc is configured there is one channel for the downlink and one for the uplink, and if uplinkOnlyROHC is configured there is only one channel for the uplink. There is thus one set of parameters for each channel, and if rohc is configured the same values shall be used for both channels belonging to the same PDCP entity.These parameters are categorized in two different groups, as defined below:

-	M:	Mandatory and configured by upper layers;

-	N/A: Not used in this specification.

The usage and definition of the parameters shall be as specified below.

-	MAX_CID (M): This is the maximum CID value that can be used. One CID value shall always be reserved for uncompressed flows. The parameter MAX_CID is configured by upper layers (maxCID in TS 38.331 [3]);

-	LARGE_CIDS: This value is not configured by upper layers, but rather it is inferred from the configured value of MAX_CID according to the following rule:

-	If MAX_CID > 15 then LARGE_CIDS = TRUE else LARGE_CIDS = FALSE;

-	PROFILES (M): Profiles are used to define which profiles are allowed to be used by the UE. The list of supported profiles is described in clause 5.7.1. The parameter PROFILES is configured by upper layers (profiles for uplink, downlink and sidelink in TS 38.331 [3]);

-	FEEDBACK_FOR (N/A): This is a reference to the channel in the opposite direction between two compression endpoints and indicates to what channel any feedback sent refers to. Feedback received on one ROHC channel for this PDCP entity shall always refer to the ROHC channel in the opposite direction for this same PDCP entity;

-	MRRU (N/A): ROHC segmentation is not used.

### 5.7.4	Header compression

If header compression is configured, the header compression protocol generates two types of output packets:

-	compressed packets, each associated with one PDCP SDU;

-	standalone packets not associated with a PDCP SDU, i.e. interspersed ROHC feedback.

A compressed packet is associated with the same PDCP SN and COUNT value as the related PDCP SDU. The header compression is not applicable to the SDAP header and the SDAP Control PDU if included in the PDCP SDU.

Interspersed ROHC feedback are not associated with a PDCP SDU. They are not associated with a PDCP SN and are not ciphered.

NOTE:	If the MAX_CID number of ROHC contexts are already established for the compressed flows and a new IP flow does not match any established ROHC context, the compressor should associate the new IP flow with one of the ROHC CIDs allocated for the existing compressed flows or send PDCP SDUs belonging to the IP flow as uncompressed packet.

### 5.7.5	Header decompression

If header compression is configured by upper layers for PDCP entities associated with user plane data, the PDCP Data PDUs are decompressed by the header compression protocol after performing deciphering as explained in clause 5.8. The header decompression is not applicable to the SDAP header and the SDAP Control PDU if included in the PDCP Data PDU.

### 5.7.6	PDCP Control PDU for interspersed ROHC feedback

#### 5.7.6.1	Transmit Operation

When an interspersed ROHC feedback is generated by the header compression protocol, the transmitting PDCP entity shall:

-	submit to lower layers the corresponding PDCP Control PDU as specified in clause 6.2.3.2 i.e. without associating a PDCP SN, nor performing ciphering.

#### 5.7.6.2	Receive Operation

At reception of a PDCP Control PDU for interspersed ROHC feedback from lower layers, the receiving PDCP entity shall:

-	deliver the corresponding interspersed ROHC feedback to the header compression protocol without performing deciphering.

## 5.8	Ciphering and deciphering

The ciphering function includes both ciphering and deciphering and is performed in PDCP, if configured. The data unit that is ciphered is the MAC-I (see clause 6.3.4) and the data part of the PDCP Data PDU (see clause 6.3.3) except the SDAP header and the SDAP Control PDU if included in the PDCP SDU. The ciphering is not applicable to PDCP Control PDUs.

For downlink and uplink, the ciphering algorithm and key to be used by the PDCP entity are configured by upper layers TS 38.331 [3] and the ciphering method shall be applied as specified in TS 33.501 [6].

The ciphering function is activated/suspended/resumed by upper layers TS 38.331 [3]. When security is activated and not suspended, the ciphering function shall be applied to all PDCP Data PDUs indicated by upper layers TS 38.331 [3] for the downlink, the uplink and the sidelink, respectively.

For downlink and uplink ciphering and deciphering, the parameters that are required by PDCP for ciphering are defined in TS 33.501 [6] and are input to the ciphering algorithm. The required inputs to the ciphering function include the COUNT value, and DIRECTION (direction of the transmission: set as specified in TS 33.501 [6]).The parameters required by PDCP which are provided by upper layers TS 38.331 [3] are listed below:

-	BEARER (defined as the radio bearer identifier in TS 33.501 [6]. It will use the value RB identity –1 as in TS 38.331 [3]);

-	KEY (the ciphering keys for the control plane and for the user plane are KRRCenc and KUPenc, respectively).

For NR sidelink communication, the ciphering function as specified in TS 33.501 [6] is applied with KEY (PEK), COUNT (derived from KD-sess Identity and PDCP SN as specified in TS 33.536 [xx2]), BEARER and DIRECTION (which value shall be set is specified in TS 33.536 [xx2]) as input.

Editor’s Note: The ciphering protection procedure of NR sidelink communication will be updated based on SA3 progress, e.g., what/how to use the Key ID included in the PDCP PDU header.

## 5.9	Integrity protection and verification

The integrity protection function includes both integrity protection and integrity verification and is performed in PDCP, if configured. The data unit that is integrity protected is the PDU header and the data part of the PDU before ciphering. The integrity protection is always applied to PDCP Data PDUs of SRBs and sidelink SRBs except for the unprotected PC5-S message (e.g. Direct Communication Request). The integrity protection is applied to PDCP Data PDUs of DRBs and sidelink DRBs of NR sidelink communication for unicast for which integrity protection is configured. The integrity protection is not applicable to PDCP Control PDUs.

For downlink and uplink, the integrity protection algorithm and key to be used by the PDCP entity are configured by upper layers TS 38.331 [3] and the integrity protection method shall be applied as specified in TS 33.501 [6].

The integrity protection function is activated/suspended/resumed by upper layers TS 38.331 [3]. When security is activated and not suspended, the integrity protection function shall be applied to all PDUs including and subsequent to the PDU indicated by upper layers TS 38.331 [3] for the downlink, the uplink and the sidelink, respectively.

NOTE:	As the RRC message which activates the integrity protection function is itself integrity protected with the configuration included in this RRC message, this message needs first be decoded by RRC before the integrity protection verification could be performed for the PDU in which the message was received.

NOTE:	As the PC5-S message which activates the integrity protection function is itself integrity protected with the configuration included in this PC5-S message, this message needs first be decoded by upper layer before the integrity protection verification could be performed for the PDU in which the message was received.

For downlink and uplink integrity protection and verification, the parameters that are required by PDCP for integrity protection are defined in TS 33.501 [6] and are input to the integrity protection algorithm. The required inputs to the integrity protection function include the COUNT value, and DIRECTION (direction of the transmission: set as specified in TS 33.501 [6]). The parameters required by PDCP which are provided by upper layers TS 38.331 [3] are listed below:

-	BEARER (defined as the radio bearer identifier in TS 33.501 [6]. It will use the value RB identity –1 as in TS 38.331 [3]);

-	KEY (the integrity protection keys for the control plane and for the user plane are KRRCint and KUPint, respectively).

For the SLRB that needs integrity protection and verification, the parameters that are required by PDCP for integrity protection are defined in TS 33.536 [xx2] and are input to the integrity protection algorithm. The required inputs to the integrity protection function include the KEY (PIK), COUNT (derived from KD-sess Identity and PDCP SN as specified in TS 33.536 [xx2]), BEARER and DIRECTION (which value shall be set is specified in TS 33.536 [xx2]).

At transmission, the UE computes the value of the MAC-I field and at reception it verifies the integrity of the PDCP Data PDU by calculating the X-MAC based on the input parameters as specified above. If the calculated X-MAC corresponds to the received MAC-I, integrity protection is verified successfully.

Editor’s Note: The integrity protection procedure of NR sidelink communication will be updated based on SA3 progress.

## 5.10	Handling of unknown, unforeseen, and erroneous protocol data

When a PDCP PDU that contains reserved or invalid values is received, the receiving PDCP entity shall:

-	discard the received PDU.

## 5.11	PDCP duplication

### 5.11.1	Activation/Deactivation of PDCP duplication

For the PDCP entity configured with pdcp-Duplication, the transmitting PDCP entity shall:

-	for SRBs:

-	activate the PDCP duplication;

-	for DRBs:

-	if the activation of PDCP duplication is indicated:

-	activate the PDCP duplication;

-	if the deactivation of PDCP duplication is indicated:

-	deactivate the PDCP duplication.

### 5.11.2	Duplicate PDU discard

For the PDCP entity configured with pdcp-Duplication, the transmitting PDCP entity shall:

-	if the successful delivery of a PDCP Data PDU is confirmed by one of the two associated AM RLC entities:

-	indicate to the other AM RLC entity to discard the duplicated PDCP Data PDU;

-	if the deactivation of PDCP duplication is indicated:

-	indicate to the secondary RLC entity to discard all duplicated PDCP Data PDUs.

# 6	Protocol data units, formats, and parameters

## 6.1	Protocol data units

### 6.1.1	Data PDU

The PDCP Data PDU is used to convey one or more of followings in addition to the PDU header:

-	user plane data;

-	control plane data;

-	a MAC-I.

### 6.1.2	Control PDU

The PDCP Control PDU is used to convey one of followings in addition to the PDU header:

-	a PDCP status report;

-	an interspersed ROHC feedback.

## 6.2	Formats

### 6.2.1	General

A PDCP PDU is a bit string that is byte aligned (i.e. multiple of 8 bits) in length. In the figures in clause 6.2, bit strings are represented by tables in which the most significant bit is the leftmost bit of the first line of the table, the least significant bit is the rightmost bit on the last line of the table, and more generally the bit string is to be read from left to right and then in the reading order of the lines. The bit order of each parameter field within a PDCP PDU is represented with the first and most significant bit in the leftmost bit and the last and least significant bit in the rightmost bit.

PDCP SDUs are bit strings that are byte aligned (i.e. multiple of 8 bits) in length. A compressed or uncompressed SDU is included into a PDCP Data PDU from the first bit onward.

### 6.2.2	Data PDU

#### 6.2.2.1	Data PDU for SRBs

Figure 6.2.2.1-1 shows the format of the PDCP Data PDU with 12 bits PDCP SN. This format is applicable for SRBs.

Figure 6.2.2.1-1: PDCP Data PDU format for SRBs

#### 6.2.2.2	Data PDU for DRBs with 12 bits PDCP SN

Figure 6.2.2.2-1 shows the format of the PDCP Data PDU with 12 bits PDCP SN. This format is applicable for UM DRBs and AM DRBs.

Figure 6.2.2.2-1: PDCP Data PDU format with 12 bits PDCP SN

#### 6.2.2.3	Data PDU for DRBs with 18 bits PDCP SN

Figure 6.2.2.3-1 shows the format of the PDCP Data PDU with 18 bits PDCP SN. This format is applicable for UM DRBs and AM DRBs.

Figure 6.2.2.3-1: PDCP Data PDU format for DRBs with 18 bits PDCP SN

#### 6.2.2.4	Data PDU for SLRBs

Editor’s Note: SDU Type field and PDCP SN field in the PDCP PDU format is needed. For unicast, D/C field is necessary. The detail PDCP PDU format is FFS.

### 6.2.3	Control PDU

#### 6.2.3.1	Control PDU for PDCP status report

Figure 6.2.3.1-1 shows the format of the PDCP Control PDU carrying one PDCP status report. This format is applicable for AM DRBs.

Figure 6.2.3.1-1: PDCP Control PDU format for PDCP status report

#### 6.2.3.2	Control PDU for interspersed ROHC feedback

Figure 6.2.3.2-1 shows the format of the PDCP Control PDU carrying one interspersed ROHC feedback. This format is applicable for UM DRBs and AM DRBs.

Figure 6.2.3.2-1: PDCP Control PDU format for interspersed ROHC feedback

## 6.3	Parameters

### 6.3.1	General

If not otherwise mentioned in the definition of each field then the bits in the parameters shall be interpreted as follows: the left most bit string is the first and most significant and the right most bit is the last and least significant bit.

Unless otherwise mentioned, integers are encoded in standard binary encoding for unsigned integers. In all cases the bits appear ordered from MSB to LSB when read in the PDU.

### 6.3.2	PDCP SN

Length: 12, or 18 bits as indicated in table 6.3.2.1. The length of the PDCP SN is configured by upper layers (pdcp-SN-SizeUL, pdcp-SN-SizeDL, or sl-PDCP-SN-Size in TS 38.331 [3]).

Table 6.3.2-1: PDCP SN length

| Length | Description |
| --- | --- |
| 12 | UM DRBs, AM DRBs, SRBs, sidelink DRBs and sidelink SRBs |
| 18 | UM DRBs, AM DRBs and sidelink DRBs |

NOTE:	For NR sidelink communication for groupcast and broadcast, only 18bits PDCP SN length is used for the sidelink DRBs.

Editor’s Note: the PDCP SN length for initial PC5-S signaling for SL unicast is FFS.

### 6.3.3	Data

Length: Variable

This field includes one of the followings:

-	Uncompressed PDCP SDU (user plane data, or control plane data);

-	Compressed PDCP SDU (user plane data only).

### 6.3.4	MAC-I

Length: 32 bits

This field carries a message authentication code calculated as specified in clause 5.9.

For SRBs, the MAC-I field is always present. If integrity protection is not configured, the MAC-I field is still present but should be padded with padding bits set to 0.

For DRBs, the MAC-I field is present only when the DRB is configured with integrity protection.

For sidelink DRBs in NR sidelink communication for groupcast and broadcast, the MAC-I field is not present.

For sidelink DRBs in NR sidelink communication for unicast, the MAC-I field is present only when the sidelink DRB is configured with integrity protection.

For sidelink SRBs, the MAC-I field is always present except for the unprotected PC5-S message (e.g. Direct Communication Request).

### 6.3.5	COUNT

Length: 32 bits

The COUNT value is composed of a HFN and the PDCP SN. The size of the HFN part in bits is equal to 32 minus the length of the PDCP SN.

Figure 6.3.5-1: Format of COUNT

NOTE:	COUNT does not wrap around.

### 6.3.6	R

Length: 1 bit

Reserved. In this version of the specification reserved bits shall be set to 0. Reserved bits shall be ignored by the receiver.

### 6.3.7	D/C

Length: 1 bit

This field indicates whether the corresponding PDCP PDU is a PDCP Data PDU or a PDCP Control PDU.

Table 6.3.7-1: D/C field

| Bit | Description |
| --- | --- |
| 0 | Control PDU |
| 1 | Data PDU |

### 6.3.8	PDU type

Length: 3 bits

This field indicates the type of control information included in the corresponding PDCP Control PDU.

Table 6.3.8-1: PDU type

| Bit | Description |
| --- | --- |
| 000 | PDCP status report |
| 001 | Interspersed ROHC feedback |
| 010-111 | Reserved |

### 6.3.9	FMC

Length: 32 bits

First Missing COUNT. This field indicates the COUNT value of the first missing PDCP SDU within the reordering window, i.e. RX_DELIV.

### 6.3.10	Bitmap

Length: Variable. The length of the bitmap field can be 0.

This field indicates which SDUs are missing and which SDUs are correctly received in the receiving PDCP entity. The bit position of Nth bit in the Bitmap is N, i.e., the bit position of the first bit in the Bitmap is 1.

Table 6.3.10-1 Bitmap

| Bit | Description |
| --- | --- |
| 0 | PDCP SDU with COUNT = (FMC + bit position) modulo 232 is missing. |
| 1 | PDCP SDU with COUNT = (FMC + bit position) modulo 232 is correctly received. |

### 6.3.11	Interspersed ROHC feedback

Length: Variable

This field contains one ROHC packet with only feedback, i.e. a ROHC packet that is not associated with a PDCP SDU as defined in clause 5.7.4.

### 6.3.X	SDU Type

Length: 2bits

PDCP SDU type, i.e. Layer-3 Protocol Data Unit type as specified in [xx]. PDCP entity may handle the SDU differently per SDU Type, e.g. ROHC is applicable to IP SDU but not Non-IP SDU.

Table 6.3.X-1: SDU Type

| Bit | Description |
| --- | --- |
| 00 | IP |
| 01 | Non-IP |
| 10-11 | reserved |

# 7	State variables, constants, and timers

## 7.1	State variables

This sub clause describes the state variables used in PDCP entities in order to specify the PDCP protocol. The state variables defined in this clause are normative.

All state variables are non-negative integers, and take values from 0 to [232 – 1].

PDCP Data PDUs are numbered integer sequence numbers (SN) cycling through the field: 0 to [2[pdcp-SN-SizeUL] – 1] or 0 to [2[pdcp-SN-SizeDL] – 1] or 0 to [2[sl-PDCP-SN-Size] – 1].

The transmitting PDCP entity shall maintain the following state variables:

a)	TX_NEXT

This state variable indicates the COUNT value of the next PDCP SDU to be transmitted. The initial value is 0.

The receiving PDCP entity shall maintain the following state variables:

a)	RX_NEXT

This state variable indicates the COUNT value of the next PDCP SDU expected to be received. The initial value is 0, except for sidelink broadcast and groupcast. For NR sidelink communication for broadcast and groupcast, the initial value of the SN part of RX_NEXT is (x +1) modulo (2[sl-PDCP-SN-Size] ), where x is the SN of the first received PDCP Data PDU.

Editor’s Note: The HFN part of RX_NEXT is FFS.

b)	RX_DELIV

This state variable indicates the COUNT value of the first PDCP SDU not delivered to the upper layers, but still waited for. The initial value is 0, except for sidelink broadcast and groupcast. For NR sidelink communication for broadcast and groupcast, the initial value of the SN part of RX_DELIV is (x – 0.5 * 2[sl-PDCP-SN-Size-1]) modulo (2[sl-PDCP-SN-Size] ), where x is the SN of the first received PDCP Data PDU.

Editor’s Note: The HFN part of RX_DELIV is FFS.

c)	RX_REORD

This state variable indicates the COUNT value following the COUNT value associated with the PDCP Data PDU which triggered t-Reordering.

## 7.2	Constants

a) Window_Size

This constant indicates the size of the reordering window. The value equals to 2[pdcp-SN-SizeDL] – 1.

## 7.3	Timers

The transmitting PDCP entity shall maintain the following timers:

a) discardTimer

This timer is configured only for DRBs. The duration of the timer is configured by upper layers TS 38.331 [3]. In the transmitter, a new timer is started upon reception of an SDU from upper layer.

The receiving PDCP entity shall maintain the following timers:

b) t-Reordering

The duration of the timer is configured by upper layers TS 38.331 [3], except for the case of NR sidelink communication. For NR sidelink communication, the t-Reordering timer is determined by the UE implementation. This timer is used to detect loss of PDCP Data PDUs as specified in clause 5.2.2. If t-Reordering is running, t-Reordering shall not be started additionally, i.e. only one t-Reordering per receiving PDCP entity is running at a given time.

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New Version |
| 2017.03 | RAN2#97bis | R2-1703512 | - | - | - | First version. | x.y.z |
| 2017.04 | RAN2#97bis | R2-1703916 | - | - | - | Change section name "Retransmission" to "Data recovery" | 0.0.1 |
| 2017.05 | RAN2#98 | R2-1704076 | - | - | - | Initial draft TS capturing outcome of e-mail discussion [97bis#24] | 0.0.5 |
| 2017.06 | RAN2 NR AH | R2-1706868 | - | - | - | Capture agreements made in RAN2#98 | 0.1.0 |
| 2017.08 | RAN2 NR AH | R2-1707507 | - | - | - | Capture agreements made in RAN2 NR AH#2 | 0.2.0 |
| 2017.08 | RAN2#99 | R2-1709097 | - | - | - | Adding integrity protection in section 5.1.2 | 0.2.1 |
| 2017.08 | RAN2#99 | R2-1709753 | - | - | - | Capture agreements made in RAN2#99 | 0.3.0 |
| 2017.09 | RANP#77 | RP-171993 | - | - | - | Provided for information to RAN | 1.0.0 |
| 2017.10 | RAN2#99bis | R2-1713660 | - | - | - | Capture agreements made in RAN2#99bis | 1.0.1 |
| 2017.11 | RAN2#100 | R2-1714273 | - | - | - | Capture agreements made in RAN2#100 | 1.1.0 |
| 2017.12 | RP-78 | RP-172335 | - | - | - | Provided for approval to RAN | 2.0.0 |
| 2017/12 | RP-78 |  |  |  |  | Upgraded to Rel-15 (MCC) | 15.0.0 |
| 2018/03 | RP-79 | RP-180440 | 0002 | 1 | F | Corrections to PDCP specification | 15.1.0 |
| 2018/06 | RP-80 | RP-181215 | 0006 | 3 | F | Corrections to PDCP specification | 15.2.0 |
|  | RP-80 | RP-181215 | 0009 | 1 | B | Introduction of PDCP duplication | 15.2.0 |
| 2018/09 | RP-81 | RP-181942 | 0011 | 4 | F | Clarification on PDCP transmission | 15.3.0 |
| 2018/12 | RP-82 | RP-182650 | 0022 | 1 | F | Suspend and resume of security | 15.4.0 |
|  | RP-82 | RP-182655 | 0023 | - | F | Introducing PDCP suspend procedure | 15.4.0 |
|  | RP-82 | RP-182656 | 0024 | - | F | Clarification on ciphering MAC-I | 15.4.0 |
| 2019/03 | RP-83 | RP-190544 | 0025 | 2 | F | Correction on the PDCP re-establishment for AM DRB | 15.5.0 |
|  | RP-83 | RP-190540 | 0027 | 1 | F | Correction on PDCP SN length | 15.5.0 |
| 2019/06 | RP-84 | RP-191375 | 0031 | 1 | F | PDCP association with RLC for RBs configured with PDCP duplication | 15.6.0 |