3GPP TSG RAN WG2#109-e	R2-2001973

Electronic meeting, 24th February - 6th March, 2020

Title:	[Draft] LS to SA3 on NR V2X Security issues on PDCP

Response to:

Release:	Rel-16

Work Item:	5G_V2X_NRSL-Core

Source:	CATT [to be RAN2]

To:	SA3

Cc:

Contact Person:

#### Name:	Da Wang

#### E-mail Address:	wangda@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1. Overall Description

RAN2 discussed the NR V2X Security issues about AS ciphering and integrity protection for unicast, groupcast and broadcast. RAN2 would like to inform SA3 about the following agreements:

1:	For SL DRBs of groupcast and broadcast, the MAC-I field is not present.

2:	For SL DRBs of unicast, if the integrity protection is not configured, the MAC-I field is not present.

3:	For the first PC5 Signalling, i.e., Direct Communication Request, the MAC-I field is not present.

4:	Except for Direct Communication Request, the MAC-I field is always present in the PDCP format for other PC5 Signallings and SL RRC signallings.

5:	4 different LCIDs are allocated for the following SL SRBs:

- The PC5-S signalling that is not protected, e.g., Direct Communication Request.

- The PC5-S signalling to activate security, i.e., Direct Security Mode Command and Direct Security Mode Complete.

- Other PC5-S signallings that are protected.

- PC5-RRC signallings that are protected.

6:	Working assumption: The PDCP SN size of Direct Communication Request message is 18 bits.

In terms of some other security related parameters that might have RAN2 impact, RAN2 respectively request SA3 feedbacks for the following questions:

Question 1: How/whether can the LCID be used in integrity algorithms and ciphering algorithms (considering LCID in NR SL communication is agreed to be 6 bit)?

Question 2: What is the required size of Key ID and MAC-I in the PDCP header?

Question 3: Since PDCP SN and HFN are aligned between UEs for V2X unicast, is it needed to consider the HFN together with PDCP SN in security counter for NR V2X as NR Uu?

Question 4: Is the PGK and PTK related security mechanism still used for NR SL groupcast and broadcast? If yes, what is the length of PGK index and PTK identity to be included in the PDCP header?

In the solution #12 of TR 33.836, for the signaling messages that are not protected, the Key ID and PDCP SN in PDCP format are set to zeros in the PDCP header. However, the PDCP SN is also used for PDCP reordering in AS layer, i.e. it should not be zero regardless of security protection. Thus, RAN2 would like to inform SA3 that the PDCP SN should not be zero even if security protection is not used.

2. Actions:

RAN2 respectfully asks SA3 to take the above agreements into account in the related work and provide their feedback for the above questions and information, if any.

3. Date of Next RAN2 Meetings:

TSG-RAN2 Meeting #109bis	20 - 24 April 2020     	   e-meeting

TSG-RAN2 Meeting #110				 25 - 29 May 2020     	 		Athens, Greece