3GPP TSG-RAN WG2 Meeting #109-e					R2-20xxxxx

Electronic meeting, 24th Feb – 6th Mar 2020

Agenda Item:		6.12.2	MDT

Source:			Huawei (Summary rapporteur)

Title:				Email discussion report: [AT109e][803][MDT] MDT open issues

WI code(s):			NR_SON_MDT-Core

Document for:		Discussion and Decision

## 1	Introduction

This document captures the important open issues for MDT:

- [AT 109e][803][MDT]MDT open issues (Huawei)

Intended outcome: email discussion report in R2-2001991

Deadline: CET 23:00, 2020/03/01

## 2	Discussion

### 2.1	Location related issues

Issue 1: For MR-DC case, how the UE handle WLAN/BT measurements?

Solutions:

- Option 1: Proposal 1: For location information of SCGFailureInformationEUTRA/ SCGFailureInformationNR, UE includes available WLAN measurement results and Bluetooth measurement results which are only from SN configured wlanNameList /btNameList of imediate MDT. ([1], QC)

- Option 2: … are only from MN…

  - Note: The above option 2 is captured in the latest 38.331 CR and 36.331 CR

We request companies to provide comments into the following table.

| Company name | Preferred option(s) | Does SON/MDT still work without this? (Yes/No/May be) | Additional comments |
| --- | --- | --- | --- |
| Huawei, HiSilicon | Option 2 | No | During email discussion 108#43, our comments were as below: Regarding the RRC format for location info in SCGFailureInformationNR message, I see that there are slightly more companies supporting to use LTE RRC format, and additionally there may be more impacts with using NR RRC format (as I mentioned in my previous email), so I suggest to keep using LTE RRC format for this CR. Companies who support to use NR RRC format could bring contributions at RAN2#109 and then we could further discuss Pros/Cons for it. Our above comment is aligned with option 2, and we think it is more simpler. The feature of location (GPS/WLAN/BT) in SCGFailureInformationEUTRA/ SCGFailureInformationNR is priority for some operators, so it is better to do it in Rel-16. |
| Ericsson | Option 2 | Yes | By including the MN configured WLAN and Bluetooth measurements, the UE can aid the MN to identify where (location) the SCG failure has occurred. Using this information, the MN can initiate relevant actions to set up new measurements to find relevant SN candidates. |
| QUALCOMM | Option 1 | Yes | We think the location of information in the SCGFailureinformation should come from the node where RLF occurs. For immediate MDT with DC cases, as both MN and SN can configure separate list of WLAN APs and Bluetooth beacons for measurement to obtain location information independently, it brings additional functionality and memory requirement for UE, what’s more, to include the location information in SCGFailureInformationNR/ SCGFailureInformationEUTRA, UE needs to convert SN configured WLAN/Bluetooth measurement result to the MN via the MN RRC format, which requires UE to have across-RAT measurement result report capability. It is necessary to introduce new UE capability to support immMeasBT for DC and immMeasWLAN for DC and immMeasSensor for DC. |
| LG |  | Yes | Measurement results configured by SCG should be included in the MeasResultSCG in the form of OCTET STRING. However, according to the current CR, the locationInfo is outside the MeasResultSCG, so only the WLAN/Bluetooth measurement results configured by MCG can be in the locationInfo. However, if UE needs to report the WLAN/Bluetooth measurement results configured by SCG, we are ok to include it in the measResultsSCG. |
| CATT | Option1 or Option 2 | Yes | If BT/WLAN info is used by MN, option 2 can assist MN to optimize SN addition/Modification procedure directly. But if BT/WLAN info should be transferred from MN to SN, we think option1 is more suitable. |
| Samsung | Option 1 | Yes | SCG failure information received in MN is forwarded to SN via CG-ConfigInfo IE. We assume that the location information should be also forwarded together. Accordingly, the option 1 seems better. The option 2 would result in additional complexity unnecessarily. |

Proposal XX: To be added

### 2.2	CEF related issues

Issue 1: How to design multiple CEF reports

Solutions:

- Option 1: Only the latest CEF report + number. (P1, P2, P4, [3], CATT), (P1, P3, [7], ZTE), (P2, [22], Samsumg). The following proposals are main proposals for option 1 (from [3], CATT)

  - Proposal 1: UE only records one entry of the detail parameters about the last CEF, while for the other previously experienced CEFs, only the per cell “Number of connection failures” field could be recorded.

  - Proposal 2: If a new CEF occurs, the old CEF records except the per cell numberOfConnFail in UE variable should be replaced by the new CEF records and the counter of the newest cell where the UE experiences CEF should be added with 1.

  - Proposal 3: UE reports the latest number of consecutive connection failures per cell the UE has experienced within the last 48 hours.

- Option 2: A list of CEF reports.

- Note: both option 1 and option 2 have been shown in the latest 38.331 CR (according to revision marks)

We request companies to provide comments into the following table.

| Company name | Preferred option(s) | Does SON/MDT still work without this? (Yes/No/May be) | Additional comments |
| --- | --- | --- | --- |
| Huawei, HiSilicon | Option 1 | No | Option 1 is simpler and we think it has more supports. Multiple CER reports were studied and agreed from SI, so it should be supported. |
| Ericsson | Option 2 | No | If the UE experience CEF while attempting connection establishment in the following order, {cell-A, cell-A, cell-A, cell-B} and then if the UE is able to connect to cell-A on the 5th attempt, then if we adopt option-1, the UE just includes one CEF report which contains details regarding CEF towards cell-B on the 4th attempt and the numberOfConnFail will be 1. This is very limited information. By including option-2, the UE can indicate that there have been 3 successive CEFs towards cell-A and one CEF towards cell-B before the UE was able to perform successful connection establishment. This will give more details to the OAM to identify the locations where there is a coverage issue and also identify which cells might be good candidates for tuning the coverage related parameters. |
| QUALCOMM | Option 1 | No | Option 1 is agreed by RAN2. Multiple CEF reports can be studied in next release. |
| LG | Option2 | No | We have the same view as Ericsson. Option 2 is simpler in terms of UE behavior and NW also can get further details for all cells. |
| CATT | Option 1 | No | Option 1 is the agreement in RAN2#106, and option 2 has not been agreed before. Since no enhancement will be further considered in R16, UE should only record one entry of detailed CEF and record the number of connection failures per cell. Based on this option, the detail of this solution needs to be decided: - How to record the number of connection failures per cell? CATT propose “If a new CEF occurs, the old CEF records except the per cell numberOfConnFail in UE variable should be replaced by the new CEF records and the counter of the newest cell where the UE experiences CEF should be added with 1”; - Whether the limitation of 48h is applied only for the detailed CEF entry or also applied for the number of connection failures per cell? And how? CATT doesn’t prefer to let two parts of records (detailed entry and failure number per cell) associate with a single timer which may introduce high complexity in UE side. |
| Samsung | Option 1 | No | We assume that option 1 follows the intention of RAN2 agreement. |

Proposal XX: To be added

### 2.3	MDT conf and report related issues

Issue 1: What is the the maximum number of cellIndentity to be configured as part of  AreaConfigForNeighbour?

Solutions:

- Option 1: 32 (P17, 108#42 email report)

- Option 2: separate numbers for FR1 and FR2 (P3, [5], Vivo)

We request companies to provide comments into the following table.

| Company name | Preferred option(s) | Does SON/MDT still work without this? (Yes/No/May be) | Additional comments |
| --- | --- | --- | --- |
| Huawei, HiSilicon | Option 1 | No | We think the option has more supports. We need to decide on the number as it is impacting ASN.1. |
| Ericsson | Option-1 | No | It is simple to provide only one list that can be used for both FR1 and FR2 related configurations. |
| QUALCOMM | Option 1 | No |  |
| LG | Option 1 | No |  |
| CATT | Option 1 | No |  |
| Samsung | Option 1 | No |  |

Proposal XX: To be added

Issue 2: UE behaviour on logged results

Solutions:

- Option 1: Proposal 7: For the report of logged measurement result, the UE will firstly consider the logged measurement result available in case the RAT Type of logged measurement result is align with the RAT type of either PCell or PSCell. (P7, [6], ZTE)

We request companies to provide comments into the following table.

| Company name | Preferred option(s) | Does SON/MDT still work without this? (Yes/No/May be) | Additional comments |
| --- | --- | --- | --- |
| Huawei, HiSilicon | Option 1 or nothing | Yes | Nothing means that it can be left to UE implementation. It seems not a critical issue, and perhaps it can be clarified later. |
| Ericsson | Parts of Option-1 | Yes | The UE need not check the PSCell related RAT type as this is about logged MDT reporting and PSCell has nothing to do with it. In our understanding, if the UE was configured with logged MDT by an EUTRA node, then the UE shall report the associated logged MDT report to an EUTRA PCell and if the UE was configured with logged MDT by an NR node, then the UE shall report the associated logged MDT report to an NR PCell.. |
| QULACOMM | NO | Yes | we only consider MN PCell type for logged MDT report as logged MDT with DC is not discussed in Rel-16. UE enters RRC_Connected with single connectivity first, so only consider Pcell RAT type. |
| LG |  | Yes | Agree with Ericsson. |
| CATT | Parts of Option 1 or nothing | Yes | We do not agree to support logged MDT for SN in R16, only the immediate MDT is support for SN node. |
| Samsung | NO | Yes | Same understanding with Qualcomm. This proposal is not required in this release. |

Proposal XX: To be added

Issue 3: Whether to control the report of beam measurements

Solutions:

- Option 1: Proposal 2	Include an indicator in the logged MDT configuration that configures the UE regarding whether the UE shall include the beam level measurements in the logged MDT report or not. (P2, [9], Ericsson)

We request companies to provide comments into the following table.

| Company name | Preferred option(s) | Does SON/MDT still work without this? (Yes/No/May be) | Additional comments |
| --- | --- | --- | --- |
| Huawei, HiSilicon | Option 1 or nothing | May be | In the latest 38.331 CR, it is best effort to include the beam results and that is “nothing” in our opinion. We also see some benefits for option 1. |
| Ericsson | Option 1 | May be | As beam measurement information is not required in all deployments, for example, in low band where the number of SSBs transmitted by the cell is one, then the cell measurement will be same as beam measurement. By avoiding this duplication of measurements in the log, both the available logged MDT memory can be optimized and also the logged MDT report size can be reduced. |
| QUALCOMM | Nothing | yes | According to our understanding, the intention of the proposal is request UE to log neighboring cell beam information. RAN2 has discussed the beam measurement in logged MDT for a long time and agreed that UE only logs camped cell beam information. No need to log neighboring cell beam information. Neighboring cells have lots of beam information and it costs UE pretty much memory. |
| LG | Nothing | Yes | We prefer to keep the current CR as it is simpler. We don’t want to complicate the logged MDT configuration just for optimization. |
| CATT | Option1 or nothing | Maybe | RAN2 agreed to introduce the following beam measurement information in the logged MDT measurement results: - the best beam index(SSB index) of the camped cell - the beam RSRP/RSRQ of the best beam of camped cell - the ‘number of good beams’ associated to the cells within the range (which is configured by network for cell reselection) of the R value of the highest ranked cell Since beam specific results for log MDT are limited to the info above, the beam level information will not use too many bits, so we slightly think it’s better to report them without indicator. |
| Samsung | nothing | Yes | We would like to avoid any optimization. |

Proposal XX: To be added

Issue 4: Out of coverage design

Solutions:

- Option 1: (P1, P2, [20], Samsung)

  - Proposal 1: In the out-of-coverage event, the entering condition is that UE enters Any Cell Selection at least.

    - Proposal 1-1: RAN2 to study whether to log measurmenet results during Camped on Any Cell state

  - Proposal 2: In the out-of-coverage event, the leaving condition is that UE enters Camped Normally state.

We request companies to provide comments into the following table.

| Company name | Preferred option(s) | Does SON/MDT still work without this? (Yes/No/May be) | Additional comments |
| --- | --- | --- | --- |
| Huawei, HiSilicon | Nothing | Yes | During online session of MDT/SON in week 1, there were some conclusions on OOC, so we think option 1 may not be needed. |
| Ericsson | Proposal 1and 2 are okay but not proposal 1-1 | Yes | The current proposal 1 and 2 are the definition of OOC related event entering and leaving condition. This is already captured in the running CR. Regarding the proposal 1-1, we think that this is not agreeable as this could potentially mean that the UE is collecting measurements on a different PLMN (where it can receive the ETWS info only) and reporting this to the home PLMN. |
| QUALCOMM | Only Proposal 2 | Yes | Firstly, Any cell selection state is logged by UE in logged MDT. Secondly, when UE enters any cell selection state, UE may come back to camp normally state, so we don’t think the entering condition of OOC is when UE enters Any Cell Selection. For the online agreement “UE configured for the event-triggerd logged MDT logs the cell ID, location information if available and time stamp information on - last serving cell UE camped on before OOC happens, and - first serving cell after leaving the OOC It is the leaving condition.” We should remove “last serving cell UE camped on before OOC happens” as it is agreed online that it is only the leaving condition. As when UE camped normally, it doesn’t knows it will enter OOC and thus the last serving cell information may not be available at UE when it enters OOC. |
| LG | P1, 2 +@ | No (UE behavior is ambiguous.) | What if UE moves from ‘Any Cell Selection’ to ‘Camped on Any Cell’? In this case, If we agree only single entering condition and single leaving condition as in P1 and P2, the entering condition has been met but the leaving condition is not yet. This means that the UE performs the logging during Camped on Any Cell state. So RAN2 should choose one between following options: Option1-1: UE performs the logging during Camped on Any Cell state. Option 1-2: In the out-of-coverage event, the second leaving condition is that UE enters Camped on Any Cell state. (The first leaving condition is P2.) |
| CATT | Proposal 1and parts of proposal 2 | Yes | UE treats the state of not suitable or not acceptable as “Any Cell Selection”, and only in Any Cell Selection state the UE will trigger the out-of-coverage event. Vice versa, the leaving condition could be that UE enters Camped Normally state or enters Camped on Any Cell state. |
| Samsung | option 1 | No | Basically, the entering and leaving conditions are required to support the event-triggered logged MDT. If either is not defined, how can UE start or stop the (event-triggered) logged MDT? A controversial issue is if Camped on Any Cell state is regarded as OOC. We have no strong opinion on it. |

Proposal XX: To be added

Issue 5: Values of loggingInterval

Solutions:

- Option 1:

  - Proposal 1: Include ‘infinity’,’640ms’ and ‘320ms’ as options for loggingInterval value range.	(P1, [11], Ericsson), (P2, [2], CATT).

Note: In the latest 38.331 CR, the values of loggingInterval are as below:

LoggingInterval-r16 ::=			ENUMERATED {

ms1280, ms2560, ms5120, ms10240, ms20480,

ms30720, ms40960, ms61440}

We request companies to provide comments into the following table.

| Company name | Preferred option(s) | Does SON/MDT still work without this? (Yes/No/May be) | Additional comments |
| --- | --- | --- | --- |
| Huawei, HiSilicon | Option 1 | Yes | We see some benefits for option 1. |
| Ericsson | Option 1 | Yes | The event triggered logged MDT benefits from having smaller logging intervals like 320ms and 640ms as the UE will be able to report more measurements in the concerned area i.e., where the event condition is satisfied. The value ‘infinity’ is used for one-shot-reporting. |
| QUALCOMM | Option 1 | Yes |  |
| CATT | Option 1 | Yes |  |
| Samsung | Option 1 | Yes | The value ‘infinity’ is also fine for one-shot logging. It would be applicable both for periodic and event-triggered. |

Proposal XX: To be added

Issue 6: Whether to indicate RAT type in the areaConfigForNeighbour IE

Solutions:

- Option 1:

  - Proposal 2: Introduce RAT-Type as part of areaConfigForNeighbour along with frequency and cell list.		(P2, [11], Ericsson)

We request companies to provide comments into the following table.

| Company name | Preferred option(s) | Does SON/MDT still work without this? (Yes/No/May be) | Additional comments |
| --- | --- | --- | --- |
| Huawei, HiSilicon | Nothing | Yes | Currently, there are frequency and cell id in the areaConfigForNeighbour, and we think it is sufficient to control the reporting. |
| Ericsson | Option-1 | Yes | In our understanding, the main purpose of introducing areaConfigForNeighbour IE was to enable mmW related measurements’ collection even when the UE is camping on LTE. This was the main concern expressed by the operators at the beginning of the SI itself. This is not at all possible if we do not allow the RATType configuration as part of areaConfigForNeighbour. |
| QUALCOMM | Option-1 | Yes | Agree with Ericsson |
| LG | Nothing | Yes | If my understanding is right, the intention is make UE perform logging according to the NR configuration even after cell re-selection on LTE. However, the same goal can be achieved by NW configuration. If the logging is needed while the UE camps on a LTE frequency, eNB would configure the logged measurement for the UE. Then, the UE will perform the logging according to the LTE configuration. |
| CATT | Nothing | Yes | The motivation to introduce areaConfigForNeighbour IE is mainly to allow UE to log some results of NR neighbor cells when UE is camping in LTE cell (For example, EN-DC deployment scenario), if we think this is the motivation, the RAT type info can be indicated in an implicit way, e.g. using field description. The RAT type info is needed if we think it’s also possible for a UE to log some results of LTE neighbor cells when UE is camping in LTE cell, but this case is not the way we do for LTE log MDT, we never discuss this use case at all. |
| Samsung | Nothing | Yes |  |

Proposal XX: To be added

Issue 7: Whether to have single D1 measurement per CG or across CG.

Solutions:

- Option 1: (P1, [5], Vivo)

  - Proposal 2: ul-DelayRatioConfig and ul-DelayValueConfig are configured per CG, i.e.,

  - -	to configure at most one measurement identity per CG using a reporting configuration with the ul-DelayRatioConfig;

  - -	to configure at most one measurement identity per CG using a reporting configuration with the ul-DelayValueConfig;

We request companies to provide comments into the following table.

| Company name | Preferred option(s) | Does SON/MDT still work without this? (Yes/No/May be) | Additional comments |
| --- | --- | --- | --- |
| Huawei, HiSilicon | Option 1 | No | This option is to address following FFSs in 38.331 CR (as below), and we think it is reasonable. - to configure at most one measurement identity (FFS: across all CGs/ per CG) using a reporting configuration with the ul-DelayRatioConfig; - to configure at most one measurement identity (FFS: across all CGs/ per CG) using a reporting configuration with the ul-DelayValueConfig; |
| Ericsson | Option 1 | No | These measurements are to be configured per CG. |
| QUALCOMM | Option 1 | No |  |
| LG | Option 1 | No |  |
| CATT | Nothing | No | We think this proposal is too high-level, we prefer the following understanding: For MN/SN terminated bearer, we think D1 can be collected by MN/SN separately. But for split bearer, only one D1 is collected by the PDCP anchor node. |
| Samsung | Option 1 | No |  |

Proposal XX: To be added

Issue 8: obtain the SSB index of neighbouring cells

Solutions:

- Option 1: UE does not log the SSB index of the neighbour cells if additional burden is introduced.

We request companies to provide comments into the following table.

| Company name | Preferred option(s) | Does SON/MDT still work without this? (Yes/No/May be) | Additional comments |
| --- | --- | --- | --- |
| Huawei, HiSilicon |  | May be |  |
| Ericsson | Option-1 (related to issue-3 of section 2.3) | May be | This is related to issue-3. We agree that there is a burden on the UE to decode the PBCH payload in mmW to understand the beam index. This overhead is larger in mmW compared to low and mid frequencies where the beam index is decoded via the DMRS sequence used for PBCH. So, we are fine with introducing some flexibility in logging beam measurements. This can be resolved by the solution proposed in issue-3 of section 2.3. |
| QUALCOMM | Option 1 | May be | It is additional burden for UE. For logged MDT, UE only logs camped cell beam information |
| CATT | Option 1 | May be | We also consider the SSB index is a burden to decode the PBCH payload. The SSB index is not used in the normal cell reselection to neighbor cell. |
| Samsung | Option 1 | No | It has been identified that it’s really critical burden to UE. It should be surely avoided. |

Proposal XX: To be added

Issue 9: UE logging behaviour

Solutions:

- Option 1:.

  - Proposal 4: The actual process of logging within the UE, takes place in RRC IDLE state could continue in RRC INACTIVE state.

We request companies to provide comments into the following table.

| Company name | Preferred option(s) | Does SON/MDT still work without this? (Yes/No/May be) | Additional comments |
| --- | --- | --- | --- |
| Huawei, HiSilicon | Option 1 | No |  |
| Ericsson | Option 1 | No |  |
| QUALCOMM | Option 1 | No |  |
| LG | Option 1 | No |  |
| CATT | Option 1 | No |  |
| Samsung | Option 1 | No |  |

Proposal XX: To be added

### 2.4	CGI info related issues

Issue 1: How to design CGI info for MDT and SON

Background:

In the latest 38.331 CR, CGI info is put in RLF report, RA-Report and CEF report. During email discussion, lots of companeis commented that CGI-InfoNR was designed for ANR and it might have too large scope for MDT and SON. So the CGI info has been changed to the following one:

CGI-INFO-LOGGING-r16 ::=                   SEQUENCE {

plmn-Identity                       	PLMN-Identity,

cellIdentity                            CellIdentity

}

Solutions:

There are the following options on CGI info for MDT and SON: (P2, [18], Samsung)

Proposal 2: Instead of the IE CGI-InfoNR, a new IE is introduced to indicate NR cell, and the following content for the new IE can be considered:

- Option 1: first PLMN-Identity (in plmn-IdentityList) and CellIdentity in first PLMN-IdentityInfo IE of PLMN-IdentityInfoList

- Option 2: plmn-IdentityList and CellIdentity in first PLMN-IdentityInfo IE of PLMN-IdentityInfoList

- Option 3: first PLMN-Identity (in plmn-IdentityList) and CellIdentity in each PLMN-IdentityInfo of PLMN-IdentityInfoList

- Option 4: plmn-IdentityList and CellIdentity in each PLMN-IdentityInfo of PLMN-IdentityInfoList

- Option 5: List of PLMN identity in PLMN-IdentityInfoList matching with the PLMN identity (i.e. MDT PLMN list) configured in MDT configuration + CellIdentity corresponding to the PLMN identity

- Option 6: Registered PLMN identity + CellIdentity

It is noted that the following proposal was agreed to be addressed in email discussion [803], and we think it has been covered by above options.

Proposal 8: The IE CGI-InfoNR is not used to indicate a NR cell.	(P1, [18], Samsung)

-	Note: During email discusison 108#42 on 38.331 CR, lots of companies also preferred to not use CGI-InfoNR. So the next step is to go to section 2.3.4 to discuss CGI info details.

We request companies to provide comments into the following table.

| Company name | Preferred option(s) | Does SON/MDT still work without this? (Yes/No/May be) | Additional comments |
| --- | --- | --- | --- |
| Huawei, HiSilicon | Option 1 | No | During email discussion 108#42 38.331 CR, it seems that more companies like to have a simple solution on NCGI, and we think option 1 is a simple option. NCGI is widely used in MDT and SON, so it should be defined in R16. |
| Ericsson | Option 5 | No | If the OAM had configured the UE with multiple PLMN list as part of the logged MDT configuration, we believe that it is an indication that the OAM wants to build coverage maps associated to cells in these PLMNs. Therefore, if a physical cell broadcast both these PLMNs’ related information, then it is beneficial to include at least these PLMNs’ related information in the logged MDT report. |
| QUALCOMM | Option 1 | No |  |
| LG | Option 1 | No | Single PLMD ID and single cell ID are sufficient to identify the cell for MDT purpose. In option 6, CellIdentity should be CellIdentity corresponding to the registered PLMN identity, but it cannot be broadcasted in SIB1 form the corresponding cell. So we prefer option 1. |
| CATT | Option 1 | No | It is a simple and useful option to identify the NCGI. |
| Samsung | Option 1 | No | The option 1 seems the simplest especially in signaling overhead aspect. It would be used to indicate NCGI in all MDT progress, i.e. configuration/logging/retrieval. |

Proposal XX: To be added

In addition, there is a proposal on PLMN ID:

Option 1:

Proposal 1: plmn-Identity in the CGI-Info-Logging identifies the PLMN of the cell for the reported cellIdentity.

Proposal 2: plmn-Identity field description in the CGI-Info-Logging is adopted as follows:

plmn-Identity Identifies the PLMN of the reported cellIdentity as given by the first PLMN entry of the plmn-IdentityList sent in the PLMN-IdentityInfo of the cell’s acquired SIB1.

(P2, [13], Nokia)

We request companies to provide comments into the following table.

| Company name | Preferred option(s) | Does SON/MDT still work without this? (Yes/No/May be) | Additional comments |
| --- | --- | --- | --- |
| Huawei, HiSilicon | Option 1 | No | Both propsoals are reasonable. |
| Ericsson | Option 5 from previous issue | No | Same explanation as previous issue. |
| QUALCOMM | Option | No | Option 1 will save UE logged MDT memory cost. |
| LG |  |  | This depends on the previous issue. |
| CATT | Option 1 | No |  |
| Samsung | Option 1 | No |  |

Proposal XX: To be added

In this RAN2#109-e meeting, there is an incoming LS from RAN3:

- R2-2000028, LS on information needed for MRO in UE RLF Report, Source:RAN3, To: RAN2

In the LS, the following text may be related to CGI info design:

- TAI of the failed cell and source cell: These two IEs could be used by the network node for routing of MRO signaling messages as well as to differentiate between intra-system HO and inter-system HO.

Issue 2: How to handle RAN3 LS R2-2000028

Solutions:

- Option 1: Add TAI into CGI info

We request companies to provide comments into the following table.

| Company name | Preferred option(s) | Does SON/MDT still work without this? (Yes/No/May be) | Additional comments |
| --- | --- | --- | --- |
| Huawei, HiSilicon | Option 1 | No | Following RAN3’s request. |
| Ericsson | Option 1 | No | TAC information is required to route the RLF report to identify the failed and the source cells in an easier way. |
| QUALCOMM | Option 1 | No | Follow RAN3 request, add TAC to enable cross-RAT RLF report routing |
| LG | Option 1 | No | Following RAN3’s request. |
| CATT | Part of Option 1 | No | Only add TAC of the failed cell. TA info of the source cell is not needed, to introduce 1 bit to differentiate between intra-/inter-system HO instead is enough. |
| Samsung | Option 1 | No |  |

Proposal XX: To be added

### 2.5	Void

.

.

### 2.6	RLF report related issues

Issue 1: ssb/rs Bitmap in RLF report

Solutions:

Proposal 12     (Based on comments from Nokia in this email discussion) – (specification clarification related) - RAN2 to discuss the way in which the ssbRLMConfigBitmap and csi-rsRLMConfigBitmap are encoded in the latest RRC specification:

- Option a.     To use the format used in the current NR RRC specification.

- Option b.     To use a 1-bit field per beam (SSB or CSI-RS) level measurements included in the RLF report.

We request companies to provide comments into the following table.

| Company name | Preferred option(s) | Does SON/MDT still work without this? (Yes/No/May be) | Additional comments |
| --- | --- | --- | --- |
| Huawei, HiSilicon | Option a or b | No | Either a o b is acceptable. |
| Ericsson | No strong view | No | Option-a gives more information compared to option-b |
| QUALCOMM | Option a | No | It is a simple solution for UE. |
| CATT | No strong view | No | Slightly prefer option b if can save some bits. |
| Samsung | Option a | No |  |

Proposal XX: To be added

### 2.7	About outgoing LS to other WGs

Issue 1: Potential LS to RAN3, SA5

Contents:

- RAN2’s understandings on NG-RAN node behaviorus 	([6])

- Inform other WGs of RAN2 agreements	([8], [9])

- Potential reply LS to:

  - R2-2000028, LS on information needed for MRO in UE RLF Report, Source:RAN3, To: RAN2

The following table is to collect comments on potential LS to other WGs, e.g. RAN3, SA5.

| Company name | Does RAN2 need to send out a LS to other WGs? | If needed, what will be captured in the LS? |
| --- | --- | --- |
| Huawei, HiSilicon | Yes | It is suggested to list RAN2 agreements (copy and paste), and the targeted WGs can finilize their work. |
| Ericsson | Yes |  |
| QUALCOMM | yes |  |
| CATT | Yes | CATT has already presented a draft reply LS R2-2000107, which could be modified based on the agreement of the e-Meeting discussion. |
| Samsung | Yes |  |

## 3	Tdocs under AI 6.12.2 MDT

- R2-2000001	Remaining issues of UE Location Information	Qualcomm Incorporated 	discussion	Rel-16	NR_SON_MDT-Core

- R2-2000100	Discussion on logged MDT	CATT	discussion	Rel-16	NR_SON_MDT-Core

- R2-2000101	Corrections for CEF  and RLF Report	CATT	discussion	Rel-16	NR_SON_MDT-Core

- R2-2000102	Failure Indication about SCG	CATT	discussion	Rel-16	NR_SON_MDT-Core

- R2-2000300	Remaining issues on MDT	vivo	discussion

- R2-2000807	Remaining issues on MDT	ZTE Corporation, Sanechips	discussion	Rel-16	NR_SON_MDT-Core

- R2-2001100	Clarification on CEF report	ZTE Corporation, Sanechips	discussion	Rel-16

- R2-2001111	[DRAFT] LS on MDT configurations	Ericsson	LS out	Rel-16	NR_SON_MDT-Core	To:TSG SA5, TSG RAN3

- R2-2001114	Measurement configuration options for immediate and logged MDT in NR	Ericsson	discussion

- R2-2001115	Neighbor frequency coverage hole indication in logged MDT	Ericsson	discussion

- R2-2001117	Open issues related to current logged MDT contents in running CR	Ericsson	discussion

- R2-2001143	A2-like measurement results in Logged MDT	Nokia, Nokia Shanghai Bell	discussion	Rel-16	NR_SON_MDT

- R2-2001144	CGI in MDT reports	Nokia, Nokia Shanghai Bell	discussion	Rel-16	NR_SON_MDT

- R2-2001145	Location information encoding in NR MDT	Nokia, Nokia Shanghai Bell	discussion	Rel-16	NR_SON_MDT

- R2-2001146	Principle on Rareport entry logging	Nokia, Nokia Shanghai Bell	discussion	Rel-16	NR_SON_MDT

- R2-2001366	Enhancements on logged MDT and accessibility measurements	Huawei, HiSilicon	discussion	Rel-16	NR_SON_MDT-Core	R2-1915855

- R2-2001367	Discusison on sensor measurement reporting	Huawei, HiSilicon	discussion	Rel-16	NR_SON_MDT-Core

- R2-2001436	Clarification on Cell ID Applied to NR MDT	Samsung	discussion	NR_SON_MDT-Core

- R2-2001438	Introducing Further Information Useful for NR MDT	Samsung	discussion	NR_SON_MDT-Core

- R2-2001439	On supporting Event-triggered Logged MDT	Samsung	discussion	NR_SON_MDT-Core

- R2-2001441	RLF Report Enhancement	Samsung	discussion	NR_SON_MDT-Core

- R2-2001443	Stage-3 Miscellaneous for NR MDT 	Samsung	discussion	NR_SON_MDT-Core

## 4 Classification and group of proposals

| Tdoc No | Tdoc | Proposal | Suggested classification | Sub-topic | Note |
| --- | --- | --- | --- | --- | --- |
| [1] | R2-2000001 Remaining issues of UE Location Information Qualcomm Incorporated | Proposal 1: For location information of SCGFailureInformationEUTRA/ SCGFailureInformationNR, UE includes available WLAN measurement results and Bluetooth measurement results which are only from SN configured wlanNameList /btNameList of imediate MDT. | (b) | Location | 108#42+43 discussed it. |
|  |  | Proposal 2: Introduce new UE capability of immMeasBT-DC and immMeasWLAN-DC to support immediate MDT for Bluetooth and WLAN under DC case. | (c) | UE cap | Could be discussed later |
| [2] | R2-2000100 Discussion on logged MDT CATT | Proposal 1: A logged MDT triggering type is needed in LogMeasReport-r16 to indicate whether it is a periodic or an event triggered logged MDT. | (c) | MDT report | 108#42+43 discussed it. |
|  |  | Proposal 2: Discuss whether a smaller logging interval (e.g. 320ms or 640ms) for event triggered logged MDT is needed. | (c) | MDT conf | Seems enhancements |
| [3] | R2-2000101 Corrections for CEF and RLF Report CATT | Proposal 1: UE only records one entry of the detail parameters about the last CEF, while for the other previously experienced CEFs, only the per cell “Number of connection failures” field could be recorded. | (b) | CEF report | 108#42+43 discussed it. Two options: - Only the latest CEF report+number - A list of CEF reports |
|  |  | Proposal 2: If a new CEF occurs, the old CEF records except the per cell numberOfConnFail in UE variable should be replaced by the new CEF records and the counter of the newest cell where the UE experiences CEF should be added with 1. | (b) | CEF report | Related to (1) |
|  |  | Proposal 3: Discuss which option could be applied for the CEF records: - Option0: The detailed CEF and each record of the failure number per cell should follow the 48 hours rule; - Option1: Only the detailed CEF follows the 48 hours rule; - Option2: Do not record any failure number, only one detailed CEF is recorded. | (c) | CEF report |  |
|  |  | Proposal 4: There is only one counter for a unique NCGI in numberOfConnFail. | (b) | CEF report | Related to (1) |
|  |  | Proposal 5: Add “No suitable cell found” flag in the NR RLF report when T311 expires. | (a) | RLF report | Related to P1 in [21] There was an agrement at RAN2#107: - “No suitable cell is found” information, if is available for reporting A small change is added into the proposal to be more clearer |
| [4] | R2-2000102 Failure Indication about SCG CATT | Proposal 1: Add source cell information into SCGFailureInformation message to assist the optimization of SN change-related failures, which may include: - previousPSCellId - failedPSCellId - timeConnFailureSCG | (c) | SCGFailure | P1 ~ P4 seem enhancements. |
|  |  | Proposal 2: Add RACH related information about SN into SCGFailureInformation message. | (c) | SCGFailure |  |
|  |  | Proposal 3: RACH related information about SN could use the same IE as in RLF report. | (c) | SCGFailure |  |
|  |  | Proposal 4: Source cell information and the RACH related information about SN could be sent to SN in CG-ConfigInfo message. | (c) | SCGFailure |  |
| [5] | R2-2000300 Remaining issues on MDT vivo | Proposal 1: ul-DelayRatioConfig and ul-DelayValueConfig are configured per CG, i.e., - to configure at most one measurement identity per CG using a reporting configuration with the ul-DelayRatioConfig; - to configure at most one measurement identity per CG using a reporting configuration with the ul-DelayValueConfig; | (a) | Delay conf | Related to P15 in email 108#42 |
|  |  | Proposal 2: RAN2 to agree that the potential values (ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240, ms20480, ms40960, min1,min6, min12, min30)of the existing ReportInterval can be reused for the field reportInterval in ul-DelayRatioConfig and ul-DelayValueConfig. | (a) | Delay conf | Related to P16 in email 108#42 |
|  |  | Proposal 3: For FR1, the maximum number of cellIndentity is 16, whereas for FR2 the the maximum number of cellIndentity is 32. | (b) | MDT conf | Related to P17 in email 108#42 |
|  |  | Proposal 4: UE speed measurement and UE orientation information should refer to TS 36.355. | (d) | Location | Related to [14] |
|  |  | Proposal 5: The maximum number of entries for logged measurements is 520. | (a) | MDT report | Related to email 108#42 |
| [6] | R2-2000807 Remaining issues on MDT ZTE | Proposal 1: Upon receiving the signalling based immediate MDT activation command from 5GC, the preferred NG-RAN node behaviour is to reject the activation if the UE involved is in IDLE mode. | (c) | MDT conf | More about NW behaviour |
|  |  | Proposal 2: If the management based immediate MDT received by the NG-RAN when UE is in RRC_INACTIVE : - No requirement for the NG-RAN to store the immediate MDT configuration in the UE context. - When the UE resumes the RRC connection in the last serving NG-RAN, the NG-RAN can configure the MDT configuration for the UE. - When the UE resumes the RRC connection in another NG-RAN, the source NG-RAN will not propagate the management based immediate MDT configuration. The source NG-RAN should inform the target NG-RAN of UE consents. | (c) | MDT conf | More about NW behaviour |
|  |  | Proposal 3: The RAN will not page INACTIVE UE only for the purpose of signalling based immediate MDT. | (c) | MDT conf | More about NW behaviour |
|  |  | Proposal 4: Upon receiving signalling based immediate MDT activation command from CN, the preferred behavior for NG-RAN is to reject the command if the involved UE is in RRC_INACTIVE mode. | (c) | MDT conf | More about NW behaviour |
|  |  | Proposal 5: If the approved behavior for NG-RAN upon receiving signalling based immediate MDT is to store configuration in UE inactive context and propagate the configuration between RAN nodes, the NG-RAN shall notify the CN that the targeting UE is in RRC_INACTIVE mode. | (c) | MDT conf | More about NW behaviour |
|  |  | Proposal 6: The MDT aspect should not prevent the NW from sending the UE to INACTIVE/IDLE mode. | (c) | MDT conf | More about NW behaviour |
|  |  | Proposal 7: For the report of logged measurement result, the UE will firstly consider the logged measurement result available in case the RAT Type of logged measurement result is align with the RAT type of either PCell or PSCell. | (b) | MDT report |  |
|  |  | Proposal 8: The UE can indicate the RAT type of the available log to the RAN node. | (c) | MDT report |  |
|  |  | Proposal 9: It’s suggested to send an LS to SA5 and RAN3 to confirm the above RAN2’s understanding. | (b) | LS related |  |
| [7] | R2-2001100 Clarification on CEF report ZTE | Proposal 1: It’s proposed to confirm the understanding that multiple CEF related to the same cell will be included in separated CEF entry of a CEF report. | (b) | CEF report | Related to P1 in [3]. |
|  |  | Proposal 2: To explicitly indicate the number of CEF per cell within 48 hours in CEF report. e.g. UE will maintain a list of numOfCEFperCell to indicate the number of CEF per cell it has been experienced for the last 48 hours. | (c) | CEF report | Related to P3 in [3]. |
|  |  | Proposal 3: UE will keep update the oldest CEF entry with new entry carrying information related to latest CEF event when all entries are completed and the CEF report haven’t been fetched yet. | (b) | CEF report |  |
|  |  | Proposal 4: After successful deliver of the UEInformationResponse carrying the CEF report, UE will discard the CEF report stored in VarConnEstFailReport except for the list of numOfCEFperCell. | (a) for the sentence before “except for” (b) for the left part | CEF report |  |
| [8] | R2-2001111 [DRAFT] LS on MDT configurations Ericsson |  | (b) | LS related | Section 2.x is to discuss LSs |
| [9] | R2-2001114 Measurement configuration options for immediate and logged MDT in NR Ericsson | Proposal 1 Send the LS [2] to SA5 to inform about the possibility of configuring rsType, includeBeamMeasurements, reportQuantityRS-Indexes, maxNrofRS-IndexesToReport and reportOnLeave as part of the immediate MDT configuration in NR related to M1 measurement. | (b) | LS related |  |
|  |  | Proposal 2 Include an indicator in the logged MDT configuration that configures the UE regarding whether the UE shall include the beam level measurements in the logged MDT report or not. | (b) | MDT conf | In the latest 38.331 CR, it is best effort to include the beam results in logged measurements. |
| [10] | R2-2001115 Neighbor frequency coverage hole indication in logged MDT Ericsson | Proposal 1 Include an indicator in the logged MDT report associated to neighbor frequencies on which the UE has performed measurements and has not detected any SSBs. | (c) | MDT report | Seems enhancements |
| [11] | R2-2001117 Open issues related to current logged MDT contents in running CR Ericsson | Proposal 1 Include ‘infinity’,’640ms’ and ‘320ms’ as options for loggingInterval value range. | (c) | MDT conf | Seems enhancements |
|  |  | Proposal 2 Introduce RAT-Type as part of areaConfigForNeighbour along with frequency and cell list. | (c) | MDT conf | Seems enhancements |
|  |  | Proposal 3 Include areaConfigForNeighbour as part of LTE RRC specification associated to logged MDT. | (c) | MDT conf | Seems enhancements |
| [12] | R2-2001143 A2-like measurement results in Logged MDT Nokia, Nokia Shanghai Bell | Proposal 1: A Threshold defining measurement trigger quantity for event triggered Logged MDT is MDT specific (the threshold configuration does not reuse RRC Connected configuration by MeasTriggerQuantity) | (a) | MDT conf |  |
|  |  | Proposal 2: Logged MDT configuration for event-triggered measurements configure ‘A2like’ event is defined as follows: loggedEventA2 SEQUENCE { Threshold LogMeasTriggerQuantity, hysteresis Hysteresis, timeToTrigger TimeToTrigger } | (a) | MDT conf | Related to P1 above It is also proposed to discuss the definition of LogMeasTriggerQuantity, e.g. LogMeasTriggerQuantity ::= CHOICE { rsrp RSRP-Range, rsrq RSRQ-Range, sinr SINR-Range } |
| [13] | R2-2001144 CGI in MDT reports Nokia, Nokia Shanghai Bell | Proposal 1: plmn-Identity in the CGI-Info-Logging identifies the PLMN of the cell for the reported cellIdentity. | (a) | MDT report |  |
|  |  | Proposal 2: plmn-Identity field description in the CGI-Info-Logging is adopted as follows: plmn-Identity Identifies the PLMN of the reported cellIdentity as given by the first PLMN entry of the plmn-IdentityList sent in the PLMN-IdentityInfo of the cell’s acquired SIB1.. | (b) | MDT report | Related to P2 in [18] |
| [14] | R2-2001145 Location information encoding in NR MDT Nokia, Nokia Shanghai Bell | Proposal 1: RAN2 dissusses reasoning of locationTimeStamp, locationError, locationSource inclusion into detailed location information. | (d) | Location |  |
|  |  | Proposal 2: Change locationEstimate to locationCoordinates, which enables direct reference to the IE in LPP specification. | (d) | Location |  |
|  |  | Proposal 3: Change TS36.355 to TS37.355 in RRC. | (d) | Location |  |
| [15] | R2-2001146 Principle on Rareport entry logging Nokia, Nokia Shanghai Bell | Proposal: In Rel-16, the UE supports RA-report retaining according to Option a): - UE will stop logging RA info if all 8 entries is filled in RA report, and starts to count the duration. If not fetched within 48 hours, then the whole RA report will be deleted. | (b) | RA report |  |
| [16] | R2-2001366 Enhancements on logged MDT and accessibility measurements Huawei, HiSilicon | Proposal 1: The UE performs logged MDT only when camping on cells/frequencies in the area configuration. | (c) | MDT conf |  |
|  |  | Proposal 2: UE logs some information (e.g. at least indicator "camped on any cell" and time stamp) to inform the detection of "camped on any cell" state. | (c) | MDT report | Seems enhancements |
|  |  | Proposal 3: UE can log out-of-coverage without the need for dedicated configuration by the network, like RLF report and RRC establishment failure report. | (c) | MDT report | Seems enhancements |
|  |  | Proposal 4: In order to ensure the management based logged MDT do not overwrite the signalling based logged MDT: - Include the logged MDT type (i.e. the management based MDT or the signalling based MDT) in the logged MDT configuration - Include the logged MDT type in the RRCConnectionComplete/RRCResumeComplete/ RRCReconfigurationComplete/RRCRestablishmentCompelete | (c) | MDT report | Seems enhancements |
|  |  | Proposal 5: Introduce one failure type in the accessibility measurement reporting results to differentiate the failure of sending RRC setup request and the failure of sending RRC resume request. | (c) | MDT report | Seems enhancements |
| [17] | R2-2001367 Discusison on sensor measurement reporting Huawei, HiSilicon | Proposal 1: The principles of the validity time of UE location information are the baseline for sensor-based data reporting in NR MDT. | (C) | MDT report |  |
|  |  | Proposal 2: Add one NOTE for the requesting to attempt to have sensor reporting in 38.331 like the NOTES of location informations. | (d) | MDT report |  |
|  |  | Proposal 3: Add one section for the manner of sensor reporting in 37.320 like the reporting of location data. | (d) | MDT report |  |
| [18] | R2-2001436 Clarification on Cell ID Applied to NR MDT Samsung | Proposal 1: The IE CGI-InfoNR is not used to indicate a NR cell. | (a) | MDT report | Related to P1 and P2 in [13] |
|  |  | Proposal 2: Instead of the IE CGI-InfoNR, a new IE is introduced to indicate NR cell, and the following content for the new IE can be considered: - Option 1: first PLMN-Identity (in plmn-IdentityList) and CellIdentity in first PLMN-IdentityInfo IE of PLMN-IdentityInfoList - Option 2: plmn-IdentityList and CellIdentity in first PLMN-IdentityInfo IE of PLMN-IdentityInfoList - Option 3: first PLMN-Identity (in plmn-IdentityList) and CellIdentity in each PLMN-IdentityInfo of PLMN-IdentityInfoList - Option 4: plmn-IdentityList and CellIdentity in each PLMN-IdentityInfo of PLMN-IdentityInfoList - Option 5: List of PLMN identity in PLMN-IdentityInfoList matching with the PLMN identity (i.e. MDT PLMN list) configured in MDT configuration + CellIdentity corresponding to the PLMN identity - Option 6: Registered PLMN identity + CellIdentity | (b) | MDT report | Related to P2 in [13] |
| [19] | R2-2001438 Introducing Further Information Useful for NR MDT Samsung | Proposal 1: The on-demand SI acquisition related information is reported for the RACH optimization. FFS on details. | (c) | RA report | Seems enhancements |
|  |  | Proposal 2: Information indicating if UE is in SUL coverage is logged during Logged MDT operation. | (c) | RA report | Seems enhancements |
|  |  | Proposal 3: A new indicator is introduced in order to informing of detection of IDC problem as in Rel-13 LTE MDT. | (c) | MDT report | Seems enhancements |
| [20] | R2-2001439 On supporting Event-triggered Logged MDT Samsung | Proposal 1: In the out-of-coverage event, the entering condition is that UE enters Any Cell Selection at least. Proposal 1-1: RAN2 to study whether to log measurmenet results during Camped on Any Cell state | (b) | MDT report |  |
|  |  | Proposal 2: In the out-of-coverage event, the leaving condition is that UE enters Camped Normally state. | (b) | MDT report |  |
|  |  | Proposal 3: UE configured for the event-triggerd logged MDT logs the information on - last serving cell UE camped on before OOC happens, and - first serving cell after leaving the OOC | (a) | MDT report | It seems following LTE design |
| [21] | R2-2001441 RLF Report Enhancement Samsung | Proposal 1: Upon the expiry of T311, a flag is added into RLF report, which indicates that no suitable cell is found. | (a) | RLF report | Related to P5 in [3] |
| [22] | R2-2001443 Stage-3 Miscellaneous for NR MDT Samsung | Proposal 1: For the neighbour cells, UE logs the cell level measurement results only, but not SSB index and the measured result of the downlink beams. | (c) | MDT report |  |
|  |  | Proposal 2: RAN2 to agree one of the following options to log the number of failed connection setup attempts per cell: - Option 1: UE logs the ‘consecutive’ failures only at last cell where the failure happens - Option 2: UE logs the ‘consecutive’ failures per cell where the failure happens | (b) | CEF report | Related to P1 in [3] |