3GPP TSG-RAN WG2 Meeting #109bis-e	R2-2002846

20-30 April 2020

Agenda item:	6.2.1

Source: 	Qualcomm Incorporated

Title:  	NR-U RRC Open Issues List

Document for:	Discussion and decision

# Introduction

This document captures the issues identified during the following email discussion:

[Post109e#38][NR-U] RRC open issues (Qualcomm)

Address known stage-3 remaining open issues from 109e for 38.331 and update E-UTRAN (36.331) with relevant agreements and any open issues that need to be discussed

Capture identified NEW, if any, stage-3 corrections/issues.  Issues that have already been discussed and not pursued should not be brought up again.

A format similar to the one used in ASN.1 discussion was used to enable merging with the list in that discussion. The guidelines for reporting issues are as follows:

[Issue #]: U + 3 digits

[Class]: Shall be set to value 2 or 3.

- Trivial e.g. editorials, commas, colon, misspelling, missing/ double spaces, italics etc. 
See procedure for Class 0 and Class 1 issues below.

- Minor e.g. quite straightforward changes e.g. correction/ addition of specification references or sub-clauses.
See procedure for Class 0 and Class 1 issues below.

- ASN.1 session issue e.g. ASN.1 issue e.g. related to need codes, extensibility, alternative encoding, ASN.1/ guidelines, general protocol (consistency) issue or issue affecting more than one WI

- WI session issue i.e. an issue that is not purely ASN.1 but has some impact on functionality but only affecting a single WI.

# Open issues for NR_unlic-Core

| Issue number | Company | Subclause | IE name | Class | Description/ correction | Status |
| --- | --- | --- | --- | --- | --- | --- |
| U501 | Samsung | 5.3.10.3 |  | 0 | “; or” is missing before the second change | Accepted, corrected in CR1528. |
| U502 | Samsung | 6.3.1 | SIB2 | 2 | the field ‘ssb-PositionQCL-Common-r16’ should have separate double-square brackets, instead of reusing the existing one. | Accepted, however it was already corrected in 38.331 16.0.0. |
| U503 | Samsung | 6.3.2 | BWP-UplinkCommon | 2 | double-square brackets should be added after extension marker. | Accepted, however it was already corrected in 38.331 16.0.0. |
| U504 | Samsung | 6.3.2 | ConfiguredGrantConfig | 0 | The field name in the table should be corrected to ‘cg-minDFI-Delay’ (i.e. ‘-‘ is missing) | Accepted, corrected in CR1528 |
| U505 | Samsung | 6.3.2 | LogicalChannelConfig | 0 | typo: channellAccessPriority in the table should be channelAccessPriority (ucces l) | Accepted, corrected in CR1528 |
| U506 | Samsung | 6.3.2 | MeasObjectNR | 2 | the field rmtc-SubframeOffset-r16 does not have to be optional: it should be mandatory | Open. Rapporteur: This was optional for LAA. However, it was optional for inter-frequency case due to the following statement “For inter-frequency measurements, this field is optional present and if it is not configured, the UE chooses a random value as rmtc-SubframeOffset for measDuration which shall be selected to be between 0 and the configured rmtc-Period with equal probability.”. RAN2 should discuss whether the same applies to NR-U. |
| U507 | Samsung | 6.3.2 | MeasObjectNR | 0 | typo: the reference of rmtc-Periodicity is TS 38.215 (i.e. 5 is missing) | Accepted, however it was already corrected in 38.331 16.0.0. |
| U508 | Samsung | 6.3.2 | PUCCH-Config | 0 | The field name can be updated to ‘dl-DCI-triggered-UL-ChannelAccess-CP-ext-r16’ (i.e. to add ‘-‘ after acronym CP). | Move to ASN.1 |
| U509 | Samsung | 6.3.2 | PUSCH-Config | 0 | The field name can be updated to ‘ul-dci-triggered-UL-ChannelAccessCP-ext-CAPC-r16’ (i.e. to remove ‘-‘ after Access and add ‘-‘ after acronym CP). | Move to ASN.1 |
| U510 | Samsung | 6.3.2 | BWP-UplinkDedicated | 2 | The field ‘useInterlacePUCCH-PUSCH-r16’ should be possible to release: can be changed to BOOLEAN with need M. | Open. Rapporteur: It is not clear if waveform change dynamically is feasible or preferable. Will need RAN1 input. |
| U511 | Samsung | 6.3.2 | ConfiguredGrantConfig | 2 | (Many) new NR-U parameters can be grouped together, and can be SetupRelease with need M. Also, some fields (e.g. cg-RetransmissionTimer-r16) within the group can be mandatory to save bits for optional fields. | Move to ASN.1. Rapporteur: Note that retransmission may be adopted for licensed as an optional feature. |
| U512 | Samsung | 6.3.2 | ControlResourceSet | 2 | Need code for ‘rb-Offset’ should be ‘Need S’, and it has to refer RAN1 specification (to use value 0 in case of absence). | Accepted. Corrected in CR1528 |
| U513 | Samsung | 6.3.2 | MAC-CellGroupConfig | 2 | Both ‘lbt-FailureRecoveryConfig-r16’ and ‘schedulingRequestID-LBT-Scell-r16’ should use SetupRelease with need M | Move to ASN.1 |
| U514 | MTK | 6.3.1 | SIB2, SIB4 | 2 | Our understanding is that Q is always needed for measurements on an NR-U cell. However in RRC CR, it appears to be optional (perhaps because it’s not needed for licensed operation). To make it clear that Q always needs to be uccessfu for an NR-U cell/frequency, perhaps “ssb-PositionQCL-Common-r16” in SIB2, SIB4 and MeasObjectNR should be made “Cond Mandatory” (mandatory for NR-U) for NR-U. Without mandatory configuration value of Q, a NR-U UE might not be able to correctly combine measurements from detected SSBs, which would affect RRM. | Open |
| U515 | MTK | 6.3.2 | MeasObjectNR | 2 |  | Open Rapporteur: According to 38.213, it doesn’t have to be signalled for cell access as below. However, this should be discussed for measobject where the UE does not have to read neighbour MIB. An alternative for measobject is also to have a default value (8) when not signalled. and NSSBQCL is either provided by ssbPositionQCL-Relationship-r16 or, if ssbPositionQCL-Relationship-r16 is not provided, obtained from a MIB provided by a SS/PBCH block according to Table 4-1. for cell access |
| U516 | Apple | 6.3.2 | MeasObjectNR | 2 | Agree with MTK002 that the “ssb-PositionQCL-Common-r16” should be “conditional mandatory” in SIB2/SIB4 and MeasObjectNR for NR-U. | Same as U515 |
| U517 | Apple | 6.3.1 | SIB2, SIB4 | 3 | For the following statement “The UE expects that a bit at position k > ssb-PositionQCL-Relationship-16 is 0, and the number of actually transmitted SS/PBCH blocks equal to the number of 1’s in the bitmap.”, we understand the “acutally transmitted” is the “intentionally transmitted” as NW may not be able to transmit all those SSB(s) uccessfully due to LBT failure. Thus, we suggest to change “equal to” to “is no more than”. Note that if the same sentence can be copied to SSB-ToMeasure as we proposed in Issue A1a, the same change is expected. | Open Pending discussion of [Post109e#38] proposals |
| U518 | Ericsson | 6.3.2 | PUCCH-Config | 1 | ‘dl-DCI-triggered-UL-ChannelAccess-CP-ext-r16’ List of the combinations of CP extension and UL channel access type (See TS 38.212 [17], Clause 7.3.1). Propose to use generic name instead of listing all parameters configured by the corresponding IE. “list” missing in the field/IE name, compare also PUSCH-TimeDomainResourceAllocationList channelAccessConfigListForDCI1-1 Proposal: Configures a set of channel access parameter combinations, where each list entry indicates a combination of UL channel access type and CP extension. The actual combination for the PUCCH transmission can be indicated in DCI format 1_1 (see TS 38.212 [17], Table 7.3.1.2.2-6). | Accepted to add “list”, corrected in CR1528. Move the IE name change to ASN.1. |
| U519 | Ericsson | 6.3.2 | PUSCH-Config | 1 | ‘ul-dci-triggered-UL-ChannelAccessCP-ext-CAPC-r16’ List of the combinations of CP extension and UL channel access mode (See TS 38.212 [17], Table 7.3.1-2-35). See above; should be channel access type. channelAccessConfigListForDCI0-1 Configures a set of channel access parameter combinations, where each list entry indicates a combination of UL channel access type, CP extension, and channel access priority class (CAPC). The actual combination for the PUSCH transmission can be indicated in DCI format 0_1 (see TS 38.212 [17], Table 7.3.1.1.2-35). | Accepted. Corrected in CR1528. |
| U520 | Ericsson | 6.3.2 | ServingCellConfigCommon | 0 | discoveryBurst-WindowLength-r16 ENUMERATED {s0dot5, s1, s2, s3, s4, s5} - The field name should be without hyphens (discoveryBurstWindowLength-r16) - As indicated in the field description, the values are prvodied in ms. Thus, the prefixes should be “ms0dot5, ms1, ms2, ms3, ms4, ms5”. | Accepted. Corrected in CR1528. |
| U521 | Ericsson | 6.3.2 | SearchSpace | 1 | Propose to start with a general description of the field, then define what the bit positions correspond to, and then describe what a value means for a bit. Propose to consistently use RB instead of mixing PRB and RB. freqMonitorLocations Defines an association of the search space to multiple monitoring locations in the frequency domain and indicates whether the pattern configured in the associated CORESET is replicated to a specific RB set. Each bit corresponds to one RB set, and the rightmost bit corresponds to the lowest RB set in the BWP. Value 1 indicates that a frequency domain resource allocation replicated from the pattern configured in the associated CORESET is mapped to the RB set. For this RB set, the first RB of the frequency domain monitoring location confined within the RB set is aligned with {the first RB of the RB set + rb-Offset provided by the associated CORESET. | Accepted “implies” to “indicates” and added “value”; Corrected in CR1528. Rapporteur: RAN2 can’t decide if the suggested first sentence is technically correct; will need RAN1 confirmation. |
| U522 | Ericsson | 6.3.2 | MeasRSSI-ReportConfig | 1 | Propose to use self-descriptive field name that matches with the field description: ‘rssi-ThresholdForCO’ instead of ’channelOccupancyThreshold’ rssi-ThresholdForCO RSSI threshold which is used for channel occupancy evaluation. | Move to ASN.1. Rapporteur: Same name has been used in LAA. |
| U523 | Ericsson | 6.3.2 | PDCCH-Config | 1 | searchSpaceSwitchingTimer The duration in slots for monitoring search space group 1 before switching to the default search space group (see TS 38.213 [13], clause 11.5.2). Justification: see TS 38.213, clause 11.5.2 | Changed to the “value of the timer in slots…”. In CR1528. Rapporteur: suggestion here does not match the description in 38.213 and the current text is correct. |
| U524 | Ericsson | 6.3.2 | ConfiguredGrantConfig | 0 | Field description of channelAccessPriority-r16 should be moved to CG-COT-Sharing | Accepted. Corrected in CR1528. |
| U525 | Ericsson | 6.3.2 | AvailableRB-SetPerCell | 0 | - ASN.1 naming: should be plural: availableRB-SetsPerCell - propose to move to SlotFormatIndicator IE Justification: - Is only used in SlotFormatIndicator - More logical to nest it there because it contains positionInDCI used in the SlotFormatIndicator - Follows RAN1 recommendation | Accepted. Corrected in CR1528. |
| U526 | Ericsson | 6.3.2 | MeasObjectNR | 2 | rmtc-Frequency Indicates the center frequency of the pre-defined measurement bandwidth (see TS 38.215, clause 5.1.aa). - Propose to use ”frequency” in the field name to provide more descriptive field name and align with other parameters in the MeasObjectNR: ssbFrequency ARFCN-ValueNR refFreqCSI-RS ARFCN-ValueNR - Propose to clarify that the bandwidth is pre-defined in TS 38.215, clause 5.1.aa. - Clause 5.1.aa can also be included for rmtc-Periodicity and also for rmtc-SubframeOffset | Accepted. Corrected in CR1528 |
| U527 | Ericsson | 6.3.2 | SemiStaticChannelAccessConfig | 3 | period Indicates the period in ms from which the maximum channel occupancy time (COT) is derived and also the periodicity at which the COT can be initiated in of the semi-static channel access mode (see TS 37.213 [XX], clause 4.3). Value ms1 corresponds to 1 ms, value ms2 corresponds to 2 ms, value ms2dot5 corresponds to 2.5 ms, and so on. - IE description: remove redundant “mode2 (channel accces mode mode) - field description: propose to clarify as indicated above. Closing square bracket missing (also in other several other places, wrong closing bracket is used) | Accepted the reference correction, second sentence starting with “value”, and removal of redundant mode. Corrected in CR1528 Rapporteur RAN2 can’t decide if the suggested first sentence is correct. RAN1 should confirm |
| U528 | Ericsson | 6.3.2 | ra-ResponseWindow | 3 | ra-ResponseWindow Msg2 (RAR) window length in number of slots. The network configures a value lower than or equal to 10 ms when ra-ResponseWindow (without suffix) is configured and 40 ms when ra-ResponseWindow-r16 is configured (see TS 38.321 [3], clause 5.1.4). UE ignores the field if included in SCellConfig. If ra-ResponseWindow-r16 is signalled, the UE shall ignore the ra-ResponseWindow (without suffix). - The new parameter has been introduced exactly for extending the RAR window. Therefore, we propose to make the value range dependent on the used parameter version rather than link it to the spectrum | Open |
| U529 | Vivo | 5.7.3.1 |  | 2 | The purpose of SCG failure information procedure needs to be extended to inform NR MN about PSCell consistent UL LBT failure. | Accepted. Corrected in CR 1528 |
| U530 | Huawei | 6.3.2 | ControlResourceSet | 1 | Based on the field description of rb-Offset, the field should be OPTIONAL needS | Accepted. Corrected in CR 1528 |
| U531 | Huawei | 6.3.2 | DownlinkConfigCommonSIB | 1 | nrofPDCCHMonitoringOccasionPerSSB-InPO. More accurately, it should be "within a PO for an SSB" | Accepted. Corrected in CR1528 |
| U532 | Huawei | 6.3.2 | PUCCH-Config | 1 | rbSetIndex no need to say that this is the first interlace, just say the RB set allocated for a PUCCH resource | Accepted. Corrected in CR1528 |
| U533 | Huawei | 6.3.2 | ServingCellConfigCommon | 1 | semiStaticChannelAccessConfig no need for the sentence “The network configures this only when channelAccessType-r16 is set to semistatic.”. With the choice structure this is self-explanatory | Accepted. Corrected in CR1528 |
| U534 | Huawei | 6.3.2 | ServingCellConfig | 0 | channelAccess-Config-r16. Does not follow ASN.1 naming rules | Accepted. It was already corrected in 16.0.0 |
| U535 | Huawei | 6.3.2 | ServingCellConfigCommon | 0 | discoveryBurst-WindowLength-r16. Does not follow ASN.1 naming rules | Accepted. It was already corrected in 16.0.0 |
| U536 | Huawei | 6.3.2 | PhysicalCellGroupConfig | 1 | why there is a spare bit here? | Accepted to remove. Corrected in CR1528. |
| U537 | Huawei | 6.3.2 | PUCCH-Config | 1 | dl-DataToUL-ACK-r16 and dl-DataToUL-ACK cannot be configured at the same time | Accepted. Corrected in CR1528 |
| U538 | Ericsson | 6.3.2 | SearchSpace | 3 | Search space switching has to be configured also for Type-3 CSS. Currently only defined for USS. freqMonitorLocations is not limited to USS, either. | Open. Rapporteur: Problem seems to be valid. Need to move these IEs from USS to make it applicable to both USS and CSS. Checking with RRC rapporteur on an acceptable format. |
| U539 | Ericsson | 6.3.2 | SlotFormatIndicator | 3 | CO-DurationPerCell. It should be possible to configure multiple COT duration, for multiple cells. Currently, only one single cell can be configured. | Accepted. Corrected in CR1528 |
| U540 | Ericsson | 5.5.1 | RSSI/CO reporting | 3 | Add: the UE measures and reports on the defined measurement bandwidth and configured time domain measurement resources on the indicated frequency. | Open Rapporteur: Current text is based on LAA. RAN2#109e decided to keep that text. |
| U541 | Ericsson | 5.5.2.10a | RSSI measurement timing configuration | 3 | The length of the RMTC occasion is defined by measDuration. On theconfigured rmtc-Frequency, the UE shall perform RSSI and channel occupancy measurements within the configured RMTC occasion, where the measurement bandwidth is defined in [TS 38.215, clause 5.1.aa] and the time domain measurement resources are defined by the RMTC occasion. | Open Rapporteur: Current text is based on LAA. RAN2#109e decided to keep that text. |
| U542 | Ericsson | 6.3.1 | SIB3, SIB4 | 2 | Proposal: Use separate list for ssb-Position-QCL-r16. | Move to ASN.1. Rapporteur: This was proposed in RAN2#109e and was part of the email discussion but was not agreed. |
| U543 | Ericsson | 6.3.2 | BWP-UplinkCommon | 3 | Remove PUCCH formats 2 and 3 from the field description since only formats 0 and 1 are allowed prior to dedicated configuration. | Accepted. Corrected in CR1528 |
| U544 | Ericsson | 6.3.2 | ServingCellConfigCommon, ServingCellConfigCommonSIB | 3 | The issue is that Q can be defined in MIB (no field name), in ssb-PositionQCL-Common, and ssb-PositionQCL, depending on what the gNB provides in its system information or in the measurement object, which would make the field description unnecessarily complex. Therefore, referring to the Information Element instead of the field could make sense, maybe something like “k > Q, where Q is the configured value for SSB-PositionQCL-Relationship.” | Open |
| U545 | Huawei | 6.3.2 | ConfiguredGrantConfig | 3 | field description for cg-COT-SharingList is missing | Accepted. Corrected in CR1528 |
| U546 | Huawei | 6.3.2 | SearchSpace |  | List of search space group IDs which the search space set is associated with.description not clear. which bit corresponds to which group and what does 0/1 stands for should be explained. | Accepted. Corrected in CR1528. Added “The network configures at most 2 search space groups per PWP where the group ID is either 0 or 1.” |
| U547 | Huawei | 6.3.2 | PUCCH-Config | 3 | dl-DataToUL-ACK-r16 SEQUENCE (SIZE (1..8)) OF INTEGER (-1 ..15) what does the value "-1" means should be explained | Accepted. Corrected in CR1528. Added “The value -1 corresponds to “non-numerical value” for the case where the A/N feedback timing is not explicitly included at the time of scheduling PDSCH.” |