3GPP TSG-RAN2 Meeting #109-bis-e	R2-2003821

# Online, 20th April 2020 - 30th April 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.355 | CR | 0257 | rev | - | Current version: | 16.0.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed changeaffects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update missed out definition for Information Element NavModel-NavIC-KeplerianSet |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Reliance Jio |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LCS_NAVIC-core |  |  |  |  |  | Date: |  |  | 2020-04-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Populated the IE NavModel-NavIC-KeplerianSet definition |  |  |  |  |  |  |  |  |
| Summary of change: |  | GNSS assistance data elements section updated with deinition of IE NavModel-NavIC-KeplerianSet |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | ASN.1 compilation breaks. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------------Start of change----------------------

## 6.5	Positioning Method IEs

< Unchanged parts are omitted >

### 6.5.2	A-GNSS Positioning

< Unchanged parts are omitted >

#### 6.5.2.2	GNSS Assistance Data Elements

< Unchanged parts are omitted >

#### –	NavModel-NavIC-KeplerianSet

-- ASN1START

# NavModel-NavIC-KeplerianSet-r16	 ::= SEQUENCE {

# navic-Toe-r16			INTEGER (0 .. 65536),

# navic-URAI				INTEGER (0 .. 15),

# navic-W-r16				INTEGER (-2147483648..2147483647),

# navic-DeltaN-r16		INTEGER (-2097152.. 2097151),

# navic-M0-r16			INTEGER (-2147483648..2147483647),

# navic-OmegaDot-r16		INTEGER (-2147483648..2147483647),

# navic-E-r16				INTEGER (0..4294967295),

# navic-IDot-r16			INTEGER (-8192..8191),

# navic-APowerHalf-r16 	INTEGER (0.. 4294967295),

# navic-I0-r16			INTEGER (-2147483648..2147483647),

# navic-Omega0-r16		INTEGER (-2147483648..2147483647),

# navic-Crs-r16			INTEGER (-32768..32767),

# navic-Cis-r16			INTEGER (-32768..32767),

# navic-Cus-r16			INTEGER (-32768..32767),

# navic-Crc-r16			INTEGER (-32768..32767),

# navic-Cic-r16			INTEGER (-32768..32767),

# navic-Cuc-r16			INTEGER (-32768..32767),

# ...

}

-- ASN1STOP

NavModel-NavIC-KeplerianSet field descriptions

navic-Toe Parameter toe, time-of-ephemeris in seconds [38]. Scale factor 24 for NAVIC

navic-URAI This field iondicates index for the User Range Accuracy (in meters) value which is a one-sigma estimate of the user range errors in the navigation data for the transmitting satellite as described under clause 6.2.1.4 in [38]

navic-W Parameter , argument of perigee (semi-circles) [38]. Scale factor 2-31 semi-circles.

navic-DeltaN Parameter n, mean motion difference from computed value (semi-circles/sec) [38] Scale factor 2-41 semi-circles/second

navic-M0 Parameter M0, mean anomaly at reference time (semi-circles) [38] Scale factor 2-31 semi-circles.

navic-OmegaDot Parameter OMEGAdot, rate of change of right ascension (semi-circles/sec) [38] Scale factor 2-41 semi-circles/second

navic-E Parameter e, eccentricity [38] Scale factor 2-33.

navic-IDot Parameter Idot, rate of change of inclination angle (semi-circles/sec) [38] Scale factor 2-43 semi-circles/second.

navic-APowerHalf Parameter sqrtA, square root of semi-major Axis in (meters)½ [38] Scale factor 2-19 meters ½.

navic-I0 Parameter i0, inclination angle at reference time (semi-circles) [38] Scale factor 2-31 semi-circles.

navic-Omega0 Parameter OMEGA0, longitude of ascending node of orbit plane at weekly epoch (semi-circles) [38] Scale factor 2-31 semi-circles.

navic-Crs Parameter Crs, amplitude of the sine harmonic correction term to the orbit radius (meters) [38] Scale factor 2-4 meters

navic-Cis Parameter Cis, amplitude of the sine harmonic correction term to the angle of inclination (radians) [38] Scale factor 2-28 radians

navic-Cus Parameter Cus, amplitude of the sine harmonic correction term to the argument of latitude (radians) [38] Scale factor 2-28 radians

navic-Crc Parameter Crc, amplitude of the cosine harmonic correction term to the orbit radius (meters) [38] Scale factor 2-4 meters

navic-Cic Parameter Cic, amplitude of the cosine harmonic correction term to the angle of inclination (radians) [38] Scale factor 2-28 radians

navic-Cuc Parameter Cuc, amplitude of the cosine harmonic correction term to the argument of latitude (radians) [38] Scale factor 2-28 radians

----------------------------End of change----------------------