3GPP TSG-RAN WG2 Meeting #109bis-e	R2-2003804

Electronic, 20 April – 30 April 2020

Source: 	Session Chair (InterDigital)

Title:	Meeting Report for NR-U, Power Savings and 2-step RACH

Organizational:

- LSs – contact companies should flag LSs that need presenting.  Otherwise we will directly note them

- Only Email discussions and summary discussions will be treated during e-meetings (indicated clearly in the meeting notes)

- All organization emails and notes will be shared over the following email discussion throughout the two meeting weeks:

- [AT109bis-e][500] Organizational Diana - NR-U, 2-step RACH, Power Savings

Scope:

    - Share plans for the meetings and list of ongoing email discussions for the sessions related to NR-U, 2-step RACH, and power saving

    - Share meetings notes and agreements for review and endorsement

Email discussions:

- [AT109bis-e][501][NR-U] CP Open and ASN.1 Issues (Qualcomm)

Scope:

    - Identify/Summarize all remaining/identified CP and ASN.1 issues

Intended outcome:

    - Set of proposals to agree by email

    - CR capturing agreements from week1 and then week2

Deadline for providing comments:

    - Companies input:  April 22nd

    - Rapporteur proposals: April 23rd

    - CR capturing agreements: April 27th

- [AT109bis-e][502][NR-U] UP Open Issues (InterDigital, Ericsson)

Scope:

    - Identify/Summarize all remaining/identified UP issues

Intended outcome:

    - Set of proposals to agree by email (InterDigital)

    - CR capturing agreements from week1 and then week2  (Ericsson)

Deadline for providing comments:

    - Companies input:  April 22nd

    - Rapporteur proposals: April 23rd

    - CR capturing agreements: April 27th

- [AT109bis-e][503][2s RA] UP Open Issues (ZTE)

Scope:

    - Identify/Summarize all remaining/identified UP issues

Intended outcome:

    - Set of proposals to agree by email

    - CR capturing agreements from week1 and then week2

Deadline for providing comments:

    - Companies input:  April 21st

    - Rapporteur proposals: April 22nd to be discussed in week1 discussion.

    - CR capturing agreements: April 27th

- [AT109bis-e][504][NR-U] CP/UE assistance Open and ASN.1 Issues (Mediatek)

Scope:

    - Identify/Summarize all remaining/identified CP UE assistance and ASN.1 issues

Intended outcome:

    - Set of proposals to agree by email

    - CR capturing agreements from week1 and then week2

Deadline for providing comments:

    - Companies input:  April 22nd

    - Rapporteur proposals: April 23rd

    - CR capturing agreements: April 27th

- [AT109bis-e][505][PowSav] RRM Open Issues (CATT, Vivo)

Scope:

    - Identify/Summarize all remaining/identified RRM issues (continuation of pre-meeting email discussion)

Intended outcome:

    - Set of proposals to agree by email (CATT)

    - CR capturing agreements from week1 and then week2 (Vivo)

Deadline for providing comments:

    - Companies input:  April 22nd

    - Rapporteur proposals: April 23rd

    - CR capturing agreements: April 27th

- [AT109bis-e][506][PowSav]  DCP Open Issues (InterDigital, Huawei)

Scope:

    - Identify/Summarize all remaining/identified DCP issues (continuation of pre-meeting email discussion)

Intended outcome:

    - Set of proposals to agree by email (InterDigital)

    - CR capturing agreements from week1 and then week2 (Huawei)

Deadline for providing comments:

    - Companies input:  April 22nd

    - Rapporteur proposals: April 23rd

    - CR capturing agreements: April 27th

- [AT109bis-e][507][2s RA] CP and ASN.1 Issues (Ericsson)

Scope:

    - Identify/Summarize all remaining/identified CP and ASN.1 issues

Intended outcome:

    - Set of proposals to agree by email

    - CR capturing agreements from week1 and then week2

Deadline for providing comments:

    - Companies input:  April 22nd

    - Rapporteur proposals: April 23rd

    - CR capturing agreements: April 27th

- [AT109bis-e][508][NR-U] CR to 36.331 for NR-U (Qualcomm)

Scope:

    - Review running 36.331 CR

Intended outcome:

    - Agreed CR for 36.331

Deadline for providing comments – April 24th

# 6	Rel-16 NR Work Items

## 6.2	NR-based Access to Unlicensed Spectrum

(NR_unlic-Core; leading WG: RAN1; REL-16; started: Dec 18; target; June 20; WID: RP-192926; SR; RP-200459, Further prioritization guidance in RP-191581). Documents in this agenda item will be handled in a break out session.

Time budget: 3 TU

Tdoc Limitation: 3

### 6.2.1	General

Including incoming LSs, rapporteur inputs, etc.
Contributions in this AI are reserved for WI rapporteur inputs and/or spec rapporteur inputs and do not count towards the tdoc limits. All comments related to 38.300, 38.304 should be given to Ozcan, spec rapporteur.   Qualcomm will produce a document with the received issues and update the CR directly

Including  [Post109e#40][NR-U] UE capabilities (Qualcomm, Vivo)

No contributions are expected for UE capabilities.  Please provide your input to the email discussion.  Vivo is expected to produce first draft of 38.304

R2-2002506	LS to RAN2 on NR-U related changes for 38.300 running CR (R1-2001300; contact: Qualcomm)	RAN1	LS in	Rel-16	NR_unlic-Core	To:RAN2

=>	Noted

R2-2002513	LS on SSB index and candidate SSB index for NR-U (R1-2001357; contact: Samsung, Charter Communications)	RAN1	LS in	Rel-16	NR_unlic-Core	To:RAN2, RAN4

=>	Noted

R2-2002514	LS on NR-U enhancements to initial access procedures (R1-2001375; contact: Charter Communications)	RAN1	LS in	Rel-16	NR_unlic-Core	To:RAN2

=>	Noted

To be treated

R2-2002516	Reply LS on consistent Uplink LBT failure detection mechanism (R1-2001397; contact: Nokia)	RAN1	LS in	Rel-16	NR_unlic-Core	To:RAN2

=>	Noted

R2-2003008	Reply LS on consistent Uplink LBT failure detection mechanism	Nokia	LS out	Rel-16	NR_unlic-Core	To:RAN1	Late

-	Huawei thinks that we may need to consider case by case

-	Nokia thinks that we should always indicates if configured.  Qualcomm thinks that the LS is clear already, if configured we always indicate and if not configured the cases are already in the LS.

=>	The LS is revised in R2-2003965

R2-2003965	Reply LS on consistent Uplink LBT failure detection mechanism	Nokia	LS out	Rel-16	NR_unlic-Core	To:RAN1	Late

=>	The LS is approved in  R2-2003971

R2-2002530	LS on UL LBT failure recovery for the target cell (R4-2002282; contact: Ericsson)	RAN4	LS in	Rel-16	NR_unlic-Core	To:RAN2	Cc:RAN1

=>	Noted

To be treated at the end of the session

R2-2002844	Report of Post109e#40][NR-U] UE capabilities 	Qualcomm Incorporated	 report

-	Mediatek is concerned that if LBT consistent failure is option then we’d have to solve the issue.    ZTE has the same view.  Samsung also agrees.

=>	Noted

Agreements

1	An indication from PHY to MAC on LBT failure or success should be supported by all NR-U UEs. How this can be grouped with other essential PHY layer NR-U capabilities can be discussed after RAN1 progress on those.

2	Detecting consistent LBT failure and recovery is optional and as a baseline the UE capability is per UE.  Whether the signalling is per band will depend on RAN1 discussion.

3	As a baseline, the capability for LBT detection and recovery capability applies to all cells (SpCell and SCells).

4   RAN2 should not further discuss the granularity of RSSI/CO measurements until RAN1 discussion concludes.

5	RAN2 should not further discuss the granularity of configured grant autonomous transmission support until RAN1 discussion concludes.

6 	As a baseline, no separate capability is needed for sharing of HARQ processes among multiple configured grants with retransmission timer.

7 	As baseline, it is mandatory to support monitoring the last two bits of SFN for RACH operation in shared spectrum.

8	When msg2 on shared spectrum and msgB, the gNB signals the last 2 bits of SFN

10 	Multiple PDCCH monitoring occasions for PO is only used for NR operation with shared spectrum channel access.

11 	As a baseline, white lists for neighbour cells broadcast in SIB are only applicable to NR operation with shared spectrum channel access.

12	From RAN2 point of view, retransmission timer for configured grant is used for only NR operation with shared spectrum channel access.

R2-2002586	Running CR to 38.306 on Introducing UE Capability for NR Shared Spectrum	vivo	CR	Rel-16	38.306	16.0.0	0266	-	B	NR_unlic-Core	Withdrawn

=>	Moved to email discussion

R2-2002584	Running CR to 38.306 on Introducing UE Capability for NR Shared Spectrum	vivo	draftCR	Rel-16	38.306	16.0.0	B	NR_unlic-Core

=>	Not treated

### 6.2.2	User plane

Including [Post109e#39][NR-U] MAC open issues (Ericsson)

Contributions related to issues addressed by the email discussions should be avoided and are discouraged for this AI.

All identified critical open issues should be provided to the rapporteur via email discussion Post109e#39 and new contributions on those topics are discouraged.  Contributions should be reserved for more complicated issued.

No individual company CRs should be submitted

To be treated

R2-2003411	Post109e#39 NR-U MAC open issues	Ericsson	discussion	Rel-16	NR_unlic-Core

=>	Revised in R2-2003951

R2-2003951	Post109e#39 NR-U MAC open issues	Ericsson	discussion	Rel-16	NR_unlic-Core

5: Consistent LBT failure is cancelled if lbt-FailureRecoveryConfig IE is reconfigured for any Serving Cell.

-	LG thinks that it is too aggressive to always cancel and suggest to cancel if LBT instance max count is greater than old one.   Ericsson thinks that this is a rare event and we don’t need to do anything.

-	Nokia asks if LBT failure is cancelled for all cells or just the ones that have been reconfigured.  Qualcomm explains that the configuration is per MAC and not per Cell.

6: The MAC entity may stop, if any, ongoing Random Access procedure due to a pending SR for consistent LBT failure, which has no valid PUCCH resources configured, for the Serving Cell that triggered the consistent LBT failure

-	Fujitsu wonders about the first two conditions.  Huawei sympathizes with Fujitsu. LG points out that they have a paper to capture general procedures that is being treated in  main session and detailed discussion don’t have to take place.

-	Nokia prefers to not go into this details and it cannot be implemented this way.

=>	Noted

Agreements 1: Remove the addition of a third type of retransmissions for Type 1 and Type 2 configured grant when cg_RetransmissionTimer is configured in the first paragraph of 5.8.2. 2: Change the last paragraph in 5.8.2 from Retransmissions are done by: - repetition of configured uplink grants; or - receiving uplink grants addressed to CS-RNTI; or - retransmission on configured uplink grants. To: Retransmissions use: - repetition of configured uplink grants; or - received uplink grants addressed to CS-RNTI; or - configured uplink grants with cg-RetransmissionTimer configured. 3: Remove “the active UL BWP of” and add “if” in 5.21.2 1> if consistent LBT failure is triggered and not cancelled in the active UL BWP of the SpCell; and 1> if the Random Access procedure is considered successfully completed (see clause 5.1) in the SpCell: 4: Reset the LBT_COUNTER when a consistent LBT failure is cancelled in 5.21.2. 5: Consistent LBT failure is cancelled if lbt-FailureRecoveryConfig IE is reconfigured. 6: FFS to be moved to email discussion - The MAC entity may stop, if any, ongoing Random Access procedure due to a pending SR for consistent LBT failure, which has no valid PUCCH resources configured, for the Serving Cell that triggered the consistent LBT failure, if: - an RRC (re-)configuration for BWP switching is received for this Serving Cell; or - a PDCCH for BWP switching is received for this Serving Cell; or - this Serving Cell is an SCell that is deactivated (see clause 5.9); or - a MAC PDU is transmitted using a UL grant other than a UL grant provided by Random Access Response, regardless of LBT failure indication from lower layers, and the MAC PDU includes an LBT failure MAC CE that indicates consistent LBT failure for this Serving Cell.

R2-2003409	Corrections of NR-U in 38.321	Ericsson	CR	Rel-16	38.321	16.0.0	0726	-	F	NR_unlic-Core

-	LG is concerned that a description is too long

=>	The CR will be updated, delete proposal 6 and further comments can be provided via email discussions

R2-2003952   Outcome of [AT109bis-e][502][NR-U] UP Open Issues  Interdigital

=>	Noted

Set of proposals with no clear majority – to be discussed online

Proposal 1: Update the agreement on incrementing the preamble counter to:

The PREAMBLE_TRANSMISSION_COUNTER is not increased if the preamble is not transmitted due to LBT failure and lbt-FailureRecoveryConfig is configured.

-	Mediatek doesn’t like this compromise as we get into different combination and complicated with testing.

-	Qualcomm indicates that this is for connected mode and they would like this to avoid IOT issues, otherwise we will leave it up to UE implementation.  Qualcomm thinks that we made this agreement under the assumption that LBT recovery would be there.  Mediatek explains that we have LBT recovery.  Qualcomm is concerned that we’d have to mandate it from the NW side and there will be problems if the network doesn’t configure it.

-	LG has a similar concern as Mediatek and the condition is not needed.

-	Nokia thinks from the network point of view this is problematic as there would be UEs with different behaviors.  From NW point of view it would be easier to support.

-	Lenovo and ZTE agree with Qualcomm that there is some issues and this is a good compromise.  ZTE thinks that we can even assume a default configuration.

-	ZTE thought this feature is mandatory.

-	Ericsson would have liked to have it mandatory and if not possible this is a good compromise.

-	Intel thinks this is a good compromise

Agreements 1 UE PHY provides LBT failure indications for all uplink transmissions when lbt-FailureRecoveryConfig is not configured (same behavior as when it is configured). Include this in the reply LS to RAN1 2 Change the modeling of capturing LBT failure indication in TS 38.321 per draft CR R2-2003050, by removing “regardless of LBT failure indication from lower layers” and not tying “transmission” to LBT success. 3 LCID set1 (below 64) is used for LBT failure MAC CE. 4 The UE behavior for type-2 configured grant activation/deactivation agreed for IIoT - including joint de-activation- to is extended for NR-U UEs. 5 Reply to RAN4 that UL LBT failure detection/recovery is applicable per current specifications to RA in R15-based handover, R15 SN addition/change, and PSCell addition, given the UE is in connected mode. LBT failure detection/recovery is not applicable per current specifications in RRC setup, resume, re-establishment, or release with redirection, as the UE does not have lbt-FailureRecoveryConfig during those procedures 6 Reply to RAN4 that no enhancements are planned in R-16 for UL LBT failure detection and recovery during handover, RRC setup, resume, re-establishment, or release with redirection. However, RAN2 will check if there is any technical issues with DAPS and CHO. 7 Update the agreement on incrementing the preamble counter to: The PREAMBLE_TRANSMISSION_COUNTER is not increased if the preamble is not transmitted due to LBT failure and lbt-FailureRecoveryConfig is configured, otherwise it is increased. Sam applies for 2-step RA. 8 Update the agreement on incrementing the SR counter to: The SR_COUNTER is not increased if the SR is not transmitted due to LBT failure and lbt-FailureRecoveryConfig is configured, otherwise it is increased 9 The MAC entity may stop an ongoing Random Access procedure initiated by a pending SR triggered by consistent LBT failure on at least one Scell, if: - The SCell(s) that triggered the corresponding SR are deactivated (see clause 5.9); or - a MAC PDU is transmitted using a UL grant other than a UL grant provided by Random Access Response, and the PDU includes an LBT failure MAC CE that indicates consistent LBT failure for the Serving Cell that triggered the SR. Note: it is up to the NR-U TS 38.321 rapporteur how to capture this, considering exact wording and the outcome of [AT109bis-e][060][NR16] RACH stopping. 10 We keep ASN.1 as is, capture in TS 38.331 that harq-ProcID-Offset2 and cg-RetransmissionTimer should not be configured simultaneously for a certain configured grant.

R2-2003966	 Reply LS to RAN4 on LBT failure recovery (R4-2002282)	InterDigital

=>	The LS is revised in R2-2003972

R2-2003972	 Reply LS to RAN4 on LBT failure recovery (R4-2002282)	InterDigital

=>	The LS is approved in R2-2003973

Will not be treated

R2-2002848	Remaining critical issues for LBT failures	Qualcomm Incorporated	discussion

R2-2002582	Clarification on the LBT Failure Indication	vivo	discussion

R2-2002583	Discussion on the UE Processing Time for Autonomous Retransmission	vivo	discussion

R2-2002613	Clash between NR-U and IIoT for the configured grant	Samsung	discussion	Rel-16	NR_unlic-Core

R2-2002614	Prioritization between initial TX and re-TX on CG in NR-U	Samsung	CR	Rel-16	38.321	16.0.0	0706	-	F	NR_unlic-Core

R2-2002837	Discussion incoming RAN1 LS on LBT failure indication	OPPO	discussion	Rel-16	NR_unlic-Core

R2-2002931	Stopping ongoing Random Access procedure	LG Electronics Inc.	discussion	Rel-16	NR_unlic-Core

R2-2003004	Remaining issue on 2-step random access in NR-U	Huawei, HiSilicon	discussion	Rel-16	NR_unlic-Core

R2-2003005	Discussion on the MAC CE for NR-U	Huawei, HiSilicon	discussion	Rel-16	NR_unlic-Core

R2-2003006	Discussion on PDCCH group switching for NR-U	Huawei, HiSilicon	discussion	Rel-16	NR_unlic-Core

R2-2003410	UEs not supporting gap-less msgA transmission	Ericsson	discussion	Rel-16	NR_unlic-Core, NR_2step_RACH-Core

R2-2003498	MsgA PUSCH LBT failure impact	CMCC	discussion	Rel-16

Not complying to guidance

R2-2003031	Flushing HARQ buffer of the pending HARQ process in NR-U	LG Electronics Polska	CR	Rel-16	38.321	16.0.0	0717	-	F	NR_unlic-Core

R2-2003050	Draft CR on LBT failure handling in MAC	Nokia, Nokia Shanghai Bell	draftCR	Rel-16	38.321	16.0.0	NR_unlic-Core

### 6.2.3	Control plane

Including [Post109e#38][NR-U] RRC open issues (Qualcomm)

Contributions related to issues addressed by the email discussions should be avoided and are discouraged for this AI.

All identified critical open issues should be provided to the rapporteur via email discussion Post109e#38 and new contributions on those topics are discouraged.  Contributions should be reserved for more complicated issued.

No individual company CRs should be submitted

To be treated

R2-2002843	Report of [Post109e#38][NR-U] RRC open issues 	Qualcomm Incorporated	report	Late

=>	Noted

Agreements 1 Introduce the field descriptions communicated by RAN1 with the following changes: - Use inOneGroup instead of mediumBitmap in ServingCellConfigCommonSIB [verify this] - Include the statement in “The UE expects that a bit at position k > ssb-PositionQCL-Relationship-16 is 0, and the number of actually transmitted SS/PBCH blocks is not larger the number of 1’s in the bitmap.” in SSB-ToMeasure - Use “For operation in licensed spectrum” instead of “without shared spectrum channel access” - Use “leftmost” instead of “MSB” 2: No other changes are introduced to RRC to address the recommendations and agreements in RAN1 LS (R2-2001357). 3. Introduce the following changes: - Replace ffsValue with 64 in: - co-DurationList-r16 SEQUENCE (SIZE(1..ffsValue)) OF CO-Duration-r16 -- FFS size upper limit 64 - Replace ffsValue below with 1120 to support 20ms duration (the new upper limit is changed from 560 to 1120 as it is needed for SCS 60Khz): - CO-Duration-r16 ::= INTEGER (0..ffsValue) -- FFS upper limit 560 The structure discussion and possible unification is moved to ASN.1 discussion. Capture this as open issue. - Add field description for CO-Duration 4. Introduce the following changes: - Replace ffsValue below with 80 (maximum needed for 20ms with SCS of 60khz): - searchSpaceSwitchingTimer-r16 INTEGER (1..ffsValue) - Put in the field description of searchSpaceSwitchingTimer that “For 15 kHz SCS, {1..20} are valid. For 30 kHz SCS, {1..40} are valid. For 60kHz SCS, {1..80} are valid. Note that this is in slots as used in 38.213. 5 Introduce the following in the field description of cp-ExtensionC2, cp-ExtensionC3: Configures the cyclic prefix (CP) extension (see TS 38.211 [16], clause 5.3.1). For 15 kHz SCS, {1..28} are valid for both cp-ExtensionC2 and cp-ExtensionC3. For 30 kHz SCS, {1..28} are valid for cp-ExtensionC2 and {2..28} are valid for cp-ExtensionC3. For 60 kHz SCS, {2..28} are valid for cp-ExtensionC2 and {3..28} are valid for cp-ExtensionC3. 6 Introduce the following changes to RRC: - Introduce a new IE in RMTC-Config called ref-SCS-CP with the values of {15 kHz, 30 kHz, 60 kHz-NCP, 60 kHz-ECP} - Remove the Editor’s Note on L3 filtering for RSSI - Wait for RAN4 conclusion on actual values for rssi-Result-r16 and channelOccupancyThreshold-r16 before introducing the indices corresponding to RAN4 table 7: Introduce the following changes in RRC: - Change the value range for cg-nrofSlots-r16 to {1,2, ..., 40} - Change the value range for cg-minDFIDelay-r16 to ENUMERATED {sym7, sym1x14, sym2x14, sym3x14, sym4x14, sym5x14, sym6x14, sym7x14, sym8x14, sym9x14, sym10x14, sym11x14, sym12x14, sym13x14, sym14x14, sym15x14, sym16x14} and introduce additional text in the field description as: cg-minDFIDelay Indicates the minimum duration (in unit of symbols) from the ending symbol of the CG-PUSCH to the starting symbol of the DFI carrying HARQ-ACK for that PUSCH. UE assumes HARQ-ACK is valid only for PUSCH transmissions ending before n - cg-minDFIDelay-r16, where n is the time corresponding to the beginning of the start symbol of the DFI (see TS 38.213 [13], clause 10.3). The following minimum delay values are supported depending on the configured subcarrier spacing [symbols]: 15 kHz: 7, m*14, where m={1, 2, 3, 4} 30 kHz: 7, m*14, where m={1, 2, 3, 4, 5, 6, 7, 8} 60 kHz: 7, m*14, where m={1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16} - Change the value ranges as follows: cg-StartingPartialBW-InsideCOT-r16 and cg-StartingPartialBW-OutsideCOT-r16 to to INTEGER (0..6) cg-StartingFullBW-InsideCOT-r16 and cg-StartingFullBW-OutsideCOT-r16r to SEQUENCE (SIZE (1..ffsValue)) OF INTEGER (0..6) - Change the ffsValue in betaOffsetCG-UCI-r16 value range to 31 9: Add the following ASN.1 to introduce multiple uplink grants: In PUSCH-Config: pusch-TimeDomainAllocationList-r16 SetupRelease { PUSCH-TimeDomainResourceAllocationList-r16 } The new PUSCH-TimeDomainResourceAllocationList-r16: PUSCH-TimeDomainResourceAllocationList-r16 ::= SEQUENCE (SIZE(1..maxNrofUL-Allocations)) OF PUSCH-TimeDomainResourceAllocation-r16 PUSCH-TimeDomainResourceAllocation-r16 ::= SEQUENCE { k2-r16 INTEGER (0..32) OPTIONAL, -- Need S multiplePUSCH-Allocations-r16 SEQUENCE (SIZE(2..maxNrofMultiplePUSCHs-r16)) OF SinglePUSCH-TimeDomainResourceAllocation-r16 } SinglePUSCH-TimeDomainResourceAllocation-r16 ::= SEQUENCE { mappingType ENUMERATED {typeA, typeB}, startSymbolAndLength INTEGER (0..127) } 10: Add to section 6.5 the UE may stop monitoring the PDCCH occasions for paging when this bit is set as defined in 304. 11: No additional values are introduced for nrofPDCCHMonitoringOccasionPerSSB-r16 and the Editor’s Note on this can be removed. 12: The following additional values are introduced for lbt-FailureInstanceMaxCount-r16: 64 and 128. No additional values are added for lbt-FailureDectectionTimer-r16 and the Editor’s Note on this can be removed. Check with ASN.1 rapporteur.

R2-2002846	NR-U RRC Open Issues List	Qualcomm Incorporated	discussion	Late

-	Ericsson thinks that U523 is incorrect description

-	Intel sympathizes with U542.  Huawei thinks that the intention and information is the same so it is a matter of signalling structure.  Qualcomm thinks that it is a matter of overhead.

=>	Noted

Agreements:

1.  close U521

2.  close U522 – the name will remain as is

3.  rapporteur will check 523 and correct accordingly

4.  U527 can be closed and RAN1 can check and tell us if there is something wrong

5.  U540 and U541 can be closed

6.  U518 is closed

R2-2003953   Outcome of [AT109bis-e][501][NR-U] CP Open and ASN.1 Issues (Qualcomm)

The UE applies default guard band when signalling is absent and no guard band is signalled by explicit IE.

-	Ericsson thinks we can’t agree to this as it is against ASN.1 convention.  Nokia disagrees.

=>	Noted

Agreements - (Issue U506 and U557) Extend RSSI/CO measurements to inter-frequency (as in LTE LAA). The IE rmtc-SubframeOffset-r16 is optional for inter-frequency (as in LTE LAA). - (Issue U510) Keep the ASN.1 for useInterlacePUCCH-PUSCH-r16 as ENUMERATED {true} with Need M. No changes to the 38.331 is needed. - FFS pending RAN1 agreements (Issue U515) The IE for signaling of Q in measurement object is kept Optional. It is added to the field description that the UE applies default value 8 when not signaled. - (Issue U528) No changes to the field description of ra-ResponseWindow is needed. - (Issue U538) Move the IEs searchSpaceGroupIdList-r16 and freqMonitorLocations-r16 from SearchSpace to SearchSpace-v16xy in order to allow search space switching for Type-3 CSS. - (Issue U544) Proposal 6b: If Proposal 6 is not agreed, introduce the following changes in order to address U544: - For ServingCellConfigCommon, add “If ssb-PositionQCL is configured” in the field description of ssb-PositionsInBurst before “the UE expects that a bit at position k > ssb-PositionQCL is 0” - For ServingCellConfigCommonSIB, modify field description of ssb-PositionsInBurst as follows: “The UE assumes that a bit at position k > NSSBQCL is 0, where NSSBQCL is obtained from MIB as specified in TS 38.213 [13], clause 4.1” - (Issue U548) No changes are made to the field description of measRSSI-ReportConfig - (Issue U555) Introduce text for setting failureType as scg-lbtFailure in 5.7.3.5 (corresponding to NR-NRU DC) - (Issue 558) No changes are made to betaOffsetCG-UCI-r16 since it can already be configured dynamically. - (Issue 559) The IE ChannelAccessMode is kept in ServingCellConfigCommon without any changes. - (Issue 561) No changes are made to the structure of IE searchSpaceGroupIdList-r16 - Agree to the editorial changes suggested in: U563: Change “neighbour” to “this serving cell” for field description of ssb-PositionQCL in ServingCellConfigCommon U564: Change semistatic to semiStatic 13 [RAN2 will capture the RAN1 agreement] (Issue 560) The UE applies default guard band when signalling is absent and no guard band is signalled by explicit IE [CB to this if there is a technical issue that companies agree with]

R2-2002847	Miscellaneous corrections for NR-U	Qualcomm Incorporated	CR	Rel-16	38.331	16.0.0	1528	-	F	NR_unlic-Core	Late

=>	The CR is endorsed as a baseline and further agreements will be captured and reviewed over email discussion

R2-2002845	E-UTRAN and NR-U interworking	Qualcomm Incorporated	discussion

=>	Noted

Agreements

1: Introduce RSSI/CO measurement and reporting of NR-U frequencies in E-UTRAN in order to improve E-UTRAN to NR-U handover (depending on whether inter-freq measurements are agreed)

2: Introduce white-list of neighbour NR-U cells in E-UTRAN (SIB24) – 16 NR-U cells just like in NR.

3: Introduce a new cause value scg-lbtFailureNR in SCGFailureInformationNR in 36.331.

4: FFS based on NR discussion – whether Per-cell Q value can be broadcasted in LTE SIB24 for NR-U neighbour cells.

R2-2003414	Mobility to NR operating with shared spectrum access	Qualcomm Incorporated	CR	Rel-16	36.331	16.0.0	4263	-	B	NR_unlic-Core

=>	The CR is moved to email discussion for approval  and the agreements based on R2-2002845 should be captured

Will not be treated

R2-2002615	Applicability of NR-U features to licensed carrier	Samsung	discussion	Rel-16	NR_unlic-Core	R2-2000535

R2-2002719	On Q-values for Measurements in NR-U	Mediatek Inc.	discussion

R2-2002910	Description on Short Message in TS38.331	LG Electronics Inc.	discussion	Rel-16

R2-2002966	Addressing RAN1 and RAN4 questions on LBT failure configuration	ZTE Corporation, Sanechips	discussion

R2-2002967	Draft-Reply LS on consistent UL LBT failure detection mechanism	ZTE Corporation, Sanechips	response	Late

R2-2002968	Draft-Reply LS on LS on UL LBT failure recovery for the target cell	ZTE Corporation, Sanechips	response

R2-2003041	Remaining control plane issues	Ericsson	discussion	NR_unlic-Core	R2-2000337

R2-2003407	LS reply to RAN4 on UL LBT failure recovery for the target cell	Ericsson	LS out	Rel-16	NR_unlic-Core	To:RAN4	Cc:None	Late

R2-2003408	UL LBT failure recovery for target cell	Ericsson	discussion	Rel-16	NR_unlic-Core

## 6.11	UE Power Saving in NR

(NR_UE_pow_sav-Core; leading WG: RAN1; REL-16; started: Mar 19; target; Jun 20; WID: RP-200494; SR: RP-200237, See also guidence in RP-192326). Documents in this agenda item will be handled in a break out session. NOTE: "SCell dormancy" like behaviour will be discussed in MR-DC WI.

Time budget: 1 TU

Tdoc Limitation: 2

### 6.11.1	Organisational

Including incoming LSs, running TS, rapporteur inputs, etc

NOTE: any stage 3 identified issues with MIMO configurations should be provided to 38.331 rapporteur (Mediatek)

Contributions in this AI are reserved for WI rapporteur inputs and/or spec rapporteur inputs and do not count towards the tdoc limits. Including outcome of email [Post109e#42][PowSav] UE capabilities (Intel)

No contributions expected for UE capabilities.  Please provide your input to the email discussion.  Intel is expected to produce first draft of 38.304

To be treated at the end of session

R2-2002601	Report of email discussion [Post109e#42][PowSav] UE capabilities	Intel Corporation	discussion	Rel-16	NR_UE_pow_sav

=>	Noted

Proposal 10.	[8 or 7 out of 10] For maxMIMO-LayerPreference, and maxBW-Preference, its characteristics are defined as “FR1-FR2 DIFF: No” (to the characteristic associated to FR1/FR2 equally applicable for FR1/FR2).

-	Intel thinks that there is no strong motivation to differentiate between FR1 and FR2.  Qualcomm explains that FR1 and FR2 operate in different HW so it is good to have separate capabilities.   Vivo, Apple agrees with Qualcomm and thinks that the UE can report different preferences.

-	Ericsson thinks that there may be some use for the network as well.

Agreements

1	A new UE capability (referred e.g. as drx-Preference) is defined to indicate its preference on DRX parameters for power saving in RRC_CONNECTED. [9 out of 10] It is included in section 4.2.2 “General parameters” of TS 38.306.

2.	A new UE capability (referred e.g. as maxBW-Preference) is defined to indicate its preference on the maximum aggregated bandwidth for power saving in RRC_CONNECTED. [8 out of 10] It is included in section 4.2.2 “General parameters” of TS 38.306.

3	A new UE capability (referred e.g. as maxCC-Preference) is defined to indicate its preference on the maximum number of secondary component carriers for power saving in RRC_CONNECTED. It is included in section 4.2.2 “General parameters” of TS 38.306.

4.	A new UE capability (referred e.g. as maxMIMO-LayerPreference) is defined to indicate its preference on the maximum number of MIMO layers for power saving in RRC_CONNECTED. [8 out of 10] It is included in section 4.2.2 “General parameters” of TS 38.306.

5	It is not agreed to bundle into a separate new UE capability the support for UE’s assistance of the maxCC, maxBW and maxMIMO.

6.	A new UE capability (referred e.g. as release-Preference) is defined to indicate its preference assistance information to transition out of RRC_CONNECTED for power saving. It is included in section 4.2.2 “General parameters” of TS 38.306.

7	New UE capability is defined for the relaxed measurement feature without signalling to the gNB, i.e. gNB will not know whether a UE supports or not this feature. It is included in section 5 “Optional features without UE radio access capability parameters” of TS 38.306.

8	For drx-Preference, maxCC-Preference, release-Preference, maxMIMO-LayerPreference, and maxBW-Preference, its characteristics are defined as as “Per: UE” (to the characteristic associated to the UE), “M: No” (to the characteristic associated with mandatory/optional features), “FDD-TDD DIFF: No” (to the characteristic associated with the applicability for FDD/TDD).

9	For drx-Preference, maxCC-Preference, and release-Preference, its characteristics are defined as “FR1-FR2 DIFF: No” (to the characteristic associated to FR1/FR2 equally applicable for FR1/FR2).

10   For maxMIMO-LayerPreference, and maxBW-Preference, its characteristics are defined as “FR1-FR2 DIFF: Yes” (to the characteristic associated to FR1/FR2 are not equally applicable for FR1/FR2).

11	Rel-16 capability maxMIMO-LayerPreference is not linked to Rel-15 capability maxLayersMIMO-Indication

R2-2002602	UE capabilities for Rel-16 Power Saving (PWS) WI	Intel Corporation	draftCR	Rel-16	38.306	16.0.0	B	NR_UE_pow_sav

=>	The CR will be moved to email discussion

R2-2002842	SRB3 for reporting UAI for power saving	OPPO	CR	Rel-16	37.340	16.1.0	0189	-	F	NR_UE_pow_sav-Core

=>	The CR is in principle agreed

### 6.11.2	PDCCH-based power saving signals/channel Additional stage-3 RAN2 aspects

Including out of [Post109e#41][PowSav] DCP open issues (InterDigital, Huawei)

Contributions related to issues addressed by the email discussions should be avoided and are discouraged for this AI.

All identified critical open issues should be provided to the rapporteur via email discussion Post109e#41 and new contributions on those topics are discouraged.  Contributions should be reserved for more complicated and critical issues.

No individual company CRs should be submitted

To be treated

R2-2003378	Summary of [Post109e#41] [PowSav] DCP open issues – Phase 1	InterDigital	discussion	Rel-16	NR_UE_pow_sav-Core	Late

=>	Noted

R2-2003379	Report of [Post109e#41] [PowSav] DCP open issues	InterDigital	discussion	Rel-16	NR_UE_pow_sav-Core	Late

=>	Noted

Agreements

1	RAN2 confirms that the flags ps-TransmitPeriodicL1-RSRP and ps-TransmitPeriodicCSI are defined per cell group

2	The flags ps-TransmitPeriodicCSI and ps-TransmitPeriodicL1-RSRP are independent, and it is possible to control UE to report all types of periodic CSI apart from L1-RSRP (i.e. cri-RSRP and ssb-Index-RSRP)

3	The flag ps-TransmitPeriodicCSI  is renamed to ps-TransmitOtherPeriodicCSI

Proposal 1: 	DCP is supported for Short DRX in Rel-16 and is configurable by the network. (7/13)

-	Intel thinks that there is no clear majority and there is some concerns from RAN1.  Vivo also thinks that RAN2 can’t make this decision.  Nokia thinks that we have already agreed in RAN2 and hasn’t come up with any technical concern.  Samsung, LG and Apple agree with Intel.

-	Ericsson thinks that this is a RAN2 topic and agrees with Nokia

Proposal 5: 	The flag ps-TransmitPeriodicCSI  is renamed to ps-TransmitOtherPeriodicCSI. (5/10)

-	Mediatek asks if we should tell RAN1.

=>	Include this agreement in a RAN1 LS

R2-2003963	LS on RAN2 DCP Open Issues	InterDigital

-	Nokia thinks that we need to indicate very clearly what is the intended behaviour and it is up to RAN2 to define the behaviour.

-	Oppo’s understanding that prioritization of search spaces are discussed in RAN1 and wants to discuss the 2-step RACH and in this case the C-RNTI has lower priority so there is some impact.  Vivo agrees.

-	Qualcomm thinks that we need to leave the decision to RAN1.   For BFR, RAR should be prioritized.

-	CATT would like to narrow down the discussion to power saving and not going into 2-step RA discussion.  The understanding is that RAR should be prioritize over DCP.  Vivo agrees.

-	Ericsson thinks we should state that some companies in RAN2 are concerned on the impact of scheduling and RAR should be prioritized.

-	ZTE doesn’t think that we should differentiate for RAR for BFR and 2-step RA and this should apply to all RAR.  Nokia agrees. Qualcomm disagrees that all RARs are the same, as some are sent in BFR search space and some are UE specific etc.  The reason it should be prioritized is that it is more urgent.

-	LG thinks that the prioritization should be for all RAR and not distinguish between different type of RAR.  CATT doesn’t see a reason why we should prioritize DCP.

=>	From RAN2 point of view, the understanding is that RAR addressed to all RNTI should be prioritized over DCP by the UE.

=>	The LS is revised in R2-2003967

R2-2003967	LS on RAN2 DCP Open Issues	InterDigital

=>	Delete actions

=>	Ask RAN1 if they have any concerns with our understanding

=>	Update title

=>	The LS is revised in R2-2003969

R2-2003967	LS on RAN2 DCP Open Issues	InterDigital

=>	The LS is approved in R2-2004175

R2-2003955 	Outcome of [AT109bis-e][506][PowSav]  Open Issues for DCP (InterDigital)

=>	Noted

Agreements

1    RAN2 to send an LS to RAN1 to clarify UE behaviour when DCP overlaps with RAR with C-RNTI.

2     When configured with multiple DCP monitoring occasions, UE behaviour when some DCP monitoring occasions overlap with active time or measurement gap or BWP switching, but at least one DCP monitoring occasion does not, will be further addressed by RAN1, if necessary.

R2-2003129	Miscellaneous corrections to 38.321 for Rel-16 UE power saving	Huawei, HiSilicon	CR	Rel-16	38.321	16.0.0	0719	-	F	NR_UE_pow_sav-Core	Late

=>	The CR will be revised in R2-2003956 capturing agreements from week1 meeting and from offline email discussion

R2-2003956	Miscellaneous corrections to 38.321 for Rel-16 UE power saving	Huawei, HiSilicon	CR	Rel-16	38.321	16.0.0	0719	-	F	NR_UE_pow_sav-Core	Late

=>	the CR will be reviewed over email discussion

Will not be treated

R2-2002797	PDCCH-WUS Mechanism	Apple	discussion	NR_UE_pow_sav-Core

R2-2002839	Remaining issues of DCP impact on SCell dormancy	OPPO	discussion	Rel-16	NR_UE_pow_sav-Core

R2-2002866	Remaining issues for DCP	vivo	discussion	Rel-16	FS_NR_UE_pow_sav

R2-2002930	Correction on RAR and DCP monitoring	Nokia, Nokia Shanghai Bell	draftCR	Rel-16	38.321	16.0.0	F	NR_UE_pow_sav-Core

R2-2003032	Remaining issue on DCP monitoring within RAR window	LG Electronics Inc.	discussion	NR_UE_pow_sav-Core

R2-2003288	Open issues UE capability, DCP, UE assistance and RRM relaxation	Ericsson	discussion	Rel-16	NR_newRAT-Core

R2-2003562	PDCCH-based power saving signal/channel	Samsung	discussion	NR_UE_pow_sav-Core

### 6.11.3	UE assistance and RRC

Including outcome of [Post109e#43][PowSav] UE Assistance and RRC open issues (Mediatek)

Contributions related to issues addressed by the email discussions should be avoided and are discouraged for this AI.

All identified critical open issues should be provided to the rapporteur via email discussion Post109e#43 and new contributions on those topics are discouraged.  Contributions should be reserved for more complicated.

No individual company CRs should be submitted

To be treated

R2-2003127	Summary of [Post109e#43][PowSav] UE Assistance and RRC open issues	MediaTek Inc.	discussion	Rel-16	NR_UE_pow_sav-Core	Late

=>	Noted

Discussion

-	Mediatek explains that we have different triggers for power saving.  Samsung maybe thinks that we can have some grouping and the current ASN.1 is not so pretty.   Mediatek thinks that maybe we can group BW, CC and MIMO.

Agreements

1	Delta signalling applies at a ‘feature’ level, where the ‘features’ for power saving are: drx-Preference, maxBW-Preference, maxCC-Preference, maxMIMO-LayerPreference, minSchedulingOffsetPreference and releasePreference.  No further grouping is considered.

2	When reporting a ‘feature’, the all parameters that the UE has a preference for are included. Parameters that are not included are interpreted as the UE having no preference for those parameters.

3	An empty ‘feature’ IE can be signalled to indicate that the UE has no preference for all parameters in the ‘feature’ (i.e. similar to overheating)

4	In NR-DC, SCG specific UAI for power saving can be configured by the network

5	The reported UAI for power saving is specific to a cell group

6 	In (NG)EN-DC, SCG specific UAI for power saving can be configured by the network via SRB1 (using nr-SecondaryCellGroupConfig) or SRB3 (using RRCReconfiguration).

7 	In (NG)EN-DC, SCG specific UAI for power saving is transmitted in ULInformationTransferMRDC on the LTE leg.

8	In (NG)EN-DC, SCG specific UAI for power saving is transmitted on the NR leg via SRB3, if SRB3 is configured.

9	In NR-DC, SCG specific UAI for power saving can be configured by the network via SRB1 (using mrdc-SecondaryCellGroup) or SRB3 (using RRCReconfiguration).

10	In NR-DC, SCG specific UAI for power saving is transmitted in ULInformationTransferMRDC on SRB1.

11	In NR-DC, SCG specific UAI for power saving is transmitted on the SCG via SRB3, if SRB3 is configured.

12   A Note is included in the RRC specification clarifying how the UE can indicate a preference for NR SCG release. (

13  The field description of maxMIMO-Layers is updated to indicate that the field does not override the value provided in PDSCH-ServingCellConfig.

14   Preferred RRC state is always reported in the release preference, and can indicate idle, inactive,   connected and out of connected

15   Inform RAN3 of our agreement on the indication of SCG release preference from the UE.

R2-2003964	LS on NR SCG release for power saving Mediatek

=>	The LS is approved in R2-2003968

Discussions

Proposal 1: UE can indicate any preferred value within its capability for maximum aggregated bandwidth, number of carriers, MIMO layers and minimum scheduling offset.

-	Apple suggests that we can limit the amount of times that the UE requests.  Mediatek explains that in the email discussion there was very limited supported.

-	Qualcomm wonders if we can make this network configurable.  ZTE, Nokia, and Ericsson don’t think we should make everything configurable.   Apple agrees with the compromise.  Ericsson thinks that the UE can just send a no preference indication and the network can configure the UE accordingly.

-	Vivo asks how the network would configure it, it would need to report UE capability.

-	Huawei supports and is ok to have the configurability.

Proposal 2: UE can indicate a preferred maximum aggregated bandwidth for a frequency range not configured with serving cells.

R2-2003125	CR for 38.331 for Power Savings	MediaTek Inc.	CR	Rel-16	38.331	16.0.0	1540	-	C	NR_UE_pow_sav-Core	Late

=>	Moved to email discussion

R2-2003126	CR for 36.331 for Power Savings	MediaTek Inc.	CR	Rel-16	36.331	16.0.0	4245	-	B	NR_UE_pow_sav-Core	Late

=>	Moved to email discussion

R2-2003957 	Outcome of [AT109bis-e][504][PowSav] CP/UE assistance Open and ASN.1 Issues (Mediatek)

Proposal 4: “connected” is removed from preferredRRC-State

-	Ericsson is concerned that this will create unnecessary signaling and the UE wouldn’t have the opportunity to cancel.

-	Intel would like to see the CR and revisit if the overall behavior allows for the UE to tell the network to cancel the preference.

-	CATT thinks that the problem if we remove connected we would have more specification to indicate that there is indeed some form of implicit cancelation.  LG has a similar understanding as CATT.  Apple agrees with CATT.

-	DT shares the concerns from Ericsson.

=>	We will review the stage 3 CR with current agreements and if there is an issue we can revisit with an actual solution.  One possible solution is that the network can configure whether the UE can send “connected” mode preference.  Continue discussing over email.

=>	Noted

Agreements

- Update relaxedMeasCondition IE to a Boolean flag ‘combineRelaxedMeasConditions’

- IEs s-SearchDeltaP and t-searchDeltaP are mandatory fields [CB if an issue is identified]

- Leave it to NW implementation to ensure that at least lowMobilityEvalutation or cellEdgeEvalutation IEs are present when relaxedMeasurement is configured.

Will not be treated

R2-2002670	Power Saving UE assistance information	Sony	discussion	Rel-16	NR_UE_pow_sav-Core

R2-2002798	Value Range for UE Assistance Information	Apple	discussion	NR_UE_pow_sav-Core

R2-2002838	Remaining issues on implicit SCG release	OPPO	discussion	Rel-16	NR_UE_pow_sav-Core

R2-2003229	Adopting general UE assistance reporting framework to UE power saving	Samsung Telecommunications	discussion	Rel-16

R2-2003289	UE assistance for connection release	Ericsson, ZTE, Deutsche Telekom	discussion	Rel-16	NR_newRAT-Core

R2-2003387	Adopting general UE assistance reporting framework to UE power saving	Samsung Telecommunications	discussion	Rel-16	Late

R2-2003472	Discussion on clarification for max MIMO layer and antenna port	Huawei, HiSilicon	discussion	Rel-16	NR_UE_pow_sav-Core

R2-2003473	TP for clarification for max MIMO layer and antenna port	Huawei, HiSilicon	discussion	Rel-16	NR_UE_pow_sav-Core

### 6.11.6	RRM measurement relaxation

Including out of [Post109e#44][PowSav] RRM open issues (CATT, Vivo)

Contributions related to issues addressed by the email discussions should be avoided and are discouraged for this AI.

All identified critical open issues should be provided to the rapporteur via email discussion Post109e#44 and new contributions on those topics are discouraged.  Contributions should be reserved for more complicated issued.

No individual company CRs should be submitted

To be treated

R2-2002791	Report of [Post109e#44][PowSav] RRM open issues	CATT	discussion	Rel-16	NR_UE_pow_sav-Core

=>	Noted

Proposal 1 If timer T330 is running, relaxed RRM measurement can be performed. No further specification impact (10/13).

-	Ericsson thinks that there is a problem with this as it will influence the logged MDT measurements.  We should make sure that two features can co-exist.  The work around of disabling RRM if the network wants RRM is not a good solution.

-	CMCC agrees with Ericsson and this is an important feature for operators and we would collect misleading results.   Samsung supports Ericsson’s view

-	CATT asks why this hasn’t been a problem with the existing Rel-15 relaxation.

-	Vivo and LG support the proposal and MDT is a feature for a number of UEs and RAN4 already agreed that the relaxation approach doesn’t stop the measurement but relaxes some measurements sample or extends the measurement period.  For such relaxation there is no impact to MDT.  LG also explains that the UE still is taking measurements and if it is not taking measurement then the UE is in low mobility.

-	ZTE supports Ericsson’s view and in some scenarios it will skew the measurement results.

-	Mediatek understands the concerns from ZTE and Ericsson, but the network can fix the problem by not configuring both RRM and MDT.  However, if the network and operators have concerns Mediatek is fine.

-	Huawei explains that one of the principles of MDT has always been that the UE should perform measurements without impacting the UE.

-	Sony thinks that this under control

-	Intel suggests a compromise that this is under control

-	Ericsson is ok with this compromise but they would also like a bit in MDT measurements to indicate that there was relaxed RRM

-	Nokia thinks that the network knows about the UE supported RRM or not.

Proposal 2 (13/13): When cellEdgeEvalutation is configured, SSearchThresholdP should be mandatory while SSearchThresholdQ is optional.

Proposal 3 (8/10): It is left up to UE implementation whether to re-evaluate the relaxation criterion when network changes the relaxed measurement parameters in system information.  The System Information change procedure is not impacted.  No specification change to capture any UE behavior.

-	Nokia is worried that the UE don’t respect the system information update.  CATT explains that the intention is not to ignore the system information update, but rather the behaviour upon the SI change.

The following issue could not be resolved and should be continued on-line during next e-meeting:

The configuration of the relaxation criteria is constant for all frequencies (6) or is per-frequency (or per-FR) configured (7).

-	Vivo would like to consider grouping of frequencies

-	Sony asks if RAN4 is discussing this.

Agreements

- If timer T330 is running, relaxed RRM measurement can be performed. No further specification impact

- When cellEdgeEvalutation is configured, SSearchThresholdP should be mandatory while SearchThresholdQ is optional

3	No new behaviour for RRM relaxation needs to be captured if the parameters in SI change and UE continues legacy behaviour of SI change/update.  The UE applies new configuration as in legacy behaviour.

4	FFS Continue offline discussion for FR1/FR2 differentiation only.  Down select per-frequency configuration of the relaxation criteria.

5	Scaling factor(s) for relaxed RRM measurements are left to RAN4 to decide

6	How/if higher priority frequencies RRM measurements are relaxed is left to RAN4 to decide

7	Whether RRM measurements relaxation is allowed on a frequency when the UE is configured to perform early measurement on that frequency and T331 is running is left to RAN4 to decide.

R2-2003954	Outcome of [AT109bis-e][505][PowSav] RRM Open Issues	CATT	discussion	Rel-16	NR_UE_pow_sav-Core

=>	Noted

R2-2003958  Outcome of [AT109bis-e][505][PowSav] RRM Open Issues (CATT, Vivo)

=>	Noted

Agreements

1	Global configuration of relaxation triggers is kept. No change is needed to the current specifications from this aspect.  Differentiation of scenarios can be done via the high priority frequency indication framework and no further behaviour is expected to be specified.

Proposal 6 (11/13): If timer T330 is running, relaxed RRM measurement can be performed. No further specification impact.

-	Ericsson is disappointed with the outcome of the email discussion and the results will be biased.

R2-2002865	CR on 38.304 for UE Power saving in NR	vivo	CR	Rel-16	38.304	16.0.0	0152	-	B	FS_NR_UE_pow_sav

=>	The CR is revised in R2-2003959 capturing additional agreements from the meeting and email discussion

R2-2003959	CR on 38.304 for UE Power saving in NR	vivo	CR	Rel-16	38.304	16.0.0	0152	-	B	FS_NR_UE_pow_sav

Will not be treated

R2-2002665	UE power saving for inter frequency measurements	Sony	discussion	Rel-16	NR_UE_pow_sav-Core	R2-2000827

R2-2002735	Configurations for RRM Measurement Relaxation	MediaTek Inc.	discussion

R2-2002867	Configurations for RRM Measurement Relaxation	vivo	discussion	Rel-16	FS_NR_UE_pow_sav

R2-2002950	Correction of SI update of relaxed measurement parameters	Nokia, Nokia Shanghai Bell, Ericsson	draftCR	Rel-16	38.304	16.0.0	F	NR_UE_pow_sav-Core

R2-2003216	EMR issue on relaxed measurement	LG Electronics Inc.	discussion	Rel-16	NR_UE_pow_sav-Core	Withdrawn

R2-2003219	EMR issue on relaxed measurement	LG Electronics Inc.	discussion	Rel-16	NR_UE_pow_sav-Core

## 6.13	2-step RACH for NR

(NR_2step_RACH-Core; leading WG: RAN1; REL-16; started: Dec 18; target; Mar 20; WID: RP-200085; SR: RP-200488). Documents in this agenda item will be handled in a break out session

Time budget: 1 TU

Tdoc Limitation: 1

### 6.13.1	General

Running CRs, Incoming LSs, Contributions in this AI are restricted for  WI rapporteur inputs and/or spec rapporteur inputs and do not count towards the tdoc limits.

All comments related to 38.300 should be given directly to Eswar rapporteur.   ZTE will update CRs according to received comments offline

R2-2003009	4-step RA type description	Nokia (rapporteur), Nokia Shanghai Bell, ZTE	CR	Rel-16	38.300	16.1.0	0214	-	F	NR_2step_RACH-Core	Late

-	Huawei asks if this is Rel-15 CR as well. ZTE explains that the explanation is only needed because we added the 2-step RACH

=>	The CR is in principle agreed

Incoming LS

R2-2004164		Reply LS on the starting point of MsgB window (R1-2002794; contact: ZTE)

=>	Noted

R2-2004165		Reply LS on the support of 2-step CFRA (R1-2002795; contact: ZTE)

Question: whether or not to support CSI-RS based 2-step CFRA with the assumption that work on CFRA is not pursued in RAN1 as stated in the WID

-	Nokia thinks that there is nothing to agree, as we have already agreed and included it in the RAN2 specs and it is UE capability.  ZTE thinks that we can discuss whether we support this as a UE capability.

-	Qualcomm indicates that if this has RAN1 impact we should just agree to not support

=>	continue discussion over email discussion on UP open issues

=>	Noted

R2-2004167	LS Response on preamble-to-PRU mapping for 2-step CFRA (R1-2002901; contact: Ericsson)

=>	Continue discussion over email discussion on UP open issues

=>	Noted

### 6.13.2	User plan aspects

A single CR will be produced by Rapporteur. No individual company CRs are expected.  Comments should be given directly to rapporteur preferable.  Contribution should be reserved for more complicated issued, but they should be critical issues

To be treated

R2-2003960   Outcome of [AT109bis-e][503][2s RA] UP Open Issues (ZTE)

=>	Noted

Agreements 1 Once the UE transmits PRACH preamble then it shall transmit the associated PUSCH regardless of the occurrence of measurement gap 2 No changes are made to the MAC spec for the issue related to the MSGA PUSCH resources overlapping with measurement gap 3 Nothing is specified (assumption being the network only uses the optional PUSCH configuration features on dedicated BWPs and only for UEs that support these features) – no change to specification needed. Inform RAN1 about this conclusion 4 If there is no valid PUSCH resource associated to the PRACH resource selected: o then MAC doesn’t determine the UL grant or the associated HARQ information o MAC will only indicate the preamble resource to L1 o The UE will still generate the MAC PDU based on the TB size 5 msgA-TransMax is configured for 2 step CFRA in rachConfigDedicated and that the UE is not allowed to switch to 4-step RACH if this is not configured in rachConfigDedicated 6 From RAN2 perspective, if the UE supports 2-step RA, it shall support 2-step CFRA for the SSB case (i.e. same as 4-step RACH) 7 FFS If RAN1 indicates support for CSI-RS for 2-step CFRA, there is no need for a separate capability bit for this in 38.306 from RAN2 perspective. 8 Change the LCID to one byte eLCID for the Absolute Timing Advance Command 9 Move the initialisation of RSRP_THRESHOLD_RA_TYPE_SELECTION after the BWP operation since the threshold can be different in different BWPs 10 The current running CR (updated per the above agreements) can be used as baseline for further review until 27th - Swapping of order of succssRAR and fallbackRAR in section 5.1.4a: Further comments can be provided for this issue (including the necessary clarification on the need etc) during the CR review phase - BFR for SpCell: Changes can be kept, text can be reviewed further - Ambiguity in PRACH prioritisation for 4-step RA: HO part to be fixed and the changes will be kept but to be reviewed further - Addition of LBT parameters for MSGB: Changes can be kept and CR can be reviewed further - Other misc corrections: Fix the typo in section 5.1.1 and review other changes in the next round

R2-2002965	Updates to MAC spec for 2-step RACH	ZTE (CR editor), Nokia, Samsung, Vivo	CR	Rel-16	38.321	16.0.0	0714	-	F	NR_2step_RACH-Core, NR_unlic-Core

=>	The CR is revised with agreements above in R2-2003962 and will be reviewed over email

R2-2003962	Updates to MAC spec for 2-step RACH	ZTE (CR editor), Nokia, Samsung, Vivo	CR	Rel-16	38.321	16.0.0	0714	1	F	NR_2step_RACH-Core, NR_unlic-Core

-	Nokia still thinks that we should re-order the success before the fallback.

=>	The CR is in principle agreed

R2-2003961	LS to RAN1 on agreements related to 2-step RACH  ZTE

=>	The LS is revised in R2-2003970

R2-2003970	LS to RAN1 on agreements related to 2-step RACH  ZTE

=>	The LS is approved R2-2004188.

Will not be treated

R2-2002585	Remaining Issues on Resource Selection in 2-setp RACH	vivo	discussion

R2-2002668	msgB-RNTI ambiguity for CFRA and CBRA of 2-Step RACH	Sony	discussion	Rel-16	NR_2step_RACH-Core	R2-2000833

R2-2002840	Remaining issues of 2-step RACH	OPPO	discussion	Rel-16	NR_2step_RACH-Core	Late

R2-2003007	Discussion on remaining issues of 2-step RA	Huawei, HiSilicon	discussion	Rel-16	NR_2step_RACH-Core

R2-2003356	Handling invalid POs for MsgA transmissions	Ericsson	discussion	Rel-16	NR_2step_RACH-Core

R2-2003357	Change LCID to eLCID for Absolute Timing Advance Command	Ericsson	CR	Rel-16	38.321	16.0.0	0722	-	F	NR_2step_RACH-Core

R2-2003362	Correction of Handling of invalid POs for MsgA transmissions	Ericsson	CR	Rel-16	38.321	16.0.0	0725	-	F	NR_2step_RACH-Core

R2-2003666	Further clarifications on parameters for Random Access procedure	LG Electronics	discussion	NR_2step_RACH-Core

### 6.13.3	RRC stage-3 related aspects

A single CR will be produced by Rapporteur. No individual company CRs are expected.  Comments should be given directly to rapporteur preferable.  Contribution should be reserved for more complicated issued, but they should be critical issues

To be treated

R2-2004101   RRC ASN.1 open issues Ericsson

=>	Noted

Agreements

1	Proposal 1: Configure msgA-PUSCH-ResourceGroupA and msgA-PUSCH-ResourceGroupB (the latter being conditional on group B being present).  Proposal 3:  msgA-PUSCH-ResourceGroupA and msgA-PUSCH-ResourceGroupB is separately configured, the parameter msgA-PUSCH-PreambleGroup is not needed

2	Proposal 4: Time domain resource allocation can also be provided through PUSCH-Config if provided (CFRA); 2) Clarification for the absence of PUSCH-TimeDomainAllocation.

3	Proposal 14: For messagePowerOffsetGroupB and ra-MsgA-SizeGroupA absence description does not apply since this is a need M field and is removed.

4	Proposal 15: Need code for cfra-TwoStep-r16 should be same with that for 4-step CFRA, Need S. Add IE field description similar to 4 Step RA to include “If this field is absent, the UE performs contention based random access.”

5	Proposal 16: Agree correction of IE for 2 step CFRA PUSCH resource configuration, MsgA-PUSCH-Resource-r16

6	Proposal 18: Delete sentence on ignoring parameters in field description for rach-ConfigGenericTwoStepRA as there is optionality in signalling whereas this was mandatory in legacy.  Add a guidance to not configure those parameters for CFRA, expect for msgatransmax

7	Proposal 19: Agree preambleTransMax-r16 to be optional with condition 2StepOnly

8	Proposal 20: Agree correction so that msgA-TransMax-r16 is applicable if switching to 4 step RA is supported.  msgA-TransMax-r16 should be configured in dedicated RACH config.

9	[CB] Proposal 2: msgA-TransmformPrecoder and msgA-DeltaPreamble-r16 are changed to Optional Need R, and sentence “If the field is absent, the UE shall use the parameter msg3-DeltaPreamble of 4-step type RA in the configured BWP if 4-step type RA is configured.” Is removed.

10   Proposal 12: Agree change for msgA-SubcarrierSpacing. For msgA-PRACH-RootSequenceIndex and msgA-RestrictedSetConfig, agree change with the added conditional sentence changed from “This field is only configured for the case of separate ROs between 2-step and 4-step type random access.” to “When both 2-step and 4-step type random access is configured, this field is only configured for the case of separate ROs between 2-step and 4-step type random access.”.  Double check for 2-step only case

11	Proposal 13: Agree the clarification in ra-ContentionResolutionTimer field description with the removal of “and the UE shall use the corresponding value from the RACH-ConfigCommon”

12	Proposal 17: Agree to remove parameter totalNumberOfRA-Preambles since preambles for msg1 based SI request does not apply for 2-Step RA type.

R2-2004173	RRC Open issues    Ericsson

=>	Noted

Agreements

1	Confirm that: msgA-TransmformPrecoder and msgA-DeltaPreamble-r16 are changed to Optional Need R, and sentence “If the field is absent, the UE shall use the parameter msg3-DeltaPreamble of 4-step type RA in the configured BWP if 4-step type RA is configured.” Is removed.

2	Correct editorial ”not” to ”no” in existing field description for InitialBWPConfig.

3	Merge the two IEs “msgA-RSRP-Threshold-r16” and “msgA-RSRP-ThresholdSUL-r16” into using a single msgA-RSRP-Threshold-r16

4	Remove the corresponding fields and parameters for msgA-RSRP-ThresholdSUL-r16.

5	Remove redundant parameter msgA-RSRP-ThresholdSSB-SUL.

6	Agree to capture change to specify msgA PRACH and payload should be either absent or present at the same time and that 2-step RACH can only be configured on SpCell.

7	The fields ra-MsgA-SizeGroupA and messagePowerOffsetGroupB are mandatory conditioned to if preamble group B is configured.

Not treated

R2-2002556	Issues - 2 step RA	Samsung Electronics Co., Ltd	discussion	Rel-16	NR_2step_RACH-Core

R2-2002878	RAN2 related UE capability for 2-step RACH	Intel Corporation	discussion	Rel-16	NR_2step_RACH-Core

R2-2003255	Remaining issue on 2-step CFRA	Qualcomm Incorporated	discussion	Rel-16	NR_2step_RACH-Core

R2-2003649	Correction on 2-step RACH configurations in RRC	ASUSTeK	discussion	Rel-16	38.331	NR_2step_RACH-Core

Email discussions for next meeting:

- [Post109e-bis#xx][NR-U] MAC open issues (Ericsson)

Address stage-3 remaining open issues from 109e

Capture identified NEW, if any, stage-3 corrections/issues from other companies.  Issues that have already been discussed and not pursued should not be brought up again.

Intended outcome: CR for 38.321 addressing open issues (including editorials received offline)

- [Post109e-bis#xx][NR-U] RRC open issues (Qualcomm)

Address stage-3 remaining open issues

Capture identified NEW, if any, stage-3 corrections/issues from ASN.1 review.  Issues that have already been discussed and not pursued should not be brought up again.

Intended outcome: Agreable proposals and CR for 38.331 addressing open issues

- [Post109e#yy][NR-U] UE running capabilities (Vivo)

Intended outcome: draft CR for 38.306 and 38.331 to reflect the latest RAN2 capabilities agreements (on R2 capabilities and the description of related field on 38.331 email discussion) and to discuss if any new R2 capability is required

- [Post109e-bis#xx][ PowSav] MAC open issues (Huawei)

Address stage-3 remaining open issues

Capture identified NEW, if any, stage-3 corrections/issues from other companies.  Issues that have already been discussed and not pursued should not be brought up again.

Intended outcome: CR for 38.321 addressing open issues (including editorials received offline)

- [Post109e-bis#xx][ PowSav] RRC open issues (Mediatek)

Address stage-3 remaining open issues

Capture identified NEW, if any, stage-3 corrections/issues from ASN.1 review.  Issues that have already been discussed and not pursued should not be brought up again.

Intended outcome: Agreable proposals and CR for 38.331 addressing open issues

 [Post109e-bis#xx][ PowSav] RRM open issues (vivo)

Address stage-3 remaining open issues

Capture identified NEW, if any, stage-3 corrections/issues from other companies.  Issues that have already been discussed and not pursued should not be brought up again.

Intended outcome: CR for 38.304 addressing open issues (including editorials received offline)

- [Post109e-bis#xx][PowSav] UE capabilities (Intel)

Intended outcome: draft CR for 38.306 and 38.331 to reflect the latest RAN2 capabilities agreements (on R2 capabilities and the description of related field on 38.331 email discussion) and to discuss if any new R2 capability is required

- [Post109e-bis#xx][ 2s-RA] UP and other open issues (ZTE)

Address stage-3 remaining open issues from 109e-bis, including CFRA

Capture identified NEW, if any, stage-3 corrections/issues from other companies.  Issues that have already been discussed and not pursued should not be brought up again.

Intended outcome: CR for 38.321 addressing open issues (including editorials received offline) and way forward for specification of CFRA (proposals and CRs)

- [Post109e-bis#xx][2s-RA] RRC open issues (Ericsson)

Address stage-3 remaining open issues

Capture identified NEW, if any, stage-3 corrections/issues from ASN.1 review.  Issues that have already been discussed and not pursued should not be brought up again.

Intended outcome: Agreable proposals and CR for 38.331 addressing open issues