3GPP TSG-RAN WG2 Meeting #109bis electronic	R2-200xxxx

20th – 24th April, 2020

Agenda item:       5.4.1.1

Source: 	Huawei, ZTE

Title: 	Summary of offline [005][NR15] L1 Configuration

Document for:		Discussion and Decision

- 1.	Introduction

This is a summary of offline discussion for the following documents:

L1 Configuration

- [AT109bis-e][005][NR15] L1 Configuration (Huawei, ZTE)

Scope: Treat R2-2002551, R2-2003537, R2-2003538, R2-2002697, R2-2002698

Part 1: Determine which issues that need resolution, find agreeable proposals. Deadline: April 23 0700 UTC

Part 2: For the parts that are agreeable, discussion will continue to agree on CRs.

R2-2002508	Reply LS for clarification of PUCCH configuration (R1-2001306; contact: Huawei)	RAN1	LS in	Rel-15	NR_newRAT-Core	To:RAN2

=> Revised in R2-2002551

R2-2002551	Reply LS for clarification of PUCCH configuration (R1-2001306; contact: Huawei)	RAN1	LS in	Rel-15	NR_newRAT-Core	To:RAN2

R2-2003537	Correction on PUCCH configuration	Huawei, HiSilicon	CR	Rel-15	38.331	15.9.0	1567	-	F	NR_newRAT-Core

R2-2003538	Correction on PUCCH configuration	Huawei, HiSilicon	CR	Rel-16	38.331	16.0.0	1568	-	A	NR_newRAT-Core

R2-2002697	Clarification on SRS-CarrierSwitching structure	ZTE Corporation, Sanechips, Qualcomm Incorporated	discussion	Rel-15	NR_newRAT-Core

R2-2002698	CR on SRS-CarrierSwitching	ZTE Corporation, Sanechips, Qualcomm Incorporated	CR	Rel-15	38.331	15.9.0	1518	-	F	NR_newRAT-Core

- 2. Discussion

## 2.1	Correction on PUCCH configuration (R2-2003537, R2-2003537)

The reason for change is:

RAN2 sent an LS to RAN1 in R2-1916481 to ask about PUCCH configuration for NR standalone and late drop architectures, and RAN1 replied the LS in R1-2001306 (which is R2-2002551 above) with the following clarification: The restriction for PUCCH configuration for EN-DC in the LS is also applied to NGEN-DC and NE-DC. For NR-CA (without configured SCG), the restriction in the LS is not applied. The maximum number of PUCCH groups is two, i.e. only primary PUCCH group and secondary PUCCH group are allowed at most. For NR-DC, the maximum number of PUCCH groups in each CG is one. Only the same numerology is supported for the CG with carriers only in FR2. Note that for NR standalone (i.e. NR-CA above), there is already a clarification, “If supported by the UE, the network may configure at most one additional SCell of a cell group with PUCCH-Config (i.e. PUCCH SCell)” in the specification.

Therefore, it is proposed to capture the PUCCH configuration restriction as indicated in R2-2002551 for NGEN-DC, NE-DC and NR-DC..

Q1) Do companies agree with the changes in the CR R2-2003537?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |

## 2.2	Clarification on SRS-CarrierSwitching (R2-2002697, R2-2002698)

In discussion paper R2-2002697, it observes that the SRS-CarrierSwitching configuration is defined differently in LTE and NR spec. In LTE, the TypeA field “typeA-SRS-TPC-PDCCH-Group-r14” is defined under PCell configuration (i.e. per-UE configured). So the definition of “SEQUENCE (SIZE (1..32)) OF… ” structure is used to indicated the mapping relationship between each PUSCH-less SCell and CC set. While for Type B, the field “typeB-SRS-TPC-PDCCH-Config-r14” is per SCell configured.

However, in NR, both Type A and Type B configuration are defined within a single structure “SRS-CarrierSwitching”, which can be per-cell configured. So for Type A, by including SRS-CarrierSwitching field in PUSCH-less SCell’s configuration, the network is able to indicate the “switch from serving cell”, “monitoring cells” and “CC set” configuration for each PUSCH-less SCell. Then it is unclear how to interpret the “SEQUENCE (SIZE (1..32)) OF …” structure defined for Type A, e.g. what is the meaning of each entry of this list?

SRS-CarrierSwitching ::=            SEQUENCE {

srs-SwitchFromServCellIndex         INTEGER (0..31)                           OPTIONAL,   -- Need M

srs-SwitchFromCarrier               ENUMERATED {sUL, nUL},

srs-TPC-PDCCH-Group                 CHOICE {

typeA                               SEQUENCE (SIZE (1..32)) OF SRS-TPC-PDCCH-Config,

typeB                               SRS-TPC-PDCCH-Config

}                                                                            OPTIONAL,   -- Need M

monitoringCells                     SEQUENCE (SIZE (1..maxNrofServingCells)) OF ServCellIndex               OPTIONAL,   -- Need M

...

}

Considering each PUSCH-less SCell can be configured with different “switch from” serving cell (as in LTE). To solve this problem, it is proposed to clarify in spec that only the first entry of “SEQUENCE” can be configured in this release, and it corresponds to the serving cell which provides the SRS-CarriersSwitching configuration (i.e. SRS “switch to” SCell).

Proposal 1: For typeA within SRS-CarrierSwitching of a serving cell, clarify that only the first entry of “SEQUENCE(SIZE(1.32) OF…” list can be configured in this release, wherein the first entry corresponds to this serving cell.

Q1: For Type A, do companies agree the meaning of each entry of “SEQUENCE (SIZE (1..32)) OF ..” structure is unclear? (If answers “Disagree”, please provide your explanation on how to interpret this list)

| Company | Agree/Disagree | Comments |
| --- | --- | --- |

Q2: If answers “Agree” to Q1, do companies agree the proposed solution (i.e. only use the first entry), and any comments to the CR in R2-2002698? (If answers “Disagree”, please provide your solution on how to solve this issue)

| Company | Agree/Disagree | Comments |
| --- | --- | --- |

For Type B in NR, there is no need to configure any SRS CC set to UE, thus it is meaningless to invoke SRS-TPC-PDCCH-Config structure when the CHOICE is set to ‘typeB’. However, the field descriptions only mention the cc-SetIndex and cc-IndexInOneCC-Set sub-field are not applicable to typeB. Then it is confused whether network should provide srs-CC-SetIndexlist (with empty sub-fields) to UE. So it is proposed to add the similar restriction to srs-CC-SetIndexlist field.

Q3: Do companies agree to clarify in field description that srs-CC-SetIndexlist is not configured by network in case of Type B (as in R2-2002698)?

| Company | Agree/Disagree | Comments |
| --- | --- | --- |

# Conclusion

In the previous sections we made the following observations:

Based on the discussion in the previous sections we propose the following:

# References

[1] R2-2002508	Reply LS for clarification of PUCCH configuration (R1-2001306; contact: Huawei)	RAN1	LS in	Rel-15	NR_newRAT-Core	To:RAN2

=> Revised in R2-2002551

[2] R2-2002551	Reply LS for clarification of PUCCH configuration (R1-2001306; contact: Huawei)	RAN1	LS in	Rel-15	NR_newRAT-Core	To:RAN2

[3] R2-2003537	Correction on PUCCH configuration	Huawei, HiSilicon	CR	Rel-15	38.331	15.9.0	1567	-	F	NR_newRAT-Core

[4] R2-2003538	Correction on PUCCH configuration	Huawei, HiSilicon	CR	Rel-16	38.331	16.0.0	1568	-	A	NR_newRAT-Core

[5] R2-2002697	Clarification on SRS-CarrierSwitching structure	ZTE Corporation, Sanechips, Qualcomm Incorporated	discussion	Rel-15	NR_newRAT-Core

[6] R2-2002698	CR on SRS-CarrierSwitching	ZTE Corporation, Sanechips, Qualcomm Incorporated	CR	Rel-15	38.331	15.9.0	1518	-	F	NR_newRAT-Core