3GPP TSG-RAN WG2 Meeting #109bis electronic	R2-200xxxx

20th – 24th April, 2020

Agenda item:	5.4.1.1

Source: 	Huawei, HiSilicon, Google, China Unicom

Title: 	Summary of	[AT109bis-e][009][NR15] Conn Control Miscellaneous II (Huawei, Google, China Unicom)

Document for:		Discussion and Decision

- 1.	Introduction

This is a summary of offline discussion for the following documents:

[AT109bis-e][009][NR15] Conn Control Miscellaneous II (Huawei, Google, China Unicom)

Scope: Treat R2-2003690, R2-2003691, R2-2003692, R2-2003693, R2-2003694, R2-2003695, R2-2003670, R2-2003671, R2-2003778,

Part 1: Determine which issues that need resolution, find agreeable proposals. Deadline: April 23 0700 UTC

Part 2: For the parts that are agreeable, discussion will continue to agree on CRs.

- 2. Discussion

## 2.1	R2-2003690 and R2-2003691 Correction on the need for reconfiguration with sync in (NG)EN-DC, NR-DC and NE-DC

Reason for change: The condition for mandatory presence in reconfigurationWithSync does not indicate the concerned CG(s) and, in case of key change, whether it is the primary or the secondary key. This results in several unclear scenarios: - for NR-DC, it is unclear whether reconfigurationWithSync is always used for the SCG at PCell change, or for the MCG at PSCell change - for NR-DC and NE-DC, for PSCell addition and update of PSCell SI, it can be misunderstood that reconfiguration with sync of the MCG is always performed - for (NG)EN-DC and NR-DC, when there is at least on no S-KgNB (i.e. there are only MN-terminated bearers), it is unclear whether the network always performs reconfiguration with sync upon KeNB/KgNB change. - for NR-DC and NE-DC, when there is at least one SN-terminated MCG bearer, in case of S-KgNB/S-KeNB change, it is unclear whether the network always performs reconfiguration with sync of the SCG

Q) Do companies agree with the changes in the CR R2-2003690 (Rel-15) and R2-2003691 (Rel-16)?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| Huawei, HiSilicon | Yes | Proponent |
| Ericsson | Yes | Co-signed |
| Qcom | partially | We agree with the changes made to the conditional presence table. However the added note, is not clear and more clarification is needed: - the suggested behaviour yields to a DRB with no RLC (only the PDCP configuration) - and SRB with no active DRB (which is conflicting with the spec) |
| Nokia | Yes, please see questions here | We also agree that clarification would be useful however we have feedback to update the cover sheet as follows: Please let me know if the additions in yellow below are okay for you? Reason for change:; The condition for mandatory presence in reconfigurationWithSync does not indicate the concerned CG(s) and, in case of key change, whether it is the primary or the secondary key. This results in several unclear scenarios: - for NR-DC, it is unclear whether reconfigurationWithSync is always used for the SCG at PCell change, or for the MCG at PSCell change. - The current understanding is that reconfigurationWithSync is mandatorily required for SCG at PCell change, but not for MCG at PSCell change - for NR-DC and NE-DC, for PSCell addition and update of PSCell SI, it can be misunderstood that reconfiguration with sync of the MCG is always performed - The current understanding is that reconfigurationWithSync is only needed for the SCG. - for (NG)EN-DC and NR-DC, when there is at least on no S-KgNB (i.e. there are only MN-terminated bearers), it is unclear whether the network always performs reconfiguration with sync upon KeNB/KgNB change. - The current understanding is that if even if there is no DRB mapped to the configured SCG, the reconfiguration with sync is still needed for the SCG upon KeNB/KgNB change; as SRB3 may be configured. - for NR-DC and NE-DC, when there is at least one SN-terminated MCG bearer, in case of S-KgNB/S-KeNB change, it is unclear whether the network always performs reconfiguration with sync of the SCG - The current understanding is that if no configured SCG, the reconfiguration with sync is needed on the MCG side upon S-KgNB/S-KeNB change. Summary of change:; Clarify the condition for the mandatory presence of reconfigurationWithSync by separating conditions for each CG and, for key change cases, distinguishing primary and secondary key, in order to address the above scenarios based on the understanding captured above. Impact analysis Impacted 5G architecture options: (NG)EN-DC, NR-DC, NE-DC Impacted functionality: Reconfiguration with sync Inter-operability: For (NG)EN-DC: - it the UE implements the CR and the network does not, the consequences if not approved remain - if the network implements the CR and the UE does not, there is no issue. For NR-DC and NE-DC: - if the network implements the CR but not the UE, or vice-versa, the consequences if not approved remain.. Consequences if not approved:; The UE may consider as invalid certain reconfigurations in which the network does not perform reconfiguration on (both) the NR MCG/SCG.. |
| ZTE | Yes with comments | We agree to the intention of the CR. But we have some comments: Regarding the following bullet, we think the last part can be removed, because in any case, there is SRB1 with keyToUse set to primary. - in the masterCellGroup at change of AS security key derived from KgNB while the UE is configured with at least one radio bearer with keyToUse set to primary, Regarding the highlighted part in the following bullet, it seems unclear whether it means the configuration before this RRCReconfiguration message, or the configuration after this RRCReconfiguration message, or both? - change of AS security key derived from S-KgNB while the UE is configured with at least one radio bearer with keyToUse set to secondary, In our understanding, if the UE was configured with one RB with secondary key, but this RRCReconfiguration is sent to release that RB, seems reconfigurationWithSync is not needed. Only if the RB is kept after applying this RRCReconfiguration message, reconfigurationWithSync operation is required. So we suggest to change it as below: - change of AS security key derived from S-KgNB while the UE is configured with at least one radio bearer with keyToUse set to secondary after applying this RRCReconfiguration message, |

## 2.2	R2-2003692 and R2-2003693 Correction on reestablishRLC

Reason for change: " When PDCP anchor changes in MR-DC, the L2 handling can be RLC bearer release and addition, which is captured in TS 37.340. For example, in case of change between MN terminated split bearer and SN terminated split bearer in EN-DC, the old NR RLC entity can be released and a new NR RLC entity is added for this bearer. In the existing field description of reestablishRLC, it is specified that the network shall set it to true when the security key used for the radio bearer associated with this RLC entity changes. According to 5.3.5.5.1, rlc-BearerToReleaseList is processed (according to 5.3.5.5.3) before rlc-BearerToAddModList (according to 5.3.5.5.4). In 5.3.5.5.3, RLC entities are released without re-establishment, so reestablishRLC has no effect. In 5.3.5.5.4, there is: 1> if the UE's current configuration contains an RLC bearer with the received logicalChannelIdentity within the same cell group: 2> if reestablishRLC is received: 3> re-establish the RLC entity as specified in TS 38.322 [4]; Since the RLC bearer with the same logicalChannelIdentity was previously deleted, the above condition is not met and reestablishRLC is ignored. Then it makes no sense to say that the network always include it in this case....".

Q) Do companies agree with the changes in the CR R2-2003692 (Rel-15) and R2-2003693 (Rel-16)?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| Huawei, HiSilicon | Yes | Proponent |
| Ericsson | No | We think the CR is not needed. In case of RLC bearer release and add during security key change, the security key will not change for the added RLC bearer. In fact, the old RLC bearer is released (without reset), and a new RLC bearer is added, which is associated with the radio bearer using the new key. According to this, we really fail to understand the proposed change. |
| Qcom | NO | Based on the described scenario, it’s a network issue, as network should ensure that RLC bearer can be only re-established if it exists. i.e. network shall not ask the UE to release a specific RLC bearer and to re-establish the same RLC bearer in same or different RRC message. Therefore CR is not needed. If insist, network restriction should be added with similar text to this. |
| Nokia | No | We don't think this is needed and can even be harmful: There are other cases when the field is used, and this is just one special case. Yes, we agree that in case the RLC bearer is released, UE does nothing, but UE also knows that. Is there an IODT problem if the field is included? Wouldn't this also change Rel-15 behaviour now for some UEs if they do check whether the field is included? Also, once again: The cover page is not acceptable as we don't say "consequences if not approved remain" in inter-operability analysis! |
| ZTE | No | In our understanding, when using release/addition, the “ the security key used for the radio bearer associated with this RLC entity changes ” is not satisfied anyway. So the CR is not needed. |

## 2.3	R2-2003694 and R2-2003695 Clarification on SCell release

Reason for change: According to the current version of 38.331, in 5.3.5.5.8, the SCell configuration is released but it is not mentioned what happens with parts of the UE configuration that have a parameter of type ServCellIndex referring to the deleted serving cell. For example, the CSI configuration of another (not deleted) serving cell of the same cell group can include a report to be sent on that serving cells: - CellGroupConfig-> sCellToAddModList-> SCellConfig-> sCellConfigDedicated-> ServingCellConfig->csi-MeasConfig-> csi-ReportConfigToAddModList-> CSI-ReportConfig-> carrier-> ServCellIndex - CellGroupConfig-> SpCellConfig-> spCellConfigDedicated-> ServingCellConfig-> csi-MeasConfig-> csi-ReportConfigToAddModList-> CSI-ReportConfig-> carrier-> ServCellIndex If the network does not explicitly remove any reference to a non-existing serving cell, it is unclear whether the UE will autonomously delete the corresponding items (as they cannot be used) or suspend them for later reuse and another serving cell is later configured with the same value of ServCellIndex.

Q) Do companies agree with the changes in the CR R2-2003694 (Rel-15) and R2-2003695 (Rel-16)?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| Huawei, HiSilicon | Yes | Proponent |
| Ericsson | No | We think the CR is not needed. At this stage, there should be no room for misinterpretation. We think it is obvious that if the gNB releases an SCell, it must remap CSI for other cells so they no longer point towards the released SCell. Therefore, we think the current text that only says that the UE releases the SCell can be kept because with "release" the SCell should not be interpreted that the UE autonomously changes parameters (CSI-mapping in this case) for other SCells. Further, we have the same formulation in LTE and we did not have any problems so far. Not sure we need to over-clarify in this case. |
| Qcom | Maybe not | Agree with Ericsson … not sure we need this change. |
| Nokia | No | This could create orphan configurations that make no sense. Normally we assume network is responsible for releasing the unused values where needed. And in most cases, it doesn't really matter if there are some configurations that are simply not used - if some stupid UE implementation goes berserk with those, well, that's a problem for the implementation, not for the system. So the question to ask is that if we do nothing and assume network does its job, what is broken? Because if we do as CR asks, the UE may release some mandatory(!) fields that are otherwise not possible to release, causing lot of confusion elsewhere. So it's safer not to do anything here. |
| ZTE | No | Similar view with Ericsson, better not to over-specify it. |

- 2.4	R2-2003670 and R2-2003671 Correction to RadioBearerConfig

Reason for change: - It is missing that the radioBearerConfig can be included in SN RRCReconfiguration message transmitted over EUTRA SRB1 to configure SRB3 in EN-DC and NGEN-DC. - It is not clear whether the radioBearerConfig can be used in NGEN-DC, NR-DC and NE-DC

Q) Do companies agree with the changes in the CR R2-2003694 (Rel-15) and R2-2003695 (Rel-16)?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| Huawei, HiSilicon | No | We think that "(NG)" is missing before "EN-DC" but no other change seems needed because there is no restriction for other cases. |
| Ericsson | No | P1: Disagree  In EN-DC, the SRB3 configuration is provided to MN in field scg-RB-Config of CG-Config, then encapsulated in nr-RadioBearerConfig1 or 2 of RRCConnectionReconfiguration message which MN sends to the UE, see field description: nr-RadioBearerConfig1, nr-RadioBearerConfig2 Includes the NR RadioBearerConfig IE as specified in TS 38.331 [82]. The field includes the configuration of RBs configured with NR PDCP. P2. Disagree  There is no need to clarify this. Only restrictions need to be mentioned. P3. Disagree  The behaviour is already clear from stage 3 field description. There is no need to update stage-2 as we have agreed long ago not to update stage-2 if behaviour is clear from stage-3. |
| Nokia | No | Agree with Huawei and Ericsson here. The text already seems clear from the understanding of usage of the RadioBearerConfig. |
| ZTE | No | Agree with Ericsson, for P1, the SRB3 radioBearerConfig is encapsulated in nr-RadioBearerConfig1 or 2 of LTE RRC message. As Huawei indicated, we can update EN-DC into (NG)EN-DC, and this can be captured in rapporteur’s CR. |

- 2.5	R2-2003778 Clarification on the using of RRCSetup in 38.331

Reason for change: RRCSetup can be used to establish SRB1, and can be used for RRC connection establishment, RRC connection re-establishment and RRC connection resume when falling to RRC connection establishment. However, in Annex B.1 Protection of RRC messages (informative), the comment for RRCSetup is Justification for A-I and A-C: the message can be sent in SRB0 in RRC_INACTIVE state, after the security is activated. - In which only RRC_INACTIVE in included, actually, CONNECTED state should also be included here, because RRCSetup can also be sent to a connected UE, for example, for RRC connection re-establishment when falling to RRC connection establishment, as discussed above.

Q) Do companies agree with the changes in the CR R2-2003778 (Rel-15)?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| Huawei, HiSilicon | Yes | Co-signed |
| Ericsson | Maybe (Rapporteur CR) | This change is a purely editorial as it impacts an informative text in an ANNEX. Even if we agree in principle with what the CR proposes, if also other companies want to do this change, this should be captured in the Rapporteur’s CR. |
| Qcom | Yes |  |
| Nokia | Rapporteur CR | If most companies agree to do something then this can be pushed to rapporteur CR. |
| ZTE | Yes |  |

# 3	Conclusion

Based on the discussion in the previous sections we propose the following: