3GPP TSG-RAN WG2 #109Bis-e	R2-2003826

E-meeting, April 2020							 									revision of R2-2002716

Agenda item:	6.1.4

Source: 	Samsung (rapporteur)

Title: 	Summary of IAB User Plane open issues and corrections

Document for:	Discussion and decision

# Introduction

This is a summary document, containing the overview of perceived impacts of remaining Rel-16 IAB work on the MAC specification (and other User Plane aspects – although the vast majority of the remaining issues is to do with the MAC layer), as captured in the companies’ submissions to this Agenda Item. Key areas of perceived impact are:

- Finalizing Pre-emptive BSR;

- Guard Symbols MAC CE(s) interpretation and remit;

- Newly agreed eLCID (one-byte extension) space and its use for IAB-specific MAC CEs;

- Miscellaneous issues (issues not easy to categorize and/or where there was only one submission per issue, but which are nevertheless captured in the summary).

The main identified issues match well the open issues document (please see R2-2002690) submitted to this meeting. In the present document, there is a main section for each of the above key areas, with some sections containing sub-sections. Each section or sub-section contains proposals drawn up by the rapporteur based on relevant submissions, and taking into account guidance from the Chair on aiming for consensus. The proposals are then repeated again in the Conclusions section, for RAN2’s attention.

# Pre-emptive BSR

## Cancellation of SR triggered by Pre-emptive BSR

In the latest version of the NR MAC spec (TS38.321v16.0.0), no distinction is made between cancellation procedures for SRs triggered by Pre-emptive BSR and BSRs other than Pre-emptive BSRs. This is the relevant text from the spec (Section 5.4.4):

‘Except for SCell beam failure recovery, all pending SR(s) for BSR triggered according to the BSR procedure (clause 5.4.5) prior to the MAC PDU assembly shall be cancelled and each respective sr-ProhibitTimer shall be stopped when the MAC PDU is transmitted, regardless of LBT failure indication from lower layers, and this PDU includes a Long or Short BSR MAC CE which contains buffer status up to (and including) the last event that triggered a BSR (see clause 5.4.5) prior to the MAC PDU assembly. Except for SCell beam failure recovery, all pending SR(s) for BSR triggered according to the BSR procedure (clause 5.4.5) shall be cancelled and each respective sr-ProhibitTimer shall be stopped when the UL grant(s) can accommodate all pending data available for transmission. Pending SR triggered prior to the MAC PDU assembly for beam failure recovery of an SCell shall be cancelled when the MAC PDU is transmitted and this PDU includes an SCell BFR MAC CE or truncated SCell BFR MAC CE which contains beam failure recovery information of that SCell. If all the SR(s) triggered for SCell beam failure recovery are cancelled the MAC entity shall stop sr-ProhibitTimer of corresponding SR configuration.’

The rapporteur would like to make the following observations:

- There are two conditions for cancelling the SR:

  - When the relevant BSR MAC CE is included in the transmitted MAC PDU; or

  - When the UL grant can accommodate all pending data.

- The condition a) also works for Pre-emptive BSR (although, as pointed out in [1] and [2], a clarification is needed to the underlined text, since Pre-emptive BSR does not use a Long or Short BSR MAC CE (it uses its own format, which admittedly is identical to Long BSR).

- The condition b) however is in rapporteur’s opinion a bigger problem (not raised in [1] but raised in [2]). If we make no changes, then the SR triggered by Pre-emptive BSR would be cancelled when the UL grant(s) can accommodate all pending data available for transmission. And we may not want that – what we would ideally like (in rapporteur’s understanding) is to make sure the Pre-emptive BSR is in fact transmitted in that case, since it is not linked to the currently pending data for transmission.

The rapporteur therefore proposes the following:

- SR triggered by (the impossibility to send) Pre-emptive BSR is only cancelled if a MAC PDU containing the relevant Pre-emptive BSR MAC CE is sent.

| Company | Comments |
| --- | --- |

- RAN2 to agree the following change to the MAC spec:

Except for SCell beam failure recovery, all pending SR(s) for BSR other than Pre-emptive BSR triggered according to the BSR procedure (clause 5.4.5) prior to the MAC PDU assembly shall be cancelled and each respective sr-ProhibitTimer shall be stopped when the MAC PDU is transmitted, regardless of LBT failure indication from lower layers, and this PDU includes a Long or Short BSR MAC CE which contains buffer status up to (and including) the last event that triggered a BSR (see clause 5.4.5) prior to the MAC PDU assembly. All pending SR(s) for Pre-emptive BSR triggered according to the BSR procedure (clause 5.4.5) prior to the MAC PDU assembly shall be cancelled and each respective sr-ProhibitTimer shall be stopped when the MAC PDU is transmitted, regardless of LBT failure indication from lower layers, and this PDU includes a Pre-emptive BSR MAC CE which contains expected change in buffer status up to (and including) the last event that triggered a Pre-emptive BSR (see clause 5.4.5) prior to the MAC PDU assembly. Except for SCell beam failure recovery, all pending SR(s) for BSR other than Pre-emptive BSR triggered according to the BSR procedure (clause 5.4.5) shall be cancelled and each respective sr-ProhibitTimer shall be stopped when the UL grant(s) can accommodate all pending data available for transmission. Pending SR triggered prior to the MAC PDU assembly for beam failure recovery of an SCell shall be cancelled when the MAC PDU is transmitted and this PDU includes an SCell BFR MAC CE or truncated SCell BFR MAC CE which contains beam failure recovery information of that SCell. If all the SR(s) triggered for SCell beam failure recovery are cancelled the MAC entity shall stop sr-ProhibitTimer of corresponding SR configuration.

| Company | Comments |
| --- | --- |

## Cancellation of Pre-emptive BSR

At RAN2#109-e, the following was agreed:

- Pre-emptive BSR shall be cancelled when a MAC PDU that contains the pre-emptive BSR MAC CE is sent. FFS other cancellation conditions, e.g. implementation specific.

In [3] it is proposed that we should consider leaving the cancellation of Pre-emptive BSR to implementation. The reasoning behind this is that “we left the triggering of Pre-emptive BSR largely up to implementation. Therefore, we believe that the IAB node should similarly have the flexibility to cancel a Pre-emptive BSR based on IAB node implementation” [3].

The rapporteur would like to share his understanding on the triggering conditions, which are in his understanding not left “largely up to implementation” as claimed in [3]:

- Whether Pre-emptive BSR is used or not is configurable;

- If it is used, then there are only two triggering conditions as specified in TS38.321v16.0.0;

- In other words, if an IAB node is configured to report Pre-emptive BSR, the triggering conditions are not left to implementation.

It was further rapporteur’s understanding that – while whether to use Pre-emptive BSR is configurable – if we do use it, we do need to send it (once triggered). Therefore the rapporteur proposes first of all for RAN2 to confirm this understanding:

- RAN2 to confirm that a Pre-emptive BSR – if configured, and once triggered (according to two triggering conditions as specified in TS38.321v16.0.0) – shall be sent.

| Company | Comments |
| --- | --- |

If this is indeed the current understanding and intention, then the cancellation should in rapporteur’s view (as well as according to views explicitly expressed in [6]) also be governed by the specs:

- RAN2 will not allow implementation-specific cancellation conditions of Pre-emptive BSR in Rel-16.

Otherwise, we would face (at least) two issues:

- Extending this (implementation-based) flexibility to SR cancellation, which defeats the purpose of agreeing to find a normative solution for scheduling latency reduction in the first place, and requires further discussion; and

- Failing to ensure consistency in multi-vendor environments.

| Company | Comments |
| --- | --- |

In [4], a different direction is taken – trying to add additional normative conditions for Pre-emptive BSR cancellation. More specifically, [4] proposes that Pre-emptive BSR can be cancelled when all or high priority expected data has been received after Pre-emptive BSR triggering. ‘High priority’ data would need to be defined and agreed by RAN2 and this would take up considerable time and is in rapporteur’s view not essential – rapporteur views this as optimization. However, cancelling Pre-emptive BSR when all data that Pre-emptive BSR refers to is received (and Pre-emptive BSR is yet to be sent) does appear to make sense and the rapporteur would like to put it forward for RAN2 consideration:

- RAN2 to discuss whether we should cancel a Pre-emptive BSR which has not yet been sent if all data this Pre-emptive BSR refers to is received by the node where the Pre-emptive BSR is triggered.

| Company | Comments |
| --- | --- |

## Editorial clarifications: whether to separate out Pre-emptive BSR into a separate section

For the moment, Pre-emptive BSR procedure is explained in the same section as “legacy” BSR (Section 5.4.5). Great care has been taken to make it clear which procedures and parameters apply only to Pre-emptive BSR. However, [5] proposes that Pre-emptive BSR should be captured in a separate section. This was already proposed at RAN2#109-e, but the proposal was rejected by RAN2.

While the rapporteur agrees with many of the observations made in [5], in reality the BSR section as it is right now (incorporating Pre-emptive BSR) is unambiguous and it is clear that Pre-emptive BSR only applies to IAB-MT, and which procedures and parameters apply only to Pre-emptive BSR. It is also unclear what is referred to exactly in [5] when the following is observed: “Separated section for pre-emptive BSR will minimize the impacts to legacy BSR procedure.” There has been no impact so far to the actual legacy BSR procedure itself. Perhaps what was meant is that editing becomes easier and disentangled.

Nevertheless it is perhaps worth discussing this again, just so it can be confirmed (or not, as the case may be):

- RAN2 to confirm the current approach to capturing Pre-emptive BSR procedure as part of Section 5.4.5 on “legacy” BSR.

| Company | Comments |
| --- | --- |

# Guard Symbols MAC CE

As a reminder, at the RAN2#109-e meeting in February, the following was agreed:

- RAN2 will design one single fixed-length Guard Symbols MAC CE, containing values (or indices mapped thereto) of all 8 parameters introduced by RAN1.

It was additionally agreed that the cell information should not be signalled explicitly, as captured in the following Editor’s Note to TS38.321v16.0.0:

Editors Note: The cell information is not signalled explicitly i.e. it is not included in the Guard Symbol MAC CE. It is FFS whether the information received in the Guard Symbol MAC CE applies only to the cell on which it is received, or to the entire cell group (if configured).

All the submissions to the present meeting treating the Guard Symbols MAC CE(s) focus on the yellow-highlighted issue immediately above. In summary, 3 options are proposed:

- Guard Symbol MAC CE applies to all the cells within the cell group: supported by 4 companies [7], [9], [11]

- Guard Symbol MAC CE applies only to the cell on which it is received: supported by 2 companies ([8], plus support for this option raised by Qualcomm in the pre-meeting email discussion)

- Revert existing agreement and explicitly include cell information needs in the Guard Symbols MAC CE: supported by 1 company [10]

Proponents of having an option to indicate different values of parameters for different cells within a cell group (Options 2 and 3 above) believe that the RAN1 agreement calls for this approach (something that is refuted by proponents of Option 1), and that different propagation delays for cells within a cell group mean that we need a per-cell granularity for the parameters in the Guard Symbols MAC CE(s) (this issue is considered to be negligible by proponents of Option 1).

An additional issue has been raised in the run-up to the meeting (RAN2 email discussion [Pre109bis-e][IAB] Summary of 6.1.4 (IAB User Plane open issues and corrections)) concerning cell-specific duplexing constraints. More specifically, a claim has been made that some cells of the MT CG may actually be able to transmit/receive during the guard symbols period if they have large enough frequency spacing to the collocated DU cells, while others cannot since they have comparatively small frequency spacing to the collocated DU cells. The argument is therefore that guard symbols should be specified per each cell within a cell group.

In rapporteur’s understanding, the need for guard symbols comes from the discrepancy between UL RX timing and DL TX timing within the IAB-DU. So, if cells within a cell group are controlled by one DU, the issues of cell-specific duplexing constraints raised above should not apply.

And finally, on top of this, the company that wishes to explicitly indicate the cell information in the Guard Symbols MAC CE (thereby reverting the existing agreement, as in Option 3) claims that this is required for cross-carrier scheduling [10].

Given the existing agreement that the cell information should not be signalled explicitly (and that only one company wishes to revert this agreement), and the numbers which speak in favour of Option 1, the rapporteur of the discussion proposes the following:

- The information contained in the Guard Symbol MAC CE applies to the entire cell group.

| Company | Comments |
| --- | --- |

Based on submissions received, and if Proposal 7 is agreed, it would appear there is no further work required for RAN2. In other words, there are no proposals or intention in specifying how the parameters are chosen. The following is therefore additionally proposed:

- RAN2 will not specify the information or procedure used to derive Guard Symbol MAC CE parameters which apply across all cells within a cell group.

| Company | Comments |
| --- | --- |

# LCID space issues

The 5 submissions treating these issues focus on two key issues – these issues and key options are summarized in the Table below:

| Issue | Proposed solutions |
| --- | --- |
| Whether to keep the reserved values in the two-byte eLCID space | - Yes: [14], [15] - No: [12], [13], [1] |
| What to do with the IAB-related MAC CEs | - Put them all in set2 of the one-byte eLCID space: [12], [13], [14], [15], [1] - Decide whether it is set2 or set1 of the one-byte eLCID space on case-by-case basis, based on priority and frequency: [15] |

The split on the first issue seems fairly even. This issue is in rapporteur’s view. Additionally, since RAN3 have further limited the number of BH RLC channels to 2^14, and we will therefore have more IDs than we need to identify all BH RLC channels for the case of ‘full’ 1:1 mapping, the rapporteur feels it is ok to keep the reserved values in the two-byte eLCID space, and this would be rapporteur’s preferred proposal. However, there is some opposition to this so the most we can do right now is propose to discuss this issue further:

- RAN2 to discuss whether to keep the reserved values in the two-byte eLCID space.

| Company | Comments |
| --- | --- |

On the second issue, the situation is much clearer:

- All of the MAC CEs introduced by the IAB WI shall have their identifiers selected from set2 of the one-byte eLCID space.

| Company | Comments |
| --- | --- |

One example of a suitable starting point to implement Proposal 10 is given in [14].

# Miscellaneous issues

PDCP duplication support for DRB

[16] deals with PDCP duplication in IAB networks. The contribution points out that – for DRB duplication – the state can be dynamically controlled using a MAC CE; since (according to [16]) the parent node has no sufficient information to control the state of duplication, the contribution argues against support of PDCP duplication for DRBs.

In rapporteur’s understanding, IAB links should be sufficiently stable for SRB transmission, and SRB duplication may not be essential. This being said, if use of split SRB for IAB is agreed, adding duplication feature will not require much effort. As for the DRBs, reliability can be under control of NW, and duplication for the case of IAB may not be essential, as pointed out in [16].

This being said, it would appear that confirmation from RAN2 on these issues is required:

- For the case of DC in IAB, RAN2 to discuss:
- whether split SRB should be supported;
- whether PDCP duplication for SRBs should be supported; and
- whether PDCP duplication for DRBs should be supported.

| Company | Comments |
| --- | --- |

Remaining RACH issues

In addition to RACH-ConfigCommon (available to all UEs and IAB-MTs), RAN1 introduced RACH-ConfigCommonIAB specifically for IAB-MTs. RAN1 recommends that, if the IAB specific RACH configuration is not provided, that the IAB node will use the configured Rel-15 RACH configuration for IAB node initial access. [17] would like to ensure that the IAB specific RACH configuration is prioritized, if configured. [17] would like this to be clarified in the MAC or RRC spec. It is rapporteur’s understanding that this is already RAN1’s intention – but it should be ok for RAN2 to discuss whether this should also be captured in RAN2’s specs.

In [17], a request is additionally made to clarify the case of Msg1 based SI request: if RACH-ConfigGeneric is not configured, in this specific case [17] points out that IAB-MT would use RACH-ConfigCommon for UE, rather than the RACH-ConfigCommonIAB for IAB (even if configured). This is different with the other RA case, where IAB specific RACH is prioritized. And finally, [17] would like to make an additional clarification that IAB-specific parameters will be applied to RACH-ConfigCommon for Rel-15 UE if the RACH-ConfigCommonIAB is not configured – in other words, that prach-ConfigurationPeriodScaling, prach-ConfigurationFrameOffset, prach-ConfigurationSOffset can be used/configured without dependence on RACH-ConfigCommonIAB.

- RAN2 to confirm the following understanding, and discuss whether action is needed to capture all or some of the below in the RAN2 specs:
- the IAB specific RACH configuration is prioritized, if configured;
- for the case of Msg1-based SI request, IAB-MT should always use RACH-ConfigCommon if configured; and
- prach-ConfigurationPeriodScaling, prach-ConfigurationFrameOffset, prach-ConfigurationSOffset can be used/configured without dependence on RACH-ConfigCommonIAB.

| Company | Comments |
| --- | --- |

Introducing a section for handling of Tdelta MAC CE

In the current version of the MAC specification, we have a section on the Timing Delta MAC CE itself, but not a section describing related procedural aspects, such as the one we have for Guard Symbols MAC CE and most other MAC CEs closely related to work done in other WGs. [18] therefore proposes that a short section on procedural aspects for Timing offset adjustment for IAB should be introduced. It further proposes to place this section within overarching Section 5.18 in the NR MAC, which is typically used for the MAC CEs from other WGs.

- RAN2 to discuss introducing a short section on procedural aspects for Timing offset adjustment for IAB.

| Company | Comments |
| --- | --- |

- RAN2 to discuss moving the sections on Timing offset adjustment for IAB and Guard Symbols to 5.18.x.

| Company | Comments |
| --- | --- |

One example of a suitable starting point to implement Proposal 13 is given in [18].

# Conclusions

In this summary tdoc being submitted to RAN2#109Bis-e, we presented the overview of key issues pertaining to the impacts of remaining Rel-16 IAB work on the MAC specification, as captured in companies’ submissions to this Agenda Item. Based on this overview, the majority views, and the focus on finalizing existing design without reversing decisions already made or introducing any new unnecessary functionality, the rapporteur has made the following proposals for RAN2’s consideration:

- SR triggered by (the impossibility to send) Pre-emptive BSR is only cancelled if a MAC PDU containing the relevant Pre-emptive BSR MAC CE is sent.

- RAN2 to agree the following change to the MAC spec:

Except for SCell beam failure recovery, all pending SR(s) for BSR other than Pre-emptive BSR triggered according to the BSR procedure (clause 5.4.5) prior to the MAC PDU assembly shall be cancelled and each respective sr-ProhibitTimer shall be stopped when the MAC PDU is transmitted, regardless of LBT failure indication from lower layers, and this PDU includes a Long or Short BSR MAC CE which contains buffer status up to (and including) the last event that triggered a BSR (see clause 5.4.5) prior to the MAC PDU assembly. All pending SR(s) for Pre-emptive BSR triggered according to the BSR procedure (clause 5.4.5) prior to the MAC PDU assembly shall be cancelled and each respective sr-ProhibitTimer shall be stopped when the MAC PDU is transmitted, regardless of LBT failure indication from lower layers, and this PDU includes a Pre-emptive BSR MAC CE which contains expected change in buffer status up to (and including) the last event that triggered a Pre-emptive BSR (see clause 5.4.5) prior to the MAC PDU assembly. Except for SCell beam failure recovery, all pending SR(s) for BSR other than Pre-emptive BSR triggered according to the BSR procedure (clause 5.4.5) shall be cancelled and each respective sr-ProhibitTimer shall be stopped when the UL grant(s) can accommodate all pending data available for transmission. Pending SR triggered prior to the MAC PDU assembly for beam failure recovery of an SCell shall be cancelled when the MAC PDU is transmitted and this PDU includes an SCell BFR MAC CE or truncated SCell BFR MAC CE which contains beam failure recovery information of that SCell. If all the SR(s) triggered for SCell beam failure recovery are cancelled the MAC entity shall stop sr-ProhibitTimer of corresponding SR configuration.

- RAN2 to confirm that a Pre-emptive BSR – if configured, and once triggered (according to two triggering conditions as specified in TS38.321v16.0.0) – shall be sent.

- RAN2 will not allow implementation-specific cancellation conditions of Pre-emptive BSR in Rel-16.

- RAN2 to discuss whether we should cancel a Pre-emptive BSR which has not yet been sent if all data this Pre-emptive BSR refers to is received by the node where the Pre-emptive BSR is triggered.

- RAN2 to confirm the current approach to capturing Pre-emptive BSR procedure as part of Section 5.4.5 on “legacy” BSR.

- The information contained in the Guard Symbol MAC CE applies to the entire cell group.

- RAN2 will not specify the information or procedure used to derive Guard Symbol MAC CE parameters which apply across all cells within a cell group.

- RAN2 to discuss whether to keep the reserved values in the two-byte eLCID space.

- All of the MAC CEs introduced by the IAB WI shall have their identifiers selected from set2 of the one-byte eLCID space.

- For the case of DC in IAB, RAN2 to discuss:
- whether split SRB should be supported;
- whether PDCP duplication for SRBs should be supported; and
- whether PDCP duplication for DRBs should be supported.

- RAN2 to confirm the following understanding, and discuss whether action is needed to capture all or some of the below in the RAN2 specs:
- the IAB specific RACH configuration is prioritized, if configured;
- for the case of Msg1-based SI request, IAB-MT should always use RACH-ConfigCommon if configured; and
- prach-ConfigurationPeriodScaling, prach-ConfigurationFrameOffset, prach-ConfigurationSOffset can be used/configured without dependence on RACH-ConfigCommonIAB.

- RAN2 to discuss introducing a short section on procedural aspects for Timing offset adjustment for IAB.

- RAN2 to discuss moving the sections on Timing offset adjustment for IAB and Guard Symbols to 5.18.x.

# Reference

- R2-2003019, “Discussion on the SR cancelation for pre-BSR and LCID values for IAB”, Huawei, HiSilicon.

- R2-2003644, “Issue of SR triggered by Pre-emptive BSR”, ASUSTeK.

- R2-2003720, “Remaining issues for IAB MAC”, Futurewei.

- R2-2002890, “Remaining Issues of Preemptive BSR”, vivo.

- R2-2003018, “Clarification of BSR and Pre-emptive BSR”, Huawei, HiSilicon.

- R2-2002852, “Discussion on IAB User plane aspects”, ZTE, Sanechips.

- R2-2002679, “On interpretation and use of the Guard Symbols MAC CE”, Samsung, CATT.

- R2-2002852, “Discussion on IAB User plane aspects”, ZTE, Sanechips.

- R2-2002999, “Determining a cell to apply a Guard Symbols MAC CE”, LG Electronics Inc.

- R2-2003016, “Remaining issues of Guard Symbols MAC CE”, Huawei, HiSilicon.

- R2-2003048, “Cell information in Guard symbols MAC CE”, Nokia, Nokia Shanghai Bell.

- R2-2003000, “Consideration on LCID set for IAB MAC CE and reserved LCID values”, LG Electronics Inc.

- R2-2003049, “Handling of IAB specific MAC CEs”, Nokia, Nokia Shanghai Bell.

- R2-2003359, “Change LCID to eLCID for IAB MAC CEs”, Ericsson.

- R2-2002680, “Open issues with IAB LCID space extension”, Samsung.

- R2-2003098, “PDCP duplication in IAB DC v2.0”, Lenovo, Motorola Mobility.

- R2-2003017, “Clarification on the RACH configuration used in MAC procedure for IAB”, Huawei, HiSilicon.

- R2-2002715, “CR (Introducing a section for handling of Tdelta MAC CE)”, Samsung.