3GPP TSG-RAN WG2 Meeting #110e	R2-2005557

Electronic 1 – 12 June, 2020

Agenda Item:	5.3.1.1

Source: 	ASUSTeK

Title:  	Discussion on presence of IEs in BeamFailureRecoveryConfig

Document for:	Discussion and Decision

- Introduction

We found the contradiction on the presence of rsrp-ThresholdSSB, powerRampingStep, preambleReceivedTargetPower, preambleTransMax, and ra-ResponseWindow in BeamFailureRecoveryConfig between NR MAC and RRC specifications. And we propose options for clarification on the expected behaviours.

- Discussion

According to NR MAC specification [1], MAC layer assumes rsrp-ThresholdSSB, powerRampingStep, preambleReceivedTargetPower, preambleTransMax, and ra-ResponseWindow are configured by BeamFailureRecoveryConfig for contention-free Random Access (CFRA) for SpCell beam failure recovery:

### 5.1.1 Random Access procedure initialization […] RRC configures the following parameters for the Random Access procedure: […] - rsrp-ThresholdSSB: an RSRP threshold for the selection of the SSB. If the Random Access procedure is initiated for beam failure recovery, rsrp-ThresholdSSB used for the selection of the SSB within candidateBeamRSList refers to rsrp-ThresholdSSB in BeamFailureRecoveryConfig IE; […] When the Random Access procedure is initiated on a Serving Cell, the MAC entity shall: […] 1> if beamFailureRecoveryConfig is configured for the active UL BWP of the selected carrier: 2> start the beamFailureRecoveryTimer, if configured; 2> apply the parameters powerRampingStep, preambleReceivedTargetPower, and preambleTransMax configured in the beamFailureRecoveryConfig;

### 5.1.4 Random Access Response reception Once the Random Access Preamble is transmitted and regardless of the possible occurrence of a measurement gap, the MAC entity shall: 1> if the contention-free Random Access Preamble for beam failure recovery request was transmitted by the MAC entity: 2> start the ra-ResponseWindow configured in BeamFailureRecoveryConfig at the first PDCCH occasion as specified in TS 38.213 [6] from the end of the Random Access Preamble transmission; […] 1> if ra-ResponseWindow configured in BeamFailureRecoveryConfig expires and if a PDCCH transmission on the search space indicated by recoverySearchSpaceId addressed to the C-RNTI has not been received on the Serving Cell where the preamble was transmitted; or

However, according to NR RRC specification [2], rsrp-ThresholdSSB and rach-ConfigBFR (including powerRampingStep, preambleReceivedTargetPower, preambleTransMax, and ra-ResponseWindow) are actually optional in BeamFailureRecoveryConfig:

BeamFailureRecoveryConfig ::=       SEQUENCE {

rootSequenceIndex-BFR               INTEGER (0..137)                    OPTIONAL, -- Need M

rach-ConfigBFR                      RACH-ConfigGeneric                  OPTIONAL, -- Need M

rsrp-ThresholdSSB                   RSRP-Range                          OPTIONAL, -- Need M

candidateBeamRSList                 SEQUENCE (SIZE(1..maxNrofCandidateBeams)) OF PRACH-ResourceDedicatedBFR   OPTIONAL, -- Need M

ssb-perRACH-Occasion                ENUMERATED {oneEighth, oneFourth, oneHalf, one, two,

four, eight, sixteen}OPTIONAL, -- Need M

ra-ssb-OccasionMaskIndex            INTEGER (0..15)                     OPTIONAL, -- Need M

recoverySearchSpaceId               SearchSpaceId                       OPTIONAL, -- Need R

ra-Prioritization                   RA-Prioritization                   OPTIONAL, -- Need R

beamFailureRecoveryTimer            ENUMERATED {ms10, ms20, ms40, ms60, ms80, ms100, ms150, ms200}            OPTIONAL, -- Need M

...,

[[

msg1-SubcarrierSpacing              SubcarrierSpacing                   OPTIONAL  -- Need M

]]

}

Observation:	MAC layer does not handle the scenario when CFRA is performed for SpCell beam failure recovery and rsrp-ThresholdSSB, powerRampingStep, preambleReceivedTargetPower, preambleTransMax, and ra-ResponseWindow are not configured by BeamFailureRecoveryConfig.

Hence, we think that the clarification on the presence of rsrp-ThresholdSSB, powerRampingStep, preambleReceivedTargetPower, preambleTransMax, and ra-ResponseWindow in BeamFailureRecoveryConfig is needed and there are two options:

Opt 1:	network always configures these parameters by BeamFailureRecoveryConfig when CFRA resources for BFR are configured.

Opt 2:	if these parameters are not configured by BeamFailureRecoveryConfig, the UE use the corresponding values provided in RACH-ConfigCommon for beam failure recovery.

Proposal:	Select one of the options to clarify the presence of rsrp-ThresholdSSB, powerRampingStep, preambleReceivedTargetPower, preambleTransMax, and ra-ResponseWindow in BeamFailureRecoveryConfig.

- Conclusion

We have the following observations and proposal:

Observation:	MAC layer does not handle the scenario when CFRA is performed for SpCell beam failure recovery and rsrp-ThresholdSSB, powerRampingStep, preambleReceivedTargetPower, preambleTransMax, and ra-ResponseWindow are not configured by BeamFailureRecoveryConfig.

Proposal:	Select one of the options to clarify the presence of rsrp-ThresholdSSB, powerRampingStep, preambleReceivedTargetPower, preambleTransMax, and ra-ResponseWindow in BeamFailureRecoveryConfig.

Opt 1:	network always configures these parameters by BeamFailureRecoveryConfig when CFRA resources for BFR are configured.

Opt 2:	if these parameters are not configured by BeamFailureRecoveryConfig, the UE use the corresponding values provided in RACH-ConfigCommon for beam failure recovery.

[3][4] are CR corresponding to option 1 and [5][6] are CR corresponding to option 2.

- Reference

- TS 38.321 V15.8.0, NR MAC specification.

- TS 38.331 V15.9.0, NR RRC specification.

- R2-2005558, CR1679, Clarification on presence of IEs in BeamFailureRecoveryConfig.

- R2-2005559, CR1680, Clarification on presence of IEs in BeamFailureRecoveryConfig.

- R2-2005560, CR0754, Handling on absence of IEs in BeamFailureRecoveryConfig.

- R2-2005561, CR0755, Handling on absence of IEs in BeamFailureRecoveryConfig.