3GPP TSG RAN WG2 #110-e	draft R2-20xxxxx

Electronic meeting, 1 – 12 Jun 2020

Source: 	Huawei, HiSilicon

Title: 	[draft] Summary for Offline [008][NR15] Mobility

Agenda Item:	5.4.1.1

Document for:	Discussion and decision

# Introduction

This document contains a list of TDocs covered in the following offline discussion:

- [AT110e][008][NR15] Mobility (Huawei)

Scope: Treat R2-2004768, R2-2004769, R2-2005270, R2-2005271, R2-2005703, R2-2005704, R2-2005636, R2-2005637 (proponents are responsible to explain and drive)

Part 1: Decision whether to make corrections or not, identify agreeable corrections. Deadline: June 4, 0700 UTC.

Part 2: For agreeable parts, continuation to agree CRs. Deadline: June 10, 0700 UTC

Companies are invited to share their views on each TDoc submitted.

# Discussion

## R2-2004768, R2-2004769

Regarding which issues that need resolution, it is suggested to use the reason for change as the input:

The current field description of ran-Area is incorrect since it is indicated that RAN area code(s) can be used alone it is indicated that the network uses only TA code(s) or RAN area code(s) to configure a UE. According to 38.300 description and ASN.1 structure, the RAN area code(s) shall be configured together with TA code(s).

In R2-2004768, R2-2004769, it is proposed to correct the field description of ran-Area as below.

ran-Area Indicates whether TA code(s) or RAN area code(s) are used for the RAN notification area. The network uses only TA code(s) or both TA code(s) and RAN area code(s) to configure a UE. The total number of TACs across all PLMNs does not exceed 16.

Q1.1: Do you agree with the reason for change?

| Company | YES/NO | Comments |
| --- | --- | --- |

Q1.2: If your answer is Yes for Q1.1, do you agree with the changes?

| Company | YES/NO | Comments |
| --- | --- | --- |

## R2-2005270, R2-2005271

Q2: Do you agree with the changes?

| Company | YES/NO | Comments |
| --- | --- | --- |

## R2-2005703, R2-2005704

Problem statement:

Issue 1:

1>	if the UE is configured with E-UTRA nr-SecondaryCellGroupConfig (UE in (NG)EN-DC):

2>	if the RRCReconfiguration message was received via E-UTRA SRB1 as specified in TS 36.331 [10]:

3>	submit the RRCReconfigurationComplete via E-UTRA embedded in E-UTRA RRC message RRCConnectionReconfigurationComplete as specified in TS 36.331 [10];

When performing HO from EN-DC to NR:

1) The UE is in EN-DC, so the yellow part applies;

2) UE receives MobilityFromEUTRACommand (which contains an RRCReconfiguration) via LTE SRB1, so the green part applies.

As a result, the blue part will be executed, that’s where the problem lies:

UE should send the RRCReconfigurationComplete directly to target NR node, instead of embedding it in an LTE message.

Issue 2:

1>	else (RRCReconfiguration was received via SRB1):

2>	submit the RRCReconfigurationComplete message via SRB1 to lower layers for transmission using the new configuration;

Since it is in 38.331, the SRB1 here refers to NR SRB1, therefore inter-RAT HO to NR is excluded, which is wrong.

Q3: Do you agree with the issues?

| Company | YES/NO | Comments |
| --- | --- | --- |

Solutions to Issue 1:

Option A: Explain that UE should send the RRCReconfigurationComplete directly to the target NR node. (That’s the change provided in 2005703/2005704);

Option B: Prevent the UE from entering “1> if the UE is configured with E-UTRA nr-SecondaryCellGroupConfig (UE in (NG)EN-DC)” :

1>  if the RRCReconfiguration message was not received within the E-UTRA MobilityFromEUTRACommand message, as specified in TS 36.331 [10]; and

1>  if the UE is configured with E-UTRA nr-SecondaryCellGroupConfig (UE in (NG)EN-DC):

Q4: For Issue 1, which solution do you prefer?

| Company | Option A/B | Comments |
| --- | --- | --- |

Solution to Issue 2:

1>  else (RRCReconfiguration was received via SRB1 or within the MobilityFromEUTRACommand message):

Q5: For Issue 2, do you agree with the solution?

| Company | YES/NO | Comments |
| --- | --- | --- |

## R2-2005636, R2-2005637

Q6: Do you agree with the changes?

| Company | YES/NO | Comments |
| --- | --- | --- |

# Conclusion

To be added.