3GPP TSG RAN WG2 #110-e	draft R2-20xxxxx

Electronic meeting, 1 – 12 Jun 2020

Source: 	Huawei, HiSilicon

Title: 	[draft] Summary for Offline [008][NR15] Mobility

Agenda Item:	5.4.1.1

Document for:	Discussion and decision

# Introduction

This document contains a list of TDocs covered in the following offline discussion:

- [AT110e][008][NR15] Mobility (Huawei)

Scope: Treat R2-2004768, R2-2004769, R2-2005270, R2-2005271, R2-2005703, R2-2005704, R2-2005636, R2-2005637 (proponents are responsible to explain and drive)

Part 1: Decision whether to make corrections or not, identify agreeable corrections. Deadline: June 4, 0700 UTC.

Part 2: For agreeable parts, continuation to agree CRs. Deadline: June 10, 0700 UTC

Companies are invited to share their views on each TDoc submitted.

# Discussion

## R2-2004768, R2-2004769

Regarding which issues that need resolution, it is suggested to use the reason for change as the input:

The current field description of ran-Area is incorrect since it is indicated that RAN area code(s) can be used alone it is indicated that the network uses only TA code(s) or RAN area code(s) to configure a UE. According to 38.300 description and ASN.1 structure, the RAN area code(s) shall be configured together with TA code(s).

In R2-2004768, R2-2004769, it is proposed to correct the field description of ran-Area as below.

ran-Area Indicates whether TA code(s) or RAN area code(s) are used for the RAN notification area. The network uses only TA code(s) or both TA code(s) and RAN area code(s) to configure a UE. The total number of TACs across all PLMNs does not exceed 16.

Q1.1: Do you agree with the reason for change?

| Company | YES/NO | Comments |
| --- | --- | --- |
| Ericsson | Maybe | From an ASN.1 point of view and from 38.300, is already clear that the RAN area code should be used together with the TA code. For this reason, we do not think that the proposed change is critical. However, if majority of companies believe that this is necessary, we would fine to include this in the Rapporteur’s CR. |
| MediaTek | Maybe | Similar view as Ericsson, it is not essential and if necessary could be include in Rapporteur’s CR |
| Nokia | Disagree | There is a point but don’t know how this changes anything. |
| Huawei | Maybe | Same view with Ericsson and MediaTek. |
| Apple | Yes | We are the proponent company, and would like to avoid any ambiguity in the spec. We are fine to include this in the Rapporteur’s CR. |
| Samsung | Maybe | Not essential correction i.e. it is already clear in ASN.1 structure. Rap CR seems fine if it is the majority's preference. |

Q1.2: If your answer is Yes for Q1.1, do you agree with the changes?

| Company | YES/NO | Comments |
| --- | --- | --- |
| Ericsson | Maybe No | From an ASN.1 point of view and from 38.300, is already clear that the RAN area code should be used together with the TA code. For this reason, we do not think that the proposed change is critical. However, if majority of companies believe that this is necessary, we would fine to include this in the Rapporteur’s CR. |
| MediaTek | Maybe not | We are not sure the original intention of adding the sentence “The network uses only TA code(s) and RAN area code(s) to configure a UE”. With the additional code, it looks correct but useless. Our suggestion would be simply delete the whole sentence in Rapporteur’s CR. |
| Nokia | Disagree | There is a point but don’t know how this changes anything. |
| Huawei | Maybe | The change is correct but not essential, can be merged to the rapporteur CR. |
| Apple | Yes | We understand it is not critical, and we are fine to include it in the Rapporteur’s CR. |
| Samsung | Maybe not | See comment in Q1.1 |

## R2-2005270, R2-2005271

Q2: Do you agree with the changes?

| Company | YES/NO | Comments |
| --- | --- | --- |
| Ericsson | Yes | We are the proponent company. The reason why we brought this CRs is because, first, we want to align the NR specification to the LTE ones regarding the stop of the timers during mobility from NR. Further, stopping T309/390 is clearly needed to ensure that the UE will not refrain from accessing the target RAN due to current UAC restrictions. Nevertheless, also timer T310 need to be stopped to avoid the UE to reverts back to the source RAT and triggers RRC re-establishment, in case T310 expires during ongoing inter RAT handover. Please, note that in 36.331, the UE procedure upon receiving mobilityFromEUTRACommand starts by stopping timers T310, T312, and T309 [36.331, section 5.4.3.3]. |
| MediaTek | Yes | Looks reasonable to us. |
| Nokia | Yes | This is to align with the handling in LTE. It is reasonable to stop T310 when UE receiving Handover Command, to avoid UE trigger re-establishment during handover. Addtionally, for NR, Rel-16 introduced T312 should be stopped as well. |
| Huawei | Yes | Agree that UE should stop T310 upon inter-RAT HO. |
| Apple | Yes | It’s reasonable. |
| Samsung | Yes | We are fine to stop T310. |

## R2-2005703, R2-2005704

Problem statement:

Issue 1:

1>	if the UE is configured with E-UTRA nr-SecondaryCellGroupConfig (UE in (NG)EN-DC):

2>	if the RRCReconfiguration message was received via E-UTRA SRB1 as specified in TS 36.331 [10]:

3>	submit the RRCReconfigurationComplete via E-UTRA embedded in E-UTRA RRC message RRCConnectionReconfigurationComplete as specified in TS 36.331 [10];

When performing HO from EN-DC to NR:

1) The UE is in EN-DC, so the yellow part applies;

2) UE receives MobilityFromEUTRACommand (which contains an RRCReconfiguration) via LTE SRB1, so the green part applies.

As a result, the blue part will be executed, that’s where the problem lies:

UE should send the RRCReconfigurationComplete directly to target NR node, instead of embedding it in an LTE message.

Issue 2:

1>	else (RRCReconfiguration was received via SRB1):

2>	submit the RRCReconfigurationComplete message via SRB1 to lower layers for transmission using the new configuration;

Since it is in 38.331, the SRB1 here refers to NR SRB1, therefore inter-RAT HO to NR is excluded, which is wrong.

Q3: Do you agree with the issues?

| Company | YES/NO | Comments |
| --- | --- | --- |
| Ericsson | No | Our understanding is that having fullconfig mandatory in EN-DC to NR handover it was a mistake made by RAN2. So basically, the fullconfig flag needs to be included for this case but it has no meaning (in my understanding) – since once the UE is performing handover from EN-DC to NR, it should check its target configuration solely against what the target node provided (there is no delta in this case). According to this, our point is that for inter-RAT handover, in general, it would not make sense to take into account the configuration on the source node i.e. the UE would anyway have a new “pointer” to the target RAT configuration (while it has to anyway keep the pointer to the source RAT configuration, since it has to revert back to such configuration if the handover fails at the end). Therefore, such procedures should be handled independently. This mean that when the UE executes 5.3.5.3, the UE is considering itself as in standalone and therefore it will not enter in this if statement: 1> if the UE is configured with E-UTRA nr-SecondaryCellGroupConfig (UE in (NG)EN-DC): But it will rather enter in this one: 1> else (RRCReconfiguration was received via SRB1): 2> submit the RRCReconfigurationComplete message via SRB1 to lower layers for transmission using the new configuration; 2> if this is the first RRCReconfiguration message after successful completion of the RRC re-establishment procedure: 3> resume SRB2 and DRBs that are suspended; So maybe it would be also ok to have a note by clarifying that when performing inter-RAT handover, the UE only cares about the target node configuration, unless stated otherwise (this last sentence would be due to NR to LTE/5GC case, where we do have some delta configuration). However, we figure out that one part in this procedure is missing and that is the case when fullconfig is included during handover from E-UTRA/5GC to NR. In such a case, we have: 1> if the RRCReconfiguration is received via other RAT (i.e., inter-RAT handover to NR): 2> if the RRCReconfiguration does not include the fullConfig and the UE is connected to 5GC (i.e., delta signalling during intra 5GC handover): 3> re-use the source RAT SDAP and PDCP configurations if available (i.e., current SDAP/PDCP configurations for all RBs from source E-UTRA RAT prior to the reception of the inter-RAT HO RRCReconfiguration message); 1> else: 2> if the RRCReconfiguration includes the fullConfig: 3> perform the full configuration procedure as specified in 5.3.5.11; That is, the highlighted part is executed only for the intra-RAT handover case. But in the field descriptions for fullconfig, we have the condition: FullConfig; The field is mandatory present in case of inter-system handover from E-UTRA/EPC to NR. It is optionally present, Need N, during reconfiguration with sync and also in first reconfiguration after reestablishment; or for intra-system handover from E-UTRA/5GC to NR. It is absent otherwise.. Thus, our understanding is that when we do handover from EN-DC to NR, we need to set the fullconfig flag, while for handover from NGEN-DC to NR (or eLTE to NR), fullconfig is optional (as we can reuse SDAP/PDCP configuration). Thus, the above line in the RRCReconfiguration should have been: 1> if the RRCReconfiguration is received via other RAT (i.e., inter-RAT handover to NR): 2> if the RRCReconfiguration does not include the fullConfig and the UE is connected to 5GC (i.e., delta signalling during intra 5GC handover): 3> re-use the source RAT SDAP and PDCP configurations if available (i.e., current SDAP/PDCP configurations for all RBs from source E-UTRA RAT prior to the reception of the inter-RAT HO RRCReconfiguration message); 1> else: 12> if the RRCReconfiguration includes the fullConfig: 23> perform the full configuration procedure as specified in 5.3.5.11; |
| MediaTek | No | UE is no longer configured with EN-DC after EN-DC -> NR handover configuration is performed. The UE is configured with NR SA. The last else branch would be used in this case 1> else (RRCReconfiguration was received via SRB1): So, we think the proposed change is incorrect. We also think that the full configuration related change proposed by Ericsson is NOT needed. In case of inter-system handover, all source configuration is cleared no matter the fullConfig flag is set or not. This issue has been discussed before and we think that the current text is fine. |
| Nokia | No | If UE handover from EN-DC to NR, it should follow below in 5.3.5.3 instead of UE is configured with EN-DC (after HO). 1> else (RRCReconfiguration was received via SRB1): 2> submit the RRCReconfigurationComplete message via SRB1 to lower layers for transmission using the new configuration; |
| Huawei | Yes | [Proponent] First, in response to Ericsson’s comments: The changes in this CR does not involve the text related to fullConfig. Our CR only trie to correct UE behavior of sending RRCReconfigurationComplete in target NR side for HO from ENDC to NR. BTW, in this case, UE even will not look at the full config flag. And we agree with MediaTek that the issue related to fullConfig has been discussed and nothing is wrong with the current text. Second, in response to MediaTek and Nokia’s comments: Our understanding is that when UE perform inter-RAT HO, UE will release source config after HO success. So when UE apply the NR configuration in HO command, UE still is configured with EN-DC. This is why we think according to current spec, UE would go to this branch “1> if the UE is configured with E-UTRA nr-SecondaryCellGroupConfig (UE in (NG)EN-DC):”. |
| Apple | No | We have same understanding ans Nokia and MediaTek that UE should follow the else branch for the handover from EN-DC to NR. 1> else (RRCReconfiguration was received via SRB1): |
| Samsung | No | We agree with Ericsson that the concerned part is only for intra-RAT mobility (i.e. it is for NR SCG management). Although we can acknowledge that this section is a bit complicated we think the intention is clear and see no need for any update. Like MediaTek, we are not convinced there is a need for changes regarding fullConfig as suggested by Ericsson. |

Solutions to Issue 1:

Option A: Explain that UE should send the RRCReconfigurationComplete directly to the target NR node. (That’s the change provided in 2005703/2005704);

Option B: Prevent the UE from entering “1> if the UE is configured with E-UTRA nr-SecondaryCellGroupConfig (UE in (NG)EN-DC)” :

1>  if the RRCReconfiguration message was not received within the E-UTRA MobilityFromEUTRACommand message, as specified in TS 36.331 [10]; and

1>  if the UE is configured with E-UTRA nr-SecondaryCellGroupConfig (UE in (NG)EN-DC):

Q4: For Issue 1, which solution do you prefer?

| Company | Option A/B | Comments |
| --- | --- | --- |
| Ericsson | No | See comment to Q1. We can add a note to clarify that by clarifying that when performing inter-RAT handover, the UE only cares about the target node configuration, unless stated otherwise (this last sentence would be due to NR to LTE/5GC case, where we do have some delta configuration). |
| MediaTek | No | We think no change is needed for this. |
| Nokia | No | We think no change is needed for this. |
| Huawei | Option A or B | As stated in Q3, before the HO is finished (in other words, upon receiving the HO command), UE is still in EN-DC, so UE anyway will fit into “UE is configured with E-UTRA nr-SecondaryCellGroupConfig (UE in (NG)EN-DC)”. |
| Samsung | No | No change is needed. |

Solution to Issue 2:

1>  else (RRCReconfiguration was received via SRB1 or within the MobilityFromEUTRACommand message):

Q5: For Issue 2, do you agree with the solution?

| Company | YES/NO | Comments |
| --- | --- | --- |
| Ericsson | No | See commend to Q1. If the note is adopted, there is no need for specification change. |
| MediaTek | Maybe | We are fine to the following change. We think that it also solve issue 1. We could also accept no change. 1> else (RRCReconfiguration was received via SRB1 or within the MobilityFromEUTRACommand message as specified in TS 36.331 [10]): |
| Huawei | Yes | The “SRB1” here refers to NR SRB1, therefore inter-RAT HO to NR is excluded, which is wrong. |
| Samsung | No | No change is needed. As indicated in the answer for Q3, we think the concerned parts are only for intra-NR mobility. |

## R2-2005636, R2-2005637

This CR is about aligning the UE behaviour in NR with the LTE spec, as we don’t see the need to change the behaviour across specs for the transmission of the UEAssistanceInformation when it’s resent to target cell due HO (within 1 sec).

In addition, it makes more sense no to retransmit the same contents of the UEAssistanceInformation to target cell, when configuration and/or overheating requirements have changed as this might have negative impact the UE performance.

Q6: Do you agree with the changes?

| Company | YES/NO | Comments |
| --- | --- | --- |
| Ericsson | Yes and No | We do not recall why for NR slightly different wording and behaviour was agreed. But we agree with the intention of the CR to align the NR behaviour with LTE. In LTE the UE sends the UAI message again, but the content is set again. We think that section 5.7.4.2 should be omitted, i.e. this is the “initiation” section, where the prohibit timers are set, and checked if the UE can send the UAI message. This should not be checked after HO, i.e. the UE should send the message again, but possibly update the content. To align with LTE we propose: 3> initiate transmission of a UEAssistanceInformation message to re-send the UE assistance information that UE is still configured to provide with the same contents in accordance with section 5.7.4.3; 36.331: 2> if the UE is configured to provide power preference indications, overheating assistance information, SPS assistance information, delay budget report or maximum bandwidth preference indications: 3> if the UE has transmitted a UEAssistanceInformation message during the last 1 second preceding reception of the RRCConnectionReconfiguration message including mobilityControlInfo: 4> initiate transmission of the UEAssistanceInformation message in accordance with 5.6.10.3; |
| Qualcomm | Yes | Agree with Ericson, the initiation section (5.7.4.2) can be removed. |
| MediaTek | Yes | We agree the intention and also the suggestion from Ericsson is fine. |
| Nokia | No | Our understanding is that the intent was that UE doesn't change the content based on whose network it connects to. If UE has indicated overheating before within 1s, it's quite unlikely it would have reduced the overheating. And for the examples, network would far better know that it has complied with the overheating report, so that should go away soon. We don’t get the real issue here. What is really broken to force the alignment? |
| Huawei | Yes | Agree with the wording from Ericsson. |
| Apple | Yes | We are fine with Ericsson’s wording. |
| Samsung | Yes | Intention seems fine but reference to 5.7.4.2 suggests UE does not send if contents is same, which is incorrect. We can just do a slight rephrase, to what is below: 3> initiate transmission of a UEAssistanceInformation message to re-send the UE assistance information that UE is still configured to provide even if contents is the same; |

# Conclusion

To be added.