3GPP RAN WG2 Meeting #110e	Draft_R2-200xxxx
June 01th– 12th, 2020

Agenda Item:	6.2.2

Source:	Lenovo

Title:	Summary of NR-U User plane

Document for:	Discussion, Decision

# Introduction

This is the summary report for tdocs submitted to the NR-U user plane agenda items (6.2.2):

# Random Access

## 2-step RACH

[2] points out that according to current specified behavior, UE needs to monitor the PDCCH for Random Access Response identified by the C-RNTI even for cases when MSGA PUSCH wasn’t transmitted due to LBT failure and C-RNTI MAC CE was included in the MSGA MAC PDU. Accordingly, the proposal is to relax the UE behaviour, i.e. UE doesn’t need to monitor PDCCH addressed to C-RNTI for cases when MSGA PUSCH wasn’t transmitted due to LBT failure. Even though the observation made in [2] is correct, the proposed change seems to be an optimization.

Question 1: Do companies see a need to change the current UE behavior such, that UE does not need to monitor the PDCCH identified by C-RNTI when LBT fails for msgA PUSCH transmission

| Company | Reply (y/n) | Additional comments |
| --- | --- | --- |

# LBT related issues

## Discussion on LBT Failure Detection and Recovery During HO with DAPS and CHO

According to the NR-U discussion on the consistent LBT failure detection in the RAN2#109bis meeting, RAN2 made the following agreements [1]:

Agreements: 5 Reply to RAN4 that UL LBT failure detection/recovery is applicable per current specifications to RA in R15-based handover, R15 SN addition/change, and PSCell addition, given the UE is in connected mode. LBT failure detection/recovery is not applicable per current specifications in RRC setup, resume, re-establishment, or release with redirection, as the UE does not have lbt-FailureRecoveryConfig during those procedures 6 Reply to RAN4 that no enhancements are planned in R-16 for UL LBT failure detection and recovery during handover, RRC setup, resume, re-establishment, or release with redirection. However, RAN2 will check if there is any technical issues with DAPS and CHO.

### DAPS HO

In case of DAPS HO, the UE does not detach from source cell upon receiving the RRCReconfiguration message but continues the DL reception from source gNB until the release of source cell as well as continues the UL transmissions to source gNB until successful completion of RACH procedure to target gNB. From UE perspective when being configured with DAPS Handover the UE has two MAC entities: one for the source cell and one for the target cell.

As observed by the contributions the current specification support LBT failure detection and recovery procedure at source cell and target cell for the DAPS HO case. Basically the LBT failure detection and recovery mechanism (lbt-FailureRecoveryConfig-r16) can be configured for both MAC entities during a DAPS HO. Since UE will transmit UL data to source gNB and target gNB simultaneously during the DAPS HO, consistent UL LBT failure may occur in both source and target cell during DAPS HO.

In [12,13] it is pointed out, that the current LBT failure detection procedure may force the UE to perform RACH in the source cell – for cases when multiple BWPs with RACH are configured - while attempting to connect to the target cell, i.e. parallel RACH procedures in source and target cell.

In order to avoid e.g. parallel RACH procedures in source and target cell [13, 21] propose to suspend LBT failure detection and recovery procedure in the source cell during DAPS HO, while for cases when the DAPS handover fails and the UE stays in the source cell, the UE shall resume LBT failure detection and recovery in the source cell. [12] instead proposes to disable BWP switching for the LBT failure detection and recovery procedure at the source cell, which can be achieved by gNB configuration, i.e. configuring RACH resources only at the current BWP.

The following options are possible:

- Option 1: suspend LBT failure detection and recovery at source cell during DAPS HO

- Option 2: disable BWP switching for the LBT failure detection and recovery procedure at source cell

- Option 3: legacy LBT failure detection and recovery procedure at source cell, i.e. no change required

Question 2: Which option do you prefer?

| Company | Preferred option | Additional comments |
| --- | --- | --- |

It should be noted that according to current TS38.331 specification (section 5.3.10.3), consistent uplink LBT failures indication from MCG MAC is not considered as source radio link failure condition. Therefore, even if source MCG MAC indicates consistent uplink LBT failures to the RRC, DRB(s) with DAPS configured will not be suspended and the source connection is not released.

In [11,12,16,17,19,20,22] it is proposed, that if DAPS HO is applied for a NR-U cell, the detection of radio link failure should also consider the case of consistent LBT failure at the source cell. UE triggers source RLF upon indication of consistent UL LBT failures, so that the all the DRBs are suspended in the source and the source connection is released.

Question 3: Do companies support that for DAPS HO, upon indication of consistent uplink LBT failures from source MCG MAC UE declares RLF, i.e. the UE should suspend all DRBs in the source and release the source connection, i.e. same behaviour as for source RLF?

| Company | Reply (y/n) | Additional comments |
| --- | --- | --- |

According to the submitted contribution, the majority doesn’t seem to see a need to change the behaviour for cases when consistent LBT failure is detected at the target cell. For the target MCG, the behaviour should be same as for legacy non-DAPS handover.

[15] proposes as an enhancement, that the information on consistent UL LBT failures occurring during Random Access in target cell should be reported to source gNB for RRM purpose.

In [20,21] it is proposed that the UE should consider T304 timer as expired upon indication of consistent UL LBT failure in target cell.

The following options are possible:

- Option 1: UE considers RLF to be detected upon detection of consistent UL LBT failures at the target cell

- Option 2: UE doesn’t consider RLF to be detected upon detection of consistent UL LBT failures at the target cell, i.e. rely on T304 timer

- Option 3: UE considers T304 timer as expired upon detection of consistent UL LBT failures at the target cell

Question 4: Which option do you prefer for handling of consistent UL LBT failures detected at the target cell for DAPS HO?

| Company | Preferred option | Additional comments |
| --- | --- | --- |

### CHO/CPC

All contributing companies [11,12,13,14,16,17,18,19,23] conclude, that UL LBT failure and recovery mechanism can be supported in CHO and CPC without additional specification change. Basically for the CHO case UE will handle LBT detection and recovery as during a normal handover, which was agreed by RAN2 in RAN2#109bis. If the consistent LBT failure occurred in source cell before one candidate cell met the execution condition, the UE can perform the recovery in source cell as in normal connected state, e.g. UE will perform BWP switch and/or trigger the RLF. For cases when the execution condition is met, configuration for that selected candidate CHO cell will contain the UL LBT failure and recovery mechanism just like in regular handover command. Hence CHO inherently supports LBT failure detection and recovery mechanism for target cell, if it is configured. Also for the CPC procedure no enhancements are required, since UL LBT failure detection/recovery is applicable per current specifications to RA in PSCell addition.

Question 5: Do companies agree that UL LBT failure and recovery mechanism can be supported in CHO and CPC without additional specification change?

| Company | Reply (y/n) | Additional comments |
| --- | --- | --- |

### Fast MCG link recovery

In R16 CA/DC enhancement, fast MCG link recovery is supported which means UE can be configured to report the MCG failure information to the MN via the SCG without performing connection re-establishment. [18] raises the question whether UE should report MCG failure instead of performing RRC re-establishment upon detecting consistent UL LBT failures for cases when fast MCG link recovery is configured.

Question 6: Do companies support that UE reports MCG failure instead of performing RRC re-establishment upon detecting consistent UL LBT failures for cases when fast MCG link recovery is configured?

| Company | Reply (y/n) | Additional comments |
| --- | --- | --- |

## UE declaring beam failure due to LBT failures during active TCI switching

RAN4 has sent a LS to RAN2 on the topic of the UE declaring beam failure due to LBT failures during active TCI state switching [R2-2004369]. It should be understood that, the LS addresses only RRC-based TCI state switching, not MAC-CE based TCI state switching. RRC-based TCI state switching is used only in the special case that a single TCI state is configured, and TCI state is only applied for PDCCH, whereas MAC-CE based TCI state switching is used for PDCCH when there is more than one configured TCI state and for PDSCH. According to the RAN4 LS and related discussions, it’s the rapporteur’s understanding, that the scenario that RAN4 is discussing is that UE is provided with a new single TCI state (for PDCCH) by RRC re-configuration message. The UE flushes its memory of the old TCI state according to current procedures. Due to LBT failures occuring at the gNB for the DRS (SS/PBCH block and/or CSI-RS) configured in the new TCI state, the UE is not able to actually perform the TCI state switch. Since the UE has deleted the old TCI state and LBT failure prevents the UE from receiving the DL RSs in the new TCI state, the UE does not have a defined QCL reference for reception of PDCCH.

RAN4 has an FFS on whether or not the UE should declare beam failure in this case after the number of SSB occasions that are not available due to LBT failure exceeds a specified threshold.

In [6,7,8,9] it is suggested that UE would declare beam failure for the described scenario, which further triggers the UE to perform the BFR procedure in order to recover from the failures. In addition, it’s stated in [6,7,9], that no additional mechanism is required to declare beam failure for the RRC-based TCI switching failure case, i.e. the NR Rel-15 BFD and BFR mechanism is sufficient to handle RRC-based TCI state switching failure caused by DL LBT failures. [8] proposes that in addition to declaring beam failure, MAC indicates “DL LBT failure” to higher layer (RRC) which in turn may trigger RLF.

[10] proposes that RRC-based TCI state switching failure in PCell triggers the RRC connection re-establishment, whereas for cases when the TCI state switching failure occurs in PSCell UE triggers the SCG failure report.

Question 7: Do companies agree that UE declares beam failure for the RRC-based TCI switching failure case due to DL LBT failures

| Company | Reply (y/n) | Additional comments |
| --- | --- | --- |

Question 8: Do companies agree that current Rel-15 BFD and BFR mechanism is enough to handle the RRC-based TCI switching failure case, i.e. no additional enhancements are needed for Rel-16.

| Company | Reply (y/n) | Additional comments |
| --- | --- | --- |

## UL LBT failure MAC CE

In [3] it is pointed out, that according to the current specifications it’s not clear when UE shall use the one-octet LBT failure MAC CE respectively the four-octet LBT failure MAC CE. Similar to the BFR case it is proposed to specify the conditions for choosing the corresponding LBT failure MAC CE format. More specifically it’s suggested that for cases when the highest ServCellIndex of this MAC entity's SCell for which LBT failure is detected is less than 8 UE uses the one-octet LBT failure MAC CE format, otherwise the four-octet format is used.

Question 9: Do companies agree to clearly specify the conditions when to use the one-octet LBT failure MAC CE or the four-octet LBT failure MAC CE, i.e. one-octet format is used when the highest ServCellIndex of this MAC entity's SCell for which LBT failure is detected is less than 8, otherwise four-octet format is used?

| Company | Reply (y/n) | Additional comments |
| --- | --- | --- |

## LBT Counter

[4] makes the observation, that the variable LBT_COUNTER should be per Serving Cell. However, from current specification it’s not clear though, that LBT_COUNTER is maintained per Serving Cell. Rapporteur agrees that from past discussions of the LBT failure detection procedure, the common understanding was that the LBT_COUNTER is per Serving Cell.

Question 10: Do companies agree that LBT_COUNTER is per Serving cell and that specification should be correspondingly updated to clearly capture this?

| Company | Reply (Y/N) | Additional comments |
| --- | --- | --- |

# Configured Grant

## Initial “Pending” status of a HARQ process

For NR-U a new status, i.e. “pending/not pending”, associated with a HARQ process was introduced in order to allow for autonomous retransmissions for cases where CG PUSCH transmission couldn’t be performed due to LBT failure. Whether a HARQ process is considered as pending or as not pending depends on whether a corresponding transmission has been performed or not. In [5] it’s identified that the initial status [pending/not pending] of a HARQ process upon activation/configuration of a configured grant is currently not clearly specified, which leads to a situation that UE is not able to perform the very first (initial) transmission on a HARQ process according to current MAC procedures.

Question 11: Do companies agree, that all HARQ processes associated with a Configured grant shall be considered as not pending upon activation/configuration of the Configured grant?

| Company | Reply (Y/N) | Additional comments |
| --- | --- | --- |

[5] proposes to specify in TS38.331 (field description of harq-ProcID-Offset) that all HARQ processes associated with a CG grant are considered as not pending upon activation/configuration of the configured grant. Rapporteur thinks, that since the [pending/not pending] status of a HARQ process is a MAC internal status, it may be more suitable to specify the intial status of a HARQ process in TS38.321 (section 5.4.2.2). Therefore the following options are possible:

- Option 1: 	Specify in TS38.331 (field description of harq-ProcID-Offset) that initial status of HARQ
 processes associated with a CG grant is “not pending” upon configuration/activation of a configured grant

- Option 2: 	Specify in TS38.321 that initial status of a HARQ process associated with a configured grant is not pending upon configuration/activation of a configured grant (if cg-RetransmissionTimer  is configured for the HARQ process)

- Option 3:	No specification change required (spec is already clear on this)

Question 12: Which option do you prefer for specifying the initial status [pending/not pending] of a HARQ process?

| Company | Preferred option | Additional comments |
| --- | --- | --- |

# Conclusion

RAN2 should discuss the above and agree to the following:

[TBA]

# References

- R2-2003804, Session minutes for NR-U, Power Savings, NTN and 2-step RACH, RAN2 Session chair (InterDigital)

- R2-2004975, Remaining issue on 2-step random access, Huawei

- R2-2004425, Clarification on when to use one-octet or four-octet LBT failure MAC CE, Samsung

- R2-2004426, Clarification on the variable LBT_COUNTER, Samsung

- R2-2005054, Text proposal on pending status for HARQ process in NRU, Huawei

- R2-2005330, UE declaring beam failure due to LBT failures during active TCI switching, Ericsson

- R2-2004671, Beam failure declaration due to TC switching failure, Intel Corporation

- R2-2004616, UE declaring beam failure due to LBT failures during active TCI switching, ZTE Corporation, Sanechips

- R2-2004598, Discussion about LBT failure and beam failure relation, Nokia, Nokia Shanghai Bell

- R2-2004420, LBT Impacts on the TCI State Switching, Vivo

- R2-2005713, Consideration on DAPS and CHO in NR-U, ZTE Corporation, Sanechips

- R2-2005550, Handling of UL LBT failure for DAPS and CHO, Qualcomm Incorporated

- R2-2005329, LBT failure detection and recovery for DAPS and CHO, Ericsson

- R2-2005050, Consistent LBT failure in CHO,	 Spreadtrum Communications

- R2-2005049, Consistent LBT failure in DAPS, Spreadtrum Communications

- R2-2004974, LBT failure recovery for DAPS and CHO, Huawei, HiSilicon

- R2-2004659, UL LBT failures interactions with DAPS and CHO, Intel Corporation

- R2-2004549, Remaining issues on UL LBT failure, OPPO

- R2-2004419, Discussion on LBT Failure Detection/ Recovery During HO with DAPS and CHO, vivo

- R2-2004529, Consistent LBT Failure Handling during Handover, Samsung Electronics Co., Ltd

- R2-2004694, On Applicability of DAPS Handover in NR-U	, MediaTek Inc.

- R2-2005617, Discussion on issues with DAPS in NR-U, LG Electronics Deutschland

- R2-2004696, On Conditional Handover in NR-U, MediaTek Inc.