3GPP TSG-RAN WG2 Meeting #111-e	R2-2006923

Online, 17 - 28 August 2020

Agenda item:	8.5.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Configured Grant Enhancement Harmonization for NR-U and URLLC

WID/SID:	NR_IIOT_URLLC_enh

Document for:	Discussion and Decision

# 1	Introduction

According to the objectives of the revised WID for Rel-17 IIoT/URLLC work item [1], we should examine how enhancement of configured grants introduced in Rel-16 for both licensed and unlicensed bands can be harmonized:

WI Objectives (R2-201310): - Study, identify and specify if needed, required Physical Layer feedback enhancements for meeting URLLC requirements covering - UE feedback enhancements for HARQ-ACK [RAN1] - CSI feedback enhancements to allow for more accurate MCS selection [RAN1] Note: DMRS-based CSI feedback is not in scope of this WI - Uplink enhancements for URLLC in unlicensed controlled environments [RAN1, RAN2]: - Specify support for UE-initiated COT for FBE with minimum specification effort - Harmonizing UL configured-grant enhancements in NR-U and URLLC introduced in Rel-16 to be applicable for unlicensed spectrum - Intra-UE multiplexing and prioritization of traffic with different priority based on work done in Rel.16 [RAN1]: - Specify multiplexing behavior among HARQ-ACK/SR/CSI and PUSCH for traffic with different priorities, including the cases with UCI on PUCCH and UCI on PUSCH. - Specify PHY prioritization of overlapping dynamic grant PUSCH and configured grant PUSCH of different PHY priorities on a BWP of a serving cell including the related cancelation behavior for the PUSCH of lower PHY priority, taking the solution developed during Rel-16 as the baseline - Enhancements for support of time synchronization: - RAN impacts of SA2 work on uplink time synchronization for TSN, if any. [RAN2] - Propagation delay compensation enhancements (including mobility issues, if any). [RAN2, RAN1, RAN3, RAN4] - RAN enhancements based on new QoS related parameters if any, e.g. survival time, burst spread, decided in SA2. [RAN2, RAN3]

This paper aims to analyse whether there is a need to modify CG enhancements relating to IIoT/URLLC introduced by RAN2 during Rel-16, in order to optimize their feasibility and/or performance in unlicensed spectrum, with considerations of NR-U mechanisms. This contribution does not address specific aspects related to UE-initiated COT for FBE, as we consider that some RAN1 progress should be made before discussing potential RAN2 impacts (e.g. RRC parameters).

# 2	Discussion

In this section, the Rel-16 enhancements for CG relating to IIoT/URLLC and NR-U are summarized first. Then, an analysis is provided to identify the aspects to be harmonized in Rel-17. Note that we mainly focus on RAN2-oriented enhancement in this paper.

## 2.1	Rel-16 CG Enhancements in IIoT/URLLC

In Rel-16, the following key enhancements relating to Configured Grants have been introduced for IIoT/URLLC operating in licensed band scenarios.

- Multiple active CG per BWP

In IIoT/TSC use cases, it is anticipated that 5GS should be capable to concurrently support multiple traffic flows with different QoS requirements. To enable low-latency transmissions for multiple flows of delay-critical data, Rel-16 can support up to 12 active CG configurations in a BWP. Moreover, the RAN is able to obtain information relating to deterministic and periodic traffics from the core network (i.e. TSCAI), which allows the gNB to allocate the multiple CGs appropriately in accordance to the traffic characteristics.

- LCH Mapping Restrictions of allowedCG-List

As the multiple active CGs could be configured to handle different traffic flows, a new LCH mapping restriction rule is introduced to ensure each CG configuration is only used to serve data from certain flows. In other words, whenever MAC is processing a configured grant resource, LCH selection should consider whether the LCH is configured with an allowedCG-List that includes the index of the CG in processing.

- Intra-UE Prioritization

To resolve the conflict between two uplink grants whose PUSCH resources are overlapping in time, RAN2 has introduced intra-UE prioritization mechanism based on LCH priority. More specifically, the MAC should select the grant that will carry data from LCHs with the higher priority for further processing (i.e. MAC PDU generation and deliver it to PHY). This includes collisions involving both dynamic grants and configured grants, as well as collisions among multiple configured grants.

- HARQ process split with offset for multiple CG

HARQ process ID for a CG occasion is derived from timing, the set of HARQ processes configured for the CG and an offset. HARQ process sharing among different CGs is not supported.

- Autonomous Transmission

In cases where the generated MAC PDU for a CG cannot be (completely) transmitted due to either intra-UE or inter-UE prioritization, the UE may store the MAC PDU in the HARQ buffer and transmit it autonomously using a subsequent CG resource with the same HARQ process. Note that since multiple CG configurations do not share HARQ processes in licensed band, essentially autonomous transmission is limited to the same CG configuration. Autonomous transmission cannot be continued in cases of Type-2 CG reactivation if the TBS is altered and not matchable.

## 2.2	Rel-16 CG Enhancements in NR-U

In Rel-16, the following key enhancements relating to Configured Grants have been introduced for NR-U:

- HARQ process selection

Differently from CG in IIoT where HARQ process ID is calculated based on timing, in NR-U it is up to UE implementation which HARQ process ID (among a set of configured HARQ processes), RV and NDI are selected and indicated in CG-UCI along with COT sharing information. Retransmissions are prioritized over new transmission when selecting the processes.

- HARQ process sharing

Multiple CGs can be configured with same set or subset of HARQ processes and the UE implemetation selects available process based on the status of configuredGrantTimer and cg-RetransmissionTimer of the processes.

- Explicit ACK support

HARQ feedback for UL CG HARQ processes is provided by CG-DFI through DCI format 0_1. The CG-DFI includes a bitmap with up to 16 bits carrying the HARQ-ACK. The parameter cg-minDFIDelay-r16 defines the minimum duration between the last symbol of the CG PUSCH and the starting symbol of the DFI for the UE to consider that DFI as carrying valid HARQ-ACK for that PUSCH.  If an ACK is received for a given HARQ process in a CG-DFI, a UE should terminate a TB repetition in a PUSCH with that HARQ processes.

- Autonomous retransmission

Different from IIoT, autonomous retransmission is perfomed when cg-RetransmissionTimer expires. The timer is started at each transmission and is stopped if feedback provided by CG-DFI or UL grant for the process is received from the NW or when configuredGrantTimer expires.

- Time domain consecutive allocations

In NR-U CG, a burst of consecutive slots allocations in a CG transmission period can be configured by RRC with the parameter cg-nrofSlots-r16. In addition, the number of allocations of same duration within a slot can be configured by the parameter cg-nrofPUSCH-InSlot-r16. This differs from the NR IIoT intra-slot repetition design.

- CG-UCI and PUCCH multiplexing

If the parameter cg-UCI-Multiplexing the CG-UCI and the HARQ-ACK bits are jointly encoded whenever PUCCH overlaps with CG PUSCH. If the parameter is not configured, PUCCH carrying HARQ-ACK is transmitted and the CG PUSCH is skipped.

It could be noted that all the enhancements introduced for NR-U are basically to cope with potential LBT failure.

## 2.3	Analysis of Harmonization

As described in WID, the operation in controlled environments assumes that interference from other systems using different radio access technology is not expected or only sporadically happens. However, even without coexistence with other systems in a controlled facility, CCA procedures need to be executed by the equipments operating in unlicensed spectrum according to regulation. Therefore, the possibility of LBT failure will then depend on the deployment, for example, LBT failures may occur due to interference from another cell. In our view, as long as there is potential LBT failure, NR-U CG operation should be adopted to support IIoT on unlicensed band. While if a deployment ensures that no LBT failures occur, then what is defined for licensed can be applicable without NR-U specific enhancement.

Observation 1: The configuration of NR-U features for CG is beneficial for IIoT in unlicensed band whenever there is possibility for LBT failure in the controlled environment deployment.

Here we review some features summarized above to identify aspects that potentially can be harmonized between features introduced from IIoT and NR-U:

- Multiple active CG per BWP

Multiple active CG per BWP introduced for IIoT was concluded to be supported for NR-U as well [2].

- LCH Mapping Restrictions of allowedCG-List

It was not explicitly discussed if this is applicable to NR-U. But the field description in RRC for allowedCG-List does not put any restriction on not to be configured for unlicensed band. This restriction applies only when the UL grant is a configured grant. If present, UL MAC SDUs from this logical channel can only be mapped to the indicated configured grant configurations. If the size of the sequence is zero, then UL MAC SDUs from this logical channel cannot be mapped to any configured grant configuration. If the field is not present, UL MAC SDUs from this logical channel can be mapped to any configured grant configurations. “allowedCG-List corresponds to "allowedCG-List" as specified in TS 38.321”. Thus it should be applicable to NR-U, as well.

Observation 2: allowedCG-List configuration is supported for NR-U as well with current specification.

- Autonomous retransmission for LBT Failure v.s. Autonomous Transmission for Intra-UE prioritization

For NR-U, a TB is considered as pending when LBT failure happens and will be transmitted in subsequent available occasion. In that sense, autonomous transmission for IIoT is very similar, espcially if we consider deprioritized MAC PDU (due to intra/inter-UE prioritization) as pending in view of autonomous retransmission as well. And on top autonomous retransmission is also supported when the cg-retransmission-timer is expired in case the NW does not send feedback or UL grant for retransmission. Thus the mechanism defined for NR-U could be extended and used to support intra/inter-UE prioritization for IIoT on NR-U, without the need of using autonomus transmission introduced by IIoT, i.e. autonomousTx should not be configured in shared spectrum with LBT failure constraints, where deprioritized MAC PDU could be potentially handled by autonomous retransmission. We think this aspect is worth exploring by RAN2.

Proposal 1: RAN2 should discuss if NR-U autonomous retransmission can supersede IIoT autonomous transmission with some possible extensions.

- HARQ process selection

For NR-U, HARQ process selection is up to UE implemenation with the configuredGrantTimer and cg-RetransmissionTimer maintained to determine which processes are available for retransmission or retransmission for the CG occasion, while for IIoT on licensed band it is determined based on timing.

- HARQ process sharing among multiple CG

HARQ process sharing was also motivated by potential LBT failure on one of the CG to allow retransmissions on another CG.

Aligned with the observation above, as long as there is potential LBT failure, HARQ operation defined for NR-U should be adopted to support IIoT on unlicensed band as well instead of the opertion on licensed one. While if no LBT failures is considered, then HARQ operation defined for NR can be adopted.

Proposal 2: As long as there is potential LBT failure, HARQ operation for CG introduced for NR-U can be applied to support IIoT as well. And if there is no potential LBT failure, NR based HARQ operation for CG can be adopted in unlicensed band (i.e. without cg-retransmission-timer, CG-UCI, explicit HARQ-ACK feedback etc.).

However, whether CG-UCI less operation is possible for unlicensed band should be decided in RAN1 since there might be other NR-U related information in CG-UCI as well apart from the HARQ related information.

Proposal 3: Whether CG-UCI less operation is supported for NR-U should be decided by RAN1.

# 3	Conclusions

This contribution discussed harmonization between CG enhancements introduced for NR-U and IIoT in Rel-16. We have made the following observations and proposals:

Observation 1: The configuration of NR-U features for CG is beneficial for IIoT in unlicensed band whenever there is possibility for LBT failure in the controlled environment deployment.

Observation 2: allowedCG-List configuration is supported for NR-U as well with current specification.

Proposal 1: RAN2 should discuss if NR-U autonomous retransmission can supersede IIoT autonomous transmission with some possible extensions.

Proposal 2: As long as there is potential LBT failure, HARQ operation for CG introduced for NR-U can be applied to support IIoT as well. And if there is no potential LBT failure, NR based HARQ operation for CG can be adopted in unlicensed band (i.e. without cg-retransmission-timer, CG-UCI, explicit HARQ-ACK feedback etc.).

Proposal 3: Whether CG-UCI less operation is supported for NR-U should be decided by RAN1.

# References

- RP-201310, Revised WID: Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR, Nokia, Nokia Shanghai Bell, RAN-P #88e, Jun-Jul 2020.

- R2-2003952, Summary of NR-U User plane, InterDigital, RAN2#109bis-e, Apr. 2020.