3GPP TSG-RAN WG2 Meeting #111	R2-2007407

# Electronic, 17 – 28 August 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | draft | rev | - | Current version: | 15.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR on network specific uac-AccessCategory1-SelectionAssistanceInfo in TS36.331-R15 solution |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE corporation, Sanechips, CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_5GCN_connect-Core |  |  |  |  |  | Date: |  |  | 2020-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - The UE behavior of forwarding uac-AC1-SelectAssistInfo to upper layers is missing in 5.2.2.32. - Among all the PLMNs sharing the same cell, some PLMNs may want to configure access barring for delay tolerant service while others do not. With the current ASN.1 structure, if one PLMN wants to configure access barring for delay tolerant services, other PLMNs sharing the same cell have to configure a “a”, “b” or “c” either via plmnCommon or individualPLMNList even though they do not want to do so. As a result, UE configured with delay tolerant service may select access category 1 and perform barring check based on the factor and timer associated with access category 1 upon receiving such configuration although it should have selected another access category and perform access barring accordingly. Thus, it is suggested to change the UE beahvior in Access Category 1 selection by also taking the configured uac-barringInfoSetIndex associated with Access Category 1 into consideration. The AS layer forward uac-AC1-SelectAssistInfo along with the uac-barringInfoSetIndex associated with Access Category 1, if present, to upper layers. The upper layer will select Access Category 1 for concerned UE when both uac-AC1-SelectAssistInfo along with the uac-barringInfoSetIndex associated with Access Category 1 are provided. In this way, a PLMN not willing to configure Access Category will not configure uac-barringInfoSetIndex for Access Category 1 and the upper layer will never select Access Category 1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Change the UE behaviour in 5.2.2.32 to also forward the uac-barringInfoSetIndex associated with Access Category 1, if present, to upper layers for Access Category 1 selection. Impact Analysis Impacted 5G architecture options: EUTRA connected to 5GC, NG-ENDC Impacted functionality: Unified Access control Inter-operability: No inter-operability is forseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Among all the PLMNs sharing the same cell, the current ASN.1 for configuring uac-AC1-SelectAssistInfo does not allow some PLMNs to configure Access Category 1 while others not to configure it. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.32 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change

#### 5.2.2.32	Actions upon reception of SystemInformationBlockType25

Upon receiving SystemInformationBlockType25 the UE shall:

1>	if uac-AC1-SelectAssistInfo is included and the value is set to a, b or c:

2>	forward uac-AC1-SelectAssistInfo to upper layers;

1>	if uac-barringInfoSetIndex associated with Access Category 1 is included:

2>	forward uac-barringInfoSetIndex associated with Access Category 1 to upper layers;

End of change