3GPP TSG-RAN WG2 Meeting #111 Electronic	R2-2007601

Elbonia, 17 – 28 August  2020

Agenda item:	8.10.3.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Adjusting timers according to delay variations

WID/SID:	NR_NTN_solutions-Core - Release 17

Document for:	Discussion and Decision

# 1	Introduction

- MAC

  - Random access:

    - Definition of an offset for the start of the ra-ResponseWindow for NTN.

    - Introduction of an offset for the start of the ra-ContentionResolutionTimer to resolve Random access contention

    - Solutions for resolving preamble ambiguity and extension of RAR window.

    - Adaptation for Msg-3 scheduling

      - Only for the case with pre-compensation of timing and frequency offset at UE side)

  - Enhancement on UL scheduling to reduce scheduling latency.

  - DRX:

    - If HARQ feedback is enabled, introduction of offset for drx-HARQ-RTT-TimerDL and drx-HARQ-RTT-TimerUL.

    - If HARQ is turned off per HARQ process, adaptions in HARQ procedure

  - Scheduling Request: Extension of the value range of sr-ProhibitTimer

- RLC

  - Status reporting: Extension of the value range of t-Reassembly

  - Sequence Numbers: extension of the SN space only for GEO scenarios

- PDCP

  - SDU discard: Extension of the value range of discardTimer.

  - Sequence Numbers: extension of the SN space for GEO scenarios.

The work item description [2] defines a number of timers need to be adjusted:

In this contribution a method to adjust the timers simultaneously, across communication layers is discussed.

# 2		Adjusting timers according to delay variations

In the TR [1] and the WI [2] it has been defined that certain timers on e.g. MAC, RLC and PDCP layer need to be changed to enable NR over NTN. The timers are part of control loops, which define actions based on the time since another event. For example, the MAC sr-ProhibitTimer defines a period after sending a scheduling request (SR), where the UE is not allowed to send a new SR.

In LEO scenarios, the timers need to be updated regularly to handle longer and varying delays of NTN, which depend heavily on the deployment scenario; i.e. earth-fixed or earth-moving cell, satellite altitude, satellite constellation, feeder link.

As an example, Figure 1 provides the propagation delay for a few earth-fixed cell scenarios using a transparent LEO satellite. The first line (LEO-600 service link) defines the service link delay for an earth-fixed cell with a reference point at nadir and a satellite at 600 km altitude moving from a 30 degrees elevation angle on one side to 30 degrees elevation angle at the opposite side. The LEO-600 feeder link case 1 assumes the NTN gateway is at the exact same location, resulting the total delay illustrated with LEO-600 total case 1. A second feeder link case (LEO-600 feeder link case 2), is defined for a scenario where the NTN gateway is on the path of the satellite’s orbit, but 1200 km away. Furthermore, it is assumed the satellite at 600 km altitude is moving away from the gateway. The resulting total delay is given by LEO-600 case 2 total. Finally, LEO-1500 total case 1, replicates the scenario of case 1, but using 1500 km altitude. Based on these examples it can be observed how the propagation delay varies significantly over time.

Figure 1 Examples of earth-fixed cell propagation delays.

Extending the timers to static values, which reflect the maximum observable delay is suboptimal, because the maximum value can be significantly larger than the minimum delay observed in a transparent satellite scenario, where both service and feeder link delays must be included. If the maximum delay is applied the network will use too conservative timers for long periods of time, eventually reducing the user experience and general system efficiency.

- Using maximum delay (sum of service and feeder links) for timers impacts user experience and system efficiency negatively.

On the contrary, if a variable value is used per timer it will be necessary to update each timer with a certain rate. Such an update per timer will be UE-specific and therefore result in significant control overhead, when considering the larger number of control loop timers in NR.

- Using variable delay values per timer increases control overhead due to per-value, per-UE signalling.

However, the service and feeder link delays can be estimated, by the network, based on satellite ephemeris, ground station location, processing times, and deployment scenario. The sum of the service and feeder link delays are defining the total delay, which the UE timers need to handle, in the transparent satellite case. In principle, the total delay can therefore be predicted per UE, if their location is known. Knowing the delay as a function of time may be used to adjust the timers at various layers, but would still require specific signalling to each UE.

- The delay between a gNB and a UE depends on among others satellite ephemeris, ground station location, satellite processing times, and UE location.

One issue is that all the required knowledge may not be available in the UE, at least not with sufficient accuracy. Furthermore, a continuous estimation of the changing delay may imply a significant processing burden on the UE.

Another option may be to define a common reference point for a NR cell slightly above Earth. In this way the delta delay between each UE and the common reference point is minimal, while a single total delay as a function of time can be broadcasted. Using such a broadcasted function, each UE can then adjust their timers, potentially with a conservative safety margin to account for the residual delay between the reference point and the UE’s location. By ensuring the cell common reference point is close to the UEs and assuming relatively small cell sizes, the UE-specific delay from the reference point to the actual UE location may be so small that it does not impact the overall choice of the timer value.

- The delay between a gNB and a cell common reference point can be broadcasted and applied by UEs to update timers.

A key issue for further study is the required accuracy and granularity per timer, which depends on the communication layer the timer resides in.

If UEs autonomously adjust the timers on various layers, the communication with the network may be jeopardized. Therefore, the broadcasted delay function may include information on when the UEs shall update their timers. Thus, the UE-network synchronization, in terms of a common understanding of the currently used UE timers, is maintained.

- The broadcasted delay function may also include information on specific time instances where the UEs shall update the timers.

Based on the above description, the solution to handle the long and varying delays of NTN by use of a cell common reference point and well-defined time instances for updating the UE timers based on the common reference delay, is proposed to RAN2 for discussion.

- RAN2 to discuss the use of a cell common reference point delay for updating UE timers.

# 7	Conclusion

In the previous sections we made the following observations:

- Using maximum delay (sum of service and feeder links) for timers impacts user experience and system efficiency negatively.

- Using variable delay values per timer increases control overhead due to per-value, per-UE signaling.

- The delay between a gNB and a UE depends on among others satellite ephemeris, ground station location, satellite processing times, and UE location.

- The delay between a gNB and a cell common reference point can be broadcasted and applied by UEs to update timers.

- The broadcasted delay function may also include information on specific time instances where the UEs shall update the timers.

Based on the discussion in the previous sections we propose the following:

- RAN2 to discuss the use of a cell common reference point delay for updating UE timers.

# References

- 3GPP TR 38.821 V16.0.0, Solutions for NR to support non-terrestrial networks (NTN), December 2019.

- 3GPP NR_NTN_solutions WI description, RP-201256, Solutions for NR to support non-terrestrial networks (NTN), July 2020.

- R2-1907069, Impacts of long RTTs to Random Access Response, 3GPP TSG-RAN WG2 Meeting #106, Reno, USA, 13 – 17 May 2019