3GPP TSG-RAN WG2 Meeting #111 electronic	R2-2007739

Online, August 17th – 28th, 2020	Revision of R2-2005571

Agenda Item:	7.3.3

Source: 	ASUSTeK

Title:  	HARQ feedback in RRC_IDLE

Document for:	Discussion and Decision

- Introduction

Due to PUR, the need for the UE to transmit HARQ feedback in RRC_IDLE is introduced in Rel-16. UL synchronization check before transmitting the HARQ feedback should be updated to incorporate the new scenario.

- Discussion

To prevent UL transmissions when UL is not synchronized, the MAC entity would decide whether to indicate HARQ feedback to the physical layer based on if the timeAlignmentTimer is running [1]:

#### 5.3.2.2 HARQ process […] The MAC entity then shall: […] - if the HARQ process is associated with a transmission indicated with a Temporary C-RNTI and the Contention Resolution is not yet successful (see clause 5.1.5); or - if the HARQ process is equal to the broadcast process; or - if the timeAlignmentTimer, associated with the TAG containing the serving cell on which the HARQ feedback is to be transmitted, is stopped or expired: - do not indicate the generated positive or negative acknowledgement to the physical layer. - else: - indicate the generated positive or negative acknowledgement for this TB to the physical layer.

Due to introduction of PUR, the UE also needs to transmit HARQ feedback in RRC_IDLE in order to indicate whether PUR response is successfully received by the UE or not. However, the timeAlignmentTimer may not be running in RRC_IDLE. If the condition check is kept the same, the MAC entity may not be able to indicate the generated positive or negative acknowledgement to the physical layer.

In RAN2 #110e meeting, the issue was discussed and there was no consensus on how to fix the condition check. Instead of based on the check of whether pur-timeAlignmentTimer is running, we have two possible alternatives to check whether to transmit HARQ feedback of PUR response:

- Using PUR response window

Because the DL TB for PUR is received during PUR response window, the UE could check whether to transmit HARQ feedback based on if the DL TB is received during PUR response window and not based on if timeAlignmentTimer is running.

- Using PUR-RNTI

Because the DL TB for PUR is associated with PUR-RNTI, the UE could check whether to transmit HARQ feedback based on if the DL TB is associated with PUR-RNTI and not based on if timeAlignmentTimer is running.

These two alternatives are based on the assumption that uplink TA would be still valid when the UE transmit HARQ feedback of PUR response.

Proposal: In order to exclude TA timer check for transmitting HARQ feedback of PUR response, RAN2 to discuss which one of the following two alternatives is adopted:

- HARQ feedback of a TB is transmitted if the TB is received during PUR response window

- HARQ feedback of a TB is transmitted if the TB is associated with PUR-RNTI

- Conclusion

We have the following proposal:

Proposal: In order to exclude TA timer check for transmitting HARQ feedback of PUR response, RAN2 to discuss which one of the following two alternatives is adopted:

- HARQ feedback of a TB is transmitted if the TB is received during PUR response window

- HARQ feedback of a TB is transmitted if the TB is associated with PUR-RNTI

Corresponding text proposals are also provided in below.

- Reference

- TS 36.321 V16.1.0, LTE MAC specification.

- Text proposal

//////////////////////// First alternative: Using PUR response window ////////////////////////

#### 5.3.2.2	HARQ process

For each TTI where a transmission takes place for the HARQ process, one or two (in case of downlink spatial multiplexing) TBs and the associated HARQ information are received from the HARQ entity.

For each received TB and associated HARQ information, the HARQ process shall:

-	if the NDI, when provided, has been toggled compared to the value of the previous received transmission corresponding to this TB; or

-	if the HARQ process is equal to the broadcast process and if this is the first received transmission for the TB according to the system information schedule indicated by RRC; or

-	if this is the very first received transmission for this TB (i.e. there is no previous NDI for this TB):

-	consider this transmission to be a new transmission.

-	else:

-	consider this transmission to be a retransmission.

The MAC entity then shall:

-	if this is a new transmission:

-	attempt to decode the received data.

-	else if this is a retransmission:

-	if the data for this TB has not yet been successfully decoded:

-	combine the received data with the data currently in the soft buffer for this TB and attempt to decode the combined data.

-	if the data which the MAC entity attempted to decode was successfully decoded for this TB; or

-	if the data for this TB was successfully decoded before:

-	if the HARQ process is equal to the broadcast process:

-	deliver the decoded MAC PDU to upper layers.

-	else if this is the first successful decoding of the data for this TB:

-	deliver the decoded MAC PDU to the disassembly and demultiplexing entity.

-	generate a positive acknowledgement (ACK) of the data in this TB.

-	else:

-	replace the data in the soft buffer for this TB with the data which the MAC entity attempted to decode.

-	generate a negative acknowledgement (NACK) of the data in this TB.

-	if the HARQ process is associated with a transmission indicated with a Temporary C-RNTI and the Contention Resolution is not yet successful (see clause 5.1.5); or

-	if the HARQ process is equal to the broadcast process; or

-	if this TB was received while pur-ResponseWindowTimer is not running, and the timeAlignmentTimer, associated with the TAG containing the serving cell on which the HARQ feedback is to be transmitted, is stopped or expired:

-	do not indicate the generated positive or negative acknowledgement to the physical layer.

-	else:

-	indicate the generated positive or negative acknowledgement for this TB to the physical layer.

The MAC entity shall ignore NDI received in all downlink assignments on PDCCH for its Temporary C-RNTI when determining if NDI on PDCCH for its C-RNTI has been toggled compared to the value in the previous transmission.

NOTE 1:	When the MAC entity is configured with more than one serving cell, UE behaviors for storing data to the soft buffer is specified in TS 36.213 [2].

NOTE 2:	If the MAC entity receives a retransmission with a TB size different from the last valid TB size signalled for this TB, the UE behavior is left up to UE implementation.

//////////////////////// Second alternative: Using PUR-RNTI ////////////////////////

#### 5.3.2.2	HARQ process

For each TTI where a transmission takes place for the HARQ process, one or two (in case of downlink spatial multiplexing) TBs and the associated HARQ information are received from the HARQ entity.

For each received TB and associated HARQ information, the HARQ process shall:

-	if the NDI, when provided, has been toggled compared to the value of the previous received transmission corresponding to this TB; or

-	if the HARQ process is equal to the broadcast process and if this is the first received transmission for the TB according to the system information schedule indicated by RRC; or

-	if this is the very first received transmission for this TB (i.e. there is no previous NDI for this TB):

-	consider this transmission to be a new transmission.

-	else:

-	consider this transmission to be a retransmission.

The MAC entity then shall:

-	if this is a new transmission:

-	attempt to decode the received data.

-	else if this is a retransmission:

-	if the data for this TB has not yet been successfully decoded:

-	combine the received data with the data currently in the soft buffer for this TB and attempt to decode the combined data.

-	if the data which the MAC entity attempted to decode was successfully decoded for this TB; or

-	if the data for this TB was successfully decoded before:

-	if the HARQ process is equal to the broadcast process:

-	deliver the decoded MAC PDU to upper layers.

-	else if this is the first successful decoding of the data for this TB:

-	deliver the decoded MAC PDU to the disassembly and demultiplexing entity.

-	generate a positive acknowledgement (ACK) of the data in this TB.

-	else:

-	replace the data in the soft buffer for this TB with the data which the MAC entity attempted to decode.

-	generate a negative acknowledgement (NACK) of the data in this TB.

-	if the HARQ process is associated with a transmission indicated with a Temporary C-RNTI and the Contention Resolution is not yet successful (see clause 5.1.5); or

-	if the HARQ process is equal to the broadcast process; or

-	if the HARQ process is not associated with a transmission indicated with a PUR-RNTI, and the timeAlignmentTimer, associated with the TAG containing the serving cell on which the HARQ feedback is to be transmitted, is stopped or expired:

-	do not indicate the generated positive or negative acknowledgement to the physical layer.

-	else:

-	indicate the generated positive or negative acknowledgement for this TB to the physical layer.

The MAC entity shall ignore NDI received in all downlink assignments on PDCCH for its Temporary C-RNTI when determining if NDI on PDCCH for its C-RNTI has been toggled compared to the value in the previous transmission.

NOTE 1:	When the MAC entity is configured with more than one serving cell, UE behaviors for storing data to the soft buffer is specified in TS 36.213 [2].

NOTE 2:	If the MAC entity receives a retransmission with a TB size different from the last valid TB size signalled for this TB, the UE behavior is left up to UE implementation.