- 3GPP TSG RAN WG2 Meeting #112-e      	                           R2-200xxxx
E-Conference, 2nd – 13th Nov. 2020

Agenda item:	5.4.1.1

Source:	Qualcomm Incorporated

Title:	Email discussion summary of [005][NR15] RRC Conn Control I

WID/SID:	NR_newRAT-Core – Release 15

Document for:	Discussion and Decision

# Introduction

It is summary for below email discussion:

- [AT112-e][005][NR15] RRC Conn Control I (Qualcomm)

Treat R2-2008715, R2-2009183, R2-2009184, R2-2009185, R2-2010563, R2-2010665, R2-2010666, R2-2009355, R2-2009356, R2-2009844, R2-2009845, R2-2010530, R2-2010531, R2-2010557, R2-2010558

Intended outcome: Intermediate: Determine agreeable parts. Final: For agreeable parts, agreed CRs.

Deadline: Intermediate deadline(s) by Rapporteur, Final: Discussion stop at Wed Nov 11, 1200 UTC

As indicated in its scoping, it includes the following topics and their related CR(s):

- SRS carrier switching

  - Treat CRs: R2-2008715, R2-2009183, R2-2009184, R2-2009185

- Handing of SRS Resource Set upon PUCCH Release

  - Treat CRs: R2-2010563

- Configuration of first active BWPs

  - Treat CRs: R2-2010665, R2-2010666

- HARQ-ACK spatial bundling and CBG

  - Treat CRs: R2-2009355, R2-2009356

- FH configuration for 1-symbol PUCCH

  - Treat CRs: R2-2009844, R2-2009845

- Clarification on p-Max in FR2

  - Treat CRs: R2-2010530, R2-2010531

- Deactivation of SCells for Overheating Mitigation

  - Treat CRs: R2-2010557, R2-2010558

According Chair’s indication, rapporteur would like to have the following two phases:

- Phase 1: collect companies’ view, by Friday 2020-11-06 12:00 UTC

- Phase 2: rapporteur will share summary report and TP based on input of phase 1 for review, by Monday 2020-11-11 12:00 UTC

# Discussion

## SRS carrier switching

The related CRs are:

R2-2008715	LS reply on NR SRS carrier switching (R1-2007395; contact: Qualcomm)	RAN1	LS in	Rel-15	NR_newRAT-Core	To:RAN2

Moved from 5.1

R2-2009183	Discussion on SRS carrier switching based on RAN1 reply LS (R1-2007395)	Qualcomm Incorporated, ZTE Corporation, Sanechips, Ericsson, MediaTek Inc.	discussion	Rel-15	NR_newRAT-Core

R2-2009184	Correction for configuration of SRS Carrier Switching	Qualcomm Incorporated, ZTE Corporation, Sanechips, Ericsson, MediaTek Inc.	CR	Rel-15	38.331	15.11.0	2039	-	F	NR_newRAT-Core

R2-2009185	Correction for configuration of SRS Carrier Switching	Qualcomm Incorporated, ZTE Corporation, Sanechips, Ericsson, MediaTek Inc.	CR	Rel-16	38.331	16.2.0	2040	-	A	NR_newRAT-Core

These CRs are based on RAN1 LS (R2-2008715) with key part copied below:

RAN1 concluded that, for type A, carrier switching to SUL is not supported in Case 1 and Case 2.

For the case of type A carrier switching to NUL in Case 2: while in RAN1 there are multiple companies that understand that this case is not allowed from RAN1 specification point of view, there are two companies which have a different view. Consequently, RAN1 can’t reach a conclusion on this issue.

Note that RAN1 discussion on RAN2 LS was summarized in R1-2007044 [1].

So basically, there are two issues:

- Carrier switching to SUL (highlighted part)

Since RAN1 had concluded this issue, R2-2009183 proposed to implement RRC spec change for both Rel-15/Rel-16:

Related part of R2-2009183

Observation 1: RAN1 had concluded that type A SRS carrier switching to SUL is not supported in Case 1 and Case 2 because it is conflicted with the objective of SRS carrier switching (i.e. support reciprocity-based beamforming in TDD bands for DL CA). Transmitting SRS in SUL without PUSCH/PUCCH transmission has no benefit or use case.

Proposal 1: In Rel-15/Rel-16 TS 38.331 CR, capture RAN1 conclusion that type A SRS carrier switching to SUL is not supported in Case 1 and Case 2.

And it is corresponding to 2nd change in R2-2009184/R2-2009185:

Corresponding spec change (2nd change) of R2-2009184/R2-2009185

typeA

Type A trigger configuration for SRS transmission on a PUSCH-less SCell (see TS 38.213 [13], clause 11.4). In this release, the network configures at most one entry (the first entry) of typeA, and the first entry corresponds to the serving cell in which the SRS-CarrierSwitching field is configured. SRS carrier switching to SUL carrier is not supported in this version of the specification.

Rapporteur think this change is straight forward.

Q1: Do you agree the 2nd change in Rel-15/Rel-16 CR R2-2009184/R2-2009185 (i.e. , capture RAN1 conclusion that type A SRS carrier switching to SUL is not supported)?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | Yes | As indicated in RAN1 LS, it was concluded in RAN1 that type A SRS carrier switching to SUL is not supported. Thus, it just implemented RAN1 conclusion in RRC spec. |
| Apple | Yes | Based on RAN1 LS |
| Huawei, HiSilicon | Yes |  |
| Nokia | Yes |  |
| ZTE | Yes |  |

- Whether SUL can be configured without PUSCH/PUCCH (highlighted part)

As R2-2009183 clarified, for the issue RAN1 can’t conclude in Case 2, the only ambiguity is whether it is a valid configuration that SUL is not configured with PUCCH/PUSCH. Based on analysis of R2-2009183, the use case is not clear why Network configures and maintains a SUL not configured with PUCCH/PUSCH. So, it proposed that the network does not configure SUL carrier without PUSCH/PUCCH:

Related part of R2-2009183

Observation 4: The use case is not clear why Network configures and maintains a SUL not configured with PUCCH/PUSCH. Furthermore, according to TS 38.300, Network is required to avoid overlapping PUSCH transmission through scheduling instead of through configuration.

Proposal 2: In Rel-15/Rel-16 TS 38.331 CR, capture the clarification that the network does not configure SUL carrier without PUSCH/PUCCH.

And it is corresponding to 1st change in R2-2009184/R2-2009185:

Corresponding spec change (1st change) of R2-2009184/R2-2009185

supplementaryUplink

Network may configure this field only when supplementaryUplinkConfig is configured in ServingCellConfigCommon or ServingCellConfigCommonSIB. The network does not configure SUL carrier without PUSCH/PUCCH.

Q2: Do you agree the 1st change in Rel-15/Rel-16 CR R2-2009184/R2-2009185 (i.e. , capture the clarification that the network does not configure SUL carrier without PUSCH/PUCCH)?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | Yes | We think it is an invalid configuration that Network configures a SUL carrier without PUSCH/PUCCH. As we know, the purpose of SUL is for adding one more carrier for uplink transmission to improve UL coverage of high frequency scenarios. The use case is not clear to us why Network configures and maintains a SUL not configured with PUCCH/PUSCH. If it is allowed, why doesn’t NW release such serving cell’s SUL instead? Furthermore, according to section 6.9 of TS 38.300, Network is required to avoid overlapping PUSCH transmission through scheduling instead of through configuration, which implies that at least PUSCH should be configured in SUL. |
| Apple | Yes | Agree with this change. |
| Huawei, HiSilicon | No | RAN1 has concluded long time ago that SRS can be configured independently as below in R1-1719112: - SRS related RRC parameters are independently configured for SRS on the SUL carrier and SRS on the non-SUL UL carrier - SRS can be configured on the SUL carrier and non-SUL UL carrier, irrespective of the carrier configuration for PUSCH and PUCCH The RAN1 LS was discussing SRS carrier switching case 2, which is not relevant on whether SRS can be configured on SUL even without PUCCH/PUSCH. The stage 2 38300 description is to avoid overlapping PUSCH transmission, which means if two UL are configured the schedule needs to avoid this. There is nothing implicating SRS cannot be configured on SUL. SRS configuration on SUL without PUCCH/PUSCH is used to measure the UL signal. We believe the proposal is NBC and are not able to accept it. |
| Nokia | Yes, fine with intention | Instead of PUCCH/PUSCH we could say PUCCH or PUSCH (SUL may have either and not necessarily have both). Then it would be good to also clarify that different BWP’s may have different configuration. |
| ZTE | Yes | Same view as Qualcomm. Regarding the RAN1 agreement quoted by Huawei, we’ve checked RAN1 discussion, and found the complete RAN1 agreements are: Agreement: - UE specific RRC signalling (re-)configures the location of the PUCCH, either on the SUL carrier or on a non-SUL UL carrier in a SUL band combination o The default location of the PUSCH is the same carrier as used by PUCCH - UE specific RRC signalling may (de-)configure that PUSCH may be dynamically scheduled on the other (i.e. non-PUCCH) carrier in the same cell as the SUL o In this case, a carrier indicator field in the UL grant is used to indicate dynamically whether the PUSCH is transmitted on the PUCCH carrier or on the other carrier § Note: Simultaneous PUSCH transmission on the SUL carrier and non-SUL UL carrier is not supported according to existing RAN2 agreement § FFS in DCI discussion whether the SUL CIF is always present o There is one active BWP on the SUL carrier and one active BWP on the non-SUL UL carrier - SRS related RRC parameters are independently configured for SRS on the SUL carrier and SRS on the non-SUL UL carrier in the SUL band combination o SRS can be configured on the SUL carrier and non-SUL UL carrier, irrespective of the carrier configuration for PUSCH and PUCCH ~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~ Except the last two bullets, the agreements also include other aspects related to PUSCH and PUCCH configuration. So the way we interprete the last sentence is that, SRS can be configured in both SUL carrier and non-SUL UL carrier regardless how PUSCH and PUCCH are configured. It does not mean "regardless of whether PUSCH and PUCCH are configured". It is just saying SRS configuration is independent on how PUSCH and PUSCH are configured. But not mean all the combinations of PUSCH and PUCCH configurations are allowed. Otherwise, there should be an explicit agreement like what R1-1719112 originally proposed. |

In addition, R2-2009185 raised the issue that the clarification “The network does not configure this field to 3 in this release of specification” was mistakenly put in field description of cc-IndexInOneCC-Set. Instead, it should be put in field description of cc-SetIndex. Note that there is no issue in Rel-15 RRC spec.

Rapporteur think this change is straight forward.

Q3: Do you agree the 3rd change in Rel-16 CR (R2-2009185) (i.e., Move the clarification “The network does not configure this field to 3 in this release of specification” from cc-IndexInOneCC-Set to cc-SetIndex)?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | Yes | It was a typo in Rel-16 CR |
| Apple | Yes |  |
| Huawei, HiSilicon | Yes |  |
| Nokia | Yes |  |
| ZTE | Yes |  |

## Handing of SRS Resource Set upon PUCCH Release

The related CRs are:

R2-2010563	SRS Resource Set upon PUCCH Release	Qualcomm Incorporated	CR	Rel-16	38.331	16.2.0	2245	-	F	NR_newRAT-Core

According to R2-2010563, the reason for the change is:

Reason for change:

Upon PUCCH Release (e.g. due HO), the SRS resources are implicitly released by the UE. the SRS Resource sets are not released, and if subsequent reconfiguration message doesn't associate existing SRS resources sets with SRS Resource, the configuration is considered invalid and call will be dropped.

Current spec doesn't specify the UE behavior on how to handle such scenario, i.e. dangling SRS Resource Sets with no SRS Resources associated to, nor mandate on the network to configure the SRS Resource sets appropriately (reassociate SRS Resoures or release the SRS Resource Sets).

Example:

- UE In NR (SA/ENDC) connected with SRS-Resources / SRS-ResourceSets are configured

- UE receives RRC ReconfigurationWithSync (HO)–

  - As per spec UE implicitly releases SRS-resources / PUCCH CSI / SR config

- Network reconfigures the UE with new SRS Resources (not the same SRS Resource instances are reconfigured)

  - this will leave some of the SRS-ResourceSets with no associated SRS-resources which is against the spec.

And corresponding spec change is:

### 5.3.12	UE actions upon PUCCH/SRS release request

Upon receiving a PUCCH release request from lower layers, for all bandwidth parts of an indicated serving cell the UE shall:

1>	release PUCCH-CSI-Resources configured in CSI-ReportConfig;

1>	release SchedulingRequestResourceConfig instances configured in PUCCH-Config.

Upon receiving an SRS release request from lower layers, for all bandwidth parts of an indicated serving cell the UE shall:

1>	release SRS-Resource instances and SRS-ResourceSets configured in SRS-Config.

Q4: Do you agree that if “SRS Resource Set” is configured but no SRS Resources were assigned, is a violation to the spec?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | Yes | According to RRC spec, at least 1 SRS resource is included in one SRS resource set: SRS-Config ::= SEQUENCE { srs-ResourceSetToReleaseList SEQUENCE (SIZE(1..maxNrofSRS-ResourceSets)) OF SRS-ResourceSetId OPTIONAL, -- Need N srs-ResourceSetToAddModList SEQUENCE (SIZE(1..maxNrofSRS-ResourceSets)) OF SRS-ResourceSet OPTIONAL, -- Need N srs-ResourceToReleaseList SEQUENCE (SIZE(1..maxNrofSRS-Resources)) OF SRS-ResourceId OPTIONAL, -- Need N srs-ResourceToAddModList SEQUENCE (SIZE(1..maxNrofSRS-Resources)) OF SRS-Resource OPTIONAL, -- Need N … SRS-ResourceSet ::= SEQUENCE { srs-ResourceSetId SRS-ResourceSetId, srs-ResourceIdList SEQUENCE (SIZE(1..maxNrofSRS-ResourcesPerSet)) OF SRS-ResourceId OPTIONAL, -- Cond Setup |
| Apple | Not sure if we can say it’s a violation | The ASN.1 forces the NW to always have atleast 1 resource in a set at configuration time. |
| Huawei | Not sure | It is better to clarify what specification text that the configuration is against for better checking. But it is correct that the configuration should not be an intended configuration. |
| Nokia | Network behavior | As the proposed change does not seem compatible with Rel-15 UE implementation, we would consider that the network must ensure that such a scenario does not occur. So we propose not to do anything. |
| ZTE | Not sure | Tend to agree with Apple, the ASN.1 does not allow network to configure SRS resource set with no SRS resource ID. |

Q5: Do you agree that the current spec doesn’t mandate on network to ensure when reconfiguring the SRS Resource Sets to have SRS Resources associated to each Set?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | Yes | Current RRC spec only mandates Network to release SRS-Resource instances upon reception of an SRS release request from lower layers: Upon receiving an SRS release request from lower layers, for all bandwidth parts of an indicated serving cell the UE shall: 1> release SRS-Resource instances configured in SRS-Config. Then, it may leave some of the SRS-ResourceSets with no associated SRS-resources upon HO, as explained in the example of cover sheet of R2-2010563. In this case, it’s not clear how UE should handle this situation. |
| Apple | No (But) | Per ASN.1, the NW has to associate atleast a resource to a set. In this sense the NW is mandated. But the current UE action (based on UE procedures) can leave the set to be empty. |
| Huawei, HiSilicon | Yes, but | Not sure if there is any problem identified if there is any SRS resource set associating an empty SRS resource ID. |
| Nokia | No | This should be network responsibility to ensure a resource is mapped to a set. Then agree with Huawei what is the exact issue in leaving the set to be empty? |
| ZTE | Yes, but | Same question as Huawei, maybe it is good to first clarify the problem when SRS reource set is associated with an empty SRS resource ID. |

We suggest having one of the changes to be included in the spec in order to handle this case (open to other suggestion):

- Option#1: UE to implicitly release the SRS Resource Sets upon PUCCH release request (this CR).

- Option#2: Network to ensure that all configured SRS Resource Sets shall have SRS Resources re-associated to it at all time.

- Option#3: Network to explicitly release the SRS Resource Sets in the reconfiguration message (e.g. RRC Reconfiguration that contains RRCReconfigurationWithSync) that have no SRS Resources associated to.

Q6: Which one of the above 3 options do you prefer? In addition, please indicate from which release (Rel-15 or Rel-16) the CR has to be introduced if you support the CR or any change

| Company | Option 1/2/3? | Comments |
| --- | --- | --- |
| Qualcomm | Prefer Option#2/3 (from Rel-15) Option#1 is also acceptable (from Rel-16) | All of them can resolve the issue. We are open for discussion solution, as long as this issue this addressed in the spec. We prefer either Option#2 or 3 which rely on Network configuration. It should be simpler than changing UE Rel-15 behavior. So, if majority prefer Option#2/3, we think CR should be introduced from Rel-15. Option#1 is also acceptable to us. But it will introduce different UE behavior from Rel-15 (i.e. not release SRS resource set). Thus, if majority prefer Option#1, we think CR should be introduced from Rel-16. |
| Apple | Prefer opt #3 | We are not sure how NW can ensure opt#2. Opt#1 needs changes for Rel-15 UE, so might be better to just go with option #3. |
| Huawei, HiSilicon | Up to network implementation | Option 1 is obviously an NBC change. Option 2 and 3 are basically the same solution, which is up to network implementation. |
| Nokia |  | Agree with Huawei here that the proposed change is NBC. |
| ZTE | Up to network implementation | Same view as Huawei and Nokia. |

## Configuration of first active BWPs

The related CRs are:

R2-2010665	Corrections on configuration of first active BWPs	Huawei, HiSilicon	CR	Rel-15	38.331	15.11.0	2269	-	F	NR_newRAT-Core

R2-2010666	Corrections on configuration of first active BWPs	Huawei, HiSilicon	CR	Rel-16	38.331	16.2.0	2270	-	A	NR_newRAT-Core

According to R2-2010665/R2-2010666, the reason for the change is:

Reason for change:

The condition for firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id is SyncAndCellAdd which is describe as:

This field is mandatory present for a SpCell upon PCell change and PSCell addition/change and upon RRCSetup/RRCResume.

The field is mandatory present for an SCell upon addition.

For SpCell, the field is optionally present, Need N, upon reconfiguration without reconfigurationWithSync, and upon reconfiguration with reconfigurationWithSync to the same SpCell.

In all other cases the field is absent.

For SpCell, the downlink/uplink first active BWP can be seen as one-shot configuration, as they are only used immediately upon the RRC reconfiguration. The need code can be Need N.

However, for SCell, the downlink/uplink first active BWPs are used for each SCell activation using MAC CE. After the SCell addition, the SCell configuration may be reconfigured, so it should be clarified that the configured firstActiveUplinkBWP-Id/firstActiveDownlinkBWP-Id should be maintained.

And corresponding spec change is:

ServingCellConfig ::=               SEQUENCE {

tdd-UL-DL-ConfigurationDedicated    TDD-UL-DL-ConfigDedicated                                   OPTIONAL,   -- Cond TDD

initialDownlinkBWP                  BWP-DownlinkDedicated                                       OPTIONAL,   -- Need M

downlinkBWP-ToReleaseList           SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Id                  OPTIONAL,   -- Need N

downlinkBWP-ToAddModList            SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Downlink            OPTIONAL,   -- Need N

firstActiveDownlinkBWP-Id           BWP-Id                                                      OPTIONAL,   -- Cond SyncAndCellAdd

| Conditional Presence | Explanation |
| --- | --- |
| MeasObject | This field is mandatory present for the SpCell if the UE has a measConfig, and it is optionally present, Need M, for SCells. |
| SCellOnly | This field is optionally present, Need R, for SCells. It is absent otherwise. |
| ServingCellWithoutPUCCH | This field is optionally present, Need S, for SCells except PUCCH SCells. It is absent otherwise. |
| SyncAndCellAdd | This field is mandatory present for a SpCell upon PCell change and PSCell addition/change and upon RRCSetup/RRCResume. The field is mandatory present for an SCell upon addition, and absent for SCell in other cases, Need M. For SpCell, the field is optionally present, Need N, upon reconfiguration without reconfigurationWithSync, and upon reconfiguration with reconfigurationWithSync to the same SpCell.. In all other cases the field is absent. |
| TDD | This field is optionally present, Need R, for TDD cells. It is absent otherwise. |

Q7: Do you agree the intention and the spec change in Rel-15/Rel-16 CR R2-2010665/R2-2010666 on configuration of first active BWPs?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | Yes | We echo the issue raised by Huawei. Unlike the SpCell that is used as one-shot configuration, first active BWP should be stored by the UE and maintained for SCell upon SCell deactivation. |
| Apple | Ok, but | If we are making corrections, then the field is only mandatory of an SCell with Uplink, i.e., for DL-only SCells, there will be on firstActiveUplinkBWP-Id (which also uses the tag SyncAndCellAdd). Maybe that can be clarified as well? |
| Huawei, HiSilicon | Yes as proponents | Ok to further clarify as Apple suggested. |
| Nokia | Yes, is CR NBC? | We are supportive of this CR but afraid it is NBC based on following interpretation. Once UE is configured with the first active BWP, that is taken into use even if SCell is deactivated. So from that perspective, this may not be needed. That said, we think this interpretation is easier than the Need N. |
| ZTE | Yes | We support the CR, regarding the comment from Apple, for DL-only SCell, seems the description of parent “uplinkConfig” field can already avoid misinterpretation (the parent field is not included, so the Condition of child field won’t work). uplinkConfig Network may configure this field only when uplinkConfigCommon is configured in ServingCellConfigCommon or ServingCellConfigCommonSIB. Addition or release of this field can only be done upon SCell addition or release (respectively). |

## HARQ-ACK spatial bundling and CBG

The related CRs are:

R2-2009355	Corrections on the configurations of HARQ-ACK spatial bundling and CBG in 38.331	CATT	CR	Rel-15	38.331	15.11.0	2058	-	F	NR_newRAT-Core

R2-2009356	Corrections on the configurations of HARQ-ACK spatial bundling and CBG in 38.331	CATT	CR	Rel-16	38.331	16.2.0	2059	-	A	NR_newRAT-Core

According to R2-2009355/R2-2009356, the reason for the change and spec changes are:

| Reason for change: | （1）According to RAN1#91, the following conclusion has been made: Conclusion: - In Rel-15, there is no consensus to support spatial domain HARQ-ACK bundling when a UE is configured with CBG It makes a restriction that the spatial bundling of HARQ-ACKs and the CBG transmission for any PDSCH serving cell within the corresponding cell group can’t be configured simultaneously. Therefore, once configures spatial bundling of HARQ-ACKs, the CBG transmissions for any PDSCH serving cell within the corresponding cell group can’t be configured. （2）maxNrofSRS-ResourceSets-1 is the maximum number of SRS resource minus 1, but not the maximum number of SRS resource in an SRS resource set minus-1. |
| --- | --- |
| Summary of change: | （1）Modify the filed description of the following parameters to restrict that the spatial bundling of HARQ-ACKs and the CBG transmission for any PDSCH serving cell within the corresponding cell group can’t be configured simultaneously in 6.3.2. - harq-ACK-SpatialBundlingPUCCH - harq-ACK-SpatialBundlingPUSCH - codeBlockGroupTransmission （2）Delete ”in an SRS resource set”from the definitions of maxNrofSRS-ResourceSets-1 in 6.4. |

As we see, there are two kinds of spec changes:

（1）Modify the filed description of the following parameters to restrict that the spatial bundling of HARQ-ACKs and the CBG transmission for any PDSCH serving cell within the corresponding cell group can’t be configured simultaneously in 6.3.2:

PhysicalCellGroupConfig field descriptions

harq-ACK-SpatialBundlingPUCCH Enables spatial bundling of HARQ ACKs. It is configured per cell group (i.e. for all the cells within the cell group) for PUCCH reporting of HARQ-ACK. It is only applicable when more than 4 layers are possible to schedule. When the field is absent, the spatial bundling is disabled (see TS 38.213 [13], clause 9.1.2.1). If the field is configured, the codeBlockGroupTransmission cannot be configured for any PDSCH serving cells within the corresponding cell group.

harq-ACK-SpatialBundlingPUSCH Enables spatial bundling of HARQ ACKs. It is configured per cell group (i.e. for all the cells within the cell group) for PUSCH reporting of HARQ-ACK. It is only applicable when more than 4 layers are possible to schedule. When the field is absent, the spatial bundling is disabled (see TS 38.213 [13], clauses 9.1.2.2 and 9.1.3.2). If the field is configured, the codeBlockGroupTransmission cannot be configured for any PDSCH serving cells within the corresponding cell group.

PDSCH-ServingCellConfig field descriptions

codeBlockGroupTransmission Enables and configures code-block-group (CBG) based transmission (see TS 38.213 [13], clause 9.1.1). The filed cannot be configured if the spatial bundling of HARQ-ACKs on PUCCH or PUSCH is configured for the corresponding PUCCH group.

Q8: Do you agree the intention and the 1st spec change in Rel-15/Rel-16 CR R2-2009355/R2-2009356 on HARQ-ACK spatial bundling and CBG (i.e. clarify the spatial bundling of HARQ-ACKs and the CBG transmission for any PDSCH serving cell within the corresponding cell group can’t be configured simultaneously) ?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | See comments (agree the intention for Rel-15 only, but the change is not correct) | We agree with the intention: per RAN1 conclusion, spatial bundling for HARQ can't be configured with CBG is configured (on UL). However, the CR failed to capture that not to support spatial domain HARQ-ACK bundling when a UE is configured with CBG. It only captured not to support CBG if spatial bundling is configured. In addition, the RAN1 decision was for Rel.15 only. We are not sure why the CR is for Rel.16 as well. Based on our understanding, we recommend to change the correction for Rel-15 as: "UE cannot be configured with both spatial bundling and codeBlockGroupTransmission within the same PUCCH group" |
| Apple | Yes, but | Should the change be “"UE cannot be configured with both spatial bundling and codeBlockGroupTransmission within the same cell group" |
| Huawei, HiSilicon | Yes |  |
| Nokia | CR is not needed | Based on our RAN1 delegate’s view the limitation is true, but the two functionalities don't work together anyway as one is designed to bundle HARQ-ACKs of two TBs into a single bit, when the other is designed to split the HARQ-ACK of one TB to multiple sub-HARQ-ACKs. The 38.213 defines that if both are configured, then you UE follows the spatial bundling branch. |
| ZTE | Yes | Agree with Apple. |

（2）Delete ”in an SRS resource set”from the definitions of maxNrofSRS-ResourceSets-1 in 6.4.

maxNrofSRS-Resources-1                  INTEGER ::= 63      -- Maximum number of SRS resources minus 1.

Q9: Do you agree the intention and the 2nd spec change in Rel-15/Rel-16 CR R2-2009355/R2-2009356 on HARQ-ACK spatial bundling and CBG (i.e. delete ”in an SRS resource set” from the definitions of maxNrofSRS-ResourceSets-1 in 6.4) ?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | Yes | It looks like a typo |
| Apple | Yes |  |
| Huawei, HiSilicon | Yes |  |
| ZTE | Yes |  |

## FH configuration for 1-symbol PUCCH

The related CRs are:

R2-2009844	FH configuration for 1-symbol PUCCH	Ericsson	CR	Rel-15	38.331	15.11.0	2137	-	F	NR_newRAT-Core

R2-2009845	FH configuration for 1-symbol PUCCH	Ericsson	CR	Rel-16	38.331	16.2.0	2138	-	A	NR_newRAT-Core, TEI16

According to R2-2009844/R2-2009845, the reason for the change are:

| Reason for change: | At #109e,RAN2 agreed after email discussion [AT109e][068][NR15] (R2-2002324) to capture the following in the meeting notes: RAN2 Understanding [AT109e][068][NR15], (no need to update TS): - From RAN2 perspective, It is understood from RAN1 discussions that the presence or absence of the intra-slot frequency hopping configuration (a.k.a intraSlotFrequencyHopping and secondHopPRB) does not have any impact on how the 1-symbol PUCCH is transmitted. a) For UEs supporting PUCCH format 0/2 without intra-slot frequency hopping, intra-slot frequency hopping configuration can be provided but will be ignored for 1-symbol PUCCH. b) For UEs NOT supporting PUCCH format 0/2 without intra-slot frequency hopping, to avoid possible different interpretations by the UE, 1-symbol PUCCH can still be configured no matter intra-slot frequency hopping configuration is provided or not. To avoid future inter-operability issues, this purely signalling related requirement should be captured in the RRC specification. |
| --- | --- |

And corresponding spec change is:

PUCCH-Resource field descriptions

intraSlotFrequencyHopping Enabling intra-slot frequency hopping, applicable for all types of PUCCH formats. For long PUCCH over multiple slots, the intra and inter slot frequency hopping cannot be enabled at the same time for a UE. The UE shall ignore this field if 1-symbol PUCCH is configured (nrofSymbols is set to 1). See TS 38.213 [13], clause 9.2.1.

pucch-ResourceId Identifier of the PUCCH resource.

secondHopPRB Index of first PRB after frequency hopping of PUCCH. This value is applicable for intra-slot frequency hopping (see TS 38.213 [13], clause 9.2.1) or inter-slot frequency hopping (see TS 38.213 [13], clause 9.2.6).The UE shall ignore this field if 1-symbol PUCCH is configured (nrofSymbols is set to 1).

Q10: Do you agree the intention and the spec change in Rel-15/Rel-16 CR R2-2009844/R2-2009845 on FH configuration for 1-symbol PUCCH?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | Yes | We think it should be the common RAN1 understanding from previous discussions, and should be captured in RRC spec. |
| Apple | Yes | Ok with the change |
| Huawei, HiSilicon | No | Agree with the intention but the change is not needed. There is no point for the network to configure intraSlotFrequencyHopping in the case of 1-symbol PUCCH. Even in case intraSlotFrequencyHopping is configured for 1-symbol PUCCH (mis-configuration), the only way the UE can do is to ignore the intraSlotFrequencyHopping configuration. Cannot see a need to specify anything in specifications. |
| Nokia | No | This was agreed to be captured in chairman’s notes based on Nokia TDOC and CR. At that time companies said no need to capture this officially to CR, now why we want to capture this? |
| ZTE |  | If this has already been agreed to be captured in Chairman notes (as Nokia mentioned), we are fine to not capture anything in spec. |

## Clarification on p-Max in FR2

The related CRs are:

R2-2010530	clarification on p-Max in FR2 rel-15	NTT DOCOMO, INC.	CR	Rel-15	38.331	15.11.0	2236	-	F	NR_newRAT-Core

R2-2010531	Clarification on p-Max in FR2	NTT DOCOMO, INC.	CR	Rel-16	38.331	16.2.0	2237	-	A	NR_newRAT-Core

According to R2-2010530/R2-2010531, the reason for the change are:

Reason for change:

As Pmax was not specified for a cell on FR2 carrier frequency in rel-15 RAN4 spec (R4-1910262), if the Pmax in FrequencyInfoUL is absent, UE shall apply maximum transmit power according to TS 38.101-2 in case of an FR2 cell. Also, if p-Max is present on a carrier frequency in FR2, the UE shall ignore the field and applies the maximum power according to TS 38.101-2.

And corresponding spec change is:

FrequencyInfoUL field descriptions

p-Max Maximum transmit power allowed in this serving cell. The maximum transmit power that the UE may use on this serving cell may be additionally limited by p-NR-FR1 (configured for the cell group) and by p-UE-FR1 (configured total for all serving cells operating on FR1). If absent, the UE applies the maximum power according to TS 38.101-1 [15] in case of an FR1 cell or TS 38.101-2 [39] in case of an FR2 cell. In this release of the specification, if p-Max is present on a carrier frequency in FR2, the UE shall ignore the field and applies the maximum power according to TS 38.101-2 [39]. Value in dBm.

Q11: Do you agree the intention and the spec change in Rel-15/Rel-16 CR R2-2010530/R2-2010531 on clarification on p-Max in FR2?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | Yes | We have the same understanding. |
| Apple | Yes but | We prefer the wording “ the network does not configure p-Max for a carrier frequency in FR2”. Since there are no RAN4 requirements, we wonder on why NW would configure p-Max for FR2. |
| Huawei, HiSilicon | Yes but | We also understand the network would not configure P-max for FR2 as it was not defined in RAN4. Thus we think the wording can be improved. |
| Nokia | Yes | Reference correction is good and seems needed, but the latter sentence should not be added as the signaling is dedicated and for that NW does not configure this field if not necessary. So we would be okay with this after the wording change. |
| ZTE | Yes |  |

## Deactivation of SCells for Overheating Mitigation

The related CRs are:

R2-2010557	Allowing Deactivation of SCells for Overheating Mitigation	Qualcomm Incorporated	CR	Rel-15	38.331	15.11.0	2242	-	F	NR_newRAT-Core

R2-2010558	Allowing Deactivation of SCells for Overheating Mitigation	Qualcomm Incorporated	CR	Rel-16	38.331	16.2.0	2243	-	A	NR_newRAT-Core

According to R2-2010557/R2-2010558, the reason for the change are:

Reason for change:

When overheating is detected, the UE can use reducedCCsDL and reducedCCsUL to indicate to the network the desired number of SCCs to keep after reduction of SCells in DL and UL respectively. In the current field description of the two IEs, the wording seems to imply that the SCC reduction will only be done via SCell deconfiguration (and not by SCell deactivation).

reducedCCsDL Indicates the UE's preference on reduced configuration corresponding to the maximum number of downlink SCells indicated by the field, to address overheating. This maximum number includes both SCells of the MCG and PSCell/SCells of the SCG.

reducedCCsUL Indicates the UE's preference on reduced configuration corresponding to the maximum number of uplink SCells indicated by the field, to address overheating. This maximum number includes both SCells of the MCG and PSCell/SCells of the SCG.

Comparing to activation of SCells, (re)configuration of SCells can take longer time. In order for the network and the UE to have flexible choices, the above wording should be updated to reflect that the network may choose to either deconfigure or deactivate the concerned SCells.

And corresponding spec change is:

UEAssistanceInformation field descriptions

reducedCCsDL Indicates the UE's preference on reduction of the maximum number of active downlink SCells indicated by the field, to address overheating. This maximum number includes both SCells of the MCG and PSCell/SCells of the SCG.

reducedCCsUL Indicates the UE's preference on reduction of the maximum number of active uplink SCells indicated by the field, to address overheating. This maximum number includes both SCells of the MCG and PSCell/SCells of the SCG.

Q12: Do you agree that the term “reduction configuration” carries both meaning, de-configuring or deactivating an SCell?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | Yes | We think both de-configuring and deactivating an SCell should be allowed by the network to reduce the number of active CCs. |
| Apple | Yes | Agree with the changes in the CR. |
| Huawei, HiSilicon | No | It has been discussed in Rel-15 and Rel-16 power saving topic but not agreed. The change impacts the UE interpretation, it is too late for Rel-15. Besides, although the IE indicates the reduced number of configured CCs, it does not prevent the NW from deactivating the SCells to alleviate the overheating. |
| Nokia | No | Not needed. It is still just UE 'suggestion'. NW may or may not use that (this is stated in stage 2). |
| ZTE | No | Same view as Huawei and Nokia. |

Q13: Do you agree the intention and the spec change in Rel-15/Rel-16 CR R2-2010557/R2-2010558 on deactivation of SCells for Overheating Mitigation?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | Yes | The intention is just to clarify both de-configuring and deactivating an SCell can work. And network has flexibility to choose either deconfigure or deactivate the concerned active SCells. |
| Apple | Yes | We want to emphasize that operation is only on the active SCells. |
| Huawei, HiSilicon | No | Please see comments for Q12. |
| Nokia | No | Not needed. It is still just UE 'suggestion'. NW may or may not use that (this is stated in stage 2). |
| ZTE | No |  |

# Conclusion

TBD

# References

[1] R1-2007044, [102-e-LS-AI5-04] Discussion on NR SRS carrier switching

# Contact Information

| Company | Email |
| --- | --- |
| Qualcomm (Peng Cheng) | chengp@qti.qualcomm.comm |
| Apple (Naveen Palle) | naveen_palle@apple.com |
| Huawei (Zhenzhen Cao) | caozhenzhen@huawei.com |
| ZTE (LiuJing) | liu.jing30@zte.com.cn |