3GPP TSG-RAN WG2 #112-e	R2-20xxxxx

E-meeting, 02 – 13 November 2020

Agenda item:	6.2.2

Source: 	Samsung (rapporteur)

Title: 	Discussion summary - [AT112-e][018][IAB] BAP

Document for:	Discussion and decision

# Introduction

This tdoc contains summary of issues raised in individual companies’ submissions, pertaining to AI 6.2.2.

More specifically, before proceeding to propose a way forward, this document collects views on issues raised in the following 4 submissions:

        - R2-2009748	Miscellaneous corrections to 38.340 for IAB	Huawei, HiSilicon, Ericsson	(CR)

        - R2-2009178	BAP behaviour at RLF	Samsung Electronics GmbH	(CR)

        - R2-2009662	The case of traffic of child nodes of a migrating node	Samsung, ZTE, Nokia, Nokia Shanghai Bell	(discussion)

        - R2-2009927	Handling descendant node traffic at HO	Samsung, ZTE	(CR)

# Issues raised

## Focus on R2-2009748 – the BAP spec rapporteur co-sourced miscellaneous corrections

Below is a list of issues covered in R2-2009748, numbered for easier reference:

        - Introduce the ‘DSCP’ abbreviation.

        - Add the defaultUL-BAP-RoutingID and defaultUL-BH-RLC-Channel in the beginning of sec. 5.2.1.2.1 and 5.2.1.4.2 – this is currently missing, the only configuration option currently being listed is via F1AP.

        - Improving the wording in 5.2.1.2.1 for the condition of using the default Routing ID, which was missed from the agreed changes in last meeting. [=> [027] Merge the 2nd change (change “until”) in R2-2007484 into the Rapporteur CR].

        - Update defaultUL-BAP-routingID to defaultUL-BAP-RoutingID in accordance with the RRC spec.

        - Various small editorial changes (typos etc.).

Please share your views on the changes proposed above. If you agree all the issues need fixing, and are happy with the fix proposed in R2-2009748, you can just write ‘ok’. Otherwise please reference the specific issue you would like to discuss further, and explain why you disagree – Is the issue not valid in your opinion and why? Is the issue valid, but you would like to see it fixed in a different way (e.g. different wording)? Please refer to the full document and not just the discussion rapporteur summary above, before sharing your views.

| Company | Views |
| --- | --- |

## R2-2009178 – BAP behaviour in RLF

Below is a list of issues covered in R2-2009178, numbered for easier reference:

        - Introduce a reference to the section on routing (5.2.1.3) into the existing NOTE in section 5.2.1.1.

        - Introduce a new NOTE in section 5.2.1.3 specifying that in the case of RLF the selection of the alternative path is down to implementation while conforming to destination-based routing.

As per the explanation given in R2-2009178, the first change above is suggested because the meaning of “alternative path” in the existing NOTE in 5.2.1.1 is not defined, which could mistakenly imply that the choice of the alternative path at RLF is fully down to implementation (whereas the agreements made mean that destination-based routing should be used).

As per the explanation given in R2-2009178, the second change above is suggested for the same reason, and also since (according to R2-2009178) the RLF behaviour is difficult to infer from the spec.

Please share your views on the changes proposed above. If you agree all the issues need fixing, and are happy with the fix proposed in R2-2009178, you can just write ok. Otherwise please reference the specific issue you would like to discuss further, and explain why you disagree – Is the issue not valid in your opinion and why? Is the issue valid, but you would like to see it fixed in a different way (e.g. different wording)? Please refer to the full document and not just the discussion rapporteur summary above, before sharing your views.

| Company | Views |
| --- | --- |

## R2-2009662 and R2-2009927 – The case of upstream traffic of child nodes of a migrating node

R2-2009662 is a discussion document focusing on the case of upstream traffic from child nodes of a migrating node. In ibid. the following scenario is described: the child node of a migrating node receives the BAP data PDU from its descendant node, but the BH routing configuration has not been configured by F1AP after the last configuration of default BH RLC channel by RRC. (This can happen, according to an interpretation of TS 38.401 [Section 8.2.3.1] as argued in R2-2009662.)

In this scenario, according to the current specification, child node shall use the out-of-date F1AP based routing configuration to forward this BAP data PDU to the migrating node, as the egress link between child node and migrating node is still available. So the child node will forward the BAP data PDU to the migrating IAB node. However, the packet will eventually be discarded at migrating IAB node as its source path is not available.

Two solutions are proposed in R2-2009662 for RAN2’s consideration:

- Adding a NOTE which clearly identifies this issue (R2-2009927, submitted to this meeting)

  - This does not change the normative behavior – what happens with the packets is left to implementation (e.g. discarding) – but it identifies a potentially problematic scenario, which can lead to data loss, and to the inability to mitigate or at least prepare for and make allowances for such data loss (according to R2-2009927).

- Use the normative approach such as the one in R2-2007484 (submitted to the previous RAN2 meeting meeting)

  - This approach would prevent the packets from being passed on to the migrating node in the first place

Please share your views on the issue discussed above (where the migrating node has at least one child node, which in turn receives a BAP data PDU from its descendant node, and where the BH routing configuration has not been configured by F1AP after the last configuration of default BH RLC channel by RRC) and share your views. If you agree that the issue needs fixing, please provide your preferred solution (A? B? A third option not listed above?). Otherwise please explain why you disagree – Is the issue not valid in your opinion and why? Please refer to the full documents and not just the discussion rapporteur summary above, before sharing your views.

| Company | Views |
| --- | --- |

# Proposed way forward for RAN2

In line with the analysis above the discussion rapporteur proposes the following:…