- 3GPP TSG RAN WG2 Meeting #112-e      	                           R2-200xxxx
E-Conference, 2nd – 13th Nov. 2020

Agenda item:	6.16

Source:	Qualcomm Incorporated

Title:	[AT112-e][027][NR TEI16] NeedForGap (QC)

WID/SID:	NR Others

Document for:	Discussion and Decision

# Introduction

It is summary for below email discussion:

- [AT112-e][027][NR TEI16] NeedForGap (QC)

Treat R2-2009401, R2-2010547, R2-2010548, R2-2010555, R2-2010556, R2-2010549, R2-2010550, R2-2010553, R2-2010554, R2-2010551, R2-2010552

Intended outcome: Intermediate: Determine agreeable parts. Final: For agreeable parts, agreed CRs.

Deadline: Short UE Cap

According to Chair’s indication, rapporteur would like to have your response by Thursday 2020-11-05 12:00 UTC, to have time to converge to the 2nd phase if needed.

# Discussion

## 1-bit capability for gap requirement info for LTE

Although the current NeedForGap capability provide high granularity for the gap requirement info, it comes with a huge drawback, the size of the IE can exceed the 200KB, which is inefficient and impractical.

The CR is introducing a 1-bit capability to inform network about gap requirement information across an NR frequency range (FR1/FR2) while UE is on LTE.

R2-2010551	1 bit capbility for gap requirment info for LTE	Qualcomm Incorporated	CR	Rel-16	36.306	16.2.0	1799	-	C	LTE_feMob-Core

R2-2010552	1 bit capbility for gap requirment info for LTE	Qualcomm Incorporated	CR	Rel-16	36.331	16.2.1	4521	-	C	LTE_feMob-Core

Q1: Do you agree that the current LTE NeedForGap capability can significantly increase the size of the capability info message?

| Company | Yes/No | Comments |
| --- | --- | --- |
| QC | Yes | - The LTE NeedForGap capability IE can increase the size of the capability info to more than 200KB. - Given that more NR bands will be added in the future, this will cause a further increase in the size  impractical and inefficient. |
| MediaTek | No | As concluded online, we will not have 1-bit approach in Rel-16 |

Q2: Do you agree with suggested change in Rel-16 CR (R2-2010551/R2-2010552)?

| Company | Yes/No | Comments |
| --- | --- | --- |
| QC | YES | The change is definitely needed as current capability is impractical. |
| MediaTek | No | As concluded online, we will not have 1-bit approach in Rel-16 |

## 1-bit capability for gap requirement info for NR SA

There is a drawback in the current dynamic RRC reporting for gap requirement info, as it’s a 2-step configuration, i.e. UE will be inquired by the network about its gap requirement information in the 1st RRC message, then a 2nd RRC message is needed to configure the gap requirement.

This inefficient approach and may introduce undesired delay when configuring the gap.

The CRs is introducing of 1-bit capability to inform network about gap requirement information across a frequency range (FR1/FR2) while UE is on NR SA, and similar capability while UE is on NR-DC.

These are optional features, so it’s a backward compatible change.

R2-2010549	1 bit capbility for gap requirment info for NR	Qualcomm Incorporated	CR	Rel-16	38.306	16.2.0	0463	-	B	TEI16

R2-2010550	1 bit capbility for gap requirment info for NR	Qualcomm Incorporated	CR	Rel-16	38.331	16.2.0	2239	-	B	TEI16

Q3: Do you agree with suggested change in Rel-16 CR (R2-2010549 & R2-2010550)?

| Company | Yes/No | Comments |
| --- | --- | --- |
| QC | Yes | Proponent - Suggested capability helps in some cases avoiding the 2-step configuration for gap. - It’s a 1-bit optional capability  BC change |
| MediaTek | No | As concluded online, we will not have 1-bit approach in Rel-16 |

## NeedForGap for NR-DC and NE-DC

The CRs below are discussing the support of the dynamic reporting of the NeedForGap for NR-DC and NE-DC. the CRs carry opposite views, hence the CRs suggestions will be discussed separately under the same section (2.3).

R2-2009401	Clarification on NeedForGap reporting in NR-DC and NE-DC	MediaTek Inc., ZTE Corporation, Sanechips	CR	Rel-16	38.331	16.2.0	2067	-	F	NR_newRAT-Core

Reason for Change

In current 38.331, the description of the IE NeedForGapsInfoNR specifies that it is NOT used in NR-DC or in NE-DC.

“The IE NeedForGapsInfoNR indicates whether measurement gap is required for the UE to perform SSB based measurements on an NR target band while NR-DC or NE-DC is not configured.”

However, in 5.3.5.3, it is specified that the UE may report the gap requirement information to MN while it receives the MN RRC message (i.e. RRCReconfiguration received via SRB1). It is unclear that whether the UE still reporting the information to the MN after NR-DC or NE-DC is configured.

RAN2 does not have enough time to conclude how to report the gap requirement information while DC is configured. It requires further discussion on the issues like:

- What UE should do while NeedForGap is changed due to Reconfiguration from SRB3 ? Should the UE report to SN or report to both MN and SN?

- Do we need additional inter-node message for the MN and SN to exchange the NeedForGap info?

- For intra-freq NeedForGap Info, does the UE report SN’s serving cell(s) to MN or report MN’s serving cell(s) to SN?

Therefore, we propose to clarify the UE only reports the gap requirement information to MN while NR-DC or NE-DC is not configured.

Q4: Do you agree with suggested change in Rel-16 CR (R2-2009401)?

| Company | Yes/No | Comments |
| --- | --- | --- |
| QC | NO | We don’t understand why a capability supported in NR-SA is being shut down in NR-DC, especially when it can be an optional feature. The three items provided in the reason of change can be easily addressed: - What UE should do while NeedForGap is changed due to Reconfiguration from SRB3 ? Should the UE report to SN or report to both MN and SN? - Suggestion: No need to support this feature on SRB3 - Do we need additional inter-node message for the MN and SN to exchange the NeedForGap info? - Suggestion: Current Inter-node messages can be enhanced if needed - For intra-freq NeedForGap Info, does the UE report SN’s serving cell(s) to MN or report MN’s serving cell(s) to SN? - Suggestion: SN’s serving cells reported to MN and SN. MN’s serving cells reported to MN. |
| MediaTek | Yes (Proponent) | Since online we have concluded that no extension of dynamic reporting to DC case. This should be a reasonable clarification. |
| Nokia | No | We understand that, in current specification, the dynamic NeedForGap reporting is limited to NR SA as concluded in the online session. However, the NeedForGapsNR field description already limit the reporting to NR SA (while NR-DC or NE-DC is not configured). We understand the CR intention is to make this also clear in procedure text, while it seems not very necessary. |
| ZTE | Yes | Since we have the restriction that NeedForGap is only applicable to NR SA, it is worth making it clear in text procedure for consistency. Otherwise, it will cause confusion to the readers. |
| Google | No | The network should only consider the NeedForGapsNR when the UE is in NR SA and neither in NR-DC nor in NE-DC. It doesn’t matter whether the UE reports the NeedForGapsNR field while the UE is in NR SA (neither NR-DC nor NE-DC), NR-DC or NE-DC. |
| Intel | Yes | We are ok with the clarification. It seems reasonable to make it clear in the procedure text. |
| vivo | No | This clarification is not essential, as there is already limitation in the field description of NeedForGapsInfoNR. |

R2-2010553	gap capability dynamic reporting for NR-DC	Qualcomm Incorporated	CR	Rel-16	38.306	16.2.0	0464	-	B	TEI16

R2-2010554	gap capability dynamic reporting for NR-DC	Qualcomm Incorporated	CR	Rel-16	38.331	16.2.0	2240	-	B	TEI16

CR is suggesting to extend the support of the NR SA dynamic reporting to NR-DC in a similar fashion, by introducing a new capability “nrdc-NeedForGap-Reporting-r16”, to indicate whether UE supports reporting the measurement gap requirement information for NR target while UE is in NR-DC.

Q5: Do you agree with the suggested capability that has been introduced in Rel-16 CR (R2-2010553 & R2-2010554)?

| Company | Yes/No | Comments |
| --- | --- | --- |
| QC | Yes | Proponent - this capability is already supported on NR-SA and it can be introduced to NR-DC with minimal modification to the current ASN.1 and the RRC procedural text. - the introduction of this capability is done in a backward compatible manner, so UE that is not capable of supporting this feature, that can simply refrain from advertising this capability - significant performance gain can be obtained if this capability was part of Rel.16 specifications. |
| MediaTek | No | As concluded online, no dynamic reporting in DC. |

## 1-bit capability for gap requirement info for EN-DC

Per current spec, gap is always configured when performing NR measurement while on EN-DC.

The CR introduces a 1-bit capability to inform network about gap requirement information across an NR frequency range (FR1/FR2) while UE is on EN-DC.

R2-2010547	1 bit capbility for gap requirment info for EN-DC	Qualcomm Incorporated	CR	Rel-16	38.306	16.2.0	0462	-	B	TEI16

R2-2010548	1 bit capbility for gap requirment info for EN-DC	Qualcomm Incorporated	CR	Rel-16	38.331	16.2.0	2238	-	B	TEI16

Q6: Do you agree with suggested change in Rel-16 CR (R2-2010547, R2-2010548 )?

| Company | Yes/No | Comments |
| --- | --- | --- |
| QC | Yes | Proponent - per current spec, gap is always required for NR measurement while in EN-DC  this capability allows measurement without the need of gap (if supported) which bring significant performance gain - An optional capability so it’s a BC change |
| MediaTek | No | As concluded online, we will not have 1-bit approach in Rel-16 |

## NeedForGap for EN-DC

The NeedForGap capability was defined for LTE SA to NR target bands in Rel.16. however the support of this feature didn’t discuss the expected UE/network behaviour when UE transitions from LTE AS to EN-DC and whether can use the same capability.

this CR proposes to extend this capability for the case when UE is in EN-DC and performing measurement on NR target band.

The support of this feature while UE is in EN-DC, is critical for optimum system performance in the field.

R2-2010555	NeedForGap for EN-DC	Qualcomm Incorporated	CR	Rel-16	38.306	16.2.0	0465	-	B	TEI16

R2-2010556	NeedForGap for EN-DC	Qualcomm Incorporated	CR	Rel-16	38.331	16.2.0	2241	-	B	TEI16

Q7: Do you agree with suggested change in Rel-16 CR (R2-2010555 & R2-2010556 ) (support of dynamic reporting for NR-DC)?

| Company | Yes/No | Comments |
| --- | --- | --- |
| QC | Yes | - Similar capability is already supported in LTE, why not extending it to EN-DC - It’s an optional feature so the change is BC. |
| MediaTek | No |  |

# Conclusion

TBD

# References