x3GPP TSG-RAN WG2 Meeting #112-e	R2-20xxxxx

Electronic, 02nd – 13th November 2020

Agenda item:	x.x.x

Source: 	Futurewei

Title: 	Summary of [AT112-e][032][NR17] eNPN LS

Document for:	Discussion and Decision

# Introduction

This document is for the following offline discussion.

- [AT112-e][032][NR17] eNPN LS (Futurewei)

Scope: Treat R2-2010691. Determine status / collect comments among RAN2 companies regarding the asked questions. Attempt agreements in RAN2 on aspects for which agreement seems feasible (if any). Create a reply LS. Depending on progress, some aspects may be brought online week2

Intended outcome: Report and Approved LS out

Deadline: Final: End of meeting. Intermediate deadlines by rapporteur.

# Introduction

This document collect companies’ views on how RAN2 should reply to SA2’s incoming LS in R2-2010691/S2-2007827 [1]. The discussion will be in two phases:

Phase 1: Collect company views for a response LS.

Phase 2: Agree reply LS to SA2

For Phase 1 let’s set a deadline of Friday, November 6, 23:59:59 UTC. This is to allow time for the moderator to prepare a draft LS reply reflecting RAN2 concensus view.

The deadline for Phase 2, End of meeting.

# Discussion

LS in R2-2010691/S2-2007827 identified two radio architectures for a UE in the context of simultaneous communication with both SNPN and PLMN:

a)	Dual radio UE using independent Rx/Tx per network (SNPN and PLMN)

b)	Dual radio UE using independent Rx per network (SNPN and PLMN) and a single Tx for one of the two networks only, e.g. the SNPN (whereby UL user-plane and NAS traffic for the other network is tunnelled via the first network using existing IP-based OTT mechanisms)

The LS then goes on to ask RAN2 to consider and respond to 3 questions:

Q1: is a) technically feasible without any new Access-Stratum mechanism and standardization?

Q2: is b) technically feasible taking into account the uplink Access Stratum activity in each network?

Q3: whether in case of b) is it feasible to achieve a very low PER for low latency multicast traffic without sending access stratum feedback to the network (e.g. the SNPN RAN)?

In the following, we provide some background drawn from TR 23.700-07 [2] and TS 22.263 [3] with the purpose of illuminating each of these questions. Furthermore, we pose a number of sub-questions to further clarify company’s understanding and views on the issues involved.

# Questionnaire

## Technical Feasibility of independent Rx/Tx dual radio (a)

For Q1, SA2 has considered a UE with two independent recievers/transmitters (two independent transceivers). One of the independent transceivers is used to communicate with the SNPN, while the second independent transceiver is used to simultaneously communicate with the PLMN.

As an illustrative example of case (a) (Dual radio UE using independent Rx/Tx per network (SNPN and PLMN)) please consider Figures 6.13.2.2-1/6.13.2.2-2 reproduced below from [2] (solution #13):

Figure 6.13.2.1-1 UPF PSA in SNPN

Figure 6.13.2.1-2: UPF PSA in PLMN

Please note that the architectures illustrated in figures 6.13.2.2-1/6.13.2.2-2  provide examples of solutions for case (a). These should not be considered as the only solutions relevant to SA2’s Q1. TR 23.700-07 currently defines 16 solutions that map to KI#2.,

Regarding SA2’s Q1, companies are kindly requested to provide their views on the following questions:

Question 4.2-1: For the case of a dual radio UE that can provide an independent Rx/Tx per network (SNPN and PLMN), is it technically feasible for the UE to simultaneously communicate with both networks, without new AS mechanisms and standardization?

| Company | Answer (Yes/No) | Comments |
| --- | --- | --- |
| OPPO | Yes |  |
| Huawei | Yes |  |

Question 4.2-2: Are there any other issues that RAN2 should consider in order to formulate a response to SA2 for Q1?

| Company | Answer (Yes/No) | Comments |
| --- | --- | --- |

## Technical Feasibility of single Tx dual RX radio (b)

For Q2, SA2 has considered a UE with two independent recievers. One RX is used to receive transmissions from the SNPN, while the second RX is used to receive transmissions from the PLMN. The UE has a single transmitter that can be used to transmit to only one of the networks (either SNPN or PLMN) at a given time. While the TX is used to transmit to the first network, UL user-plane and NAS traffic for the other network is tunnelled via the first network using existing IP-based OTT mechanisms.

As an illustrative example of case (b) (Dual radio UE using independent Rx per network (SNPN and PLMN) and a single Tx for one of the two networks only) please consider Figure 6.18.2.2-1 reproduced below from [2] (solution #18):

Figure 6.18.2.2-1: Architectural overview of a 2Rx/1Tx dual radio UE simultaneously connected to a SNPN and a PLMN

Please note that the architecture illustrated in figure 6.18.2.2-1 provides an example of a solution for case (b), and should not be considered as the only solution relevant to SA2’s Q2. TR 23.700-07 currently defines 16 solutions that map to KI#2.,

Regarding SA2’s Q2, companies are kindly requested to provide their views on the following questions:

Question 4.2-1: In the case of a single TX dual RX radio UE (case b), if the single TX is used to transmit to the SNPN only, is it technically feasible to tunnel UL user-plane and NAS traffic for the PLMN via the SNPN using existing IP-based OTT mechanisms, taking into account the uplink Access Stratum activity in each network?

| Company | Answer (Yes/No) | Comments |
| --- | --- | --- |
| OPPO | No | Rx-only for the second network (i.e., w/o Tx for UL) is not feasible, since UL is required for UEs in RRC_CONNECTED, to carry L12 control messages, i.e., PHY control channel, MAC CE, RLC Control PDU and PDCP Control PDU. |
| Huawei | - | “Single TX dual RX” is in the scope of WID Multi-SIM (RP-201309). This LS can be postponed and wait for the progress of Multi-SIM. |

Question 4.2-2: In the case of a single TX dual RX radio UE (case b), if the single TX is used to transmit to the PLMN only, is it technically feasible to tunnel UL user-plane and NAS traffic for the SNPN via the PLMN using existing IP-based OTT mechanisms, taking into account the uplink Access Stratum activity in each network?

| Company | Answer (Yes/No) | Comments |
| --- | --- | --- |
| OPPO | No | Rx-only for the second network (i.e., w/o Tx for UL) is not feasible, since UL is required for UEs in RRC_CONNECTED, to carry L12 control messages, i.e., PHY control channel, MAC CE, RLC Control PDU and PDCP Control PDU. |
| Huawei | - | Same comment as Q4.2-1. |

Question 4.2-3: Are there any other issues that RAN2 should consider in order to formulate a response to SA2 for Q2?

| Company | Answer (Yes/No) | Comments |
| --- | --- | --- |

## Technical Feasibility of multicast traffic without AS feedback with single Tx dual RX radio

TS 22.263 [3] defines performance requirements for video, imaging and audio for professional applications (VIAPA) (KI#2). In particular, table 6.3.1-1 (reproduced below) provides performance requirements for low latency multicast service for audio and video production applications:

Table 6.3.1-1: Performance requirements for low latency deterministic periodic traffic with multicast service [3].

| Profile | # of active UEs | # of UL streams | # of DL streams | UE Speed | Service Area | E2E latency (Note 1) | Transfer interval (Note 1) | Packet error rate (Note 2, Note 3) | Data rate UL | Data rate DL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Integrated audience services | 50000 | - | 30 multicast streams | 5 km/h | 1.5 km x 1.5 km | 7 ms DL | 3 ms | 10-4 | - | 200 kbit/s |
| Intercom system | 1000 | 240 (Note 4) | 30 multicast streams | 5 km/h | 1.5 km x 1.5 km | 7 ms DL 7 ms UL | 3 ms | 10-4 | 100 kbit/s | 100 kbit/s |
| NOTE 1: Transfer interval refers to periodicity of the packet transfers. It has to be constant during the whole operation. The value given in the table is a typical one, however other transfer intervals are possible as long as the end-to-end latency is ≤ (10 ms – Transfer interval). NOTE 2: Packet error rate is related to a packet size of (Transfer interval × data rate). Packets that do not conform with the end-to-end latency are also accounted as error. NOTE 3: The given requirement for a packet error rate assumes a uniform error distribution. The requirement for packet error rate is stricter if packet errors occur in bursts. NOTE 4: The UL stream originating from a UE may be the source of a DL multicast stream. |  |  |  |  |  |  |  |  |  |  |

Per the NR MBS WID [4] RAN WGs are defining enhancements to support broadcast and multicast service delivery via both point-to-multipoint (PTM) and point-to-point (PTP) transmissions. Therefore, RAN2 should consider both the PTP and PTM cases when addressing Q3 from SA2.

Regarding SA2’s Q3, companies are kindly requested to provide their views on the following questions:

Question 4.3-1: For multicast services delivered via PTP, in the case of a single TX dual radio UE (case b), is it feasible to achieve the performance requirements of table 6.3.1-1 [3] without sending access stratum feedback to the network

| Company | Answer (Yes/No) | Comments |
| --- | --- | --- |
| OPPO | No | Our understanding is PTP cannot be supported for RRC_IDLE/INACTIVE UEs, so that it cannot achieve “without sending access stratum feedback to the network”, as replied to 4.2-2. |
| Huawei | - | Same comment as Q4.2-1. |

Question 4.3-2: For multicast services delivered via PTM, in the case of a single TX dual radio UE (case b), is it feasible to achieve the performance requirements of table 6.3.1-1 [3] without sending access stratum feedback to the network

| Company | Answer (Yes/No) | Comments |
| --- | --- | --- |
| OPPO | See comment | This requires further evaluation by RAN1, i.e., when UE works in RRC_IDLE/INACTIVE state, and thus requires Rx only, can achieve the KPI as above. In MBS session, there are tools being considered to secure the KPIs, e.g., HARQ A/N feedback, which also requires the Tx, so colliding with the requirement of “without sending access stratum feedback to the network”. |
| Huawei | - | Same comment as Q4.2-1. |

Question 4.3-3: Are there any other issues that RAN2 should consider in order to formulate a response to SA2 for Q3?

| Company | Answer (Yes/No) | Comments |
| --- | --- | --- |

# Summary

To be completed

…

# Reference

[1]	R2-2010691, LS on questions to RAN WGs on dual Radio UE (2Rx/2Tx or 2Rx/1Tx) support for simultaneous communication with both SNPN and PLMN (S2-2007827; contact: Futurewei) SA2

[2]	TR 23.700-07, Study on enhanced support of non-public networks (Release 17), V1.1.0

[3]	TS 22.263, Service requirements for video, imaging and audio for professional applications (VIAPA); Stage 1 (Release 17), V17.0.0

[4]	RP-201038, WID revision: NR Multicast and Broadcast Services, Huawei, HiSilicon