3GPP TSG-RAN WG2 Meeting #112-e	R2-2011064

Electronic, 02nd – 13th November 2020

Agenda item:	8.16

Source: 	Futurewei

Title: 	Discussion Summary of [AT112-e][032][NR17] eNPN LS

Document for:	Discussion and Decision

# Introduction

This document is for the following offline discussion.

- [AT112-e][032][NR17] eNPN LS (Futurewei)

Scope: Treat R2-2010691. Determine status / collect comments among RAN2 companies regarding the asked questions. Attempt agreements in RAN2 on aspects for which agreement seems feasible (if any). Create a reply LS. Depending on progress, some aspects may be brought online week2

Intended outcome: Report and Approved LS out

Deadline: Final: End of meeting. Intermediate deadlines by rapporteur.

# Scope

This document collect companies’ views on how RAN2 should reply to SA2’s incoming LS in R2-2010691/S2-2007827 [1]. The discussion will be in two phases:

Phase 1: Collect company views for a response LS.

Phase 2: Agree reply LS to SA2

For Phase 1 let’s set a deadline of Friday, November 6, 23:59:59 UTC. This is to allow time for the moderator to prepare a draft LS reply reflecting RAN2 consensus view.

The deadline for Phase 2, End of meeting.

# Discussion

LS in R2-2010691/S2-2007827 identified two radio architectures for a UE in the context of simultaneous communication with both SNPN and PLMN:

a)	Dual radio UE using independent Rx/Tx per network (SNPN and PLMN)

b)	Dual radio UE using independent Rx per network (SNPN and PLMN) and a single Tx for one of the two networks only, e.g. the SNPN (whereby UL user-plane and NAS traffic for the other network is tunnelled via the first network using existing IP-based OTT mechanisms)

The LS then goes on to ask RAN2 to consider and respond to 3 questions:

Q1: is a) technically feasible without any new Access-Stratum mechanism and standardization?

Q2: is b) technically feasible taking into account the uplink Access Stratum activity in each network?

Q3: whether in case of b) is it feasible to achieve a very low PER for low latency multicast traffic without sending access stratum feedback to the network (e.g. the SNPN RAN)?

In the following, we provide some background drawn from TR 23.700-07 [2] and TS 22.263 [3] with the purpose of illuminating each of these questions. Furthermore, we pose a number of sub-questions to further clarify company’s understanding and views on the issues involved.

# Questionnaire

## Technical Feasibility of independent Rx/Tx dual radio (a)

For Q1, SA2 has considered a UE with two independent receivers/transmitters (two independent transceivers). One of the independent transceivers is used to communicate with the SNPN, while the second independent transceiver is used to simultaneously communicate with the PLMN.

As an illustrative example of case (a) (Dual radio UE using independent Rx/Tx per network (SNPN and PLMN)) please consider Figures 6.13.2.1-1/6.13.2.1-2 reproduced below from [2] (solution #13):

Figure 6.13.2.1-1 UPF PSA in SNPN

Figure 6.13.2.1-2: UPF PSA in PLMN

Please note that the architectures illustrated in figures 6.13.2.1-1/6.13.2.1-2 provide examples of solutions for case (a). These should not be considered as the only solutions relevant to SA2’s Q1. TR 23.700-07 currently defines 16 solutions that map to KI#2.,

Regarding SA2’s Q1, companies are kindly requested to provide their views on the following questions:

Question 4.1-1: For the case of a dual radio UE that can provide an independent Rx/Tx per network (SNPN and PLMN), is it technically feasible for the UE to simultaneously communicate with both networks, without new AS mechanisms and standardization?

| Company | Answer (Yes/No) | Comments |
| --- | --- | --- |
| OPPO | Yes |  |
| Huawei | Yes |  |
| CATT | Yes |  |
| Lenovo | Yes | We assume that there is no need for coordination between PLMN and SNPN. |
| ZTE | Yes |  |
| Sony | Depends | It depends on the carrier frequencies of the connections in the dual radio UE. The carrier frequencies may be intra band or inter band. The UE receiver frontend will be affected in the same way as for carrier aggregation but with uncoordinated carriers. This shall therefore be investigated. We think this is not limited to RAN4. |
| Nokia | Yes |  |
| Ericsson | Yes | If there are separate connections to the SNPN and PLMN with separate protocol stacks for the transmitter and receiver chains, the transmissions in both networks can work independent of each other. |
| LGE | Yes |  |
| Apple | Yes | There might be however co-existence concerns on UE based on band combinations chosen. It would be good to confirm with RAN4. |
| Futurewei | Yes | Assuming that the UE’s frontend is able to able to operate independently on the carrier frequencies/bands in use in each network, without significant interference between the two radios. |
| Qualcomm | Yes but | Assuming the two links are not on the same or contiguous carriers, it should be possible. In deployments, PLMN and SNPN will likely be in different bands. |

Question 4.1-2: Are there any other issues that RAN2 should consider in order to formulate a response to SA2 for Q1?

| Company | Item | Comments |
| --- | --- | --- |
| Lenovo | PLMN NG-RAN | Referring to the figures 6.13.2.1-1/6.13.2.2-2 do we need to consider E-UTRA/5GC and/or NR/5GC for the PLMN? |
| Lenovo | RF aspects | The feasibility of the dual radio UE use-cases may have RF implications as well depending on the deployment scenarios (same or different band operation of SNPN/PLMN). Therefore, it might be good to contact RAN4 to study the use-cases as well. |
| Apple | Lenovo’s concern on PLMN for NG-RAN |  |
| Qualcomm | Band and RF | Agree with Lenovo |

Summary of section 4.1:

Question 4.1-1: 10 out of 12 companies believe that the answer to this question is Yes it is feasible. 1 company thinks that the answer depends on the the carrier frequencies of the connections in the dual radio UE. 1 company believes it is feasible, assuming the two links are not on the same or contiguous carriers.

Question 4.1-2: Two companies (Lenovo and Apple) wonder if RAN2 needs to consider both E-UTRA/5GC and NR/5GC scenarios for the PLMN. The moderator’s understanding is that SA2 is not considering E-UTRA for eNPN. However, companies are free to raise questions about the scope within SA2.

Lenovo and Qualcomm also believe that the feasibility of the dual radio use cases may have RF implications as well depend on the deployment scenarios (same or different band operation of SNPN/PLMN). Therefore, proponents suggests contacting RAN4 to study these use cases. The moderator’s view is that RAN2 can point out such possibly dependencies in our response to SA2.

Based on these responses the moderator proposes the following response to SA2 on Q1:

Q1: is a) technically feasible without any new Access-Stratum mechanism and standardization?

A1: For scenario a) dual radio UE using independent Rx/Tx per network, RAN2 concluded that it is technically feasible for the UE to simultaneous communicate with both SNPN and PLMN without new AS mechanisms and standardization.

This assumes that the UE’s RF frontend is able to operate independently on the carrier frequencies/bands in use in each network, without this operation resulting in significant interference between the two radios. RAN4 may need to consider whether such independent operation can be supported by a dual radio UE for specific combinations of carrier frequencies/bands.

## Technical Feasibility of single Tx dual RX radio (b)

For Q2, SA2 has considered a UE with two independent receivers. One RX is used to receive transmissions from the SNPN, while the second RX is used to receive transmissions from the PLMN. The UE has a single transmitter that can be used to transmit to only one of the networks (either SNPN or PLMN) at a given time. While the TX is used to transmit to the first network, UL user-plane and NAS traffic for the other network is tunnelled via the first network using existing IP-based OTT mechanisms.

As an illustrative example of case (b) (Dual radio UE using independent Rx per network (SNPN and PLMN) and a single Tx for one of the two networks only) please consider Figure 6.18.2.2-1 reproduced below from [2] (solution #18):

Figure 6.18.2.2-1: Architectural overview of a 2Rx/1Tx dual radio UE simultaneously connected to a SNPN and a PLMN

Please note that the architecture illustrated in figure 6.18.2.2-1 provides an example of a solution for case (b), and should not be considered as the only solution relevant to SA2’s Q2. TR 23.700-07 currently defines 16 solutions that map to KI#2.,

Regarding SA2’s Q2, companies are kindly requested to provide their views on the following questions:

Question 4.2-1: In the case of a single TX dual RX radio UE (case b), if the single TX is used to transmit to the SNPN only, is it technically feasible to tunnel UL user-plane and NAS traffic for the PLMN via the SNPN using existing IP-based OTT mechanisms, taking into account the uplink Access Stratum activity in each network?

| Company | Answer (Yes/No) | Comments |
| --- | --- | --- |
| OPPO | No | Rx-only for the second network (i.e., w/o Tx for UL) is not feasible, since UL is required for UEs in RRC_CONNECTED, to carry L12 control messages, i.e., PHY control channel, MAC CE, RLC Control PDU and PDCP Control PDU. |
| Huawei | - | “Single TX dual RX” is in the scope of WID Multi-SIM (RP-201309). This LS can be postponed and wait for the progress of Multi-SIM. |
| CATT | - | Agree with Huawei |
| Lenovo | No |  |
| ZTE |  | Does it mean the DL user plane data is still transmitted through the AS connection between PLMN gNB and UE, while the UL data is sent as the IP packet, if it does, we don’t think it’s feasible as OPPO explained. |
| Sony | No | Agree with OPPO and in addition UE frontend aspects from Q4.1-1 are applicable. |
| Nokia | Yes | Our understanding is that in this scenario all UP and CP (NAS) traffic to/from the SNPN is tunneled over the PLMN. This scenario is feasible as far as NAS and upper layers in the UE ensure that any triggers (paging, track area update, UL UP traffic etc.) that require traffic (including CP and UP) for the SNPN is carried over the PLMN. (This is similar to the case when paging over 3GPP access can trigger connection establishment over non-3GPP access.) Therefore, the UE remains in IDLE/INACTIVE in SNPN RAN. Generally, any scenarios where the UE can remain IDLE/INACTIVE in the SNPN side can be supported. We do not think that the support of this scenario (with the restriction that UE remains in IDLE/INACTIVE in SNPN side) depends on Multi-SIM work. |
| Ericsson | No, UL AS cannot be tunneled | It is possible to tunnel UP and NAS traffic for the PLMN via the SNPN using OTT mechanisms, but UL Access Stratum activity cannot be taken into account, i.e. the SNPN cannot carry L1/L2 feedback messages of the PLMN, see examples listed by OPPO. The scope of the Multi-SIM covers “RX-only” to monitor paging in the PLMN while the UE is in RRC_IDLE/RRC_INACTIVE in the PLMN and in RRC_CONNECTED in the SNPN. In other words, the UE in Multi-SIM does not support reception of any DL traffic from the PLMN if Tx/Rx are used for the SNPN, i.e. no simultaneous reception of DL traffic from both networks is supported. However, if the UE is paged in the PLMN, it can request the release of the connection in the SNPN and use Rx/Tx in order to connect to the PLMN. So the scope for VIAPA is different. |
| LGE | No | Same understanding with OPPO |
| Apple | No | Agree with Huawei that the scope of this WID is Multi-SIM |
| Futurewei | Depends | We agree with other companies: It is possible for the UE to tunnel UP and NAS traffic for one network (e.g. PLMN) via a second network (SNPN in this example) as long as the RRC state of the UE for the first network (PLMN in this example) is either RRC_IDLE or RRC_INACTIVE. Consequently, a Rel. 16 UE that is in RRC_CONNECTED in one network (SNPN in this example) will not send any L1/L2 feedback to the other network (PLMN in this example). |
| Qualcomm | No | Agree with others that L1/L2 transmissions cannot be tunneled. The scenario here is not related to Multi-SIM where the two connections are independent and there is no tunneling between PLMNs of different USIMs. |

Question 4.2-2: In the case of a single TX dual RX radio UE (case b), if the single TX is used to transmit to the PLMN only, is it technically feasible to tunnel UL user-plane and NAS traffic for the SNPN via the PLMN using existing IP-based OTT mechanisms, taking into account the uplink Access Stratum activity in each network?

| Company | Answer (Yes/No) | Comments |
| --- | --- | --- |
| OPPO | No | Rx-only for the second network (i.e., w/o Tx for UL) is not feasible, since UL is required for UEs in RRC_CONNECTED, to carry L12 control messages, i.e., PHY control channel, MAC CE, RLC Control PDU and PDCP Control PDU. |
| Huawei | - | Same comment as Q4.2-1. |
| CATT | - | Agree with Huawei |
| Lenovo | No |  |
| ZTE |  | Same comment as Q4.2-1. |
| Sony | No | Same as Q4.2-1 |
| Nokia | Yes | Same comment as Q4.2-1 (PLMN and SNPN roles are swapped) |
| Ericsson | No, UL AS cannot be tunneled | The same applies independent of whether the single Tx is for PLMN or SNPN, see comment above. |
| LGE | No |  |
| Apple | No | Agree with Huawei. |
| Futurewei | Depends | Similar to comment on Q4.2-1: It is possible for the UE to tunnel UP and NAS traffic for one network (e.g. SNPN) via a second network (PLMN in this example) as long as the RRC state of the UE for the first network (SNPN in this example) is either RRC_IDLE or RRC_INACTIVE. Consequently, a Rel. 16 UE that is in RRC_CONNECTED in one network (PLMN in this example) will not send any L1/L2 feedback to the other network (SNPN in this example). |
| Qualcomm | No | Same response as Q4.2-1 |

Question 4.2-3: Are there any other issues that RAN2 should consider in order to formulate a response to SA2 for Q2?

| Company | Item | Comments |
| --- | --- | --- |
| Lenovo | RF aspects | Same comment as for Q4.1-2. |
| Apple | RF Aspects | Would be good to confirm with RAN4 on potential co-existence issues. |

Summary of section 4.2:

Question 4.2-1/2: 6 companies do not believe this is feasible due to L1/L2 UL feedback to the network in RRC_CONNECTED. 1 company believes that it is feasible to tunnel UL Data and NAS to one network via another network, as long as the UE is in RRC_IDLE/RRC_INACTIVE in the first network. 3 companies think that a response can be postponed pending the progress of Multi-SIM WI.

Regardless of whether companies answered the questions with Yes or No, they seem to be broad agreement that the UE can only be in RRC_CONNECTED with one network. Hence, the majority believe that AS feedback can not be sent to the other network in this scenario.

The moderator believes that Q2 raised by SA2 is a bit ambiguous, and therefore the answer really depends on their exact intent. On one hand tunnelling of UP and NAS traffic for one network (e.g. SNPN) via another network (e.g. PLMN) is technically feasible. However, the UE should be in RRC_IDLE or RRC_INACTIVE in the first network if it is in RRC_CONNECTED in the second network. Therefore, the UE will not provide any L1/L2 feedback to the first network.

Question 4.2-3: Two companies (Lenovo and Apple) think that there may also be RF aspectss that should be addressed by RAN4.

Based on these responses the moderator proposes the following response to SA2 on Q2:

Q2: is b) technically feasible taking into account the uplink Access Stratum activity in each network?

A2: For scenario b) dual radio UE using independent Rx per network (SNPN and PLMN) and a single Tx for one of the two networks only, RAN2 reiterated that if the UE’s RRC state is RRC_CONNECTED in one network (e.g. PLMN) then it’s RRC state should be RRC_IDLE or RRC_INACTIVE in the other network (e.g. SNPN). Therefore, RAN2 concluded that a Rel. 16 UE that is in RRC_CONNECTED in one network will not send any L1/L2 feedback to the other network. It is possible for the UE to tunnel UP and NAS traffic for one network (e.g. SNPN) via the other network (e.g. the PLMN) as long as the RRC state of the UE for the first network is either RRC_IDLE or RRC_INACTIVE.

## Technical Feasibility of multicast traffic without AS feedback with single Tx dual RX radio

TS 22.263 [3] defines performance requirements for video, imaging and audio for professional applications (VIAPA) (KI#2). In particular, table 6.3.1-1 (reproduced below) provides performance requirements for low latency multicast service for audio and video production applications:

Table 6.3.1-1: Performance requirements for low latency deterministic periodic traffic with multicast service [3].

| Profile | # of active UEs | # of UL streams | # of DL streams | UE Speed | Service Area | E2E latency (Note 1) | Transfer interval (Note 1) | Packet error rate (Note 2, Note 3) | Data rate UL | Data rate DL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Integrated audience services | 50000 | - | 30 multicast streams | 5 km/h | 1.5 km x 1.5 km | 7 ms DL | 3 ms | 10-4 | - | 200 kbit/s |
| Intercom system | 1000 | 240 (Note 4) | 30 multicast streams | 5 km/h | 1.5 km x 1.5 km | 7 ms DL 7 ms UL | 3 ms | 10-4 | 100 kbit/s | 100 kbit/s |
| NOTE 1: Transfer interval refers to periodicity of the packet transfers. It has to be constant during the whole operation. The value given in the table is a typical one, however other transfer intervals are possible as long as the end-to-end latency is ≤ (10 ms – Transfer interval). NOTE 2: Packet error rate is related to a packet size of (Transfer interval × data rate). Packets that do not conform with the end-to-end latency are also accounted as error. NOTE 3: The given requirement for a packet error rate assumes a uniform error distribution. The requirement for packet error rate is stricter if packet errors occur in bursts. NOTE 4: The UL stream originating from a UE may be the source of a DL multicast stream. |  |  |  |  |  |  |  |  |  |  |

Per the NR MBS WID [4] RAN WGs are defining enhancements to support broadcast and multicast service delivery via both point-to-multipoint (PTM) and point-to-point (PTP) transmissions. Therefore, RAN2 should consider both the PTP and PTM cases when addressing Q3 from SA2.

Regarding SA2’s Q3, companies are kindly requested to provide their views on the following questions:

Question 4.3-1: For multicast services delivered via PTP, in the case of a single TX dual radio UE (case b), is it feasible to achieve the performance requirements of table 6.3.1-1 [3] without sending access stratum feedback to the network

| Company | Answer (Yes/No) | Comments |
| --- | --- | --- |
| OPPO | No | Our understanding is PTP cannot be supported for RRC_IDLE/INACTIVE UEs, so that it cannot achieve “without sending access stratum feedback to the network”, as replied to 4.2-2. |
| Huawei | - | Same comment as Q4.2-1. |
| CATT | No | without AS feedback,it can not meet the reliability requirement for NR MBS |
| Lenovo | Very likely not | The support of multicast reception in the different RRC states is under discussion in RAN2. But we can assume that the UE may need to be in RRC_CONNECTED state for multicast reception of services requiring high reliability. In this case a reliable reception w/o AS feedback on the SNPN link cannot be achieved. The same applies for the PLMN link. Furthermore, it should be noted that SA2 has not specified MBS QoS requirements yet. Therefore, RAN2 cannot definitely say whether the performance requirements for VIAPA can/need to be supported in the framework of R17 MBS. |
| ZTE | Possibly not | From ran2 side, there would be no RLC ACK, at least the reliability would be affected. We think the evaluation from RAN1 is also needed |
| Sony | Very likely not | Some form of AS feedback (HARQ, ARQ) is required for PTP operation. |
| Nokia | No |  |
| Ericsson | No | PTP requires the UE to be in RRC_CONNECTED, which is not possible without sending access stratum feedback. |
| LGE | No | RAN2 is considering only UE feedback based re-transmission for reailiable multicast reception. |
| Apple | No | For PTP will not be possible without sending access stratum feedback to network. |
| Futurewei | Very likely not | Reception of multicast services delivered using PTP from the SNPN RAN requires the UE to be in RRC_CONNECTED with the SNPN RAN. We assume that for reliable operation in connected mode, the UE will send AS to the RAN. |
| Qualcomm | No | Agree with Ericsson that the UE needs to be in RRC Connected mode |

Question 4.3-2: For multicast services delivered via PTM, in the case of a single TX dual radio UE (case b), is it feasible to achieve the performance requirements of table 6.3.1-1 [3] without sending access stratum feedback to the network

| Company | Answer (Yes/No) | Comments |
| --- | --- | --- |
| OPPO | See comment | This requires further evaluation by RAN1, i.e., when UE works in RRC_IDLE/INACTIVE state, and thus requires Rx only, can achieve the KPI as above. In MBS session, there are tools being considered to secure the KPIs, e.g., HARQ A/N feedback, which also requires the Tx, so colliding with the requirement of “without sending access stratum feedback to the network”. |
| Huawei | - | Same comment as Q4.2-1. |
| CATT | No | without AS feedback,it can not meet the reliability requirement for NR MBS |
| Lenovo | Very likely not | See comments to Q4.3-1 above. |
| ZTE | Possibly not | We think the evaluation from RAN1 is also needed |
| Sony | - | RAN2 is still discussing MBS and whether broadcast solution without feedback for a multicast service can be used or not is not entirely clear. |
| Nokia | No |  |
| Ericsson | Possibly not | First, it depends on whether RAN1/RAN2 decide to support the DL-only mode (PTM without feedback), in which the UE does not need to be in RRC_CONNECTED. Second, details remain to be discussed whether the above reliability requirements can be met without the possibility for sending AS feedback. |
| LGE | No | The blind re-transmission can be done by NW implementation but it cannot guarantee the performance requirement. To achieve the performance requirements, the UE feedback based re-transmission via PTP is essential. |
| Apple | No | Same as comments in Question 4.3-1 |
| Futurewei | Possibly not | RAN WGs should define a PTM solution without feedback for IDLE and INACTIVE UEs. However, it is up to the operator to determine which services to deploy using this solution, and whether the resulting reliability is sufficient to address the requirements of the service. If the operator determines that reliability is not sufficient for a particular multicast service, then it is up to the operator to configure UEs differently (e.g. use PTP or PTM with AS feedback if supported). |
| Qualcomm | No | For PTM, RAN1 has agreed that there will be HARQ feedback and thus DL only transmission is not possible. |

Question 4.3-3: Are there any other issues that RAN2 should consider in order to formulate a response to SA2 for Q3?

| Company | Item | Comments |
| --- | --- | --- |

Summary of section 4.3:

Question 4.3-1: 7 companies answered No, 3 companies answered Very likely not, 1 company answered Possibly not. 1 company thinks that a response can be postponed pending the progress of Multi-SIM WI. No company thinks that for PTP and a single TX dual RX UE (case b), that the performance requirements of table 6.3.1-1 [3] can be met without AS feedback.

1 company (Lenovo) pointed out that although SA1 has agreed the performance requirements of table 6.3.1-1 [3] for VIAPA, SA2 has not yet specified MBS QoS requirements.

Question 4.3-2: 5 companies answered No, 1 company answered Very likely not, 3 companies answered Possibly not. 1 company believes that this requires further evaluation by RAN1 (to determine whether the required QoS can be achieved with RX only). 1 company questions whether or not a broadcast solution without feedback can be used for a multicast service and thinks that this is not entirely clear. 1 company thinks that a response can be postponed pending the progress of Multi-SIM WI.

The moderator believes that RAN1/RAN2 may specify basic functions for NR broadcast/multicast for UEs in IDLE and INACTIVE state in Rel. 17. However, this is only one possible solution for the delivery of multicast traffic. PTP and PTM will both be supported for CONNECTED UEs. Furthermore, AS will be supported for PTP, and very likely for PTM in connected mode (based on feedback from various companies). Hence, there are many options for an operator to select among for the delivery of a multicast service. It is up to the operator to determine which solution is best suited for the delivery of a particular multicast service, and whether the resulting reliability is sufficient to address the requirements of that service.

Based on these responses the moderator proposes the following response to SA2 on Q3:

Q3: whether in case of b) is it feasible to achieve a very low PER for low latency multicast traffic without sending access stratum feedback to the network (e.g. the SNPN RAN)?

A3: RAN working groups are currently considering two options for multi-cast traffic: point-to-point (PTP) and point-to-multipoint (PTM) transmission.

To receive PTP transmissions the UE must be in RRC_CONNECTED. The majority view in RAN2 is that AS feedback is required to support reliable reception of PTP multicast traffic.

The UE can receive PTM multicast transmissions in RRC_IDLE and RRC_INACTIVE, as well as in RRC_CONNECTED state. However, it is not clear whether the reliability requirements targeted by SA2 for low latency multicast traffic could be met without AS feedback. RAN1 and RAN2 are currently discussing if and how AS feedback can be supported for PTM multicast transmissions by a UE in RRC_CONNECTED. It will be up to the operator deploying a multicast service to determine which solution is best suited for the delivery of their particular service, and whether the resulting reliability is sufficient to address the requirements of that service.

# Summary

The moderator would like to thank all companies that participated in this e-mail discussion. Based on the feedback receive so far, the moderator proposes to respond to the 3 questions raised by SA2 in LS S2-2007827 (R2-2010691) as follows:

Q1: is a) technically feasible without any new Access-Stratum mechanism and standardization?

A1: For scenario a) dual radio UE using independent Rx/Tx per network, RAN2 concluded that it is technically feasible for the UE to simultaneous communicate with both SNPN and PLMN without new AS mechanisms and standardization.

This assumes that the UE’s RF frontend is able to operate independently on the carrier frequencies/bands in use in each network, without this operation resulting in significant interference between the two radios. RAN4 may need to consider whether such independent operation can be supported by a dual radio UE for specific combinations of carrier frequencies/bands.

Q2: is b) technically feasible taking into account the uplink Access Stratum activity in each network?

A2: For scenario b) dual radio UE using independent Rx per network (SNPN and PLMN) and a single Tx for one of the two networks only, RAN2 reiterated that if the UE’s RRC state is RRC_CONNECTED in one network (e.g. PLMN) then it’s RRC state should be RRC_IDLE or RRC_INACTIVE in the other network (e.g. SNPN). Therefore, RAN2 concluded that a Rel. 16 UE that is in RRC_CONNECTED in one network will not send any L1/L2 feedback to the other network. It is possible for the UE to tunnel UP and NAS traffic for one network (e.g. SNPN) via the other network (e.g. the PLMN) as long as the RRC state of the UE for the first network is either RRC_IDLE or RRC_INACTIVE.

Q3: whether in case of b) is it feasible to achieve a very low PER for low latency multicast traffic without sending access stratum feedback to the network (e.g. the SNPN RAN)?

A3: RAN working groups are currently considering two options for multi-cast traffic: point-to-point (PTP) and point-to-multipoint (PTM) transmission.

To receive PTP transmissions the UE must be in RRC_CONNECTED. The majority view in RAN2 is that AS feedback is required to support reliable reception of PTP multicast traffic.

The UE can receive PTM multicast transmissions in RRC_IDLE and RRC_INACTIVE, as well as in RRC_CONNECTED state. However, it is not clear whether the reliability requirements targeted by SA2 for low latency multicast traffic could be met without AS feedback. RAN1 and RAN2 are currently discussing if and how AS feedback can be supported for PTM multicast transmissions by a UE in RRC_CONNECTED. It will be up to the operator deploying a multicast service to determine which solution is best suited for the delivery of their particular service, and whether the resulting reliability is sufficient to address the requirements of that service.

- Reference

[1]	R2-2010691, LS on questions to RAN WGs on dual Radio UE (2Rx/2Tx or 2Rx/1Tx) support for simultaneous communication with both SNPN and PLMN (S2-2007827; contact: Futurewei) SA2

[2]	TR 23.700-07, Study on enhanced support of non-public networks (Release 17), V1.1.0

[3]	TS 22.263, Service requirements for video, imaging and audio for professional applications (VIAPA); Stage 1 (Release 17), V17.0.0

[4]	RP-201038, WID revision: NR Multicast and Broadcast Services, Huawei, HiSilicon