3GPP TSG-RAN WG2 Meeting #112-e	R2-200xxxx

Electronic, 02nd – 13th November 2020

Agenda item:	x.x.x

Source: 	Huawei

Title: 	Summary of [AT112-e][036][MBS] SA2 LS on MBS

Document for:	Discussion and Decision

# Introduction

This document is for the following offline discussion.

- [AT112-e][036][MBS] SA2 LS on MBS (Huawei)

Scope: Treat R2-2008755, and related contributions. While not overlapping with already done email discussions, collect comments and reply proposals for the questions asked by SA2 and identify easy agreements / options with some support.

Intended outcome: Report, to be treated on-line Friday Nov 6

Deadline: Nov 6

The relevant contributions submitted to RAN2#112-e meeting on the SA2 reply LS include:

R2-2008755	LS on RAN impact of FS_5MBS Study (S2-2006044; contact: Huawei)	SA2	LS in	Rel-17	FS_5MBS, NR_MBS-Core	To:SA, RAN, RAN2, RAN3

R2-2008751	Reply LS on RAN impact of FS_5MBS Study (RP-202086; contact: Huawei)	RAN	LS in	Rel-17	FS_5MBS, NR_MBS-Core	To:SA, SA2	Cc:RAN2, RAN3

R2-2008768	Reply LS on RAN impact of FS_5MBS Study (SP-200884; contact: Huawei)	SA	LS in	Rel-17	FS_5MBS, NR_MBS-Core	To:RAN, SA2	Cc:RAN2, RAN3

R2-2009335	Discussion on SA2 LS on RAN impact of FS_5MBS Study	Huawei, HiSilicon	discussion	Rel-17	NR_MBS-Core

R2-2009336	Draft reply LS to SA2 on RAN impact of FS_5MBS Study	Huawei, HiSilicon	LS out	Rel-17	NR_MBS-Core	To:SA, SA2, RAN3	Cc:RAN

R2-2009822	draft_Reply LS on RAN impact of FS_5MBS Study	ZTE, Sanechips	LS out	Rel-17	To:SA2, RAN3

R2-2009954	SA2 questions about RRC state transitions for multicast	Ericsson	discussion	Rel-17	NR_MBS-Core

R2-2009196	MBS L2 Architecture, user plane and control plane	Intel Corporation	discussion	Rel-17	NR_MBS-Core

R2-2008791	Discussion on Requirement and Architecture of MBS	CATT	discussion	Rel-17	NR_MBS-Core

# Discussion

- 2.1 Issues on RRC states for Multicast reception

The related description and questions in the SA2 LS are:

- There are different proposals how to handle the CM-IDLE/CM-CONNECTED state transitions: - UE within a multicast MBS session shall stay in CM-CONNECTED state, - UE can receive data of a multicast MBS session also while in CM-IDLE state. …

First, it should be noted that the proposals are about the multicast solution (i.e. not for the broadcast solution) defined in SA2.

The multicast solution is characterized by:

- a multicast MBS session; and

- the UE joining procedure in 3GPP networks.

So the 3GPP network is aware of which UEs are receiving the MBS service and will establish the multicast MBS sessions towards only the serving gNBs of these UEs.

The broadcast solution is characterized by:

- a broadcast MBS session; and

- No UE joining procedure defined in 3GPP networks (there can be UE joining procedures in the application layer).

So it is the application server to decide the broadcast area and indicate to the 3GPP network to broadcast the content in these areas, i.e. indicate the relevant gNBs/cells. In this sense, the LTE solutions including MBSFN and SC-PTM can be all seen as broadcast solutions from 3GPP perspective.

The email discussion in Post111-e906 MBS Idle mode support has discussed the RRC states for “multicast services”, but companies may have different understandings on the terminology of “multicast services”, e.g. whether it is from application layer point of view or from 3GPP point of view, and whether the broadcast solution defined by 3GPP can be used for the “multicast service”, e.g. services with low reliability requirement.

Therefore, in this offline email, we would like to further collect views from companies to clarify this point.

Question 0: Do you agree with the understanding that the terminologies of multicast MBS session and broadcast MBS session are defined from 3GPP network point of view, and it is NOT directly mapped to “multicast services” or “broadcast services” in application layer, e.g. a groupcast service in application layer can be mapped to a broadcast session in 3GPP networks?

| Company | RRC state(s) | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes | An example is that in LTE, the group call for public safety, which is a multicast service in application layer, can be delivered via LTE SC-PTM which is a broadcast solution defined in 3GPP. |
| Xiaomi | Yes | Agree with Huawei that the LTE SC-PTM solution which is broadcast can also support the group communication. |
| MediaTek | Yes |  |
| QC | Yes | Broadcast vs Multicast is 3GPP transport method. Based on service requirements, QoS support and target UEs, transport mechanism can be choosen. Same view as Huawei. |
| Futurewei | Yes | Terminologies should help guide clear and distinct 3GPP specifications. |
| Ericsson | Yes | We agree with the description of the multicast/broadcast solution above. What applications/services run on top of that will be a decision of the operators eventually. |
| CATT | Yes but the mapping between multicast session and multicast services need to confirm with SA2 | We agree multicast MBS session and broadcast MBS session are defined from 3GPP network point of view. But we are not sure if there is direct mapping between multicast session and multicast service, it seems there is a direct mapping between them,according to 23.757, “Multicast session: A session to deliver the multicast communication service.” So maybe we need further clarification on this from SA2. |
| ZTE | Yes | This is a good chance to clarify things. We basically agree with moderator’s summary and have two more concerns: - not a huge difference, but let us be aligned with SA2 (TR 23.757/3.1 Terms) once and for all. They are “Multicast session” and “Broadcast session”, rather than “multicast MBS session” and “broadcast MBS session”. - suggestion of WA: the term of “Multicast session” and “Broadcast session” apply in RAN (as well). In last meeting is was questioned that a “Multicast session” from core network perspective might be of no difference from “Broadcast session” from access network perspective. This might be true depending on SA2 design, however there will be difference from RAN perspective anyway, e.g., “Multicast session” will be associated with a list of UE in RAN. Therefore, a clear definition is also needed in RAN as well. |

Question 1: In the multicast solution (in contrast to the broadcast solution), in which RRC state(s) should the UE be able to receive multicast MBS session data?

| Company | RRC state(s) | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | RRC_CONNECTED | The concept of the “multicast solution” is very similar to the Unicast solution, in the sense that the 3GPP network is fully aware of which UEs are receiving the services, and establishes the unicast PDU session or multicast MBS session only towards these UEs. Different from the broadcast solution which is cell-oriented, the multicast/unicast solutions are all UE-oriented. Therefore, we would like to reuse as much as possible the unicast concept for the multicast solution, i.e. the reception of the service is only in RRC_CONNECTED state. |
| Xiaomi | RRC_CONNECTED | If the UE is released to CM_IDLE, it seems that the RAN1 is not be able to keep the UE context, and the UE would have to be pushed to RRC_IDLE. Then when the UE transits from RRC_IDLE to RRC_CONNECTED, the gNB would need to coordinate with the CN to get the UE context for multicast service. For the CM_CONNECTED, the UE can be in RRC_CONNECTED or RRC_INACTIVE. However supporting RRC_INACTIVE MBS reception for multicast service seems not very essential and brings more specification efforts. |
| MediaTek | RRC_CONNECTED | Agree with Huawei. We can use broadcast to reach Inactive/Idle mode UE when needed. |
| QC | RRC_CONNECTED | Agree with Huawei comments. In addition, Multicast is mainly intented to deliver same data to a group of UEs with high reliability (i.e Multicast reliability should be same as Unicast reliability) and deliver in radio efficient manner. High reliability can only be provided for UEs in RRC_CONNECTED state and low reliability services can be delivered by using Broadcast mode in all RRC states. |
| Futurewei | RRC_CONNECTED | Similar to the support of unicast data. |
| Ericsson | RRC_CONNECTED RRC_INACTIVE RRC_IDLE | We agree with the comments provided above that connected is the preferred state to receive multicast, because connected mode can provide good reliability, QoS, service continuity and NW efficiency. But there can be cases where the required number of multicast users cannot be supported in connected mode (e.g. in exceptional cases with public safety), i.e. when there is congestion. In such cases we may need to contiue the multicast reception for some UEs also in inactive (or idle) reusing the connected PTM configuration. The UEs that are stationary and in good coverage conditions can be released to idle/inactive to relieve the congestion. Those UEs will still receive a good multicast reception in idle/inactive. In case the UE roams into bad coverage the UE should return to connected mode, i.e. the congestion is locally handled in the cell, i.e. no CN impact. |
| CATT | RRC_CONNECTED or RRC_INACTIVE/RRC_IDLE | we consider this issue from service requirement perspective and design complexity perspective respectively, 1. From SA2 spec,the only difference between multicast and broadcast is that UE need to join the multicast session for multicast service. We can not make assumption in RAN2 that multicast is only for high reliability services and then conclude that multicast session is only suitable to be transmitted in connected mode. 2.if the concern is the design complexity to support multicast reception in idle/inactive mode, we can put a FFS on support low reliability multicast in idle if there is really complexity issues identified in R17. Moreover,whether the network has the capacity to accomdate a large number of multicast UEs in connected mode is also to be considered. |
| ZTE | RRC_CONNECTED, RRC_INACTIVE, maybe RRC_IDLE | No doubt RRC_CONNECTED shall be supported. For UE in RRC_INACTIVE, it is in CM-CONNECTED state and addressable by core network, since the UE context is maintained (in RAN and between RAN node/CN). - it was agreed as majority consensus in “Email Discussion Post111-e906 MBS Idle mode support” that “Reception of some multicast services (e.g.,multicast services with low reliability requirement) is supported in idle/ inactive mode.” - RAN node decides if UE is released to RRC_INACTIVE for a variety of reasons: network load/resource availability, UE activity (especially the uplink), QoS requirement. For Multicast session cases, it is beneficial to enable the network to release UE who is associated with Multicast session with lower reliability requirement., to reduce the network load, to support large number of receiving UE, and to resume UE when uplink is needed with low control plane latency (as required in TR 23.774, Study on mission critical services over 5G multicast-broadcast system). Therefore, it is beneficial to support Multicast session data reception for UE in RRC_INACTIVE, to provide a truly scalable solution from 3GPP. For UE in RRC_IDLE, it was supported in UMTS by broadcasting the service data in the region characterized by the list of per UE RA (routing area of UE). However, it might not be resource efficient if the RA is large and UE number is low. This is the legacy solution and it might be worth having a look by WGs (e.g., RAN2/3 and SA2) |

- 2.2 Issues on RRC states when no data ongoing for multicast MBS session

The related description and questions in the SA2 LS are:

- There are different proposals how to handle the CM-IDLE/CM-CONNECTED state transitions: … - UEs can transition into CM-IDLE while no multicast MBS data are transmitted. …

Even if the UE is only allowed to receive multicast data in RRC_CONNECTED state, there can be the case that there is no data ongoing for the multicast MBS session. The question is whether the UE is allowed to be released to RRC_IDLE or RRC_INACTIVE states in case of no traffic.

Q2: When there is no data ongoing for a multicast MBS session, in which RRC state(s) do you think the UE is allowed to be?

| Company | RRC state(s) | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | RRC_CONNECTED/RRC_INACTIVE (preferred), or RRC_IDLE | By reusing the concept of the unicast solution, when there is no traffic a multicast MBS session, the network is allowed to release the UE to RRC_INACTIVE or RRC_IDLE based on implementation, or even keep the UE in RRC_CONNECTED. The desired RRC state mostly depends on the service requirement, e.g. how long is the latency required for the UE to receive the service when the data arrives again. |
| Xiaomi | RRC_CONNECTED or RRC_INACTIVE | If a MBS session is only temporarily having no data for transmission but still configured, the UE context should be kept to avoid unnccessary signaling procedures for establishing the UE context of the MBS session for the RRC_IDLE UE. Then the UE should be in RRC_CONNECTED or RRC_INACTIVE. |
| MediaTek | RRC_CONNECTED | We did not see the need to allow the UE to keep the session but goes to RRC_IDLE or RRC_INACTIVE. When there is no data ongoing for multicast MBS session, the UE can still stay at DRX state of RRC_connected to maintain the session . Otherwise, UE can go to idle/Inactive mode, which means the session is released. |
| QC | RRC_CONNECTED or RRC_INACTIVE/RRC_IDLE | Agree with Huawei. If there is need to keep UE context in RAN, it is preferable to keep UE in RRC_INACTIVE state. Otherwise UE can go to RRC_IDLE state and some group paging mechanism can be used to trigger UEs to get into RRC_CONNECTED state when Multicast data begins. |
| Futurewei | RRC_CONNECTED, RRC_INACTIVE/RRC_IDLE | Similar to the support of unicast data. |
| Ericsson | RRC_CONNECTED | Our working assumption is that when the multicast session is active, the UE is actually receiving multicast data. When there is temporarily no data, the cDRX would kickin for power saving. The cDRX is a good power saving state for the UE when the multicast session is still active, but there is temporarily no data. We are not sure about the comparison with unicast, i.e. when the unicast data session is over, then the UE is released to idle/inactive, but when the data session is not over, the UE remains in connected mode, in our view. In case there are frequent RRC state changes during a multicast cast session, then there will be an impact on the signalling load in both RAN and CN, which is a concern. One problem with going to idle is that it depends on the data inactivity timer used in the RAN, i.e. RAN and CN may have different views when it would be efficient to transit to idle. One problem in the discussion about RRC state transitions is that we do not have a clear view on what time periods of data inactivity we are talking about. This information is needed to make a good decision. |
| CATT | RRC_CONNECTED RRC_INACTIVE Or RRC_IDLE | Agree with Huawei,it is up to network implementation. |
| ZTE | It depends. | The question goes to who to decide “no multicast MBS data are transmitted”. - If it is 5GC (e.g., UPF has got an inactivity timer), there might be explicit signaling, e.g., an indication of session “suspension”. This is an SA2 issue, and UE CM state is determined by core network. RRC_IDLE is possible if UE is released to CM-IDLE. - if it is triggered in gNB (e.g., CU-UP has got an inactivity timer), UE might be released to RRC_INACTIVE but still addressable from core network, and the reception can be resumed as the MBS session resumes. So RRC_INACTIVE is possible. For either cases, RRC_CONNECTED is possible. |

- 2.3 Issues on trigger indication to establish resources

The related description and questions in the SA2 LS are:

- There are different proposals how to handle the CM-IDLE/CM-CONNECTED state transitions: … - Some solutions propose that 5G CN may trigger notification to CM-IDLE and/or CM-CONNECTED mode UEs (e.g. paging CM-IDLE mode UEs) for establishing transmission resources for an multicast MBS session when data of an multicast MBS session are ready to be delivered. …

There can be different issues in the description from different perspectives. RAN2 should discuss how to trigger a RRC_IDLE or RRC_INACTIVE UE to establish RRC connection for multicast reception, in case the UE is allowed to be released to RRC_IDLE/INACTIVE when no data ongoing.

It is proposed to reuse the legacy CN paging or RAN paging to trigger the RRC_IDLE or INACTIVE UE respectively to establish RRC connection for multicast reception.

Q3: Do you agree that as the baseline the legacy CN paging or RAN paging can be reused to trigger the RRC_IDLE or INACTIVE UE respectively to establish RRC connection for multicast reception?

| Company | Answer | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes | By reusing the concept of the unicast solution, when there is no traffic a multicast MBS session, the network is allowed to release the UE to RRC_INACTIVE or RRC_IDLE. If the UE was released to RRC_INACTIVE, similar to unicast PDU sessions, the multicast MBS session should be kept at RAN, and when there is data coming for this multicast MBS session, RAN can trigger RAN paging to trigger the UE to establish the RRC connection for multicast reception. If the UE was released to RRC_IDLE, similar to unicast PDU sessions, the multicast MBS session should be released at RAN, and when there is data coming for this MBS service, CN can trigger CN paging to trigger the UE to establish the RRC connection for multicast reception. |
| Xiaomi | Yes except for RRC_IDLE | If the UE is in CM_IDLE/RRC_IDLE, it is not clear to us how the RAN sends the paging indication to recover the MBS service. When the UE is in CM_IDLE/RRC_IDLE, there is no UE context for a MBS session stored in the gNB. Maybe we should firstly have a better understanding on the CM_IDLE/RRC_IDLE paging solution from the SA2. Givne that a ue-Identity in the paging message is mandatory for the RAN/CN paging. If we want to reuse the legacy paging for the RRC_IDLE UE, the group paging message for a MBS session from the CN would need to include the UE ID. |
| MediaTek | No | We assume we can still use the legacy MBMS notification mechanism to notify the UE. If the UE went to RRC_IDLE or RRC_INACTIVE, then when the MBS session is activated, the UE can connect to the network following the updated MBS configuration. |
| QC | Yes | Same comment as Huawei |
| Futurewei | Yes | Similar to the support of unicast data. |
| Ericsson | Yes, but | We agree to use Paging to indicate multicast session start. Whether Paging is used to get the UE back to connected when the UE is released to idle/inactive due to inactivity, depends on the outcome of Q2. |
| CATT | Yes, but | We think the legacy CN paging or RAN paging as the baseline is workable.but in principle we should try to use a unified solution for multicast and broadcast as much as possible. If the MCCH and MCCH change notification mechanism is used for broadcast reception in idle/inactive mode,then to simplify the design,we think MCCH change notification mechanism could also be used to indicate multicast session start. |
| ZTE | Not a RAN issue. | Please note the LS says explicitly it is a 5GC initiated paging. This is related to the traffic or service model: whether the time interval between UE joins the Multicast session (NAS signaling) and the session starts is long enough; or CN has an explicit state for MBS (e.g., active, inactive), no data ongoing if the Multicast session is inactive, therefore UE is released to RRC_IDLE - if yes for any of the cases above, it might be beneficial to release UE to CM-IDLE, and paging UE when the session really starts or data arrives after an inactive period. - if no, it is better not to release UE to RRC_IDLE, either by RAN or 5GC. However, this shall be an SA2 issue rather than RAN’s. |

- 2.4 Issues on activation/deactivation of multicast MBS session

The related description and questions in the SA2 LS are:

- There are different proposals how to handle the CM-IDLE/CM-CONNECTED state transitions: … - Some solutions propose that the multicast MBS session can be deactivated by the network while no multicast MBS data are transmitted to save power. - Some solutions propose that the network can activate the multicast MBS session and trigger notification to UEs when multicast MBS data are transmitted again. …

It is rapporteur’s understanding that the activation/deactivation of a multicast MBS session is only a solution from core network’s perspective, but from RAN perspectice RAN may still see the multicast MBS session as established or released. In this sense, it would be completely up to SA2 to decide on this feature. Companies are invited to comment on the following question:

Q4: Do you agree that activation/deactivation of a multicast MBS session is not within the RAN2 scope?

| Company | Answer | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes | RAN may decide about the UE’s RRC state based on MBS session data activity as for unicast session as we clarified in our answer to Q2. |
| Xiaomi | Yes | The gNB by implementation can decide to keep the UE in RRC_CONNECTED or send the UE to RRC_INACTIVE when the CN activates/deactivates the MBS session. |
| MediaTek | Yes | We assume the “activation/deactivation of a multicast MBS session” is identical to “multicast MBS session establishment and release” from NAS perspective. This means it will lead to the MBS radio bearer establishment and release. |
| QC | Yes |  |
| Futurewei | Yes |  |
| Ericsson | Yes, but the RAN impact is within scope | The CN activation/deactivation is perhaps outside RAN2 scope, but RAN2 should evaluate the possible impact on RAN, i.e. this i also the reason why SA2 mentions this in the LS to RAN2. SA2 wrote in the LS: SA2 would like to inform that no solution or combination of solutions has been adopted yet, and would like to receive feedback from RAN2 and RAN3 on the RAN impacts, produced by some of the company proposed solutions in SA2, and for some of the solutions documented in current TR, SA2 would like to kindly ask RAN2 and RAN3 the following questions In our understanding the activation/deactivation is an explicit indication to RAN that there is multicast data inactivity. But for RAN to make a proper decision about what to do, it needs to be clarified for how long data inactivity is expected, and how quickly the multicast reception should be restored when an activation request is received from CN. When the data inactivity period is very short, or the latency requirement is very strong, then perhaps RAN should do nothing. |
| CATT | Yes | It is not clear what the deactivation really means,does it mean there will be no data for a long time?if it is,why not release the multicast session directly?therefore we can wait RAN3 and SA2 to figure it out. |
| ZTE | Might be within RAN2 scope. (just answering yes or no might be a little tricky to understand) | This could be an unilateral enhancement of RAN: - if it is triggered in gNB (e.g., CU-UP has got an inactivity timer), UE might be released to RRC_INACTIVE but still addressable from core network, and the reception can be resumed as the MBS session resumes. Might be beneficial to radio resource efficiency and UE power saving. But as described in our answer to Q2, who to decide “no MBS data”? and it could be an SA2 issue. If so, clarification is needed from SA2 and RAN3 on Session Management might be needed, e.g, additional explicit signaling indicating that session is activated/deactivated, other than existing session start/stop. Then it will be an RAN2 issue later. |

- 2.5 Issues on data forwarding

The related description and questions in the SA2 LS are:

- Some Xn/N2 handover solutions in the SA2 study are documented in the TR. - Some solutions consider to have temporary MBS data forwarding from S-RAN to the T-RAN, to address potential data loss or duplication in case of a UE moving to a T-RAN supporting 5MBS. - Some solutions have left forwarding FFS and would appreciate RAN feedback on possibilities for forwarding at Xn/N2 handovers with considerations of minimization of data loss, data duplication and complexity. …

As we have discussed the issue of data forwarding in the email discussion [Post111-e][905][MBS] Connected Mode Mobility with Service Continuity, rapporteur suggest to just wait for the conclusion of that email discussion summary.

Q5: Are you ok to reply to SA2 on this issue of data forwarding based on the conclusion of email discussion [Post111-e][905][MBS] Connected Mode Mobility with Service Continuity?

| Company | Answer | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes |  |
| Xiaomi | Yes |  |
| MediaTek | Yes |  |
| QC | Yes | We think to meet high reliability requirement, loss-less HO with data forwarding support is needed . |
| Futurewei | Yes |  |
| Ericsson | Yes |  |
| CATT | Yes |  |
| ZTE | Yes | We follow any agreements of the email discussion (after online discussion, if needed). However, maybe no scalable solution can be offered from 3GPP. “high reliability requirement” is indeed there, we have to admit. But why not Unicast from the application layer in the first place, considering the spec impacts being introduced. |

- 2.6 Issues on local MBS service

The related description and questions in the SA2 LS are:

- Some Xn/N2 handover solutions in the SA2 study are documented in the TR. … - Some solutions introduce HO for local MBS service that can only transmit data in a certain area, which has impact on RAN for service area restriction. …

It is rapporteur’s understanding that it is up to SA2 to decide on the support of local MBS service, and RAN2 may discuss based on SA2’s progress.

Q6: Do you agree that it is up to SA2 to decide on the support of local MBS service, and RAN2 may discuss based on SA2’s progress?

| Company | Answer | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes |  |
| Xiaomi | Yes |  |
| MediaTek | Yes |  |
| QC | Yes |  |
| Futurewei | Yes |  |
| Ericsson | Yes, but the RAN impact is within scope | Similar reply as for Q4. For SA2 to make an informed decision, RAN2 needs to inform SA2 about the possible RAN impact of Local MBS service. |
| CATT | Yes | But it seems SA2 is working on solutions for local MBS services, some of them may have impact on RAN2, so maybe we could provide a more clear view to SA2. For example,”the local MBS services should have no impact to RAN2, from RAN point of view we don’t expect any difference between normal MBS services and local MBS services.” |
| ZTE | Yes |  |

- 2.7 Issues on broadcast

The related description and questions in the SA2 LS are:

- SA2 is debating whether broadcast (i.e. without the network’s awareness about UEs receiving broadcast contents and for other use cases than the ones excluded already for Rel-17) should be further down-scoped in Rel-17 for remaining broadcast requirement in the SID. Some companies have provided solutions on broadcast (which are documented in the TR). SA2 would like to ask SA, RAN, RAN2 and RAN3 for feedback on broadcast support in Rel-17.

RANP in RAN#89e has concluded that broadcast is within the scope of NR MBS in Rel-17. Therefore, RAN2 can reply that RAN2 will continue the work of broadcast based on the RAN decision.

Q7: Do you agree to reply to SA2 that based on the RAN decision RAN2 will continue the work on the broadcast support?

| Company | Answer | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes |  |
| Xiaomi | Yes |  |
| MediaTek | Yes |  |
| QC | Yes |  |
| Futurewei | Yes |  |
| Ericsson | Yes |  |
| CATT | Yes |  |
| ZTE | Yes |  |

- 2.8 Issues on assistance information for PTP/PTM switch

The related description and questions in the SA2 LS are:

- Some solution suggests the 5GC sends MBS assistance information to RAN for PTP/PTM delivery method decision and switching. SA2 would appreciate RAN2 and RAN3 feedback on the above and comments, if any.

Some proposed assistance information have been summarized as below:

- Information#1: assistance information from 5GC to help gNB to decide which delivery method for specific MBS or UE [6];

- Information#2: “information of MBS services/groups subscribed by the UE, e.g. TMGI [8];

- Information#3: session start time [8].

- Information#4: QOS requirements from CN [9]

Q8: what assistance information from 5GC to RAN above do you support for PTP/PTM switch?

| Company | Answer | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | #2, #4 | The choice of the delivery method depends mainly on the QoS requirements of the service and the radio condtions of the UE. Hence, only #2 and #4 are needed from CN and the rest of the information is available directly in the gNB (e.g. based on UE measurements or feedback). |
| Xiaomi | #2, #4 | For #1, the final decision of whether to use multicast or unicast would be anyway up to the gNB. The suggested delivery methold from CN is not very useful For #3, we think that for deterministic services, the session start time should be included in the USD file downloaded from the application server. For the dynamic/proactive traffic without certain packet arrival time, the session start time seems not avaible anyway. |
| MediaTek | #2, #4 | #4 can be the legacy way of QoS notification from 5GC to RAN during session establishment |
| QC | #2, #4 |  |
| Futurewei | Not #1, #3 | It is RAN’s determination of PTM/PTP switch, based on both multicast service characterisitcs and RAN conditions. It is not clear if specific assistance information would be needed from 5GC, in addition to what RAN would already receive to establish multicast communication. |
| Ericsson | (#2, #4) | In our understanding “CN assistance info” is optional information that the CN may or may not provide, and information that RAN may or may not use. In our understanding #2 and #4 are not optional to provide or use, and not limited to the use of PTM/PTP switch. Not sure if we should call QoS/group info “CN assistance” info. |
| CATT | #2,#4, and maybe more | Solution #18 in 23.757 has listed some other assistance information that could be provided from 5GC to RAN. As for now we are not sure if any more assistance info is needed,so we prefer a more open response to SA2 such as “At the moment, besides informion #2,#4,there is no conclusion on whether RAN requires any more assistance information from 5GC. RAN2 will inform SA2 in case any more information from 5GC is needed.” |
| ZTE | #1, #2, #4 | It is RAN’s determination of PTM/PTP switch based on the QoS, and assistance information that can not be conveyed on QoS profile, like the subscription or other policy info. It can be further studied in SA2 anyway and RAN2 follows what SA2 offers. |

# Conclusion

# Reference

- R2-2008755	LS on RAN impact of FS_5MBS Study (S2-2006044; contact: Huawei)	SA2	LS in	Rel-17	FS_5MBS, NR_MBS-Core	To:SA, RAN, RAN2, RAN3

- R2-2008751	Reply LS on RAN impact of FS_5MBS Study (RP-202086; contact: Huawei)	RAN	LS in	Rel-17	FS_5MBS, NR_MBS-Core	To:SA, SA2	Cc:RAN2, RAN3

- R2-2008768	Reply LS on RAN impact of FS_5MBS Study (SP-200884; contact: Huawei)	SA	LS in	Rel-17	FS_5MBS, NR_MBS-Core	To:RAN, SA2	Cc:RAN2, RAN3

- R2-2009335	Discussion on SA2 LS on RAN impact of FS_5MBS Study	Huawei, HiSilicon	discussion	Rel-17	NR_MBS-Core

- R2-2009336	Draft reply LS to SA2 on RAN impact of FS_5MBS Study	Huawei, HiSilicon	LS out	Rel-17	NR_MBS-Core	To:SA, SA2, RAN3	Cc:RAN

- R2-2009822	draft_Reply LS on RAN impact of FS_5MBS Study	ZTE, Sanechips	LS out	Rel-17	To:SA2, RAN3

- R2-2009954	SA2 questions about RRC state transitions for multicast	Ericsson	discussion	Rel-17	NR_MBS-Core

- R2-2009196	MBS L2 Architecture, user plane and control plane	Intel Corporation	discussion	Rel-17	NR_MBS-Core

- R2-2008791	Discussion on Requirement and Architecture of MBS	CATT	discussion	Rel-17	NR_MBS-Core