3GPP TSG-RAN WG2 #112e	Tdoc R2-20xxxx

Electronic meeting, November 2nd – 13th 2020

Agenda Item:	6.2.3

Source:	Ericsson

Title:	Summary of [AT112-e][037][IAB] User Plane

Document for:	Discussion, Decision

# 1	Introduction

This paper addresses the following email discussion:

- [AT112-e][037][IAB] User Plane (Ericsson)

Treat tdocs under 6.2.3

Intended outcome: Intermediate: Determine agreeable parts. Final: For agreeable parts, agreed CRs.

Deadline: Intermediate deadline(s) by Rapporteur, Final: Discussion stop at Wed Nov 11, 1200 UTC

The rapporteur would like to set the following deadlines:

Deadline 1: Friday, Nov. 6 EOB for answers to the questionnaire in the summary

Deadline 2: Wed, Nov 11 1200 UTC for CR wording (if needed).

# 2	Discussion

This email discussion handles the following documents submitted at RAN2#112 regarding IAB user plane:

R2-2009745	Correction on Tdelta	Huawei, HiSilicon	CR	Rel-16	38.321	16.2.1	0938	-	F	NR_IAB-Core

R2-2010152	Correction to tDelta	Ericsson	CR	Rel-16	38.321	16.2.1	0963	-	F	NR_IAB-Core

R2-2010150	Pre-emptive BSR handling at MAC Reset	Ericsson	CR	Rel-16	38.321	16.2.1	0962	-	F	NR_IAB-Core

R2-2010419	Correction on the condition check in Pre-emptive BSR procedure	ASUSTeK	CR	Rel-16	38.321	16.2.1	0984	-	F	NR_IAB-Core

R2-2009324	CR to 38.322 on Backhaul RLC Channel	vivo	CR	Rel-16	38.322	16.1.0	0037	-	F	NR_IAB-Core

Moved from 6.2.6

R2-2010684	Summary of Rel-16 IAB UP issues and corrections	Samsung	discussion	Rel-16	NR_IAB-Core

## 2.1	Correction to tDelta

The following CRs handle this issue:

- R2-2009745	Correction on Tdelta	Huawei, HiSilicon	CR	Rel-16	38.321	16.2.1	0938	-	F	NR_IAB-Core

- R2-2010152	Correction to tDelta	Ericsson	CR	Rel-16	38.321	16.2.1	0963	-	F	NR_IAB-Core

Both CRs intend to fix the mismatch between the RAN1 terminology in TS 38.213 and TS 38.321. From TS 38.213 the   is the index value, while in MAC specification, the terminology Tdelta and T_delta is used to represent the index value.

The two CRs are very similar. The only difference is that R2-2009745 proposes to remove Tdelta, and T_delta and hence also to change the MAC. Instead, R2-2010152 proposes to keep the legacy MAC CE (with T_delta field) and just remove Tdelta.

Q1: Which option do you prefer to correct this issue?

- The solution in R2-2009745

- The solution in R2-2010152

| Company | Preference (1/2) | Detailed Comments |
| --- | --- | --- |
| LG | 1 |  |
| Samsung | 2 | Our preference is 2 over 1, but we think the correction to the MAC CE mentioned in 1 should be agreed. Basically we prefer how 2 handles the issue with symbols and their editorial approach, but we think 1 has a point when suggesting to correct the MAC CE as well. So a combination of two is our preference. These details can be ironed out during the CR drafting stage – in any case a change is needed. |
| Huawei | 1 | Another difference is that we need to remove the “mapped to an index value Tdelta”, since there is no so-called mapping, as proposed in 1. |
| ZTE | 1 |  |
| vivo | 1 | We prefer to decouple the descriptions between RAN1 and RAN2 regarding the RAN1 parameter Tdelta so that we don’t need to modify our spec even some relevant modifications are made in RAN1. |
| CATT | 1 | We prefer 1 than 2. It is more clear. |
| Nokia | 1 |  |
| Ericsson | 1 and 2 | We agree with Samsung way forward. It makes sense to go for option 1 when it comes to correcting the MAC CE. However, when it comes to Section 5.18.18. it is preferred a combination of option 1 and option 2. Option 1 for example left “timing adjustment Tdelta” which is not entirely correct since Tdelta is just an index value, and also remove the Tdelta from the action upon reception. This would be our proposal: |
| Apple | 1 and then 2 | Agree with Samsung’s way forward. |
| Intel | 1 |  |

## 2.2	Correction to Pre-emptive BSR handling at MAC Reset

- R2-2010150	Pre-emptive BSR handling at MAC Reset	Ericsson	CR	Rel-16	38.321	16.2.1	0962	-	F	NR_IAB-Core

The CR is to specify that a triggered pre-emptive BSR shall be cancelled at MAC reset. This is in line with many other triggered procedures, e.g. Buffer Status Reporting procedure, that shall be cancelled at MAC reset.

Q2: Is there any concern with the correction in R2-2010150?

| Company | Preference (Yes/No) | Detailed Comments |
| --- | --- | --- |
| LG | NO | We are fine with this change. |
| Samsung | No |  |
| Huawei | Yes, but | We have agreed before that pre-BSR cancellation can be implementation, “Implementation-specific cancellation conditions for Pre-emptive BSR are not precluded”, when some contributions proposed to add more pre-BSR cancellation procedure. We are wondering if anything is broken as in the current spec to allow IAB implementation to handle this case. BTW, it is strange that we don't specify the exact behaviour to trigger pre-BSR, but cover all cases to cancel pre-BSR. We are afraid that lots of CRs will come next meeting to check if any other pre-BSR cancellation is not captured, as legacy BSR, if the CR is agreed this meeting. Therefore, we think the CR is not essential, but are open to hear majority’s view on the consequence if not agreed. |
| ZTE | No |  |
| vivo | No | OK with the change. |
| CATT | No | We agree with the change. |
| Nokia |  | We think the existing text about cancelling BSR procedure can already cover the pre-emptive BSR case similarly and don’t need a new condition for that. |
| Ericsson | No | The pre-emptive BSR is a separate procedure from the legacy BSR procedure. Adding this cancellation is important otherwise when the MAC is reset the preemptive BSR is not cancelled, which would result in the IAB node transmitting a pre-emptive BSR related to a previous MAC session. That is obviously a wrong behaviour. |
| Apple | No |  |
| Intel | No |  |

## 2.3	Correction on Pre-emptive BSR procedure

- R2-2010419	Correction on the condition check in Pre-emptive BSR procedure	ASUSTeK	CR	Rel-16	38.321	16.2.1	0984	-	F	NR_IAB-Core

The CR is to fix an editorial issue, i.e the word “Pre-emptive” is missing in the Pre-emptive Buffer Status Reporting procedure.

Q3: Is there any concern with the correction in R2-2010419?

| Company | Preference (Yes/No) | Detailed Comments |
| --- | --- | --- |
| LG |  | We think that the proposed clarification may be helpful, but there is no confusion without this change. No strong view. |
| Samsung | No | We support this change, for consistency’s sake. |
| Huawei | No, but | This is kind of editorial change. |
| ZTE | No |  |
| vivo | No | Fine with the clarification. |
| CATT | No | We agree with the change. |
| Nokia |  | We can leave the text as is and we don’t need either the change to the MAC reset part. |
| Ericsson | No strong view | It is an editorial correction. We are ok with the majority view. |
| Apple | No | Agree with this change. |
| Intel | No | We just wonder in 38.321 clause 5.4.7, is it intentionally missing or also need to change similar as CR in R2-2010419? If configured, Pre-emptive BSR may be triggered for the specific case of an IAB-MT if any of the following events occur: - UL grant is provided to child IAB node or UE; - BSR or Pre-emptive BSR is received from child IAB node or UE. |

## 2.4	Correction on Backhaul RLC Channel to TS 38.322

- R2-2009324	CR to 38.322 on Backhaul RLC Channel	vivo	CR	Rel-16	38.322	16.1.0	0037	-	F	NR_IAB-Core

Intention of the CR is to clarify that in case of IAB the data are transferred between RLC and BAP via a single BH RLC channel, rather than via a single RLC channel as it is in legacy non-IAB networks. There is also a second change to clarify that the BAP procedures are specified in TS 38340.

Rapporteur´s view: If RAN2 intends to agree on the intention of this CR, Rapporteur believes that it would be enough to just state that “In case the upper layer is BAP as defined in TS 38.340 [xx], an RLC channel refers to a Backhaul RLC channel”. Otherwise, it might be misleading stating that data are transferred through a single BH RLC channel.

Q4: Do you agree with the intention of the changes in R2-2009324?

| Company | Preference (Yes/No) | Detailed Comments |
| --- | --- | --- |
| LG | Yes | Agree with rapporteur’s view. |
| Samsung | Yes but… | Agree with the intention behind the change but not with the change itself. The current wording ‘the data will be transferred through a single Backhaul RLC channel between RLC and BAP layer’ can be misleading as pointed out by the rapporteur. We are ok with rapporteur’s alternative proposal. |
| Huawei | No | Adding reference is not essential. For the NOTE, it is pretty clear on the protocol architecture from 38.300, e.g. “the BAP PDUs are carried by BH RLC channels”. We don't not need to clarify this in every spec. Fig. 4.7.2-1: Protocol stack for the support of F1-U protocol |
| ZTE | No | We think the current specification is clear enough according to IAB protocol architecture in TS 38.300 and BAP structure in TS 38.340. |
| vivo | Yes | Proponent. We also agree with the refinement from rapporteur. |
| CATT | Yes | We agree with rapporteur’s view. |
| Nokia | Yes (intention) | We can accept rapporteur’s proposal. |
| Ericsson | No strong view | We are ok with the majority view. |
| Apple | Yes | Agree with rapporteur’s view |
| Intel | No strong view | We are ok with the majority view. |

If the answer to Q4 is “Yes” related to the first change, do you agree with Rapporteur´s proposal to replace the NOTE in R2-2009324 with a NOTE stating “In case the upper layer is BAP as defined in TS 38.340 [xx], an RLC channel refers to a Backhaul RLC channel”

Q5: If the answer to Q4 is yes, do you agree with the Rapporteur proposal to add the following NOTE?

NOTE:	In case the upper layer is BAP as defined in TS 38.340 [xx], an RLC channel refers to a Backhaul RLC channel.

| Company | Preference (Yes/No) | Detailed Comments |
| --- | --- | --- |
| LG | Yes |  |
| Samsung | Yes |  |
| vivo | Yes |  |
| CATT | Yes |  |
| Nokia | Yes |  |
| Ericsson | Yes |  |
| Apple | Yes |  |

# 3	Conclusion