3GPP TSG-RAN WG2 #112-e	R2-2010722

Online, 02 – 13 November 2020

Source: 	Huawei, HiSilicon

Title: 	Report of [AT112-e][212][MOB] Mobility UE capabilities for LTE and NR (Huawei)

Agenda Item:	6.7.4

Document for:	Discussion and decision

# Introduction

This is the report of the following offline discussion:

- [AT112-e][212][MOB] Mobility UE capabilities for LTE and NR (Huawei) Scope: - Discuss which UE capability corrections to LTE and NR are seen necessary and provide merged CRs with agreeable corrections (if any) Intended outcome: - Discussion summary in R2-2010722 (by email rapporteur). - Merged CRs to 36.306 (R2-2010723), 36.331 (R2-2010724), 38.306 (R2-2010725) and 38.331 (R2-2010726) (if any) Deadline for providing comments, for rapporteur inputs, conclusions and CR finalization: - Initial deadline (for companies' feedback): 1st week Fri, UTC 0900 - Initial deadline (for rapporteur's summary in R2-2010722): 2nd week Mon, UTC 13:00 - Deadline for CR finalization: 2nd week Thu, UTC 1000

This offline discussion is divided into two phases:

First phase: collecting companies’ feedback, deadline: 1st week Fri, UTC 0900

Second phase: updating summary and CRs (if any), deadline: 2nd week Thu, UTC 1000

# Discussion

## NR UE capabilities

DAPS capabilities:

R2-2009655	Correction on CA-ParametersNR for DAPS handover	NEC	draftCR	Rel-16	38.331	16.2.0	NR_Mob_enh-Core

Reason for change: The general description of CA-ParametersNR IE inidiates that it only contains carrier aggregation related capabilities. However inter-frequency DAPS handover related capabilities are also included in this IE.

- CA-ParametersNR

The IE CA-ParametersNR contains carrier aggregation and inter-frequency DAPS handover related capabilities that are defined per band combination.

Question 1: Do companies agree to the change proposed in R2-2009655?

| Company | Yes/No | Remark |
| --- | --- | --- |

CHP/CPC capabilities:

R2-2009273	The supported combination among FRx/xDD CHO/CPC capabilities	Intel Corporation	discussion	Rel-16	NR_Mob_enh-Core, LTE_feMob-Core

R2-2009281	Clarification on the setting of FRx&xDD CHO&CPC capabilities	Intel Corporation	CR	Rel-16	38.306	16.2.0	0423	-	F	NR_Mob_enh-Core

R2-2010296	Clarification on CHO and CPC capabilities between different modes	Huawei, HiSilicon	CR	Rel-16	38.306	16.2.0	0443	-	F	NR_Mob_enh-Core

Reason for change: In current spec, it is still not clear how to set and interpret CHO and CPC capabilities between different modes. In RAN2#111e, one agreement was made as blow in this respect:

- [206] 1-5: The table in the Annex of R2-2006936 can be applied to both CHO and CPC cases. How to capture this in specifications can be discussed in the next meeting.

And two different ways are proposed to address this issue:

Option 1 proposed by R2-2009273:

Capture in TS38.306 in the field description:

- For condHandoverFR1-FR2-r16 as “condHandoverFR1-FR2-r16 can only be set if condHandover-r16 is set for at least one FR1 band and one FR2 band;”

- For “condHandoverFDD-TDD-r16 can only be set if condHandover-r16 is set for at least one TDD band and one FDD band;”

- For condPSCellChangeFR1-FR2-r16 as “condPSCellChangeFR1-FR2-r16 can only be set if condPSCellChange-r16 is set for at least one FR1 band and one FR2 band;”

- For “condPSCellChangeFDD-TDD-r16 can only be set if condPSCellChange-r16 is set for at least one TDD band and one FDD band;”

Option 2 proposed by R2-2010296:

Capture the corresponding interpretation table for CHO and CPC respectively in Annex of TS 38.306 as below:

- Annex C:
UE capability indication for UE CHO and CPC capabilities between FDD and TDD and between FR1 and FR2

Annex C clarifies the UE CHO and CPC capability indications for the case where the UE is allowed to support different functionality in all FDD-FR1 bands, all TDD-FR1 bands and all TDD-FR2 bands respectively. Table C-1 clarifies the setting of UE CHO capability fields between FDD and TDD and between FR1 and FR2 for cases where the UE supports the corresponding feature in different combinations of duplex mode and frequency range. Table C-2 clarifies the setting of UE CPC capability fields between FDD and TDD and between FR1 and FR2 for cases where the UE supports the corresponding feature in different combinations of duplex mode and frequency range.

Table C-1: UE CHO capability indication between FDD and TDD, and between FR1 and FR2

| Support for the feature |  | Setting of UE capability fields |  |  |
| --- | --- | --- | --- | --- |
|  |  |  | condHandoverFR1-FR2-r16 | condHandoverFDD-TDD-r16 |
| Case 1 | FR1 FDD: 'supported' FR1 TDD: 'supported' FR2 TDD: 'not supported' |  | condHandoverFR1-FR2-r16 should not be set to true since the UE cannot support CHO within FR2. | condHandoverFDD-TDD-r16 could be set since UE support FR1_FDD and FR1_TDD. |
| Case 2 | FR1 FDD: supported' FR1 TDD: 'not supported' FR2 TDD: supported' |  | condHandoverFR1-FR2-r16 could be set, and if set, condHandoverFR1-FR2-r16 and condHandoverFDD-TDD-r16 should be set simultaneously for this case. | condHandoverFDD-TDD-r16 could be set, and if set, condHandoverFR1-FR2-r16 and condHandoverFDD-TDD-r16 should be set simultaneously for this case. |
| Case 3 | FR1 FDD: 'not supported' FR1 TDD: 'supported' FR2 TDD: 'supported' |  | condHandoverFR1-FR2-r16 could be set since UE support FR1_TDD and FR2_TDD. | condHandoverFDD-TDD-r16 should not be set to true since the UE only supports TDD |
| Case 4 | FR1 FDD: 'not supported' FR1 TDD: 'not supported' FR2 TDD: 'supported' |  | condHandoverFR1-FR2-r16 should not be set to true since the UE only supports FR2_TDD | condHandoverFDD-TDD-r16 should not be set to true since the UE only supports FR2_TDD |
| Case 5 | FR1 FDD: 'not supported' FR1 TDD: 'supported' FR2 TDD: 'not supported' |  | condHandoverFR1-FR2-r16 should not be set to true since t the UE only supports FR1_TDD | condHandoverFDD-TDD-r16 should not be set to true since the UE only supports FR1_TDD |
| Case 6 | FR1 FDD: 'supported' FR1 TDD: 'not supported' FR2 TDD: 'not supported' |  | condHandoverFR1-FR2-r16 should not be set to true since t the UE only supports FR1_FDD | condHandoverFDD-TDD-r16 should not be set to true since the UE only supports FR1_FDD |
| Case 7 | FR1 FDD: 'supported' FR1 TDD: supported' FR2 TDD: supported' |  | condHandoverFR1-FR2-r16 could be set. | condHandoverFDD-TDD-r16 could be set. |

Table C-2: UE CPC capability indication between FDD and TDD, and between FR1 and FR2

| Support for the feature |  | Setting of UE capability fields |  |  |
| --- | --- | --- | --- | --- |
|  |  |  | condPSCellChangeFR1-FR2-r16 | condPSCellChangeFDD-TDD-r16 |
| Case 1 | FR1 FDD: 'supported' FR1 TDD: 'supported' FR2 TDD: 'not supported' |  | condPSCellChangeFR1-FR2-r16 should not be set to true since the UE cannot support CHO within FR2. | condPSCellChangeFDD-TDD-r16 could be set since UE support FR1_FDD and FR1_TDD. |
| Case 2 | FR1 FDD: supported' FR1 TDD: 'not supported' FR2 TDD: supported' |  | condPSCellChangeFR1-FR2-r16 could be set, and if set, condPSCellChangeFR1-FR2-r16 and condPSCellChangeFDD-TDD-r16 should be set simultaneously for this case. | condPSCellChangeFDD-TDD-r16 could be set, and if set, condPSCellChangeFR1-FR2-r16 and condPSCellChangeFDD-TDD-r16 should be set simultaneously for this case. |
| Case 3 | FR1 FDD: 'not supported' FR1 TDD: 'supported' FR2 TDD: 'supported' |  | condPSCellChangeFR1-FR2-r16 could be set since UE support FR1_TDD and FR2_TDD. | condPSCellChangeFDD-TDD-r16 should not be set to true since the UE only supports TDD |
| Case 4 | FR1 FDD: 'not supported' FR1 TDD: 'not supported' FR2 TDD: 'supported' |  | condPSCellChangeFR1-FR2-r16 should not be set to true since the UE only supports FR2_TDD | condPSCellChangeFDD-TDD-r16 should not be set to true since the UE only supports FR2_TDD |
| Case 5 | FR1 FDD: 'not supported' FR1 TDD: 'supported' FR2 TDD: 'not supported' |  | condPSCellChangeFR1-FR2-r16 should not be set to true since t the UE only supports FR1_TDD | condPSCellChangeFDD-TDD-r16 should not be set to true since the UE only supports FR1_TDD |
| Case 6 | FR1 FDD: 'supported' FR1 TDD: 'not supported' FR2 TDD: 'not supported' |  | condPSCellChangeFR1-FR2-r16 should not be set to true since t the UE only supports FR1_FDD | condPSCellChangeFDD-TDD-r16 should not be set to true since the UE only supports FR1_FDD |
| Case 7 | FR1 FDD: 'supported' FR1 TDD: supported' FR2 TDD: supported' |  | condPSCellChangeFR1-FR2-r16 could be set. | condPSCellChangeFDD-TDD-r16 could be set. |

Question 2: which option would companies like to choose?

| Company | Option 1 or 2 | Remark |
| --- | --- | --- |

## LTE UE capabilities

UL power sharing for LTE DAPS:

R2-2010681	Introducing power sharing for DAPS handover	Ericsson, Qualcomm, Huawei	CR	Rel-16	36.306	16.2.0	1798	1	F	LTE_feMob-Core

R2-2010682	Introducing power sharing for DAPS handover	Ericsson, Qualcomm, Huawei	CR	Rel-16	36.331	16.2.1	4517	1	F	LTE_feMob-Core

Reason for change: According to reply LS R1-2007420 from RAN1, “An IE with the same fields as PowerCoordinationInfo-r12 (up to RAN2 whether to reuse PowerCoordinationInfo-r12 or define a new IE) is introduced to configure UL power sharing for LTE DAPS.”

In addition, optional UE capabilities for the support of UL power sharing for LTE DAPS are required:

- Support of power sharing for sync DAPS.

- Support of power sharing for async DAPS.

Companies are requested to review the joint CRs proposed by concerned companies.

Question 3: Do companies agree to the changes proposed in R2-2010681 and R2-2010682?

| Company | Yes/No | Remark |
| --- | --- | --- |

LTE DAPS capabilities:

R2-2009188	Clarifications to LTE DAPS capabilities	Nokia, Nokia Shanghai Bell	discussion	Rel-16	LTE_feMob-Core

This paper tries to clarify the understading on PCell bandwidth for DAPS, and the following proposals are made:

Proposal 1: Clarify that the source and target PCell channel bandwidths may be different in intra-frequency DAPS HO.

Proposal 2: UE indicating support for inter-frequency DAPS in a BC shall support for all possible combinations of source and target cell belonging to the bands of the band combination where source carrier center frequency != target carrier center frequency.

Proposal 3: Clarify that the source and target PCell channel bandwidths may be different in inter-frequency DAPS HO.

Question 4: Regarding the clarification that source and target PCell channel bandwidths may be different in intra/inter-frequency DAPS HO, do companies agree to P1 and P3?

| Company | Yes/No | Remark |
| --- | --- | --- |

Question 5: Regarding the clarification on inter-frequency DAPS HO, do companies agree to P2?

| Company | Yes/No | Remark |
| --- | --- | --- |

------------------------------------------------------------------TP Start---------------------------------------------------------------

Annex A: Proposed 36.306 changes

4.3.5.43	interFreqDAPS-r16

This field indicates whether the UE supports DAPS handover in source PCell and inter-frequency target PCell, i.e. support of simultaneous DL reception of PDCCH and PDSCH from source and target cell on carriers belonging to this band combination. A UE indicating this capability shall also support synchronous DAPS handover, and single UL transmission for inter-frequency DAPS handover.

Annex B: Proposed 36.331 changes

| interFreqDAPS Indicates whether the UE supports DAPS handover in source PCell and inter-frequency target PCell, i.e. support of simultaneous DL reception of PDCCH and PDSCH from source and target cell on carriers belonging to this band combination. A UE indicating this capability shall also support synchronous DAPS handover, and single UL transmission for inter-frequency DAPS handover. NOTE 9 | - |
| --- | --- |

| intraFreqDAPS Indicates whether UE supports DAPS handover in source PCell and intra-frequency target PCell, i.e. support of simultaneous DL reception of PDCCH and PDSCH from source and target cell. A UE indicating this capability shall also support synchronous DAPS handover, and single UL transmission for intra-frequency DAPS handover. NOTE 9 | - |
| --- | --- |

NOTE 9:	The source and target cell bandwidths may be different in DAPS handover. UE supports the same bandwidths for source and target cell as it would in single-carrier operation.

------------------------------------------------------------------TP End---------------------------------------------------------------

Question 6: if the proposals above are agreeable, do companies further agree to the corresponding TPs?

| Company | Yes/No | Remark |
| --- | --- | --- |

Question 7: if the proposals and TPs above are agreeable, do companies think the same changes can be made for NR?

| Company | Yes/No | Remark |
| --- | --- | --- |

# Conclusion

# Annex

In order to ease possible offline discussions, all delegates having provided input in this document are requested to fill the following table.

| Company | Name | Email Address |
| --- | --- | --- |