3GPP TSG RAN WG2 Meeting #112-e		R2-200xxxx

Online meeting, 02 – 13 November 2020

Title:	[draft] LS on NB-IoT connected mode neighbour cell measurements

Response to:

Release:	Release 17

Work Item:	NB_IOTenh4_LTE_eMTC6-Core

Source:	Qualcomm Incorporated [to be RAN2]

To:	RAN4

Cc:	RAN1??

Contact person:	Mungal Dhanda

mdhanda (at) qti (dot) qualcomm (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

# 1	Overall description

RAN2 is working on the following Release 17 WI objective:

- Specify signaling for neighbor cell measurements and corresponding measurement triggering before RLF, to reduce the time taken to RRC reestablishment to another cell, without defining specific gaps. [NB-IoT] [RAN2, RAN4].

In TS 36.133 for RRC re-establishment in NB-IoT, the delay requirements are given by:

TUE-re-establish_delay_NB-IoT = 100 ms + NNB-Iot-freq*Tsearch_NB1-NC + TSI_NB1-NC + TPRACH_NB-IoT

Where Tsearch_NB1-NC is the time required by the UE to search the target cell and defined as follows:

If the target cell is known, then Tsearch_NB1-NC = 0 ms. If the target cell is unknown and signal quality is sufficient for successful cell detection on the first attempt, then Tsearch_NB1-NC = 80 ms. Otherwise, Tsearch_NB1-NC = 1400 ms.

But it is not clear what is the definition of ‘target cell is known’.

Question 1: What is the definition of ‘target cell is known’ for RRC re-establishment in NB-IoT ?

Currently UE is not required to perform neighbour cell measurements for NB-IoT in RRC_CONNECTED state hence there are no measurement requirements. For the WI objective above, RAN2 is considering specifying neighbour cell measurements for NB-IoT in RRC_CONNECTED state. Therefore, RAN2 would like to understand the measurement requirements for NB-IoT in RRC-CONNECTED state for both intra-frequency and inter-frequency neighbour cells.

Question 2: For averaging, the minimum number of measurements required and the spacing between each measurement.

# 2	Actions

To 3GPP RAN4

ACTION: RAN2 respectfully asks RAN4 to provide feedback on the questions above.

# 3	Dates of next TSG RAN WG2 meetings

TSG RAN WG2 Meeting #113-e	25 January – 05 February 2021

TSG RAN WG2 Meeting #113bis-e	12 – 20 April 2021