3GPP TSG-RAN WG2 Meeting #113-e             	R2-2100791

E-Meeting, 25th January - 5th February 2020

Source:	vivo, OPPO, Apple

Title:	Left issues on TX resource (re-)selection

Agenda Item:	6.4.3

Document for:	Discussion and Decision

# Introduction

This contribution is to discuss the last issue (number 20) in RAN2 #112e meeting offline discussion [AT112-e][713][V2X] MAC corrections (LG)[1] about TX resource (re-)selection procedure.

# Discussion

In current RAN2 specification, the TX resource (re-)selection procedure is captured as follows, as an example (similar text is also for multiple MAC PDU case as well as in TX resource (re-)selection check procedure):

1> if the MAC entity has selected to create a selected sidelink grant corresponding to transmission(s) of a single MAC PDU, and if SL data is available in a logical channel, a SL-CSI reporting is triggered: … 2> if the TX resource (re-)selection is triggered as the result of the TX resource (re-)selection check: … 3> if one or more HARQ retransmissions are selected: 4> if there are available resources left in the resources indicated by the physical layer according to clause 8.1.4 of TS 38.214 [7] for more transmission opportunities: 5> randomly select the time and frequency resources for one or more transmission opportunities from the available resources, according to the amount of selected frequency resources, the selected number of HARQ retransmissions and the remaining PDB of SL data available in the logical channel(s) allowed on the carrier by ensuring the minimum time gap between any two selected resources in case that PSFCH is configured for this pool of resources and that a retransmission resource can be indicated by the time resource assignment of a prior SCI according to clause 8.3.1.1 of TS 38.212 [9];

It can be observed that the minimum time gap should be ensured in case that PSFCH is configured for this pool when perform resource (re-)selection under certain condition.

Observation 1: In current RAN2 MAC specification, the minimum time gap is ensured in case that PSFCH is configured for this pool when a UE perform resource (re-)selection.

However, according to RAN1 agreements, when UE selects resource from the resources indicated by physical layer for a given TB transmission, the minimum time gap is ensured by UE only when a HARQ feedback for the first resources is expected.

RAN1 #98 Agreements: - The resource (re-)selection procedure includes the following steps - Step 1: Identification of candidate resources within the resource selection window - FFS details - Step 2: Resource selection for (re-)transmission(s) from the identified candidate resources - FFS details RAN1 #102e Agreements: - In Step 2, a UE ensures a minimum time gap Z = a + b between any two selected resources of a TB where a HARQ feedback for the first of these resources is expected

Observation 2: In RAN1 agreement, the minimum gap should be ensured in resource (re-)selection when the HARQ-feedback is enabled for a given TB transmission.

Therefore, there is clearly some gap between RAN1 agreement and RAN2 specification.

- In RAN1 agreement, the minimum gap should be ensured when the SL HARQ-feedback is enabled for a given TB transmission.

- In RAN2 specification, the minimum gap is ensured when a SL grant is selected in a pool configured with PSFCH resources.

During the offline discussion in last meeting, the issue is discussed without consensus as some companies are thinking ‘pool configured with PSFCH resources’ can cover the ‘HARQ enable’ case, or are wondering whether the HARQ feedback enable/disable can be known by the UE in multi-shot transmission.

Therefore, we think a LS should be sent to RAN1 for clarification on this issue for a better RAN2 understanding and how we capture the minimum gap ensuring in MAC specification. A simple way can be just to ask RAN1 whether the current specification on minimum gap ensuring is aligned with their understanding.

Proposal 1: Send LS to RAN1 to confirm whether the RAN2 specification is aligned with RAN1 understanding on minimum gap ensuring in TX resource (re-)selection.

# Conclusion

We have the following observation and proposals:

Observation 1: In current RAN2 MAC specification, the minimum time gap is ensured in case that PSFCH is configured for this pool when a UE perform resource (re-)selection.

Observation 2: In RAN1 agreement, the minimum gap should be ensured in resource (re-)selection when the HARQ-feedback is enabled for a given TB transmission.

Proposal 1: Send LS to RAN1 to confirm whether the RAN2 specification is aligned with RAN1 understanding on minimum gap ensuring in TX resource (re-)selection.

- Reference

- R2-2010950, [AT112-e][713][V2X] MAC corrections (LG), LG Electronics Inc., discussion, RAN2 #112e meeting.