3GPP TSG-RAN WG2 Meeting #113-e		R2-2100870

Electronic, Jan 25 – Feb 05 2021

Agenda item:	6.3.3

Source:	Apple, Xiaomi

Title:	Discussion on NR-U RSSI/CO measurement

Document for:	Discussion

# 1	Introduction

In this contribution, we try to emphasize several issues related to NR-U, comprising of NR-U RSSI/CO measurement reporting and corresponding UE capabilities.

# 2   Inconsistency on RSSI/CO reporting in LTE spec and NR spec

In current NR-U design, RSSI/CO reporting can happen when reportType for the associated reportConfig is either eventTriggered or periodical. This can be observed by looking into the measurement procedure and ASN.1.

### TS 38.331 ### 5.5.3 Performing measurements #### 5.5.3.1 General <Text omitted> 2> if the reportType for the associated reportConfig is periodical, eventTriggered or condTriggerConfig: 3> if a measurement gap configuration is setup, or 3> if the UE does not require measurement gaps to perform the concerned measurements: <Text omitted> 4> if the measRSSI-ReportConfig is configured in the associated reportConfig: 5> perform the RSSI and channel occupancy measurements on the frequency indicated in the associated measObject; ### 6.3.2 Radio resource control information elements <Text omitted> ReportConfigNR information element -- ASN1START -- TAG-REPORTCONFIGNR-START ReportConfigNR ::= SEQUENCE { reportType CHOICE { periodical PeriodicalReportConfig, eventTriggered EventTriggerConfig, ..., reportCGI ReportCGI, reportSFTD ReportSFTD-NR, condTriggerConfig-r16 CondTriggerConfig-r16, cli-Periodical-r16 CLI-PeriodicalReportConfig-r16, cli-EventTriggered-r16 CLI-EventTriggerConfig-r16 } } ReportCGI ::= SEQUENCE { cellForWhichToReportCGI PhysCellId, ..., [[ useAutonomousGaps-r16 ENUMERATED {setup} OPTIONAL -- Need R ]] } ReportSFTD-NR ::= SEQUENCE { reportSFTD-Meas BOOLEAN, reportRSRP BOOLEAN, ..., [[ reportSFTD-NeighMeas ENUMERATED {true} OPTIONAL, -- Need R drx-SFTD-NeighMeas ENUMERATED {true} OPTIONAL, -- Need R cellsForWhichToReportSFTD SEQUENCE (SIZE (1..maxCellSFTD)) OF PhysCellId OPTIONAL -- Need R ]] } CondTriggerConfig-r16 ::= SEQUENCE { condEventId CHOICE { condEventA3 SEQUENCE { a3-Offset MeasTriggerQuantityOffset, hysteresis Hysteresis, timeToTrigger TimeToTrigger }, condEventA5 SEQUENCE { a5-Threshold1 MeasTriggerQuantity, a5-Threshold2 MeasTriggerQuantity, hysteresis Hysteresis, timeToTrigger TimeToTrigger }, ... }, rsType-r16 NR-RS-Type, ... } EventTriggerConfig::= SEQUENCE { eventId CHOICE { eventA1 SEQUENCE { a1-Threshold MeasTriggerQuantity, reportOnLeave BOOLEAN, hysteresis Hysteresis, timeToTrigger TimeToTrigger }, eventA2 SEQUENCE { a2-Threshold MeasTriggerQuantity, reportOnLeave BOOLEAN, hysteresis Hysteresis, timeToTrigger TimeToTrigger }, eventA3 SEQUENCE { a3-Offset MeasTriggerQuantityOffset, reportOnLeave BOOLEAN, hysteresis Hysteresis, timeToTrigger TimeToTrigger, useWhiteCellList BOOLEAN }, eventA4 SEQUENCE { a4-Threshold MeasTriggerQuantity, reportOnLeave BOOLEAN, hysteresis Hysteresis, timeToTrigger TimeToTrigger, useWhiteCellList BOOLEAN }, eventA5 SEQUENCE { a5-Threshold1 MeasTriggerQuantity, a5-Threshold2 MeasTriggerQuantity, reportOnLeave BOOLEAN, hysteresis Hysteresis, timeToTrigger TimeToTrigger, useWhiteCellList BOOLEAN }, eventA6 SEQUENCE { a6-Offset MeasTriggerQuantityOffset, reportOnLeave BOOLEAN, hysteresis Hysteresis, timeToTrigger TimeToTrigger, useWhiteCellList BOOLEAN }, ... }, rsType NR-RS-Type, reportInterval ReportInterval, reportAmount ENUMERATED {r1, r2, r4, r8, r16, r32, r64, infinity}, reportQuantityCell MeasReportQuantity, maxReportCells INTEGER (1..maxCellReport), reportQuantityRS-Indexes MeasReportQuantity OPTIONAL, -- Need R maxNrofRS-IndexesToReport INTEGER (1..maxNrofIndexesToReport) OPTIONAL, -- Need R includeBeamMeasurements BOOLEAN, reportAddNeighMeas ENUMERATED {setup} OPTIONAL, -- Need R ..., [[ measRSSI-ReportConfig-r16 MeasRSSI-ReportConfig-r16 OPTIONAL, -- Need R useT312-r16 BOOLEAN OPTIONAL, -- Need M includeCommonLocationInfo-r16 ENUMERATED {true} OPTIONAL, -- Need R includeBT-Meas-r16 SetupRelease {BT-NameList-r16} OPTIONAL, -- Need M includeWLAN-Meas-r16 SetupRelease {WLAN-NameList-r16} OPTIONAL, -- Need M includeSensor-Meas-r16 SetupRelease {Sensor-NameList-r16} OPTIONAL -- Need M ]] } PeriodicalReportConfig ::= SEQUENCE { rsType NR-RS-Type, reportInterval ReportInterval, reportAmount ENUMERATED {r1, r2, r4, r8, r16, r32, r64, infinity}, reportQuantityCell MeasReportQuantity, maxReportCells INTEGER (1..maxCellReport), reportQuantityRS-Indexes MeasReportQuantity OPTIONAL, -- Need R maxNrofRS-IndexesToReport INTEGER (1..maxNrofIndexesToReport) OPTIONAL, -- Need R includeBeamMeasurements BOOLEAN, useWhiteCellList BOOLEAN, ..., [[ measRSSI-ReportConfig-r16 MeasRSSI-ReportConfig-r16 OPTIONAL, -- Need R includeCommonLocationInfo-r16 ENUMERATED {true} OPTIONAL, -- Need R includeBT-Meas-r16 SetupRelease {BT-NameList-r16} OPTIONAL, -- Need M includeWLAN-Meas-r16 SetupRelease {WLAN-NameList-r16} OPTIONAL, -- Need M includeSensor-Meas-r16 SetupRelease {Sensor-NameList-r16} OPTIONAL, -- Need M ul-DelayValueConfig-r16 SetupRelease { UL-DelayValueConfig-r16 } OPTIONAL, -- Need M reportAddNeighMeas-r16 ENUMERATED {setup} OPTIONAL -- Need R ]] } NR-RS-Type ::= ENUMERATED {ssb, csi-rs} MeasTriggerQuantity ::= CHOICE { rsrp RSRP-Range, rsrq RSRQ-Range, sinr SINR-Range } MeasTriggerQuantityOffset ::= CHOICE { rsrp INTEGER (-30..30), rsrq INTEGER (-30..30), sinr INTEGER (-30..30) } MeasReportQuantity ::= SEQUENCE { rsrp BOOLEAN, rsrq BOOLEAN, sinr BOOLEAN } MeasRSSI-ReportConfig-r16 ::= SEQUENCE { channelOccupancyThreshold-r16 RSSI-Range-r16 OPTIONAL -- Need R } CLI-EventTriggerConfig-r16 ::= SEQUENCE { eventId-r16 CHOICE { eventI1-r16 SEQUENCE { i1-Threshold-r16 MeasTriggerQuantityCLI-r16, reportOnLeave-r16 BOOLEAN, hysteresis-r16 Hysteresis, timeToTrigger-r16 TimeToTrigger }, ... }, reportInterval-r16 ReportInterval, reportAmount-r16 ENUMERATED {r1, r2, r4, r8, r16, r32, r64, infinity}, maxReportCLI-r16 INTEGER (1..maxCLI-Report-r16), ... } CLI-PeriodicalReportConfig-r16 ::= SEQUENCE { reportInterval-r16 ReportInterval, reportAmount-r16 ENUMERATED {r1, r2, r4, r8, r16, r32, r64, infinity}, reportQuantityCLI-r16 MeasReportQuantityCLI-r16, maxReportCLI-r16 INTEGER (1..maxCLI-Report-r16), ... } MeasTriggerQuantityCLI-r16 ::= CHOICE { srs-RSRP-r16 SRS-RSRP-Range-r16, cli-RSSI-r16 CLI-RSSI-Range-r16 } MeasReportQuantityCLI-r16 ::= ENUMERATED {srs-rsrp, cli-rssi} -- TAG-REPORTCONFIGNR-STOP -- ASN1STOP

In the history, 3GPP RAN2 initially agreed to only support periodical NR-U RSSI reporting. Later, some comments mentioned that LAA supports piggybacked RSSI reporting when event triggered reporting for other quantities is initiated. In order to align with LAA, NR-U changed to also support piggybacked RSSI reporting when reportType for the associated reportConfig is eventTriggered. In this case, the event trigger determination is based on other quantity reporting, i.e., RSRP/RSRQ/SINR, but not RSSI.

However, we found out that actually in LTE spec, neither LAA nor inter-RAT NR-U supports piggybacked RSSI reporting as shown below.

TS36.331 measRSSI-ReportConfig (in ReportConfigEUTRA for LAA) If this field is present, the UE shall perform measurement reporting for RSSI and channel occupancy and ignore the triggerQuantity, reportQuantity and maxReportCells fields. E-UTRAN sets this field to true only when setting triggerType to periodical and purpose to reportStrongestCells. measRSSI-ReportConfigNR (in ReportConfigInterRAT for NR RSSI reporting) If this field is present, the UE shall perform measurement reporting for RSSI and channel occupancy and ignore the triggerQuantity, reportQuantity and maxReportCells fields. E-UTRAN sets this field to true only when setting triggerType to periodical and purpose to reportStrongestCells.

Observation 1: NR-U RSSI reporting in TS 38.331 is not consistent with LAA/inter RAT NR-U RSSI reporting in TS 36.331. In details, In TS38.331, NR-U RSSI can be piggybacked in event triggered RSRP/RSRQ/SINR reporting while it’s not true for LAA and inter-RAT NR-U RSSI reporting in TS36.331.

Proposal 1: RAN2 to discuss if companies have the common understanding on the discrepancy between TS 36.331 and TS 38.331 on NR-U RSSI reporting. Further discussion is needed on whether and how RAN2 would like to remove the discrepancy.

# 3   UE Capability on RSSI/CO measurement

In TS 38.331 and TS 38.306, RSSI/CO measurement and reporting on unlicensed spectrum is defined as a per-band UE capability.

TS 38.306

However, in LTE spec, NR-U RSSI/CO measurement UE capability is only carried in UE-CapabilityRAT-ContainerList, which the eNB is not required to decode. However, in LTE spec, NR-U RSSI measurement configuration is enabled. Without UE capability, eNB cannot make such corresponding configuration to UE.

Observation 2: In LTE spec, inter-RAT NR-U RSSI/CO measurement UE capability cannot be known by eNB.

Proposal 2: Suggest to add the per band inter-RAT NR-U RSSI/CO measurement UE capability into LTE spec, as shown below.

TS 36.331 UE-EUTRA-Capability <Text omitted> UE-EUTRA-Capability-v1610-IEs ::= SEQUENCE { highSpeedEnhParameters-v1610 HighSpeedEnhParameters-v1610 OPTIONAL, neighCellSI-AcquisitionParameters-v1610 NeighCellSI-AcquisitionParameters-v1610 OPTIONAL, mbms-Parameters-v1610 MBMS-Parameters-v1610 OPTIONAL, pdcp-Parameters-v1610 PDCP-Parameters-v1610 OPTIONAL, mac-Parameters-v1610 MAC-Parameters-v1610 OPTIONAL, phyLayerParameters-v1610 PhyLayerParameters-v1610 OPTIONAL, measParameters-v1610 MeasParameters-v1610 OPTIONAL, pur-Parameters-r16 PUR-Parameters-r16 OPTIONAL, eutra-5GC-Parameters-v1610 EUTRA-5GC-Parameters-v1610 OPTIONAL, otherParameters-v1610 Other-Parameters-v1610 OPTIONAL, dl-DedicatedMessageSegmentation-r16 ENUMERATED {supported} OPTIONAL, mmtel-Parameters-v1610 MMTEL-Parameters-v1610, irat-ParametersNR-v1610 IRAT-ParametersNR-v1610 OPTIONAL, rf-Parameters-v1610 RF-Parameters-v1610 OPTIONAL, mobilityParameters-v1610 MobilityParameters-v1610 OPTIONAL, ue-BasedNetwPerfMeasParameters-v1610 UE-BasedNetwPerfMeasParameters-v1610, sl-Parameters-v1610 SL-Parameters-v1610 OPTIONAL, fdd-Add-UE-EUTRA-Capabilities-v1610 UE-EUTRA-CapabilityAddXDD-Mode-v1610 OPTIONAL, tdd-Add-UE-EUTRA-Capabilities-v1610 UE-EUTRA-CapabilityAddXDD-Mode-v1610 OPTIONAL, nonCriticalExtension UE-EUTRA-Capability-v16xy-IEs OPTIONAL } UE-EUTRA-Capability-v16xy-IEs ::= SEQUENCE { measParameters-v16xy MeasParameters-v16xy OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL } <Text omitted> MeasParameters-v1610 ::= SEQUENCE { bandInfoNR-v1610 SEQUENCE (SIZE (1..maxBands)) OF MeasGapInfoNR OPTIONAL, altFreqPriority-r16 ENUMERATED {supported} OPTIONAL, ce-DL-ChannelQualityReporting-r16 ENUMERATED {supported} OPTIONAL, ce-MeasRSS-Dedicated-r16 ENUMERATED {supported} OPTIONAL, ca-IdleInactiveMeasurements-r16 ENUMERATED {supported} OPTIONAL, endc-IdleInactiveMeasFR1-r16 ENUMERATED {supported} OPTIONAL, endc-IdleInactiveMeasFR2-r16 ENUMERATED {supported} OPTIONAL, idleInactiveValidityAreaList-r16 ENUMERATED {supported} OPTIONAL, measGapPatterns-NRonly-r16 ENUMERATED {supported} OPTIONAL, measGapPatterns-NRonly-ENDC-r16 ENUMERATED {supported} OPTIONAL } MeasParameters-v16xy ::= SEQUENCE { sharedSpectrumMeasNR-r16 SEQUENCE (SIZE (1..maxBands)) OF SharedSpectrumMeasNR-r16 OPTIONAL } SharedSpectrumMeasNR-r16 ::= SEQUENCE { rssi-ChannelOccupancyReporting-r16 ENUMERATED {supported} OPTIONAL }

# 4	Conclusions

Based on the discussion above, we have the following proposals.

Proposal 1: RAN2 to discuss if companies have the common understanding on the discrepancy between TS 36.331 and TS 38.331 on NR-U RSSI reporting. Further discussion is needed on whether and how RAN2 would like to remove the discrepancy.

Proposal 2: Suggest to add the per band inter-RAT NR-U RSSI/CO measurement UE capability into LTE spec, as shown below.