3GPP TSG-RAN WG2 Meeting #113 electronic                                             　    　　　　　　 	R2-2100939

Online, January, 2021

Source: 	KDDI Corporation

Title:	Introduction of event-based trigger for LTE MDT logging

Tdoc Type:	Discussion

Agenda Item:	9.3 EUTRA R17 Other

Document for:	Discussion and decision

Release:	Rel-17

Introduction

In NR, two types of event triggered MDT logging functions, out of coverage and event L1 were standardized. However, in LTE, those event triggered functions have not been specified yet. In this paper we propose to introduce those event triggered functions in LTE.

Discussion

When introducing SA, operators will migrate their LTE spectrum to NR, so that the operators can provide better NR coverage services to their subscribers. However, this spectrum migration would cause LTE coverage holes, LTE coverage problem. In such situations, operators want to have a mechanism to detect LTE coverage holes. If operators find LTE coverage holes, then operators take some measures, having more base stations, optimizing network parameters to compensate the coverage gap caused from the NR migration. (A mechanism to detect LTE coverage holes is also beneficial for other purposes rather than NR migration.)

LTE coverage

LTE coverage

NR only

Current 5G service (NSA)

Next 5G service (migrate to SA)

Coverage holes

Migration

Fig: Coverage gap/hole caused from the NR migration

At this moment, since the event triggered MDT logging functions on LTE is not standardized, operators have no choice but to use their dedicated proprietary solution (non 3GPP solution) to detect LTE coverage holes. However, implementing operator's dedicated proprietary solution on UEs would be difficult, and it is preferable to have the standardized solution which can be used for all operators. We guess that many operators might think that they don’t want to use their dedicated proprietary solution any more. We believe it’s beneficial to standardize the functions before many operators migrate their own LTE spectrum to NR, and we don’t think it’s too late to standardize LTE function now. Thus, we propose the following.

Proposal 1:

RAN2 agree to start the discussion (agree to standardize the feature) with regard to out-of-coverage detection trigger.

RAN2 agree to introduce the event triggered MDT logging functions, at least for out-of-coverage detection trigger.

Addition to out of coverage detection, we may want to introduce event L1 MDT logging function at the same time. Thus, we propose the following.

Proposal 2:

RAN2 discuss whether to introduce event L1 MDT logging.

Proposal 3:

Once RAN2 agree proposal1 and proposal2, RAN2 finalize the draft CR[1][2][3][4] of this enhancement.

As to in which release to be introduced, from operators’ perspective, it is preferable to introduce the function from Rel-16, but some vendors may have different views, so we suggest the following.

Proposal 4:

RAN2 discuss from which release the function is introduced, Rel-17 or Rel-16.

Conclusion

In this conclusion, we make the following proposals.

Proposal 1:

RAN2 agree to start the discussion (agree to standardize the feature) with regard to out-of-coverage detection trigger.

RAN2 agree to introduce the event triggered MDT logging functions, at least for out-of-coverage detection trigger.

Proposal 2:

RAN2 discuss whether to introduce event L1 MDT logging

Proposal 3:

Once RAN2 agree proposal1 and proposal2, RAN2 finalize the draft CR[1][2][3][4] of this enhancement.

Proposal 4:

RAN2 discuss from which release the function is introduced, Rel-17 or Rel-16.

References

[1] R2-2100818 draftCR  Introduction of event-based trigger for LTE MDT logging KDDI Corporation Rel-16 36.331
[2] R2-2100819 draftCR  Introduction of event-based trigger for LTE MDT logging KDDI Corporation Rel-17 36.331
[3] R2-2100821 draftCR  Introduction of event-based trigger for LTE MDT logging KDDI Corporation Rel-16 37.320
[4] R2-2100823 draftCR Introduction of event-based trigger for LTE MDT logging KDDI Corporation Rel-17 37.320