3GPP TSG-RAN WG2 #113-e	Tdoc R2-2101452

Electronic meeting, January 25th – February 5th 2021

Agenda Item:	6.2.2

Source:	Ericsson

Title:	Handling of Unknown and Reserved Values in the BAP Header

Document for:	Discussion, Decision

# 1	Introduction

According to the Rel.16 BAP specification, when an IAB node receives a BAP PDU with a BAP header containing reserved or unknown values, the IAB node shall discard the received BAP PDU.

In case RAN2 decides in future releases, e.g. Rel.17, to enhance the BAP header and include new fields or use some of the currently reserved fields, the present functionality would imply that an “old” Rel.16 IAB node, which receives a BAP PDU from a “new” IAB node, would discard the BAP PDU if its BAP header contains value which appear as reserved or unknown to the old IAB node.

The objective of this contribution is to raise the attention on this BAP packet discarding functionality and to discuss whether the scenario of an IAB network consisting of a mixture of “old” and “new” IAB nodes which can relay BAP PDUs between each other is a valid/common scenario from a deployment perspective. It is then discussed whether 3GPP specifications should take into account this problem in Rel.16 or in future releases.

# 2	Discussion

According to the existing Rel.16 BAP specification TS 38.340, when an IAB node receives a BAP PDU with a BAP header containing reserved or unknown values, the IAB node shall discard the received BAP PDU. Similarly, the BAP PDU is discarded if its BAP address is unknown to the IAB node, i.e. the BAP address indicated in the BAP header is not the BAP address of the IAB node and it does not appear in the BH routing table either.

From Rel.16 TS 38.340: ## 5.5 Handling of unknown, unforeseen, and erroneous protocol data When a BAP PDU that contains reserved or invalid values or contains a BAP address which is not included in the configured BH Routing Configuration and is not the BAP address of this node is received, the BAP entity shall: - discard the received BAP PDU.

In Rel.16, the event of an IAB node receiving a BAP PDU with a BAP header containing reserved or invalid values should be considered as an error case, since all IAB nodes in the network shall be capable to transmit and decode a BAP PDU compliant with the Rel.16 BAP specification. Hence, if for some reasons, an IAB node receives a BAP PDU containing reserved or unknown values, the BAP PDU shall be discarded.

According to Rel.16 BAP specification, if an IAB node receives a BAP PDU with a BAP header containing reserved or invalid values, it will discard the BAP PDU.

However, this present functionality would also imply that if the BAP header is enhanced in future releases, e.g. new fields are introduced in the BAP header or some of the currently reserved values are used, an “old” Rel.16 IAB node that receives such enhanced BAP PDU from a new IAB node would discard the packet. That could happen for example if RAN2 decides to introduce new features that imply an enhancement of the current BAP header.
Let´s for example consider the scenario in Figure 1 which depicts an IAB network consisting of “new” and “old” Rel.16 IAB nodes. Considering the downstream, the “old” Rel.16 IAB node 2, is the next hop for the “new” IAB node 1 and hence it receives BAP PDUs from it. Let´s then assume that the IAB node 1 includes in the BAP header of the BAP PDU a certain new field reflecting a certain new feature that the IAB node 1 can support. Such new field could be understood by the IAB node 3, but unfortunately this BAP PDU will never reach the IAB node 3, since the “old” IAB node 1 would discard it, as per Rel.16 TS 38.340.
Obviously, the same issue would be present also for the upstream, i.e. whenever an “old” Rel.16 IAB node is the next hop for a “new” IAB node.

In case RAN2 decides in a future release to support new features that imply enhancements to the BAP header, it might happen that a “new” IAB node uses such new features that are not supported by a peer receiving “old” IAB.

The issue in Observation 2 would result in the “new” IAB node transmitting a BAP PDU with a BAP header containing fields that a peer receiving “old” IAB node would consider as reserved or invalid. Upon reception, the “old” IAB node would discard such BAP PDU, as per current Rel.16 specification.

It is difficult at the moment to foresee how, when and if the BAP header will be really enhanced, even though there are already proposals in the Rel.17 WI that hint on possible enhancements to the BAP header for enhancing IAB performances. In any case, RAN2 cannot exclude that enhancements to the BAP header may be possible in a future IAB release.

Figure 1: IAB network made up of a mixture of “old” Rel.16 and “new” IAB nodes.

In order to assess whether the issue highlighted in Observation 3 is something that needs to be tackled, RAN2 should first discuss whether the scenario illustrated in Figure 1 is a valid scenario from a deployment perspective. In case RAN2 concludes that a scenario in which an IAB network is made up of a mixture of “old” and “new” IAB nodes is not of interest from an operator´s perspective, then there is nothing that needs to be fixed in 3GPP. Hence, RAN2 should discuss whether it is possible in a real-world deployment to have an “old” Rel.16 IAB node as next hop for a new IAB node.

RAN2 to discuss whether it is of interest a deployment scenario in which an “old” Rel.16 IAB node is the next hop for a new IAB node.

In case RAN2 concludes that the scenario depicted in Figure 1 is a valid scenario, then RAN2 should discuss if anything should be done in the specification, either in Rel.16 specification or in future releases.

In particular, if such a scenario is valid, RAN2 should discuss whether there is any value for the NW to configure the BAP transmitter with features which are not configured or supported by a peer BAP receiver. The value could be for example that another IAB node in the network, e.g. IAB3 in Figure 1, can benefit of such features, but according to the current spec. the peer BAP receiver of the IAB node 2 would discard the packet.

RAN2 to discuss whether there is any benefit for the NW to configure the BAP transmitter with features which are not configured or supported by the BAP receiver leading to that the receiver cannot understand the BAP header.

If RAN2 sees benefits, then RAN2 should discuss whether there is the need to address this issue in current of future specification.

If RAN2 sees benefit, RAN2 to discuss whether specification work (in Rel.16 or in a future release) is required to solve the issue observed in Observation 3, i.e. an “old” Rel.16 IAB node would discard a BAP PDU received from a “new” IAB node if the “new” IAB node sets some fields in the BAP header that appear as reserved/invalid at the “old” IAB node.

## 2.1 CU configuration

One approach would be for example to let the donor CU configure the IAB nodes with the BAP header version they should adopt, based on the path a certain BAP packet is following. This would imply to allow the CU to configure the IAB nodes with a certain set of features that can be supported also by the other IAB nodes in the path. 
For example, if the BAP packet is traversing a Rel.16 “old” IAB node, then the “new” IAB node of the previous hop should not use the new BAP header format. Otherwise, if all the IAB nodes traversed are “new”, then the new BAP header can be used. In this way, the receiving IAB node will always be able to decode the received BAP PDU, and no packet discard will occur.

One approach to fix the issue observed in Observation 3 is to let the CU provide the “new” IAB node with the BAP header formats it can use depending on the path/destination of a BAP PDU. This implies allowing the CU to configure an IAB node with a set of features included in the BAP header that a peer receiving IAB node supports.

## 2.2 Avoid BAP PDU discarding

An alternative approach is to allow the “old” Rel.16 IAB node to just relay the BAP PDU to the next hop without discarding the packet. In fact, already according to the existing specification, an intermediate IAB node is not required to inspect other fields than the DESTINATION and PATH fields, i.e. the BAP routing ID, which are instrumental to determine if the IAB node is the access IAB node and to map the packet to a proper egress link and BH RLC channel.

In particular, as shown below, TS 38.340 envisages two methods for passing BAP PDUs from the receiving part to the transmitting part of the BAP entity:

From TS 38.340: In the example of Figure 4.2.2-1, the receiving part on the BAP entity delivers BAP PDUs to the transmitting part on the collocated BAP entity. Alternatively, the receiving part may deliver BAP SDUs to the collocated transmitting part. When passing BAP SDUs, the receiving part removes the BAP header and the transmitting part adds the BAP header with the same BAP routing ID as carried on the BAP PDU header prior to removal. Passing BAP SDUs in this manner is therefore functionally equivalent to passing BAP PDUs, in implementation. The following specification therefore refers to the passing of BAP Data Packets. Figure 4.2.2-1. Example of functional view of BAP sublayer

From Rel.16 BAP perspective, only the DESTINATION and PATH fields in the BAP header need to be decoded correctly, in order to ensure proper BH routing operations.

So from the above reasoning, as long as the DESTINATION/PATH fields can be correctly decoded by a Rel.16 IAB node, it seems that there are no strong technical reasons to require an IAB intermediate node to discard a received BAP PDU, even if that includes some unknown/reserved fields. In fact, even if the BAP PDU would contain some unknown/reserved fields that would not affect the functionalities neither of the receiving part nor of the transmitting part of the BAP entity. The receiving part just needs to read the DESTINATION/PATH and ignore the unknown/reserved fields, i.e. without stripping-off or manipulate those fields. The transmitting part just needs to check again the DESTINATION/PATH fields to determine the egress link and the BH RLC channel, and it can also ignore those unknown/reserved fields. 
The only case in which the received BAP PDU should be discarded is when the IAB node is the IAB access node. In fact, in such case, the IAB node should pass to upper layers only the DATA part of the BAP PDU, but unfortunately if IAB node cannot comprehend all the fields of the BAP header, it is not possible for the IAB node to properly separate the BAP header from the DATA part.

An alternative approach to fix the issue observed in Observation 3 is to avoid the IAB node discarding a received BAP PDU if it contains unknown/reserved values, as long as the DESTINATION/PATH fields can be decoded, and the IAB node is not an IAB access node for the BAP PDU.

## 2.3 Summary

In conclusion, if RAN2 sees the need to fix the problem observed in Observation 3, we see two possible solutions which presented in Section 2.1 CU configuration and in Section 2.2 Avoid BAP PDU discarding.

Pros of the solution proposed in Observation 4 is that it does not require any change to the Rel.16 specifications. If RAN2 will decide in a future release to introduce a new BAP header format, then the CU should provide the IAB node with the necessary information on the version of the BAP header formats the IAB node should use, depending on the version of the IAB node at the next hop. That would basically imply the CU configuring the IAB node with a set of features affecting the BAP header that a receiving IAB node can understand.
Cons of this solution is that if a “new” IAB node has an “old” IAB node at the next hop, then the “new” IAB node cannot fully exploit its functionalities. In other words, that implies that both transmitter and receiver must support and be configured with the same functionality so both ends can understand the headers. 
For example, considering the example in Figure 1, RAN2 may introduce a new feature that the IAB node 1 could use to aid the IAB node 3 operations. Unfortunately, with this solution such feature cannot be used since the IAB node 1 will be configured to always transmit BAP PDU with the Rel.16 BAP header when transmitting to the IAB node 2.

Pros of the solution proposed in Observation 6 is that it allows a new IAB node to fully exploit its functionalities without causing any packet losses due to the presence of old Rel.16 IAB nodes.
Cons of this solution is that it requires changes to the Rel.16 specification which needs to be forward-compatible with respect to possible changes to the BAP header format. For this solution, we provide a CR in the Annex.

If RAN2 sees the need to fix the problem observed in Observation 3, RAN2 to consider one of the following approaches:

  - The CU provides the “new” IAB node with the BAP header format it can use depending on the path/destination of a BAP PDU, i.e. the CU configures the IAB node such that the BAP header can be understood by a peer receiving IAB node. No changes to Rel.16 spec are needed.

  - Avoid the IAB node discarding a received BAP PDU if it contains unknown/reserved values, as long as the DESTINATION/PATH fields can be decoded, and the IAB node is not an IAB access node for the BAP PDU. Changes to Rel.16 spec. are needed (see Annex for the possible changes required).

Pros of Proposal 3a is that it does not require changes to Rel.16 spec. Cons is that a “new” IAB node cannot fully exploit its new functionalities when the IAB node of the next hop is an “old” Rel.16 IAB node, since both transmitter and receiver must support and be configured with the same functionalities.

Pros of Proposal 3b is that it allows a “new” IAB node to fully exploit its new functionalities without causing any packet discards at the “old” IAB node of the next hop. Cons is that it requires changes to Rel.16 specification (see Annex for the possible changes required).

# Conclusion

In the previous sections we made the following observations:

Observation 1	According to Rel.16 BAP specification, if an IAB node receives a BAP PDU with a BAP header containing reserved or invalid values, it will discard the BAP PDU.

Observation 2	In case RAN2 decides in a future release to support new features that imply enhancements to the BAP header, it might happen that a “new” IAB node uses such new features that are not supported by a peer receiving “old” IAB.

Observation 3	The issue in Observation 2 would result in the “new” IAB node transmitting a BAP PDU with a BAP header containing fields that a peer receiving “old” IAB node would consider as reserved or invalid. Upon reception, the “old” IAB node would discard such BAP PDU, as per current Rel.16 specification.

Observation 4	One approach to fix the issue observed in Observation 3 is to let the CU provide the “new” IAB node with the BAP header formats it can use depending on the path/destination of a BAP PDU. This implies allowing the CU to configure an IAB node with a set of features included in the BAP header that a peer receiving IAB node supports.

Observation 5	From Rel.16 BAP perspective, only the DESTINATION and PATH fields in the BAP header need to be decoded correctly, in order to ensure proper BH routing operations.

Observation 6	An alternative approach to fix the issue observed in Observation 3 is to avoid the IAB node discarding a received BAP PDU if it contains unknown/reserved values, as long as the DESTINATION/PATH fields can be decoded, and the IAB node is not an IAB access node for the BAP PDU.

Observation 7	Pros of Proposal 3a is that it does not require changes to Rel.16 spec. Cons is that a “new” IAB node cannot fully exploit its new functionalities when the IAB node of the next hop is an “old” Rel.16 IAB node, since both transmitter and receiver must support and be configured with the same functionalities.

Observation 8	Pros of Proposal 3b is that it allows a “new” IAB node to fully exploit its new functionalities without causing any packet discards at the “old” IAB node of the next hop. Cons is that it requires changes to Rel.16 specification (see Annex for the possible changes required).

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to discuss whether it is of interest a deployment scenario in which an “old” Rel.16 IAB node is the next hop for a new IAB node.

Proposal 2	RAN2 to discuss whether there is any benefit for the NW to configure the BAP transmitter with features which are not configured or supported by the BAP receiver leading to that the receiver cannot understand the BAP header.

Proposal 3	If RAN2 sees benefit, RAN2 to discuss whether specification work (in Rel.16 or in a future release) is required to solve the issue observed in Observation 3, i.e. an “old” Rel.16 IAB node would discard a BAP PDU received from a “new” IAB node if the “new” IAB node sets some fields in the BAP header that appear as reserved/invalid at the “old” IAB node.

Proposal 4	If RAN2 sees the need to fix the problem observed in Observation 3, RAN2 to consider one of the following approaches:

a.	The CU provides the “new” IAB node with the BAP header format it can use depending on the path/destination of a BAP PDU, i.e. the CU configures the IAB node such that the BAP header can be understood by a peer receiving IAB node. No changes to Rel.16 spec are needed.

b.	Avoid the IAB node discarding a received BAP PDU if it contains unknown/reserved values, as long as the DESTINATION/PATH fields can be decoded, and the IAB node is not an IAB access node for the BAP PDU. Changes to Rel.16 spec. are needed (see Annex for the possible changes required).

# Annex - Text Proposal for Rel.16 TS 38.340

START OF CHANGE

## 5.5	Handling of unknown, unforeseen, and erroneous protocol data

When a BAP PDU that contains a BAP address which is not included in the configured BH Routing Configuration and is not the BAP address of this node is received; or when a BAP PDU that contains reserved or invalid values and that contains a BAP address which is the BAP address of this node is received, the BAP entity shall:

-	discard the received BAP PDU.

Otherwise, when a BAP PDU contains reserved or invalid values, the BAP entity shall:

-	ignore the reserved or invalid values and process the received BAP PDU as in clause 5.2.2.

END OF CHANGES