3GPP TSG-RAN WG2 Meeting #113-bis-e                             	R2-2102939

Online, April 12 – April 20, 2021

Agenda item:	8.3.2

Source:	Samsung

Title:	Considerations for Paging Collision Avoidance Solution

Document for:	Discussion & Decision

# Introduction

In previous RAN2 meeting #113-e [1], following agreements were made for paging collision avoidance:

- There is support for solution 1 (for 5GS) with something else, either solution 3 or 2b. - Option 2b is the preferred solution to address paging collision for “LTE + LTE”. - MUSIM UE determines potential paging collision on two networks and triggers actions on potential paging collision avoidance. - It is left to UE implementation as to how it selects one of the two RATs/networks for paging collision avoidance. - FFS if we can make the UE behaviour predictable for paging collision avoidance - NAS signalling is baseline for UE reporting paging collision in 5GS side (to be confirmed by SA2). - It is FFS whether assistant information is needed for paging collision in 5GS side.

This paper discusses potential solution to address paging collision issue and related considerations.

# Discussion

## Solution for paging collision avoidance

With option 1 for UE-requested 5G-GUTI reassignment, collision probability with new GUTI assigned by the network is expected to be very less. It would be extremely rare that UE would be required to again request for new GUTI reassignment in the case new 5G-GUTI does not resolve paging collision. UE may also, optionally, provide assistance information to help CN. Given the extreme low paging collision probability, we conclude that this is the simplest solution, has lowest impact on UE/Network entities and is quite effective.

Option 2b with offset approach is like option 1 but essentially separates the paging timing considerations with an ‘offset’ and provides information to devise an updated UE_ID. However, the approach may require re-negotiation when UE moves to a new cell/area with different paging configuration, however, only when it causes a paging collision. There is higher impact of this approach as compared to Option 1 e.g. UE, RAN and AMF will be impacted and NG-AP interface needs to carry UE Identity Index Value field updated with offset.

Option 3 for network repetition or retransmission, in our opinion, is not paging resource efficient. At the minimum, there is 100% paging overhead when network repeats the paging. We think RAN2 should consider to improvise this aspect before considering this as a solution.

With option 1 approach, there seems no rational or need to also complement this with Option 2b and/or Option 3.

Proposal 1: Option 1 for UE-requested 5G-GUTI reassignment is adopted as paging collision avoidance solution for 5GCS.

Proposal 2: Option 2b and Option 3 are not adopted as independent solutions or as complementary solutions to Option 1.

## Idle and Inactive mode UEs

For RAN paging in Inactive mode, DRX cycle of the UE ‘T’ is determined by the shortest of the UE specific DRX value(s), if configured by RRC and/or upper layers, and a default DRX value broadcast in system information. This implies that there may be case where RAN paging cycle can also be sub-multiple of CN paging cycle. So as a possibility, we may have case where in only alternate RAN paging occasions are affected by paging collision with other SIM and CN paging occasions are not affected at all. However, in general, CN paging collision avoidance solution can resolve this case as well. E.g. a new GUTI reassignment will change the PO for both RAN paging and CN paging and collision is avoided.

As described in [2], AMF provides the updated UE Identity Index Value field sent in the NG-AP paging message. For Idle UEs, as there is no UE context maintained at RAN, AMF needs to provide this with every paging message. For the case of Inactive UEs, the AMF also provides it as part of the Core network assistance information to RAN. When the assistance information is an offset given by the UE, UE Identity Index Value is updated with the offset value by the AMF.

We think there may possibly be some RAN based solutions for Inactive mode only, where in gNB can apply the offset by itself. But such solutions will bring in more complexity and lead to duality of approaches for Idle and Inactive modes. Moreover, as paging collision is a rare event, there seems no special reason to introduce another solution for Inactive UEs.

Proposal 3: CN based paging collision avoidance solution is equally applicable for Idle and Inactive mode UEs. There is no need for RAN based approaches to address paging collision in Inactive mode.

## Predictable UE behavior

As agreed in RAN2#113-e meeting, UE implementation determines as to how it selects one of the two RATs/networks for paging collision avoidance. However, there was a question if RAN2 can make UE behaviour predictable for the paging collision avoidance. Concern here seems that behavior from different UEs would vary. However, it is not very clear what is the real problem? e.g. different UEs may have different preferences to select one of the two RATs/Networks. As UE is the only entity that has the visibility to both networks and associated configurations, and may have some motivations for this selection e.g. enhanced power saving. It appears imposing some prioritization rule or preference on UEs (e.g. preferred RAT) or setting some selection policy (e.g. round-robin) on UE implementation by the network would be unnecessary and inefficient. Moreover, there is no coordination for the two different networks involved here, and it seems unfair for either network to apply any prioritization rule on the common MUSIM UE.

Proposal 4: No prioritization rule or preference or selection policy is imposed by the network on the UE for RAT/Network selection for paging collision avoidance.

## Need for assistance information

In general, paging configuration parameters can be symmetric or asymmetric on two uncoordinated networks for MUSIM UE e.g. USIM A (UE_ID, T, N, PF_offset) and USIM B (UE_ID’, T’, N’, PF_offset’). UE can provide potential paging collision indication, and possibly assistance information, to the network. In our understanding assistance information is not essential for paging collision resolution and its utilization may be more related to better power saving for the UE. Therefore, we propose that assistance information is kept optional and it should be left to UE implementation whether to include it or not.

Further, since Access Stratum (AS) is better equipped with paging configuration parameters information on both the networks, it seems logical that AS builds the assistance information, when needed, for paging collision avoidance and provides it to NAS.  NAS can, then, signal the same to the network.

Proposal 5: MUSIM UE may optionally provide assistance information to network to resolve paging collision since involved networks are not coordinated.

Proposal 6: Access Stratum in the MUSIM UE builds assistance information, when needed, for paging collision avoidance and provides same to NAS to signal to the network.

# Conclusion

RAN2 to discuss and agree on following proposals:

Proposal 1: Option 1 for UE-requested 5G-GUTI reassignment is adopted as paging collision avoidance solution for 5GCS.

Proposal 2: Option 2b and Option 3 are not adopted as independent solutions or as complementary solutions to Option 1.

Proposal 3: CN based paging collision avoidance solution is equally applicable for Idle and Inactive mode UEs. There is no need for RAN based approaches to address paging collision in Inactive mode.

Proposal 4: No prioritization rule or preference or selection policy is imposed by the network on the UE for RAT/Network selection for paging collision avoidance.

Proposal 5: MUSIM UE may optionally provide assistance information to network to resolve paging collision since involved networks are not coordinated.

Proposal 6: Access Stratum in the MUSIM UE builds assistance information, when needed, for paging collision avoidance and provides same to NAS to signal to the network.

# References

[1] Report of 3GPP TSG RAN2#113-e meeting (Draft_RAN2_113-e_Meeting Report_v1)

[2] TR 23.761 v1.3.0, Study on system enablers for devices having multiple Universal Subscriber Identity Modules