3GPP TSG-RAN WG2 Meeting #113-bis-e	R2-2103341

# E-meeting, 12th April – 20th April 2021

Title:	[DRAFT] LS Reply on TCI State Update for L1/L2-Centric Inter-Cell Mobility

Response to:	R2-2102627/R1-2102248

Release:	Rel-17

Work Item:	NR_feMIMO-Core

Source:	Samsung [To be RAN2]

To:	RAN1

Cc:	RAN3, RAN4

Contact Person:

Name:	Seungri Jin

E-mail Address:	seungri.jin@samsung.com

# 1. Overall Description

RAN2 would like to thank RAN1 for the LS on TCI State Update for L1/L2-Centric Inter-Cell Mobility. In RAN2#113-bis-e meeting, RAN2 has discussed RAN1’s questions on the support of TCI state update (beam indication) for DL reception from and UL transmission to non-serving cell(s) – at least on UE-dedicated PDSCH, PDCCH, PUSCH, and PUCCH.

RAN2 added the answers for the RAN1’s questions as below:

Question 1: In regard of serving cell,

- Is there a need for a UE to change a serving cell for DL reception from or UL transmission to another (non-serving) cell, at least on UE-dedicated PDSCH, PDCCH, PUSCH, and PUCCH?

[Answer 1-1] For PUCCH/PUSCH/PDCCH/PDSCH transmission/reception on non-serving cell, UE needs to know the corresponding configurations (BWP, physical channel configuration, cell group configurations, etc.).

- If so, how can the addition, release or change of a non-serving cell for DL reception and/or UL transmission be done? For example, would any of such actions require L3 handover and/or selection/activation among pre-configured candidate cells from RAN2 perspective?

[Answer 1-2] How and when are these signaled to UE would be depend on the scenario, using the pre-configured candidate cells from the serving cell is the one way if L3 handover is not intended scenario. If we assume the specific scenario (e.g. intra-DU case), the same configuration would be applied to non-serving cells by implementation, then no additional configuration is needed.

- If so, how can the TCI states associated with the previous serving cell be handled?

[Answer 1-3] From RAN2 understanding, UE is connected with source cell so the TCI states associated with the previous serving cell would be kept.

- If so, what is the impact on the system information reception by the UE?

[Answer 1-4] It is also depended on the scenario. It seems the common configurations (e.g. RACH configuration) included in MIB/SIB values change upon inter PCI/TRP change in general, so configuration of the common configurations from the non-serving cell is required to start transmitting RACH based on that configuration. However, it can be possible all common configurations are same in case of intra-DU case then UE can still use the MIB/SIB in the source cell.

- If so, what is the impact on the RACH and PUCCH-related procedures and configurations?

[Answer 1-5] As commented above, RACH and PUCCH-related procedures and configurations would be different for the scenario:

- Intra-DU case: RACH and PUCCH-related configuration of serving cells can be applied i.e. MIB/SIB of serving cell can be used.

- Inter-DU case: RACH and PUCCH-related configuration of non-serving cells would be required i.e. MIB/SIB of non-serving cell is required.

- If not, what is the impact on the applicable use cases? That is, in what scenarios can the UE be configured for DL reception from or UL transmission to another (non-serving) cell, at least on UE-dedicated PDSCH, PDCCH, PUSCH, and PUCCH, if the serving cell does not change?

[Answer 1-6] Intra-DU case can be the answer of this question i.e. same configuration of the serving cell can be used for the non-serving cell by implementation.

Question 2: In regard of RRC configuration, RAN1 is discussing whether to allow a UE to be configured for DL reception from or UL transmission to a non-serving cell on UE-dedicated PDSCH, PDCCH, PUSCH, and PUCCH. From RAN2 perspective

- Depending on the answer to question 1-1, what would be the impact of allowing the UE to transmit and/or receive on some or all of those channels and which RRC parameter(s) would need to be reconfigured for the UE?

[Answer 2-1] For PUCCH/PUSCH/PDCCH/PDSCH transmission/reception on non-serving cell, UE needs to know the corresponding configurations (BWP, physical channel configuration, cell group configurations, RACH configuration, etc.).

- Is it feasible to update some of the above RRC parameter(s) via dynamic signaling (e.g. MAC CE and/or DCI, potentially selecting pre-configured values) without any additional RRC reconfiguration signaling?

[Answer 2-2] RAN2 perspective, dynamic signaling with pre-configured RRC configuration for non-serving cell seems feasible but it requires many impacts if RRC configuration of non-serving cells is allowed.

Question 3: In regard of C-RNTI:

- Is there a need to assign a UE a separate C-RNTI for DL reception from and UL transmission to a non-serving cell, or can the same C-RNTI from the serving cell be reused, at least for transmission and reception on UE-dedicated PDSCH, PDCCH, PUSCH, and PUCCH?

[Answer 3-1] In general, C-RNTI would be different between serving cells to identify each cell but it can be assigned the same value by implementation for some cases.

- In restricting the use of the same C-RNTI for serving and non-serving cells, what would be the impact in applicable use cases and/or required specification support, if any?

[Answer 3-2] No specification support is required if the CU is same for the serving and non-serving cells i.e. CU can assign same values, it means that the same RRC configuration is required between serving cell and non-serving cells.

- If separate C-RNTIs are considered necessary in some cases, for serving and non-serving cells, how would this be configured for UE, i.e. is RRC reconfiguration signaling or some other (dynamic) signaling needed for configuring the separate C-RNTI(s)?

[Answer 3-3] RRC configuration is the general approach.

Question 4: In regard of CU-DU split, from RAN2/3 perspective, is there any difference between supporting intra-DU only and supporting inter- in addition to intra-DU, in terms of the following?

- The associated RAN2 specification impact,

[Answer 4-1] RAN2 assumes some RAN2/RAN3 impacts regarding inter-node RRC signaling/F1/F2 interface to support any intra-DU or inter-DU scenario.

- Applicable use cases (e.g. deployment scenarios), and

[Answer 4-2] RAN2 assumed that as many deployment scenarios support this feature in the real deployment (i.e. in general, supporting the limited scenario is not preferred).

- Intra-DU deployment.

- Inter-DU deployment, and each DU is implemented by same gNB vendor.

- Inter-DU deployment , but each DU is implemented by different gNB vendors.

- CU of non-serving cell is different from the CU of the serving cell.

- Network inter-operability (e.g. across different gNB vendors)

[Answer 4-3] Except for the intra-DU deployment, RAN2 expects the network inter-operability issues across different gNB vendors for real deployment scenarios (i.e. inter-node RRC signaling/F1/F2 interface are required).

Question 5: In regard of CA issues, RAN1 is discussing whether the operation is supported only for intra-band CA scenario (i.e. UE is configured to operate with serving and non-serving cells that belong to the same frequency band) or for both intra-band CA and inter-band CA scenarios. Note that one common TCI state ID associated with a non-serving cell, if supported, may be optionally applied for CCs in a band.

- Are there specific RAN2/4 issues (including higher-layer impact) that need to be considered for deciding between the two alternatives?

[Answer 5] RAN2 signaling perspective, no big issues are expected on two alternatives.

Question 6: In regard of inter-frequency issues, from RAN2/4 perspective, what would be the higher-layer and RRM impact assuming inter-frequency scenarios as opposed to intra-frequency scenarios? For intra-frequency scenario, it is assumed that SSBs of non-serving cells have the same center frequency and SCS as the SSBs of the serving cell.

- Note: RAN1 has agreed to support intra-frequency scenarios, whereas the support for inter-frequency scenarios is still for further study.

[Answer 6] RAN2 cannot have answer for this question.

2. Actions:

RAN2 respectfully asks RAN1 to take the above information into account for future work.

3. Date of Next RAN2 Meetings:

TSG RAN WG2   Meeting #114-e            	   May 19 – 27, 2021           Online

TSG RAN WG2   Meeting #115-e            	   TBD