3GPP TSG-RAN WG2 Meeting #113bis-e	R2-2103657

# Electronic Meeting, 12th – 20th April, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 2522 | rev | - | Current version: | 15.13.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the RLC entity release durimg full configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2021-04-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to section 5.3.5.11, when the UE perform full configuration it needs to release all the dedicated radio configuration except the SRB1/SRB2 configurations and DRB configurations as configured by radioBearerConfig or radioBearerConfig2. However, one issue is how to handle the release of the RLC configuration that is not included in radioBearerConfig or radioBearerConfig2. According to how the procedure should work, the UE may release the RLC configuration but it should not release the RLC entity. This basically mean that the UE should not perform and RLC bearer release and add and set the RLC variables to their initial value. But this is not the intended behaviour as the UE has already initialized the RLC entity and applied the RLC default configuration upon re-establishment and resum before to execute the full configuration and does not need to do it again. This is also inline with the latest agreement taken regarding the handling of the SRB1 configuration in the first reconfiguration after reestablishment and during resume: - RAN2 confirm that SRB1 configuration is not required in the RRCResume message and in the first reconfiguration after reestablishment in the case of fullConfig. - RAN2 confirm that SRB1 configuration is not required in the RRCResume message in the first reconfiguration after reestablishment in the case of delta signalling. - If SRB1 is included in the RRCResume or in the first reconfiguration after reestablishment, the reestablishPDCP field is not set to true for SRB1. - If SRB1 is included in the RRCResume or in the first reconfiguration after reestablishment, the reestablishRLC field is not set to true for SRB1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Section 5.3.5.11 - Clarified that upon full configuration the UE needs to release the radio configuration except the SRB1/SRB2 configurations and DRB configurations as configured by radioBearerConfig or radioBearerConfig2, and RLC-BearerConfig. Impact Analysis Impacted 5G architecture options: NR SA Impacted functionality: Full configuration Inter-operability: 1. If the network is implemented according to the CR and the UE is not, the UE may reset the RLC SN to its initial value (=0) whereas the network may believe that the value of the RLC SN is different and thus this will create a misalignement between the UE and the network that the cause the RRC reconfiguration failure. 2. If the UE is implemented according to the CR and the network is not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the CR is not approved, the UE may reset the RLC SN to its initial value (=0) whereas the network may believe that the value of the RLC SN is different and thus this will create a misalignement between the UE and the network that the cause the RRC reconfiguration failure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.5.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

#### 5.3.5.11	Full configuration

The UE shall:

1>	release/ clear all current dedicated radio configurations except for the following:

-	the MCG C-RNTI;

-	the AS security configurations associated with the master key;

NOTE 1:	Radio configuration is not just the resource configuration but includes other configurations like MeasConfig. In case NR-DC or NE-DC is configured, this also includes the entire NR or E-UTRA SCG configuration which are released according to the MR-DC release procedure as specified in 5.3.5.10. The radio configuration does not include SRB1/SRB2 configurations and DRB configurations as configured by radioBearerConfig or radioBearerConfig2, and SRBs configurations as configured by RLC-BearerConfig.

1>	if the spCellConfig in the masterCellGroup includes the reconfigurationWithSync (i.e., SpCell change):

2>	release/ clear all current common radio configurations;

2>	use the default values specified in 9.2.3 for timers T310, T311 and constants N310, N311;

1>	else (full configuration after re-establishment or during RRC resume):

2>	use values for timers T301, T310, T311 and constants N310, N311, as included in ue-TimersAndConstants received in SIB1;

1>	apply the default L1 parameter values as specified in corresponding physical layer specifications except for the following:

-	parameters for which values are provided in SIB1;

1>	apply the default MAC Cell Group configuration as specified in 9.2.2;

1>	for each srb-Identity value included in the srb-ToAddModList (SRB reconfiguration):

2>	apply the default SRB configuration defined in 9.2.1 for the corresponding SRB;

NOTE 2:	This is to get the SRBs (SRB1 and SRB2 for reconfiguration with sync and SRB2 for resume and reconfiguration after re-establishment) to a known state from which the reconfiguration message can do further configuration.

1>	for each pdu-Session that is part of the current UE configuration:

2>	release the SDAP entity (clause 5.1.2 in TS 37.324 [24]);

2>	release each DRB associated to the pdu-Session as specified in 5.3.5.6.4;

NOTE 3:	This will retain the pdu-Session but remove the DRBs including drb-identity of these bearers from the current UE configuration. Setup of the DRBs within the AS is described in clause 5.3.5.6.5 using the new configuration. The pdu-Session acts as the anchor for associating the released and re-setup DRB. In the AS the DRB re-setup is equivalent with a new DRB setup (including new PDCP and logical channel configurations).

1>	for each pdu-Session that is part of the current UE configuration but not added with same pdu-Session in the drb-ToAddModList:

2>	if the procedure was triggered due to reconfiguration with sync:

3>	indicate the release of the user plane resources for the pdu-Session to upper layers after successful reconfiguration with sync;

2>	else:

3>	indicate the release of the user plane resources for the pdu-Session to upper layers immediately;

END OF CHANGES