3GPP TSG-RAN WG2 Meeting #113bis-e	R2-2103937

# Electronic Meeting, 12th – 20th April 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 2542 | rev | - | Current version: | 16.4.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification to BAP address field description in the BAP-RoutingID IE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB-Core |  |  |  |  |  | Date: |  |  | 2021-04-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The BAP-RoutingID IE is adopted to configure the defaultUL-BAP-RoutingID, i.e. to configure the default uplink Routing ID to the IAB-node during IAB-node bootstrapping, migration, IAB-MT RRC resume and IAB-MT RRC re-establishment for F1-C and non-F1 traffic. Hence, it is evident that this BAP-RoutingID IE is only used for the UL, and the bap-Address included in the BAP-RoutingID can only be the BAP address of an IAB donor node, and not the BAP address of other IAB destination nodes, i.e. IAB access nodes. Therefore the current field description of bap-Address within BAP-RoutingID IE is clearly wrong. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR provides a field description clarification to the BAP-Address IE included in the BAP-RoutingID, so that it is clarified that the BAP-Address included in this case can only be the BAP address of the IAB donor node. Impact analysis Impacted functionality: BAP Routing ID configuration for IAB Inter-operability: If the IAB node is implemented according to this CR, but the network is not, there is no inter-operability issue foreseen, since the BAP routing ID applies anyhow only to the UL. If the network is implemented according to this CR, but the IAB node is not, there is no inter-operability issue foreseen, since the BAP routing ID applies anyhow only to the UL. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Field description of the BAP-Address IE included in the UL BAP-RoutingID IE is not correct since it seems that the network can indicate the BAP address of a destination IAB node which is not possible for the upstream traffic. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

### 6.3.2	Radio resource control information elements

<Text Omitted>

#### –	BAP-RoutingID

The IE BAP-RoutingID is used for IAB-node to configure the BAP Routing ID.

BAP-RoutingID information element

-- ASN1START

-- TAG-BAPROUTINGID-START

BAP-RoutingID-r16::=        SEQUENCE{

bap-Address-r16              BIT STRING (SIZE (10)),

bap-PathId-r16               BIT STRING (SIZE (10))

}

-- TAG-BAPROUTINGID-STOP

-- ASN1STOP

BAP-RoutingID field descriptions

bap-Address The ID of an IAB-donor-DU used in the BAP header.

bap-PathId The ID of a path used in the BAP header.

END OF CHANGE