3GPP TSG-RAN WG2 Meeting #113-bis-e	R2-2104471

Electronics Meeting, April 12 – 20, 2021

Agenda item:	6.2.3

Source: 	LG Electronics Inc. (Rapporteur)

Title: 	[AT113-e][710][V2X/SL] Miscellaneous MAC corrections (LG)

Document for:	Discussion and decision

Introduction

This document is to trigger the following email discussion:

[AT113bis-e][705][V2X/SL] Miscellaneous corrections on MAC (LG)

Scope: Discuss R2-2102983, R2-2103091, R2-2103380 and R2-2102995 by taking into account Rapporteur’s suggestions in Table 1 (R2-2104493). Also discuss whether R2-2102812, R2-2103117, R2-2103282, R2-2103296, R2-2103379, R2-2103850, and R2-2104106 are not pursued. Rapporteur may provide more details, e.g. why current spec is ok w/o change or nothing is broken, which are suggested in Table 3 (R2-2104493).

Intended outcome: Agreeable 38.321 CR in R2-2104470 and discussion summary in R2-2104471 if needed.

Deadline: 4/19, 14:00 (UTC).

In this document, Rapporteur propose to discuss the CRs listed above.

Note that the some changes do not require inputs from companies in this document e.g. if the changes are obvious e.g. reference/editorial change or the changes are obviously not needed. For some of those changes, the rapporteur’s proposals were suggested.

Discussion

- R2-2102983 (ZTE, Sanechips)

Question 1A:	Do you agree to reflect the above change in 38.321?

| Company | Yes/No | Comment |
| --- | --- | --- |
| LG | Yes | This note is needed to clarify when MAC PDU carrying CSI reporting MAC CE. For simplicity, we suggest note as below: Note: UE disables HARQ feedback for transmission of a MAC PDU only carrying CSI reporting MAC CE. |
| Samsung | No with comment | We understand the proposed change is to clarify the CSI reporting MAC CE only case but we do not think the note is not related to this subclause “Selection of logical channels” If this clarification is needed, then the change should be reflected in the related subclause e.g., Sidelink HARQ operation. |
| OPPO | Yes | The NOTE is not in the correct style. Besides, to align the wording between NOTE and normative text, one suggestion is Note: sl-HARQ-FeedbackEnabled is set to disabled for the transmission of a MAC PDU only carrying CSI reporting MAC CE. |
| Huawei, HiSilicon | Yes |  |
| ASUSTeK | Yes | Agree with OPPO’s suggestion. |
| CATT | Yes | We agree with the intension and the Note. |
| Fujitsu | Yes |  |
| Apple | Yes | Agree with OPPO’s further revision |
| vivo | Yes | Agree with the intention to capture the agreement, but we are wondering why this note is put in the section of LCP procedure? Would it be more appropriate to add this note in the following section? 5.22.1.3.1 Sidelink HARQ Entity |
| Lenovo | Yes | Agree with OPPO’s suggestion |
| Qualcomm | Yes |  |
| ZTE | Yes | Proponent. For the detailed wording, we can following majority view. |
| Intel | Yes | OPPO’s wording is fine for us |

Summary 1A:

| Answer | Number of supporting companies |
| --- | --- |
| Yes | 12 |
| No | 1 |

Recommendation 1A: the change in R2-2102983 is reflected on 38.321

- R2-2103091 (CATT)

First change in R2-2103091

Question 2A:	Do you agree to reflect the first change?

| Company | Yes/No | Comment |
| --- | --- | --- |
| LG | Yes | It needs to be aligned. |
| Samsung | Yes |  |
| OPPO | Yes |  |
| Huawei, HiSilicon | Yes |  |
| ASUSTeK | Yes |  |
| CATT | Yes |  |
| Fujitsu | Yes |  |
| Apple | Yes |  |
| vivo | Yes |  |
| Lenovo | Yes |  |
| Qualcomm | Yes |  |
| ZTE | Yes |  |
| Intel | Yes |  |

Summary 2A:

| Answer | Number of supporting companies |
| --- | --- |
| Yes | 13 |
| No | 0 |

Recommendation 2A: the first change in R2-2103091 is reflected on 38.321

Second change in R2-2103091

Question 2B:	Do you agree to reflect the second change?

| Company | Yes/No | Comment |
| --- | --- | --- |
| LG | Yes |  |
| Samsung | Yes |  |
| OPPO | Yes |  |
| Huawei, HiSilicon | Yes |  |
| ASUSTeK | No strong view |  |
| CATT | Yes | If we don’t change it, it will cause misunderstanding on the condition of the MAC entity determine the last transmission of one MAC PDU for mode 2. |
| Fujitsu | Yes |  |
| Apple | Yes |  |
| vivo | Yes |  |
| Lenovo | No strong view | We think original text has the same meaning compared with the updated one. But also ok to follow majority view |
| Qualcomm | Yes |  |
| ZTE | Yes |  |
| Intel | Yes |  |

Summary 2B:

| Answer | Number of supporting companies |
| --- | --- |
| Yes | 11 |
| No | 0 |
| No strong view | 2 |

Recommendation 2B: the second change in R2-2103091 is reflected on 38.321

Third change in R2-2103091

Question 2C:	Do you agree to reflect the third change?

| Company | Yes/No | Comment |
| --- | --- | --- |
| LG | No | The expression of latency requirement is used in other section (e.g., CSI reporting). We do not need to limit the usage. Also, if we adopt the change, Note 2 and Note 3 need to be changed. |
| Samsung | Yes | The proposed change looks fine since this if clause refers to the data in a logical channel. In current MAC specification, we find the similar text is used as “the remaining PDB of SL data available in the logical channel(s)” below: “5.22.1.1 SL Grant reception and SCI transmission” 3> randomly select the time and frequency resources for one transmission opportunity from the resources indicated by the physical layer as specified in clause 8.1.4 of TS 38.214 [7], according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) allowed on the carrier, and the latency requirement of the triggered SL CSI reporting; |
| OPPO | See comment | we tend to share the view from LG, on either no change or change with NOTE-2/3 together. But we are neutral on this. |
| Huawei, HiSilicon | Yes | We think the latency requirement is not clear and should be changed to remaining PDB for data. We also support change for NOTE2/3 together. |
| ASUSTeK | Yes | NOTE-2/3 should be changed as well. |
| CATT | Yes | In current spec. latency requirement is always used together with CSI reporting. While the remaining PDB is always used together with data in logical channel, except this one. |
| Fujitsu | No strong view |  |
| Apple | Neutral | This seems an editorial issue, I think there is no misunderstanding in UE implementation. |
| vivo | No strong view | We can go for the majority’s view. |
| Lenovo | Yes | OK update to align with “remaining PDB” phrase in other part of MAC spec. But not OK to change note 3 since it is relates to MAC CE but not data |
| Qualcomm | No | We share the view of OPPO and LG |
| ZTE | No | Share the view with OPPO, LG and Qualcomm |
| Intel | No strong view | We think the current wording is fine as is |

Summary 2C:

| Answer | Number of supporting companies |
| --- | --- |
| Yes | 5 |
| No | 3 |
| No strong view | 5 |

Recommendation 2C: the third change in R2-2103091 is reflected on 38.321

- R2-2103380 (CATT)

First change in R2-2103380

Question 3A:	Do you agree to the changes?

| Company | Yes/No | Comment |
| --- | --- | --- |
| LG | Yes | Only CSI reporting can be considered |
| Samsung | Yes |  |
| OPPO | No | Do not think we want to further complicate this issue, since by changing this to “and/or” one may mis-interpret this as even though both MAC SDU and MAC CE exist, UE only needs to consider the legacy requirement of one but not both.. so tend to keep the existing spec as it is |
| Huawei, HiSilicon | No | We agree with OPPO. |
| ASUSTeK | Yes |  |
| CATT | Yes | If we don’t change it, when discussing the resource selection for SL, the case that there is only SL data available in the logical channel, or only SL CSI reporting are all excluded in logical from the current spec. |
| Fujitsu | Yes |  |
| Apple | See commenets | I agree with the intetion, but the change may be misinterpretated, so the change needs to be: and, if CSI reporting is triggered, the latencty requirements of triggered SL-CSI reporting. |
| vivo | Yes | We think the added ‘or’ is for CSI reporting MAC CE only so it seems ok to us. |
| Lenovo | No | We share the view from OPPO that by adding “or” there may has mis-interpret. Besides, we think current text already cover SL-CSI only case, since latency requirement of SL-CSI is anyway considered and if no data transmission, PDB of dat is not considered naturally |
| Qualcomm | Yes |  |
| ZTE | Yes |  |
| Intel | Yes | We agree with the intention of the change |

Summary 3A:

| Answer | Number of supporting companies |
| --- | --- |
| Yes | 9 |
| No | 3 |

Recommendation 3A: the first change in R2-2103380 is reflected on 38.321

Second change in R2-2103380

Question 3B:	Do you agree to the changes?

| Company | Yes/No | Comment |
| --- | --- | --- |
| LG | Yes | SL-CSI reporting can be considered. |
| Samsung | Yes |  |
| OPPO | Yes with comment | As replied to 3A, the decision on “and/or” issue should be consistent in the spec. |
| Huawei, HiSilicon | Yes with comment | We agree with OPPO. |
| ASUSTeK | See comment | First, we want to clarify the intention of agreement made in RAN2#108: CSI report event shall be cancelled if the CSI report has been transmitted. CSI report is one-shot transmission. - If the agreement implies that one resource (i.e. no blind retransmission resources) is selected for CSI report-only MAC PDU, then this change is not needed. However, the UE can select retransmission number >=1 for CSI report-only MAC PDU according to current specification, so another change is needed for restricting the retransmission number to 0 in this case. - If the agreement implies that multiple resources for a single MAC PDU are selected for CSI report, according to the current specification, the UE selects number of retx allowed in MaxTxTransNumPSSCH (maximum value: 32) for a SL data transmission. If the UE only considers the resources meeting the latency requirement of CSI report (minimum value 3ms) as available resources, this change will force the UE to select quite a number of retransmission resources in a very short time period, which seems unrealistic. Instead of adding additional restriction on resource selection, we think the UE should not continue to retransmit CSI report-only MAC PDU if the latency requirement is not met. |
| CATT | Yes | In the current spec, when performing resource reservation for retransmission, according to the current spec, only “the remaining PDB of SL data available in the logical channel(s) allowed on the carrier” was considered. But in fact, the SL-CSI MAC CE may also be retransmitted. Hence it should also consider the latency requirement of the triggered SL-CSI reporting.That’s our intension for this CR. |
| Fujitsu | Yes |  |
| Apple | Yes with comment | Please see answer in 3A |
| vivo | Yes |  |
| Lenovo | See comments | Please see answer in 3A |
| ZTE | Yes |  |
| Intel | Yes | As commented above, we have to be consistent with wherever we add “and/or” |

Summary 3B:

| Answer | Number of supporting companies |
| --- | --- |
| Yes | 10 |
| No | 0 |

Recommendation 3B: the second change in R2-2103380 is reflected on 38.321

- R2-2102995 (OPPO)

Question 4A:	Do you agree to reflect the first change?

| Company | Yes/No | Comment |
| --- | --- | --- |
| LG | Yes | We need to capture the agreement from RAN1. However, instead of capturing the RAN1 agreement directly, some modification (i.e. better wording) for RAN2 specification is needed. |
| Samsung | Yes |  |
| OPPO | Yes | Proponent |
| Huawei, HiSilicon | Yes |  |
| ASUSTeK | Yes |  |
| CATT | Yes |  |
| Fujitsu | Yes |  |
| Apple | Yes |  |
| vivo | Yes |  |
| Lenovo | Yes |  |
| ZTE | Yes |  |
| Intel | Yes |  |

Summary 4A:

| Answer | Number of supporting companies |
| --- | --- |
| Yes | 13 |
| No | 0 |

Recommendation 4A: R2-2102995 can be agreed

- R2-2102812 (vivo)

Observation 1: in 38.321, the terminology ‘Sidelink process ID’ denotes the ID in SCI, and the terminology ‘HARQ Process ID’ denotes the ID in DCI. Observation 2: in 38.212, there is only the terminology ‘HARQ process number’ both in DCI and SCI. Proposal 1: Send LS to RAN1 to kindly request them to align the terminology of ‘HARQ process number’ in SCI formats with RAN2 specification. Proposal 2: Take the Draft LS provided in the Annex as baseline.

Question 5A:	Do you agree to send an LS to RAN1?

| Company | Yes/No | Comment |
| --- | --- | --- |
| LG | No | We think to align with Sidelink process in 38.212 is up to RAN1 decision. Hence, it is better to submit the CR to RAN1. |
| Samsung | No | Same view as Rapporteur |
| OPPO | No | There are even more important issues on the table, and we understand there is a trend in RAN2 that no LS to RAN1 is preferred.. if that is the case, we do not think this issue is of the first prio for consideration of LS delivery.. |
| Huawei, HiSilicon | No | We don’t think we need to align every terminology in RAN2 with that in RAN1. |
| ASUSTeK | No |  |
| CATT | No |  |
| Fujitsu | No |  |
| Apple | No |  |
| vivo | Yes | Proponent. Although in the proposal we suggest to send an LS to RAN1 to kindly request them to change 38.212, this issue is actually caused by RAN2 because it is RAN2 MAC specification which introduces a new term ‘sidelink process ID’ and is not aligned with 38.212 on SCI format. The reason why we don’t suggest to change MAC specification is the foreseen specification impact as this term is used in a lot of places in 38.321. If we don’t like to change MAC on this term ‘sidelink process ID’ in procedure text, we can add a note to solve the possible misunderstanding. And we can still consider to tell RAN1. |
| Lenovo | No |  |
| Qualcomm | No |  |
| ZTE | No |  |
| Intel | No |  |

Summary 5A:

| Answer | Number of supporting companies |
| --- | --- |
| Yes | 1 |
| No | 12 |

Recommendation 5A: the change in R2-2102821 is not pursued.

- R2-2103117 (Sharp)

Question 6A:	Do you agree to reflect the change?

| Company | Yes/No | Comment |
| --- | --- | --- |
| LG | No | Although the sentence is not needed in MAC specification, there is no technical impact. Moreover, current sentence is helpful to understand the condition of HARQ buffer flushing. But we do not have a strong opinion. |
| Samsung | No | We think that the previous if clause does not cover this “negative only acknowledgement case” so it is better to keep as it is. |
| OPPO | No | if we follow this logic, we tend to believe there should be a change on other places in MAC spec, to remove the NACK-only case, e.g., So far our evaluation is that this issue is not that big, so slightly prefer no change. |
| Huawei, HiSilicon | No | We think there is no issue if we keep the current spec as it is. Even we have some sympathy on the intention, however we think the “negative-only” sentence is more readable and straightforward for people to understand the cases to flush HARQ buffer. |
| ASUSTeK | No | Share the same view with LG. |
| CATT | No |  |
| Fujitsu | No | Agree with LG. |
| Apple | No |  |
| vivo | No | Nothing is broken and we can just keep the original text to be more informative. |
| Lenovo | No | Nothing is broken to keep the text, so we tend to agree keep it as it |
| Qualcomm | No |  |
| ZTE | No |  |
| Intel | No | Does not seem essential to us |

Summary 6A:

| Answer | Number of supporting companies |
| --- | --- |
| Yes | 0 |
| No | 13 |

Recommendation 6A: the change in R2-2103117 is not pursued.

- R2-2103282 (Fujitsu)

Question 7A:	Do you agree to reflect the change?

| Company | Yes/No | Comment |
| --- | --- | --- |
| LG | No | According to the note in BSR section: MAC PDU assembly can happen at any point in time between uplink grant reception and actual transmission of the corresponding MAC PDU. BSR and SR can be triggered after the assembly of a MAC PDU which contains a BSR MAC CE, but before the transmission of this MAC PDU. In addition, BSR and SR can be triggered during MAC PDU assembly. In rapporteur’s perspective, according to current specification, BSR will be triggered after MAC PDU has been built. |
| Samsung | No | We think current texts are fine. |
| OPPO | No | Regardless whether the change is to be applied, seems the sl-bsr generation has nothing to do with the SL mac pdu anyway. So we think the current spec does not bring misunderstanding/error that requires a CR to correct. |
| Huawei, HiSilicon | No | We think the MAC PDU in the sentence refers to Uu MAC PDU. And the “i.e.” part can be interpreted as if there is parallel Uu LCP and SL LCP, then the buffer size field may be zero. So the current specification is correct and does not need any change. |
| ASUSTeK | No |  |
| CATT | No |  |
| Fujitsu | Yes | In our understanding, the word “which” in the bracket “i.e. after the logical channel prioritization procedure, which may result the value of the Buffer Size field to zero” means that the UL LCP/MAC PDU construction may result the value of SL buffer size to zero. However, UL LCP/MAC PDU should have nothing to do with SL buffer size. As an alternative, we can delete the sentence “which may result the value of the Buffer Size field to zero” to make it clear. |
| vivo | No | We understand the intention is to avoid simple copy/paste description from Uu to sidelink, but we understand the original text may not bring much misunderstanding so we can keep it. The MAC PDU should be SL MAC PDU here, in our understanding. |
| Lenovo | No | Share the view from OPPO |
| Qualcomm | No | Agree with rapporteur |
| ZTE | No |  |
| Intel | No | We think the current text is clear |

Summary 7A:

| Answer | Number of supporting companies |
| --- | --- |
| Yes | 1 |
| No | 11 |

Recommendation 7A: the change in R2-2103282 is not pursued.

- R2-2103296 (Samsung)

Question 8A:	Do you agree to reflect the change?

| Company | Yes/No | Comment |
| --- | --- | --- |
| LG | Yes | Yes. It can be merged in rapporteur’s CR. |
| Samsung | Yes | Same view as Rapporteur |
| OPPO | Yes |  |
| Huawei, HiSilicon | Yes |  |
| ASUSTeK | No strong view |  |
| CATT | Yes |  |
| Fujitsu | Yes |  |
| Apple | Yes |  |
| vivo | Yes |  |
| Lenovo | Yes |  |
| Qualcomm | Yes |  |
| ZTE | No strong view |  |
| Intel | Yes |  |

Summary 8A:

| Answer | Number of supporting companies |
| --- | --- |
| Yes | 11 |
| No | 0 |

Recommendation 8A: the change in R2-2103296 is reflected on 38.321

- R2-2103379 (CATT)

Question 9A:	Do you agree to reflect the change?

| Company | Yes/No | Comment |
| --- | --- | --- |
| LG | Yes | Yes. It can be merged in rapporteur’s CR. |
| Samsung | Yes | Same view as Rapporteur |
| OPPO | No with comment | We do not think this is a change that R2 need to pursue, not because it is something wrong (there is indeed some redundancy), but because the root redundancy issue is anyway there and not solved by this CR. In other words, can one clarify: why not make the following change directly in 321, which can remove all the redundancy (including the one raised in 3379) Considering the following definition |
| Huawei, HiSilicon | No strong view | We are fine to follow the majority. |
| CATT | Yes | Besides redundancy, the current spec may cause some unexpected result. In detail, the “or” in bullet 3 will cause the bullet 6 invalid, this will cause unexpected result. For example, UE able to send both UL and NR SL (not V2X SL), but V2X SL is prioritized -For bullet 3,UL and NR SL will be transmitted -For bullet 6, UL and V2X SL should be transmitted Can this difference be acceptable? |
| Fujitsu | Yes |  |
| Apple | Yes | We are also open for OPPO’s suggestion to reduce redunducy further in this section |
| vivo | Yes |  |
| Lenovo | No strong view | We are open to discuss the redundancy issue for UL/SL prioritization condition thoroughly |
| Qualcomm | Yes |  |
| ZTE | No strong view |  |
| Intel | No strong view | We are fine to follow the majority |

Summary 9A:

| Answer | Number of supporting companies |
| --- | --- |
| Yes | 6 |
| No | 1 |
| No strong view | 4 |

Recommendation 9A: the change in R2-2103379 is reflected on 38.321

- R2-2103850 (Apple)

[Apple]: The intention of the CR is to limit the counting of sl-reselectAfter only on the first transmission opportunity unused/skipped, not considering retransmission opportunities skipped, so, it is equivalent to the following simple change:

Companies may also share some view on  whether the simpler change above is acceptable... Thanks!

Question 10A:	Do you agree to reflect the change?

| Company | Yes/No | Comment |
| --- | --- | --- |
| LG | No | From Rapporteur perspective, the proposed change is not critical remedy at this late timing of maintenance phase. To be specific, RAN1 already made the agreement that the number of resources cleared/released by ACK reception is not counted in CR evaluation. This means that the cleared/released resource is assumed as “not reserved resources” in Mode 2. Rapporteur think that we can keep this principle, and further optimization doesn’t need to be introduced. |
| Samsung | No with comment | We have sympathy on the issue. But we share the Rapporteur view that mode 2 can work without this change and this is an optimization. |
| OPPO | Yes for the revised change proposed by apple below | This is such a big change that may not be preferred at late stage.. [OPPO2] Yet for the revised version, we think it is feasible (actually, after some further check internally, we understand it would difficulty to take re-transmission into account for this counter in NR-V2X, considering all the features of PSFCH, preemption/re-evaluation and so on), which is probably the easiest way-out. |
| Huawei, HiSilicon | No with comment | We tend to agree with the intention. However, we think the proposed change is not that straightforward and we propose to change like below 1> if sl-ReselectAfter is configured and the number of consecutive unused new transmission opportunities on resources indicated in the selected sidelink grant is equal to sl-ReselectAfter; or |
| ASUSTeK | No strong view | Can follow majority. |
| CATT | See comments | It can be left to gNB implementation. |
| Fujitsu | No strong opinion |  |
| Apple | Yes(proponent) | For rapporteur’s comment, I think the usage of this parameter has nothing to do with CR evaluation and needs to be crystally clear in the MAC spec. We are fine to further improve the text, suh as Huawei’s suggestion. Maybe we can have a simple change as because “initial transmission opportunies” are the tems used in MAC spec for SL resource reservation: 1> if sl-ReselectAfter is configured and the number of consecutive unused initial transmission opportunities on resources indicated in the selected sidelink grant is equal to sl-ReselectAfter; or |
| vivo | No | The intention is OK but the original change is totally new feature which is obvious optimization. For Huawei’s suggestion, we can go for the majority view. |
| Lenovo | Yes | We agree the intention is to count initial transmission opportunities. We are fine with Apple’s simple change proposal in comments |
| Qualcomm | No | We appreciate the intention of this CR, but tend to agree with the comments from LG and Samsung. |
| ZTE | No | Do not see the benefits to adopt this CR, nothing is broken. |
| Intel | No | We share the view with Vivo that while it is an interesting aspect to consider, it seems like a new optimization and does not seem essential at this stage |

Summary 10A:

| Answer | Number of supporting companies |
| --- | --- |
| Yes | 4 (with revision) |
| No | 6 |
| No strong view | 3 |

Recommendation 10A: the change in R2-2103850 is not pursued.

- R2-2104106 (Huawei)

Question 11A:	Do you agree to reflect the change?

| Company | Yes/No | Comment |
| --- | --- | --- |
| LG | No | According the current specification, in terms of instructing HARQ combing operation at PHY layer, RX UE considers “received TB” as “retransmission” only when TB has not yet been successfully decoded. Also the RX UE can use only “received SCI information (e.g., NDI, SL HP ID, L1 ID)” to determine whether “received TB” is “retransmission” of the TB already has been successfully decoded and it transmits ACK to TX UE. As a result, Rapporteur thinks that further modification is not needed. Reply to LG: in MAC spec, the UE judges whether a transmission is a new transmission or retransmission based on whether NDI is toggled or not insteading of basing on “TB has not yet been successfully decoded”. So it is possible that the RX UE has already decoded successfully and allocate the SL process to handle the other TB, but then receives a retansmission corresponding to the same SL process. In this case, how the UE treats the retransmission without an assocaited SL HARQ process is not clear. |
| Samsung | No | We wonder these two if-s can coexist. Does it mean that UE keep NDI value of Sidelink identification information of previous received transmission without sidelink process of the Sidelink identification of previous received transmission? “if the NDI has not been toggled compared to the value of the previous received transmission corresponding to the Sidelink identification information and the Sidelink process ID of the SCI, and if there is no Sidelink process associated with the Sidelink identification information and the Sidelink process ID of the SCI” Reply to Samsung: Yes. We think these two if are separate and can co-exist. The RX UE should anyway keep NDI value of Sidelink identification information and the Sidelink process ID of the SCI to judge whether the next transmission is new transmission or retransmission and the Sidelink process ID is allocated by TX UE. While for the second if it means there is no SL process (which is allocated by the RX UE) associated with the Sidelink identification information and the Sidelink process ID of the SCI. So these two if has nothing to do with each other and can co-exist. |
| OPPO | No | We are not sure the intention of this proposal change – in 4106: “However, according to current specification, if the data for a TB was successfully decoded before, the RX UE will consider the SL process as unoccupied and may associate this SL process to the other TB. In this case, it is not clear how to handle this redundant retransmission without an ssociated SL process.” Why there is a problem? No matter whether the old process is used for other new data or is kept as it is, it seems OK? Since only the data with toggled-NDI will trigger new process association And the addition seems to say if the old process has been used for another new process, the redundant data still needs a new process to carry it? That we hold different view.. Reply to OPPO: the point here is that the NDI is not toggled (retransmission received) and no new process association is triggered. In this case, since the old process is used for other TB reception, then for this retransmission, there is no associated SL process. Even the RX UE has decoded successfully, however how to handle this retransmission is missing in current spec. Our suggestion is to allocate a new process to carry this retransmission. |
| Huawei, HiSilicon | Yes | It is possible that the RX UE may receive a retransmission of a TB even if it decoded the TB successfully before, e.g., for groupcast, if positive-negative acknowledgement is enabled and some UEs within the group replied NACK while the other UEs replied ACK, then TX UE will retransmit this packet and for the UEs providing ACK, this retransmission is received even if the packet has been successfully decoded before. However, according to current specification, if the data for a TB was successfully decoded before, the RX UE will consider the SL process as unoccupied and may associate this SL process to the other TB. In this case, if the old process has been used for the other TB, there is no associated SL process for this retransmission and how the UE handles this retransmission is missing in the spec. Our suggestion is to allocate a new process to carry this retransmission. [Huawei, HiSilicon 2] based on the feedback, we think companies agree there is such a case, however they think the proposed correction is not acceptable at this stage. As a compromise, we are fine to add a note e.g. the RX UE drop the TB if the NDI has not been toggled compared to the value of the previous received transmission corresponding to the Sidelink identification information and the Sidelink process ID of the SCI, and if there is no Sidelink process associated with the Sidelink identification information and the Sidelink process ID of the SCI |
| CATT | See comments | We agree with the intension of the case. As a compromise, a note can be considered added to further clarification the case. |
| Fujitsu | Yes |  |
| Apple | Yes with commet | I agree with the intention. But the change force UE to evaluate a “NDI not toggled” case and associate it with a unused SL process. It is not clear what “NDI” is used here. The old decoded TB is no longer in MAC context, and its associated NDI in the SL process is also no longer remembered. If the SL process associate with a new TB in a different HARQ process, then How a UE compare with new NDI associated with different HARQ process to a no longer existing old NDI? IN my view, this better could be carried as a NOTE to describe the corner case can be left to UE implementation. Reply to Apple: We think even the old TB is successfully decoded, the “NDI” still needs to be maintained to judge whether the next transmission associated with the Sidelink identification information and the Sidelink process ID (allocated by TX UE) of SCI is a new transmission or a retransmission. This maintenance has nothing to do with whether the RX UE associate the SL process (a SL process to handle the reception and allocate by the RX UE) with a new TB or not. |
| vivo | No | We tend to agree with the case that ‘if the data for a TB was successfully decoded before, the RX UE will consider the SL process as unoccupied and may associate this SL process to the other TB’, but then if a retransmission is received it can be left to UE implementation (e.g. to delete that retransmission TB). The procedure text modification at this late stage is not preferred. |
| Lenovo | No | We are not sure why Rx UE need to allocate a SL process for a redundant data in unexpected retransmission, since the data is already been correctly decoded and delivery to higher layer, the redundant data seems useless for Rx UE |
| Qualcomm | No | Agree with the rapporteur |
| ZTE | No | This CR is not needed. if there is no Sidelink process associated with the Sidelink identification information and the Sidelink process ID of the SCI, how UE knows the NDI is toggled or not? For this case, we think UE will identify the transmission as new transmission, current spec works well. |
| Intel | See comment | This seems like an interesting issue; we wonder if adding a note to handle the particular scenario in question can be simpler (as mentioned by Huawei) |

Summary 11A:

| Answer | Number of supporting companies |
| --- | --- |
| Yes | 5 (with revision) |
| No | 7 |

Recommendation 11A: the change in R2-2104106 is not pursued.

Conclusion and recommendation

In conclusion, Rapporteur proposes the following recommendations as the outcome of this email discussion:

Recommendation 1A: the change in R2-2102983 is reflected on 38.321

Recommendation 2A: the first change in R2-2103091 is reflected on 38.321

Recommendation 2B: the second change in R2-2103091 is reflected on 38.321

Recommendation 2C: the third change in R2-2103091 is reflected on 38.321

Recommendation 3A: the first change in R2-2103380 is reflected on 38.321

Recommendation 3B: the second change in R2-2103380 is reflected on 38.321

Recommendation 4A: R2-2102995 can be agreed

Recommendation 5A: the change in R2-2102821 is not pursued.

Recommendation 6A: the change in R2-2103117 is not pursued.

Recommendation 7A: the change in R2-2103282 is not pursued.

Recommendation 8A: the change in R2-2103296 is reflected on 38.321

Recommendation 9A: the change in R2-2103379 is reflected on 38.321

Recommendation 10A: the change in R2-2103850 is not pursued.

Recommendation 11A: the change in R2-2104106 is not pursued.