3GPP TSG-RAN WG2 Meeting #114-e	R2-2105185

# Online, 19th-27th May, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0371 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on descriptions of PDCCH features |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IIOT-Core |  |  |  |  |  | Date: |  |  | 2021-05-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In RAN1#104bis-e meeting, RAN1 has endorsed an update to TS 38.300 to add missing description of PDCCH features that have been introduced as part of the NR-U work item in Release 16: Reason for change: The description of PDCCH features added in Release 16 for shared spectrum operation are missing. Summary of change: Add description of the following PDCCH features: - Indicating Available RB set - Indicating COT duration - Trigger search space set group switching - Indicating CG-DFI indication - Triggering one-shot HARQ codebook feedback Consequences if not approved: Incomplete description of PDCCH features. Clauses affected: 5.2.3 Text proposal: #### 5.2.3 Physical downlink control channels The Physical Downlink Control Channel (PDCCH) can be used to schedule DL transmissions on PDSCH and UL transmissions on PUSCH, where the Downlink Control Information (DCI) on PDCCH includes: - Downlink assignments containing at least modulation and coding format, resource allocation, and hybrid-ARQ information related to DL-SCH; - Uplink scheduling grants containing at least modulation and coding format, resource allocation, and hybrid-ARQ information related to UL-SCH. In addition to scheduling, PDCCH can be used to for - Activation and deactivation of configured PUSCH transmission with configured grant; - Activation and deactivation of PDSCH semi-persistent transmission; - Notifying one or more UEs of the slot format; - Notifying one or more UEs of the PRB(s) and OFDM symbol(s) where the UE may assume no transmission is intended for the UE; - Transmission of TPC commands for PUCCH and PUSCH; - Transmission of one or more TPC commands for SRS transmissions by one or more UEs; - Switching a UE's active bandwidth part; - Initiating a random access procedure; - Indicating the UE(s) to monitor the PDCCH during the next occurrence of the DRX on-duration; - In IAB context, indicating the availability for soft symbols of an IAB-DU; - Triggering one shot HARQ-ACK codebook feedback; - Notifying one or more UEs of the available RB sets, COT duration and search space set group switching for shared spectrum operation; - Indicating downlink feedback information for configured grant PUSCH (CG-DFI) for shared spectrum operation. A UE monitors a set of PDCCH candidates in the configured monitoring occasions in one or more configured COntrol REsource SETs (CORESETs) according to the corresponding search space configurations. A CORESET consists of a set of PRBs with a time duration of 1 to 3 OFDM symbols. The resource units Resource Element Groups (REGs) and Control Channel Elements (CCEs) are defined within a CORESET with each CCE consisting a set of REGs. Control channels are formed by aggregation of CCE. Different code rates for the control channels are realized by aggregating different number of CCE. Interleaved and non-interleaved CCE-to-REG mapping are supported in a CORESET. Polar coding is used for PDCCH. Each resource element group carrying PDCCH carries its own DMRS. QPSK modulation is used for PDCCH.. |
| Summary of change: |  | In subcaluse 5.2.3, adding the following descriptions of PDCCH features for operation with shared spectrum, with improvements on RAN1 TP: - Triggering one shot HARQ-ACK codebook feedback; - Notifying one or more UEs of the available RB sets, COT duration and search space set group switching for operation with shared spectrum; - Indicating downlink feedback information for configured grant PUSCH (CG-DFI) for operation with shared spectrum. Impact analysis Impacted 5G architecture options: NR-SA, MR-DC Impacted functionality: PDCCH features for shared spectrum operation Inter-operability: If the UE is implemented according to this CR while the network is not, there is no inter-operability issue. If the network is implemented according to this CR while the UE is not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete description of PDCCH features in TS 38.300. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

### 5.2.3	Physical downlink control channels

The Physical Downlink Control Channel (PDCCH) can be used to schedule DL transmissions on PDSCH and UL transmissions on PUSCH, where the Downlink Control Information (DCI) on PDCCH includes:

-	Downlink assignments containing at least modulation and coding format, resource allocation, and hybrid-ARQ information related to DL-SCH;

-	Uplink scheduling grants containing at least modulation and coding format, resource allocation, and hybrid-ARQ information related to UL-SCH.

In addition to scheduling, PDCCH can be used to for

-	Activation and deactivation of configured PUSCH transmission with configured grant;

-	Activation and deactivation of PDSCH semi-persistent transmission;

-	Notifying one or more UEs of the slot format;

-	Notifying one or more UEs of the PRB(s) and OFDM symbol(s) where the UE may assume no transmission is intended for the UE;

-	Transmission of TPC commands for PUCCH and PUSCH;

-	Transmission of one or more TPC commands for SRS transmissions by one or more UEs;

-	Switching a UE's active bandwidth part;

-	Initiating a random access procedure;

-	Indicating the UE(s) to monitor the PDCCH during the next occurrence of the DRX on-duration;

-	In IAB context, indicating the availability for soft symbols of an IAB-DU;

-	Triggering one shot HARQ-ACK codebook feedback;

-	Notifying one or more UEs of the available RB sets, COT duration and search space set group switching for operation with shared spectrum;

-	Indicating downlink feedback information for configured grant PUSCH (CG-DFI) for operation with shared spectrum.

A UE monitors a set of PDCCH candidates in the configured monitoring occasions in one or more configured COntrol REsource SETs (CORESETs) according to the corresponding search space configurations.

A CORESET consists of a set of PRBs with a time duration of 1 to 3 OFDM symbols. The resource units Resource Element Groups (REGs) and Control Channel Elements (CCEs) are defined within a CORESET with each CCE consisting a set of REGs. Control channels are formed by aggregation of CCE. Different code rates for the control channels are realized by aggregating different number of CCE. Interleaved and non-interleaved CCE-to-REG mapping are supported in a CORESET.

Polar coding is used for PDCCH.

Each resource element group carrying PDCCH carries its own DMRS.

QPSK modulation is used for PDCCH.

END OF CHANGE