3GPP TSG-RAN WG2 Meeting #114 electronic             	R2-2105349

# E-Meeting, 19th May – 27th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 4659 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on priority of LTE PSSS/SSSS/PSBCH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2021-05-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the scenario of LTE V2X and NR sidelink co-existance, the UE should know the priority of LTE V2X transmission/recepetion and NR sidelink transmission/recepetion to priortize one of them. When it comes to synchronization signal, it means the UE should know the priority of LTE PSSS/SSSS/PSBCH. During RAN2 #109-e meeting, there is a CR (R2-2001967) for introduction of 5G V2X with NR Sidelink in TS 36.331. The IE sl-SSB-PriorityEUTRA-r16 was introduced as follows: RRCConnectionReconfiguration-v1610-IEs ::= SEQUENCE { conditionalReconfiguration-r16 ConditionalReconfiguration-r16 OPTIONAL, -- Need ON daps-SourceRelease-r16 ENUMERATED{true} OPTIONAL, -- Need ON tdm-PatternConfig2-r16 TDM-PatternConfig-r15 OPTIONAL, -- Need ON sl-ConfigDedicatedForNR-r16 OCTET STRING OPTIONAL, -- Need OR sl-SSB-PriorityEUTRA-r16 INTEGER (1..8) OPTIONAL, -- Need OR nonCriticalExtension SEQUENCE {} OPTIONAL } However, as sl-SSB-PriorityEUTRA-r16 was only put in RRCConnectionReconfiguration, there is no means for RRC_IDLE UE who camps on an LTE cell or an NR cell (because E-UTRA SIB 21/26 are included in NR SIB 13/14) to acquire this parameter, and therefore the UE will not know the priority of LTE PSSS/SSSS/PSBCH transmission and reception. It is not reasonable to have this limitation without agreement, and it can have impacts on UE behaviour e.g. the UE cannot perform the priority comparison between LTE PSSS/SSSS/PSBCH and NR sidelink transmissions/receptions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | sl-SSB-PriorityEUTRA-r16 is added in SystemInformationBlockType26. Impact analysis Impacted functionality: LTE V2X, NR SL Inter-operability: - If UE implements according to the CR and the network does not, the UE in RRC_IDLE in LTE/NR cell will always regard the priority configuration of LTE sidelink SSB as absent. - If the network implements according to the CR and the UE does not, the UE cannot understand the configuration of LTE PSSS/SSSS/PSBCH priority. - If one UE is implemented according to this CR while the other UE is not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RRC_IDLE UE who camps on an LTE cell or NR cell cannot know the LTE PSSS/SSSS/PSBCH priority and cannot perform priority comparison between LTE PSSS/SSSS/PSBCH and other transmissions. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------------------------------------------------[Start of Change] ------------------------------------------------

#### –	SystemInformationBlockType26

The IE SystemInformationBlockType26 contains V2X sidelink communication configurations which can be used jointly with those included in SystemInformationBlockType21.

SystemInformationBlockType26 information element

-- ASN1START

SystemInformationBlockType26-r15 ::= SEQUENCE {

v2x-InterFreqInfoList-r15			SL-InterFreqInfoListV2X-r14			OPTIONAL,	-- Need OR

cbr-pssch-TxConfigList-r15			SL-CBR-PPPP-TxConfigList-r15		OPTIONAL,	-- Need OR

v2x-PacketDuplicationConfig-r15		SL-V2X-PacketDuplicationConfig-r15	OPTIONAL,	-- Need OR

syncFreqList-r15					SL-V2X-SyncFreqList-r15				OPTIONAL,	-- Need OR

slss-TxMultiFreq-r15				ENUMERATED{true}					OPTIONAL,	-- Need OR

v2x-FreqSelectionConfigList-r15		SL-V2X-FreqSelectionConfigList-r15	OPTIONAL,	-- Need OR

threshS-RSSI-CBR-r15				INTEGER (0..45)						OPTIONAL,	-- Need OR

...,

lateNonCriticalExtension			SystemInformationBlockType26-v16xy						OPTIONAL

}

SystemInformationBlockType26-v16xy-IEs ::= SEQUENCE {

sl-SSB-PriorityEUTRA-r16					INTEGER (1..8)					OPTIONAL, -- Need OR

nonCriticalExtension						SEQUENCE {}						OPTIONAL

}

-- ASN1STOP

--------------------------------------------------[End of Change] ---------------------------------------------