3GPP TSG-RAN WG2 Meeting #114-e	R2-2105579

E-meeting, 19th – 27th May, 2021

Agenda item:	8.1.2.3

Source: 	Huawei, CBN, HiSilicon

Title: 	Service continuity during inter-cell mobility

Document for:	Discussion and Decision

# Introduction

In previous meetings, service continuity during inter-cell mobility has been discussed and some progress has been made.

During RAN2#111-e meeting, the following agreements were achieved:

Agreements: - Focus on MBS-MBS scenario initially (i.e. shared delivery), including both PTM and PTP (if applicable). Other scenarios later, TBD. - Requirements for lossless mobility are TBD. Assume for now that R2 will anyway discuss service continuity functionality for low or no data loss. - R2 assumes that for Rel-17 NR multicast Mobility in Connected mode, handover (including variants) is the baseline, TBD exactly which variants.

During RAN2#112-e meeting, the following agreements were achieved:

Agreements: - R2 aim to support lossless handover for MBS-MBS mobility for service that requires this (TBD which detailed scenario but at least PTP-PTP) - In order to support the lossless handover for 5G MBS services, at least DL PDCP SN synchronization and continuity between the source cell and the target cell should be guaranteed by the network side to realize. The design of specific approach to realize this can be involved with WG RAN3. - From network side, the source gNB may forward the data to the target gNB and the target gNB will deliver the forwarding data. Meanwhile, the SN STATUS TRANSFER should be extended to cover the PDCP SN for MBS data; Then (TBD after or in parallel) the UE receives the MBS in the target cell by the target cell according to target configuration. - From UE side, PDCP status report may be supported as well.

In RAN3#110e meeting, the mobility issue was further discussed and the following agreements were made:

Agreements: - Xn Handover Request and NG Handover Request message contain MBS context information for the UE. - MBS context information within the UE context shall contain all MBS multicast session information the UE has joined. - The MBS configuration decided at target gNB is sent to the UE via the source gNB (details e.g. RRC container etc. pending RAN2 progress).

In this contribution, we will discuss the following aspects:

- Mobility from MBS supporting gNB to MBS supporting gNB.

- Mobility from MBS supporting gNB to MBS non-supporting gNB

- Mobility from MBS non-supporting gNB to MBS supporting gNB

# Discussion

## 2.1 Mobility between MBS supporting gNBs

For mobility between MBS supporting gNBs, the target MBS configuration can be contained in the RRC container (i.e. the RRCReconfiguration message) and delivered to the UE via the handover command. In this case, after switching to the target gNB, the UE can continue receiving the MBS service.

However, there may be a progress gap between gNBs which may lead to plenty of data loss. In order to achieve lossless handover, RAN2 has agreed to support DL PDCP SN synchronization and data forwarding. For the moment, RAN2 targets at least PTP to PTP scenario, but as far as we are concerned, there is no need to limit lossless handover to this scenario only. According to the previous RAN2 agreement, “Focus on MBS-MBS scenario initially (i.e. shared delivery), including both PTM and PTP (if applicable). Other scenarios later, TBD”, we should focus on MBS bearers configured with both PTM and PTP instead of PTP-only. There is no additional complexity to support lossless handover between the cells if the UE is configured with a PTM leg in addition to the PTP leg.

Observation 1: There is no additional complexity in supporting lossless handover for a UE configured with both PTM and PTP legs compared with PTP leg only.

It should be noted that the scenario where the UE is configured with both PTP and PTM legs is very important as it allows the target gNB to deliver the data forwarded from the source cell while simultaneously providing new data over PTM leg. Based on this, we propose the following:

Proposal 1: Lossless handover can be supported during mobility as long as at least PTP leg is configured at the target gNB.

After the handover, the forwarded data should be sent to the UE without affecting the PTM transmission in the target. Therefore the packets forwarded from the source should be dedicatedly delivered via the PTP leg, while the packets from CN can be sent to UE via either PTP or PTM.

Proposal 2: The forwarded data should be delivered to the UE via PTP transmission after handover.

## 2.2 Mobility from MBS supporting gNB to MBS non-supporting gNB

For scenarios where MBS is not supported in the target gNB, SA2 has achieved the following conclusions in TS23.247:

During the mobility from a NG-RAN node that supports MBS to a target NG-RAN node that supports MBS, or from a NG-RAN node that supports MBS to a target NG-RAN node that does not support MBS, minimization of data loss should be supported. To support Handover from NG-RAN node that supports MBS to a target NG-RAN node that does not support MBS: - mapping information about unicast QoS flows and the associated multicast QoS flows are provided to the NG-RAN node. This is already performed during the PDU session modification procedure for the PDU session associated with the MBS session when the UE Joins into the MBS Session; - during the handover procedure, the delivery method is switched from 5GC Shared MBS traffic delivery method to 5GC Individual MBS traffic delivery method, i.e. the N3 tunnel of the PDU Session for 5GC Individual MBS traffic delivery needs to be activated towards the target RAN node. The SMF realizes that the target NG-RAN node does not support 5GC Shared MBS traffic delivery method. - the SMF and the MB-SMF shall also activate the GTP tunnel between the UPF and the MB-UPF for 5GC Individual MBS traffic delivery method, if needed.

Besides, in the LS from RAN3 [1], the following feedback has been given: “for Xn-based handover from MBS supporting node to non-MBS supporting node, it is assumed that switch from shared delivery to individual delivery will be take place during the path switch procedure.”

During RAN2#113e meeting, the following agreement was made based on [2]:

- [037] RAN2 assumes that from RAN2 perspective, mobility from the source gNB supporting MBS to target gNB not supporting MBS can be achieved by switching the traffic from delivery via MRB to delivery via DRB either before or during the handover. Whether and how this can be done without data losses has to be further investigated and requires progress and input from other WGs, i.e. RAN3 and SA2.

To be specific, there are two options for mobility to MBS non-supporting gNB:

Option 1: Reconfigure MRB to DRB before performing legacy handover procedure.

In this case, RAN can reconfigure the MRB to DRB for data transmission. During the reconfiguration, RAN2 can further consider how to ensure no data loss for MBS service with high reliability requirement. One possible way is to support shared delivery data to be transmitted via DRB as a transient state after the reconfiguration is done, but before the handover. After MRB to DRB conversion, the legacy data lossless handover is performed to the target gNB that does not support MBS.

Option 2: Convert MRB to DRB via full configuration during handover.

The source gNB initiates MBS handover by sending MBS context in the HANDOVER REQUEST message. The target does not recognize MBS context and thus may initiate full-config at the UE. After the handover, the UE uses the DRB established according to the full-config to receive data from the target gNB. The full-config procedure will result in data loss which is not acceptable for MBS services with high reliability.

Proposal 3: For services requiring lossless handover, the mobility from MBS gNB to non-MBS gNB is done in the following way:

- MRB is reconfigured to DRB at the beginning of the handover procedure.

- Shared delivery data is transmitted via DRB in a source node as a transient state.

Proposal 4: For services not requiring lossless handover, for mobility from MBS gNB to non-MBS gNB, MRB can be converted to DRB via full configuration.

## 2.3 Mobility from MBS non-supporting gNB to MBS supporting gNB

For the scenario of mobility from MBS non-supporting gNB to MBS supporting gNB, SA2 has achieved the following conclusions in TS 23.247:

To support Handover from a NG-RAN node that does not support MBS to a target NG-RAN node that supports MBS: - The PDU sessions, including the one associated with the MBS session and used for 5GC Individual MBS traffic delivery, are handed over to target RAN. - the 5GC terminates the 5GC Individual MBS traffic delivery method after handover procedure.

From RAN2 point of view, it is straightforward to perform legacy handover to the target gNB first. And then it is up to the CN to switch the traffic delivery through unicast PDU session to traffic delivery through the MBS session. RAN2 can further discuss whether data lossless handover should be ensured for MBS service with high reliability requirement during this procedure.

Proposal 5: For mobility from non-MBS to MBS, legacy handover can be performed before unicast is converted to MBS in the target.

# Conclusion

In this contribution, we discussed service continuity during MBS mobility and the following observations and proposals are provided:

Observation 1: There is no additional complexity in supporting lossless handover for a UE configured with both PTM and PTP legs compared with PTP leg only.

Proposal 1: Lossless handover can be supported during mobility as long as at least PTP leg is configured at the target gNB.

Proposal 2: The forwarded data should be delivered to the UE via PTP transmission after handover.

Proposal 3: For services requiring lossless handover, the mobility from MBS gNB to non-MBS gNB is done in the following way:

- MRB is reconfigured to DRB at the beginning of the handover procedure.

- Shared delivery data is transmitted via DRB in a source node as a transient state.

Proposal 4: For services not requiring lossless handover, for mobility from MBS gNB to non-MBS gNB, MRB can be converted to DRB via full configuration.

Proposal 5: For mobility from non-MBS to MBS, legacy handover can be performed before unicast is converted to MBS in the target.

# Reference

- R3-211296   Reply LS on 5MBS progress and issues to address   RAN3

- R2-2102482  Report of [Offline-037][MBS] MBS General (Huawei)