3GPP TSG-RAN WG2 Meeting #114 electronic   						    	R2-2105724

E-Meeting, 19th May – 27th May, 2021

Source: 	Xiaomi Communications

Title:  	Remaining issues of CG harmonization

Agenda Item:	8.1.3

Document for:	Discussion and Decision

# Introduction

According to the discussion on the CG harmonization in the previous RAN2 meetings, RAN2 made the following agreements:

RAN2#112-e agreements [1]: From RAN2 perspective - It is assumed that LBT failures only happen infrequently in UCE (unlicensed controlled environment). A formal definition of UCE and its relationship to semi-static or dynamic access mode is not necessary in RAN2 specifications. - cg-RetransmissionTimer can be configured optionally for shared spectrum - When cg-RetransmissionTimer is configured, Rel-16 NR-U mechanism is used for HARQ process ID and RV selection. - When cg-RetransmissionTimer is not configured, Rel-16 URLLC mechanism may be used for HARQ process ID and RV selection. - As a baseline, HARQ processes sharing between multiple CGs are allowed when cg-RetransmissionTimer is configured as in Rel-16 NR-U. - HARQ processes sharing between multiple CGs are not allowed when cg-RetransmissionTimer is not configured. - The assumption for Rel-16 is that the network will not configure autonomousTx and cg-RetransmissionTimer simultaneously per cell. No optimizations will be pursued to allow the two features be configured together in Rel-16. No CR is needed for this for now. - If a configured grant is deprioritized and/or gNB didn’t get it (e.g. LBT failure and/or tx failure) then we should be able to autonomously re-transmit it. FFS how to achieve it (using existing mechanisms should be considered as baseline)

RAN2#113-e agreements [2]: - LCH based prioritization and cg-RetransmissionTimer can be configured together in Rel-17 (consensus) - Option 1: AutoTx and CGRT are responsible for deprioritized MAC PDU and LBT-failed MAC PDU, respectively. - If CGRT is not configured, LBT-failed MAC PDU is not retransmitted. If AutoTx is not configured, deprioritized MAC PDU is not retransmitted. - The MAC entity stops cg-RetransmissionTimer when the CG resource associated with the timer is deprioritized due to LCH-based prioritization. - LBT failure is not considered when determining a grant priority for intra-UE prioritization (17/22)

According to the email discussion [3] on the URLLC in UCE, the UE behaviours regarding the different configuration combinations (i.e. whether autoTx or CGRT is configured or not) are discussed.

# Discussion

## Configurations of autoTx and CGRT

According to [3], RAN2 discussed the UE behaviors of the following scenarios:

- Scenario 1: UL grant is prioritized and LBT fails

  - Case 1.1: autonomousTx is configured and cg-RetransmissionTimer is not configured.

  - Case 1.2: autonomousTx is not configured and cg-RetransmissionTimer is configured.

  - Case 1.3: autonomousTx and cg-RetransmissionTimer are configured.

- Scenario 2: UL grant is de-prioritized

  - Case 2.1: autonomousTx is configured and cg-RetransmissionTimer is not configured.

  - Case 2.2: autonomousTx is not configured and cg-RetransmissionTimer is configured.

  - Case 2.3: autonomousTx and cg-RetransmissionTimer are configured.

However, the configuration that autonomousTx is not configured and cg-RetransmissionTimer is configured seems not a valid configuration. When the gNB configures cg-RetransmissionTimer, this means that the gNB requires the UE to perform the retransmission of the MAC PDU due to the LBT failure. When both cg-RetransmissionTimer and lch-basedPrioritization are configured, if the autonomousTx is not configured, this means that the gNB only requires the UE to autonomously retransmit the MAC PDU only due to the LBT failure, but not due to the de-prioritization of the MAC PDU. However, many companies consider that the UE should retransmit the deprioritized MAC PDU by using the cg-RetransmissionTimer. Namely, when the cg-RetransmissionTimer expires, the UE retransmits the deprioritized MAC PDU. This is also clear why the UE which can immediately retransmit the MAC PDU would wait until the expiry of the cg-RetransmissionTimer. Thus, to allow the immediate MAC PDU retransmission and to avoid the extra UE complexity on some invalid configurations, we think that when both cg-RetransmissionTimer and lch-basedPrioritization are configured, autonomousTx is always configured.

Proposal 1: When both cg-RetransmissionTimer and lch-basedPrioritization are configured, autonomousTx is always configured.

## De-prioritized autonomous retransmission

According to [3], most companies consider that when cg-RetransmissionTimer and lch-basedPrioritization are configured, for overlapping CGs, the MAC entity prioritizes the initial transmission of higher priority data over autonomous retransmission of lower priority data. Then we could have the following two ways to retransmit the de-prioritized MAC PDU:

- Option 1: The UE (re-)starts cg-RetransmissionTimer (i.e. the UE retransmits the MAC PDU when cg-RetransmissionTimer expires).

- Option 2: The UE retransmits the MAC PDU immediately in the next CG (i.e. alike the UE behaviors for the autoTx configuration).

From our understanding, the cg-RetransmissionTimer is to allow the gNB to use the dynamic grant of CS-RNTI PDCCH to retransmit the MAC PDU when the PUSCH of the MAC PDU is actually transmitted. On the other hand, when the UE prioritizes the initial transmission, the gNB may also consider that the skipped transmission of the autonomous retransmission is due to the uplink skipping, as it is also difficult for the gNB to know whether there is an uplink skipping due to no MAC PDU for transmission. As such, Option 2 is preferred.

Proposal 2: The UE retransmits the de-prioritized MAC PDU immediately in the next CG (i.e. alike the UE behaviors for the autoTx configuration), when the initial transmission is prioritized over the autonomous retransmission.

## Selection of retransmission CG

When both cg-RetransmissionTimer and autonomousTx are configured, according to the Rel-16 NR-U and the Rel-16 IIOT, the retransmission of the MAC PDU includes the following two ways:

- Option 1 (Rel-16 IIOT with autonomousTx): The MAC PDU of deprioritized CG is retransmitted in the CG of the same HARQ process and the same CG configuration.

- Option 2 (Rel-16 NR-U with cg-RetransmissionTimer): The MAC PDU of the LBT failure CG is retransmitted in the CG of the same HARQ process and the same TBS across all CG configurations.

From our understanding, if the CGRT is configured, CG HARQ processes which can be shared across multiple CG configurations is selected autonomously by the UE. Then selecting the available CG of the same HARQ process and the same TBS across all CG configurations would provide faster retransmission of the MAC PDU which is either deprioritized or has LBT failure.

Proposal 3: When cg-RetransmissionTimer is configured, the MAC PDU which is either deprioritized or has LBT failure is retransmitted via the CG of the same HARQ process and the same TBS across all CG configurations, as Rel-16 NR-U.

# Conclusions

According to the analysis given above, we have the following proposals:

Proposal 1: When both cg-RetransmissionTimer and lch-basedPrioritization are configured, autonomousTx is always configured.

Proposal 2: The UE retransmits the de-prioritized MAC PDU immediately in the next CG (i.e. alike the UE behaviors for the autoTx configuration), when the initial transmission is prioritized over the autonomous retransmission.

Proposal 3: When cg-RetransmissionTimer is configured, the MAC PDU which is either deprioritized or has LBT failure is retransmitted via the CG of the same HARQ process and the same TBS across all CG configurations, as Rel-16 NR-U.

# Reference

[1] RAN2#112-e meeting minutes.

[2] RAN2#113-e meeting minutes.

[3] [Post113bis-e][505][R17 IIoT] URLLC in UCE (LG)