3GPP TSG RAN WG2 Meeting #114-e	R2-2105985

Electronic meeting, 19th May - 27th May 2021

Source:	ESA

Title:	Guiding framework on integrity concepts for A-GNSS positioning

Agenda Item:	8.11.5

Document for:	Discussion and Decision

- Introduction

A revision of the “NR Positioning Enhancements” work item was approved in RAN#91-e ([1]) including the following objective:

- Specify the signalling, and procedures to support GNSS positioning integrity determination, including [RAN2, RAN3]: - The assistance information that will be used to support integrity determination - The information that will be used to provide the positioning integrity KPIs and integrity results - Support of integrity for UE-based and UE-assisted A-GNSS positioning. Note: This objective is applicable to NR and E-UTRA.

This contribution provides considerations on the modifications needed in the signalling, procedures and information transferred to support GNSS positioning integrity.

- Integrity concepts in the context of GNSS

Civil aviation and automotive industry

Positioning integrity has a long operational history in the field of civil aviation, and specifically GNSS integrity monitoring techniques such as SBAS, ARAIM, RAIM, etc. Many widely-spread GNSS reference stations provide coverage over large swaths of land. Measurements collected at each individual GNSS reference stations used by central processing facilities to monitor individual GNSS satellites (and their signals) and ionosphere conditions over the entire service area. Next step is to perform integrity checks (to determine error bounds) of the corrections computed in previous step. All these information is uplinked to GEO satellites which then broadcast this info in their field of view. Aviation is not a use cases identified in the Study Item phase and is used here only as background information.

With the advent of autonomous systems possibly becoming mainstream by end of the decade, land-based applications are requiring safety functions which are increasingly carried out by sophisticated electric/electronic (E/E) systems. A new functional safety standard, the ISO-26262 [3], has been created to enable the design and assess systems that can prevent dangerous failures or control them when they occur. The ISO-26262 applies to safety-related road vehicle E/E system, and addresses potential hazards due to malfunctions. A navigation system (both its HW and SW components) proposed for installation in series production passenger cars is considered an E/E system and therefore subject to ISO-26262 certification.

Unlike integrity in the aviation domain, road vehicles experience new challenges due to change in environment and operation conditions. Therefore, higher level of positioning accuracy is required for in-lane navigation and higher degree of integrity is also required for safe operations. Typically, the integrity risk (maximum allowable failure of the positioning system) is selected as 10-7/hour which corresponds to the allowable hardware failure rate for an ASIL B system according to ISO 26262 [3].

Observation 1. As per WID text, one of the objectives is to add to TS 37.355 assistance information specific to GNSS positioning integrity and to support integrity for UE-based and UE-assisted positioning. Since, 3GPP identified automotive as a use case for positioning integrity it is recommended to discuss at group level what are the implications, if any, of ISO-26262 on our work (especially for safety-critical and liability-critical use cases).

Observation 2. The authors interpretation is that position integrity concepts designed by 3GPP would need to fulfil the ISO-26262 requirements before being adopted for lane-level navigation. If this scenario is correct, UE, LMF, and LPP/transport layers may need to be subject of ISO-26262 certification.

Translation of position integrity concepts to NR: potential pitfalls

The study phase examined how to report the integrity of positioning assistance data and the overall Positioning System, distributed via 5GS. A list of considerations, based on elements identified during the study phase are brought into attention.

#1. Many use case with diverse requirements (from very relaxed to very stringent).

Three application sectors have been identified in the study phase: Automotive, Railway, and IIoT, with very different requirements which may require a flexible integrity concept, able to satisfy the high integrity levels required in automotive and railway but also the wide range of integrity levels associated to IIoT use cases. Solutions specified with Automotive use case in mind may prove too complex for low TIR level e.g. subset of IIoT applications.

Observation 3. Requirements collected during the study phase are spread and suggest the need for flexible integrity concepts.

#2. Fragmented integrity system (3GPP ecosystem) vs End-2-end integrity system (how integrity is nowadays implemented)

A comparison with EGNOS satellite augmentation system is added here to pass across the message on possible pitfall #2.

We recall from [4] the figure where different failure events that can happen when carrying out an assisted-GNSS positioning such as PPP-RTK/SSR using the LPP (see TS 37.355). In this setup, besides errors and threats linked to GNSS systems and GNSS CORS networks (from which data is used to generate e.g., SSR assistance data), there may be possible threats to position integrity linked to 5GS. To add integrity information to NR positioning one should carry out an Failure Modes Effects Analysis (FMEA) on the 5GS/NR positioning architecture and GNSS (satellites, signals, network of stations) and make a fault tree, with probability of failures. Measures to bring the probability of failure to acceptable levels are then identified and implemented.

Outside 3GPP, solutions are designed and demonstrated as an one end-2-end system usually delivered by one entity. NR positioning architecture involves different components, such as UE, LMF, assistance data providers, etc. each developed by a different actor, and a service provider. When this architecture, corresponding to a very fragmented integrity system, is brought into the context of lane-level identification and road-user charging, the roles and responsibilities are not obvious. Given the implications of ISO-26262 in automotive use case, RAN2 should discuss this item in order to reach common grounds.

#3. Solutions are implementation-based and therefore out of scope of the WID….but are they?!

TR 38.857 Sections 9.3 and 9.4 lists a series of feared event categories and proposes examples of positioning integrity assistance information required to address the feared events. These parameters are very dependent on how the GNSS assistance data were generated and, indirectly, they can point to particular implementation(s). What is more, as pointed by TR 38.857, RTCM (Radio Technical Commission for Maritime Services) SC-134 is working on the integrity message definition in parallel to the work carried out by 3GPP. The work has reached a planning and experimental stage following initial investigations on the application scenario requirements and first outputs are expected in 2021-2022 timeframe. Both [6] and [7] suggest that content from RTCM on this topic represents a potential resource for consideration within this study depending what content is available from SC-134 within the Release 17 timeframe. In the past, 3GPP has relied on RTCM specifications to carry out its work on RTK and SSR, being sure that all GNSS related items specified for RTK and SSR meet several important conditions:

- are based on an industry standard backed by GNSS industry,

- the format is open and publicly available,

- being a standard, how each field should be used is clearly documented. Besides, all the positioning algorithms employed by each GNSS methods are well known.

At this moment, the same cannot be said when it comes to integrity as RTCM SC134 did not release yet any industry backed standard for GNSS integrity messages. The first drafts from RTCM are expected to become available at the end of 2021.

Observation 4. The parameters needed for integrity are very dependent to how the GNSS assistance data were generated (OSR and SSR IEs). Therefore, the set of parameters that will end up in LPP specs will be tightly connected to a particular implementation.

Observation 5.  A standard on GNSS integrity published by RTCM or any other relevant SDOs is not available yet and there is no independent external reference 3GPP could use to check its solution for GNSS integrity against.

Proposal 1.  	Liaise with RTCM SC134 working group on GNSS assistance data for integrity.

#4. Solutions for UE-based and UE-assisted

Support of integrity for UE-based and UE-assisted A-GNSS positioning is an objective of the WID. From an  LPP standpoint, certainly, both the LMF and UE could compute and monitor the integrity related results. However, from GNSS standpoint, the situation is not trivial at all: local effects experienced by GNSS receiver e.g. multipath are errors with random behaviour and significant magnitude depending the environment. They are typically addressed at the UE, therefore suggesting an UE-based solution (suited for automotive use case). Most certainly, for high integrity levels (e.g. TIR ~10-5 and beyond) specific to safety-critical or liability-critical is very difficult to envisage UE-assisted implementations simply because local effects are best addressed by solutions running on-board the UE rather than being shifted to location server. However, for lower TIR levels, with relaxed accuracy and latency requirements, “light” integrity concepts could be envisaged working in UE-assisted (a sub-set of the IIoT applications). Therefore, the UE-assisted mode should be framed as an optimisation problem with at least two variables: signalling procedures, and mitigation of all GNSS errors, including local effects experienced by the UE.

Observation 6.  Just because LPP supports exchange of information between LMF and UE, does not mean UE-based and UE-assisted positioning integrity are applicable in all scenarios. Furthermore, for the UE-assisted mode, the ability of LMF to mitigate GNSS local effects, based on information provided by the UE, is a key decision factor and it should not be overlooked.

Proposal 2.	When deciding the details of UE-assisted mode, take into consideration the need to enable LMF to mitigate all GNSS feared events, including the local feared events.

- Considerations on the support of integrity in A-GNSS

Integrity and Positioning

It is worth noting that positioning integrity is inherently linked to positioning, that is, positioning integrity should be considered as an enhancement or an extra feature/layer over positioning. It does not make sense to consider integrity separated from positioning.

Therefore, when deciding which signalling procedures should be employed to enable the integrity feature, the same LPP procedures and messages employed for transfer of assistance data, location information and measurements should be used.

For UE-based method, assistance data can be transfer from the LMF to the UE using the (Periodic) Assistance Data Transfer procedure. LMF may send to UE the following assistance data type:

- GNSS related Feared Events (FE):

  - GNSS system FEs

  - Atmospheric FEs

  - Local FEs (following a cooperative approach the LMF can provide integrity info obtained from other receivers in the same area to cope with local FEs)

- FEs in the GNSS assistance data

- FEs during the data transmission/communications.

For UE-assisted method, assuming that UE will perform and send GNSS measurements to LMF where the integrity results will be computed, the Location Information Transfer procedure can be used. We think periodic reporting of measurements from UE to the LMF should be enabled if not already supported in the specs. An important consideration for UE-assisted is the need to mitigate all GNSS feared events including the ones in the local environment of the UE (e.g. local FEs and Rx FEs like the measured multipath error, jamming/spoofing detection and receiver noise).

Some considerations about the integrity info required to be exchanged are provided in section 4.3.

Observation 7. (Periodic) Assistance Data Transfer procedure and Location Information Transfer procedure are well adapted to deal with GNSS integrity as well.

Integrity Capabilities

In an initial assessment two important capabilities have to be taken into account in order to provide integrity. Firstly, it is clear that an integrity algorithm is needed, and secondly, the integrity algorithm will only be able to provide PLs (Protection Levels) for a certain range of TIR (Target Integrity Risk). The more stringent/lower the TIR is, the more difficult is to ensure that the computed PL satisfies it. An algorithm needs to take into account much less effects and events for computing a PL that ensures a TIR of 10-1 or 10-2 than when computing a PL that needs to ensure a TIR of 10-7. Hence, the integrity algorithm can only provide and ensure PLs for a range of TIR and this range will depend on several factors. One is the implementation of the integrity algorithm (the effects/events that it is able to take into account) and other is the information available for the computation of the PLs, which, in the case of integrity for A-GNSS, is for example the availability of quality indicators related to the GNSS corrections or the availability of information related to FEs (Feared Events).

One way to indicate the capabilities of the integrity algorithm is through the LPP capability transfer procedure. Therefore, as the RequestCapabilities and ProvideCapabilities messages in the Rel-16 Capability Transfer Procedure of LPP handle position capabilities separately for each positioning method, the A-GNSS integrity capabilities can be provided within the A-GNSS positioning capabilities. The A-GNSS integrity capabilities should at least indicate whether integrity PLs can be provided and also the range of TIR values for which the integrity algorithm, if implemented, can provide valid PLs.

Proposal 3.  	Introduce the capability of providing PLs and the range of TIRs for which the PLs can be provided as positioning integrity capabilities for A-GNSS positioning method in the LPP capability transfer procedure.

Location Request

Four integrity KPIs were identified during the SI: TIR (Target Integrity Risk), AL (Alert Limit), TTA (Time To Alert) and Integrity Availability. This section provides some considerations with respect to these four KPIs and the parameters to be included in the Location Request with respect to integrity.

Set of TIRs

When considering which parameters should be included in the location request we need to assess what is needed by user requesting the integrity results and also by the algorithm that has to compute the integrity results.

From the user perspective there can be two situations/approaches:

- The user needs to know the distance/area/volume size that bounds the error for a certain TIR (Target Integrity Risk) and then reacts depending on the obtained size (e.g. comparing it with a threshold or a set of thresholds).

- The user needs to know the confidence level or the IR (Integrity Risk) corresponding to a certain distance/area/volume size and then react depending on the obtained probability.

The first one is the approach employed in applications where the TIR is a fixed requirement, like in aviation where there is a fixed TIR and the obtained PL is compared with the required AL (Alert Limit). The second one is employed in situations where the bounding distance/area/volume is a requirement, for example a vehicle driving on a road/lane of known width that needs to find out the probability of being within the lane/road width and react accordingly (confidence level).

The existing integrity algorithms could be adapted to work with both input-output options but option a) is the one most used i.e., the integrity algorithm receives as an input the TIR and provides the corresponding PL. With this implementation approach, the situations in which the user needs to find out the IR corresponding to a certain size, are accomplished by requesting at the same time the different PLs corresponding to a set of TIR and then approximating or interpolating the IR for the required size.

There are other situations in which the user/application will need to know the PLs for several TIR at the same epoch/time. A user can have different ways of behaviour depending on the level of accomplishment of a set of TIR-AL pairs. For example a car can be driving at a certain speed and when the TIR-AL that is being employed is not satisfied then it will need to reduce its speed accordingly to the TIR-AL that is being satisfied, this will be a real-time decision so a sequential request would complicate it. Also, a user can require a TIR-AL in a nominal situation but, when an unexpected event happens, requires a different TIR-AL (e.g. animal crossing the road, tyre puncture), so the PLs corresponding to both TIRs should be available at the same time.

As a conclusion of the above reasoning, it is fundamental to include within the location request, as a parameter provided by the user, the set of TIRs (one or more TIR values) for which the integrity algorithm has to provide the PLs.

Proposal 4. 		Include within the location request, the set of TIRs for which the integrity algorithm has to provide the PLs.

AL, TTA and Integrity Availability

With respect to the other three KPIs identified during the SI (AL, TTA and Integrity Availability), in principle, their inclusion as parameters into the location request is not considered mandatory because the user can check the computed PL (or set of PLs) against their corresponding ALs. Besides, the user can also check that the time difference between the epoch for which the PL is applicable and the epoch when the PL was received by the user is lower than the TTA. In addition, Integrity Availability is just a performance indicator that can only be checked a posteriori after collecting a certain amount of data.

However, if two integrity result reporting modes are going to be supported (PL Reporting or Integrity Event Flagging), then the reporting mode selected by the user should be part of the location request parameters, and for the Integrity Event Flagging mode along with the AL and TTA.

Observation 8.  In principle, the AL, TTA and Integrity Availability KPIs do not need to be included within the location request because they can be checked by the user.

Observation 9.  If any of the integrity reporting modes includes integrity event flagging, then the AL and TTA will need to be provided by the user in the location request.

Integrity Results – Reporting Modes

Two reporting modes were proposed at RAN2#113-Bis-e:

- Mode 1 of Integrity Result Reporting : PL Reporting

- Mode 2 of Integrity Result Reporting : Integrity Event Flagging

Mode 1, as it reports the computed PL value, is considered fundamental because it provides more flexibility and allows to satisfy all the different use cases. On the other hand, computing the integrity event flag in Mode 2 only requires to compare the PL against the AL, there is not much added value in that, but this can be supported if there are users needing this type of reporting. In this scenario, Mode 1 implies the user to only provide the required TIRs when performing the location request, while Mode 2 implies the user to provide TIRs, AL, and TTA in the location request.

One of the drawbacks of the Integrity Event Flagging in Mode 2 is that, with just a red/green flag, it does not provide visibility of the degree of accomplishment of the AL (the user may need to know how far or close from the AL the PL is). The simplest and more complete way of doing it requires to provide the computed PL along with the integrity event flag within Mode 2. Thus, the only difference between Mode 1 and Mode 2 would be whether the user provides the AL and the TTA or not when sending a location request.

Observation 10.  The integrity result reporting mode in which the PL values are reported is fundamental because it provides more flexibility and allows to satisfy all the different use cases.

Observation 11.  If both modes will be supported, then the mode 2 needs the user to also provide AL, and TTA in the location request.

Proposal 5.  	Include the reporting of PL values in at least one of the integrity result reporting modes (PL Reporting).

Proposal 6.  	If Mode 2 is also supported, provide to the UE visibility of the degree of accomplishment of the AL by also including the PL value in the integrity mode 2.

- Integrity concepts at different levels

In this section we have looked at elements already included in LPP and proposed a number of integrity levels that can be achieved in NR. For some of the levels, there is no need for touching LPP while for other levels SSR IEs could be extended with new fields and new IEs could be also added.

  - Basic system-level integrity: potential satellites and signals faults

Satellites can suffer HW failures and therefore enter into a mode in which they cannot broadcast a signal altogether for a period of time or permanently, depending on the magnitude of the issue. In situations like this the health of the GNSS satellite(s) and the signal(s) must be communicated to the UE in real-time. Outside of 3GPP, this is achieved by using flags in the message broadcast by SBAS systems or directly that particular GNSS constellation.

3GPP: Is there already a solution in LPP? Yes, the GNSS-RealTimeIntegrity IE. This is the most basic form of integrity capability.

  - Basic user-level integrity

Besides the GNSS-RealTimeIntegrity IE, additional integrity information can be provided by server in the shape of User Range Accuracy – URA (deals with signal-in-space integrity information). The term URA, employed in LPP (navURA and GNSS-SSR-URA), provides the signal-in-space integrity information and is related to satellite orbit and clock errors. Thus, navURA is the signal-in-space accuracy after using the satellite orbits and clock information provided in the GNSS navigation message and GNSS-SSR-URA is the signal-in-space accuracy after applying the SSR corrections. The URA is based on the past/historic statistics of a given satellite and is provided for the worst user location (WUL) on the Earth (the location that leads to the highest ranging error when projecting the orbit and clock errors to the line-of-sight). As it is based on past/historic data, without considering the current orbits and clocks, the URA is an a-priori estimation of the signal-in-space ranging error at WUL and is conservative with respect to the actual signal error because it´s valid for several hours at the expense of resolution.

3GPP: Is there already a solution in LPP? Yes, the navURA fields and GNSS-SSR-URA IE. To be checked more carefully though.

Proposal 7.	Acknowledge the support of basic integrity features in LPP by updating TS 38.305.

  - Uncertainty of the ranging measurement

In this level, integrity information on SSR assistance data can be generated at location server by using the measurements from a GNSS ground tracking networks and provide this information to the UE. Here, we propose to enable the overabounding of the total uncertainty of the ranging measurement as, regardless of the implementation algorithms, this is a pre-requisite. This integrity information can be provided as quality indicators with the SSR correction data.

Recalling from [9], the computation of the positioning integrity (PLs) is mainly based on:

- The knowledge of the uncertainty of all the measurements and the system data employed as an input for the position computation.

- The amount of redundant information/measurements that is available for the algorithm to compute the position. A higher amount of measurements and a good satellite geometry implies a better observability of the positioning errors.

Therefore, it is important to note that, in the end, the positioning integrity and the achievable performance are based on the knowledge of the uncertainty of all the ranging measurements and the system data needed to employ those measurements (e.g. satellite orbits, clocks, etc.). The following pyramid depicts this concept.

Figure 1. Relation between ranging measurements, integrity algorithms, and error bounds (PLs) for a certain TIR

Inputs employed by the integrity algorithms

Here is an overview of the different inputs needed by the GNSS integrity algorithms to compute positioning error bounds (PLs) for a certain TIR:

- GNSS Measurements / Residuals after the position estimation. The computation of the PLs is either based on the residuals, or is based on the geometry taking assumptions on the error distribution and then the residuals are employed to check that the assumptions hold.

- Line-of-Sight / Geometry. The geometry of the receiver with respect to the satellites has also an impact on the integrity results. For example it is needed to compute the dilution of precision which relates the measurement errors with the position errors and shows the directions in which the position error can be higher.

- Overbound of the uncertainty of ranging measurement error (see Annex A for details). The total ranging error is formed by the Signal in Space (SIS) errors, the atmospheric errors, the local effects and the receiver errors. The uncertainty of the ranging measurements, or some of its components, can be hardcoded/fixed by the integrity algorithm with worst case values or with values depending on the detected environment, but, as the uncertainty changes along time, to achieve better performances some of them can be monitored by an external system and provided to the integrity algorithm by means of what is usually called an Integrity Support Message (ISM). In the context of NR, this Integrity Support Message can be seen as LPP Provide Assistance Data message and the information transported to UE the quality indicators for the SSR IEs.

- Additional parameters. Depending on the integrity algorithm, additional parameters will be needed to compute the integrity info. These parameters can be hardcoded/fixed or updated by means of an Integrity Support Message (ISM). Here are some examples:

  - Parameters to set the probability of failure of a satellite (Psat) or the whole constellation (Pconst)

  - Operation parameters: satellite status flags; the Mean-Time-To-Notify (MTTN) to the user that a satellite has become faulted; validity time indicating when these integrity parameters may be safely used.

The following table provides an overview of the inputs employed by some of the integrity algorithms (see algorithm descriptions in [9]). The table shows that all the integrity algorithms need as an input the an estimation on the uncertainty of the ranging measurements.

| Integrity Algorithm | Characteristics | Overview of Integrity Algorithm Inputs |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | GNSS Measurements / Residuals | Line-of-Sight / Geometry | Uncertainty of ranging measurements | Additional parameters |
| Classic RAIM | Snapshot integrity algorithm that applies to the least-squares navigation solution. | YES | YES | YES Fixed | YES Fixed sat fault probability values |
| ARAIM | Snapshot integrity algorithm that applies to the least-squares navigation solution, enhanced to handle multiple failures with multiple constellations and optimised availability. High computational load. Intended for aeronautical environment. | YES | YES | YES ISM provides aURE, aURA, bnom | YES ISM provides Psat, Pconst, MTTN, validity time |
| IBPL | Snapshot integrity algorithm that applies to the least-squares navigation solution capable of handling multiple failures and constellations as well. Based on the residuals and on the isotropy of the measurement errors to estimate the PLs and satisfy integrity (proper ratio of the uncertainties is more important than proper bound, which is a more relaxed requirement). Low computational load. Performances improve with the number of available measurements (multi-constellation needed in urban environment). | YES | YES | YES Can employ the same uncertainty parameters as ARAIM in aeronautic environment. | YES In aeronautic environment and for demanding TIR: Psat, Pconst |
| KFMI | Filtered integrity algorithm that applies to standard Kalman filter navigation algorithms. Intended to be employed in all types of environment (including deep-urban). | YES | YES | YES | NO |
| KIPL | Filtered integrity technique that applies to standard Kalman filter navigation algorithms, plus additional considerations (correlation). Intended to be employed in all types of environment (including deep-urban). | YES | YES | YES | YES Time correlation parameters |

Observation 12.  Overbounding the uncertainty for the ranging measurement error is a pre-requisite of any integrity algorithm known to date. It can be estimated by UE based on quality indicators for the SSR data.

The following formula can be used to statistically describe the overall error contribution for each GNSS measurement. In other words, the total uncertainty for measurements performed by the UE to each visible ith satellite can be expressed as:

σUERE, i2=σSV orbit2+σSV clocks2+ σcode and phas biases2+ σI2 + σT2+σENV2+σRx2        (1)

σUERE, i2=σURE2 +σI2+ σT2+σENV2+σRx2        (2)

Where

| Quality indicator | Meaning | Observation |
| --- | --- | --- |
| σUERE, i2 | Total uncertainty for measurements obtained from satellite i. Is formed by the summary of the total error budget affecting a pseudorange from the user's point of view, including the signal in space ranging error called URE (User Range Error), the atmospheric effects (due to the Ionosphere and Troposphere), the impact of local environment (e.g. multipath) and the quality of the receiver. |  |
| σURE2 | Uncertainty of the combined orbit, clock, and bias corrections. Could also be expressed as σclock2+σorbit2+σcode and phase biases2 | These terms are derived in real time based on measurements collected at stations part of GNSS CORS reference network. |
| σI2 | Uncertainty of the ionosphere model |  |
| σT2 | Uncertainty of the troposphere model |  |
| σENV2+σRx2 | Uncertainty of the measurements in the given environment and receiver noise. Multipath is the dominant term here. | It is computed by the UE. Is perhaps the most difficult to determine as the value is dependent on UE environment, multipath, possible spoofing and jamming, and measurement quality. |

Then, the next step would be to compute Protection Level based on the expected behaviour of the error sources introduced above. How to use the quality indicators to compute integrity PLs is related to implementation aspects and it should not be part of the scope of the work in 3GPP.

Specification impact for UE-based:

- Possible extension of GNSS-SSR IE with additional fields, representative to the quality of each GNSS error here modelled as SSR: GNSS-SSR-OrbitCorrections, GNSS-SSR-ClockCorrections, GNSS-SSR-CodeBias, GNSS-SSR-PhaseBias, etc. Alternatively, a new IE collecting quality indicators flags for all GNSS SSR IEs could be defined.

Specification impact for UE-assisted:

- Possible extension of GNSS-Measurement IE with additional fields, if any identified, or improvements to existing fields (e.g., improve resolution of mpathDet to achieve better granularity).

- The 37.355 includes period reporting of Assistance Data with direction from LMF to UE. It is not clear whether periodic reporting of measurements from UE to LMF is also supported. According to our interpretation of existing LPP, UE-assisted positioning seems to be snapshot based. This may not be enough for all possible applications in IIoT, Railway, and Road where the LCS client is outside the UE.

Proposal 8.	Add quality indicators to the SSR assistance data IEs. Details are for FFS at this moment.

Proposal 9.	Study whether periodic reporting of measurements from UE to LMF needs enabled (if not already supported).

  - Complex integrity concepts

The concepts discussed above may not be sufficient to address the most stringent TIR levels and new IEs, addressing the feared events identified during the study phase, will be needed in several instances.

- Conclusions

In this contribution, we have proposed a starting point for an integrity concept applicable UE-based and UE-assisted methods.

Observation 1. As per WID text, one of the objectives is to add to TS 37.355 assistance information specific to GNSS positioning integrity and to support integrity for UE-based and UE-assisted positioning. Since, 3GPP identified automotive as a use case for positioning integrity it is recommended to discuss at group level what are the implications, if any, of ISO-26262 on our work (especially for safety-critical and liability-critical use cases).

Observation 2. The authors interpretation is that position integrity concepts designed by 3GPP would need to fulfil the ISO-26262 requirements before being adopted for lane-level navigation. If this scenario is correct, UE, LMF, and LPP/transport layers may need to be subject of ISO-26262 certification.

Observation 3. Requirements collected during the study phase are spread and suggest the need for flexible integrity concepts.

Observation 4. The parameters needed for integrity are very dependent to how the GNSS assistance data were generated (OSR and SSR IEs). Therefore, the set of parameters that will end up in LPP specs will be tightly connected to a particular implementation.

Observation 5.  A standard on GNSS integrity published by RTCM or any other relevant SDOs is not available yet and there is no independent external reference 3GPP could use to check its solution for GNSS integrity against.

Observation 6.  Just because LPP supports exchange of information between LMF and UE, does not mean UE-based and UE-assisted positioning integrity are applicable in all scenarios. Furthermore, for the UE-assisted mode, the ability of LMF to mitigate GNSS local effects, based on information provided by the UE, is a key decision factor and it should not be overlooked.

Observation 7. (Periodic) Assistance Data Transfer procedure and Location Information Transfer procedure are well adapted to deal with GNSS integrity as well.

Observation 8.  In principle, the AL, TTA and Integrity Availability KPIs do not need to be included within the location request because they can be checked by the user.

Observation 9.  If any of the integrity reporting modes includes integrity event flagging, then the AL and TTA will need to be provided by the user in the location request.

Observation 10.  The integrity result reporting mode in which the PL values are reported is fundamental because it provides more flexibility and allows to satisfy all the different use cases.

Observation 11.  If both modes will be supported, then the mode 2 needs the user to also provide AL, and TTA in the location request.

Observation 12.  Overbounding the uncertainty for the ranging measurement error is a pre-requisite of any integrity algorithm known to date. It can be estimated by UE based on quality indicators for the SSR data.

Proposal 1.  	Liaise with RTCM SC134 working group on GNSS assistance data for integrity.

Proposal 2.	When deciding the details of UE-assisted mode, take into consideration the need to enable LMF to mitigate all GNSS feared events, including the local feared events.

Proposal 3.  	Introduce the capability of providing PLs and the range of TIRs for which the PLs can be provided as positioning integrity capabilities for A-GNSS positioning method in the LPP capability transfer procedure.

Proposal 4. 		Include within the location request, the set of TIRs for which the integrity algorithm has to provide the PLs.

Proposal 5.  	Include the reporting of PL values in at least one of the integrity result reporting modes (PL Reporting).

Proposal 6.  	If Mode 2 is also supported, provide to the UE visibility of the degree of accomplishment of the AL by also including the PL value in the integrity mode 2.

Proposal 7.	Acknowledge the support of basic integrity features in LPP by updating TS 38.305.

Proposal 8.	Add quality indicators to the SSR assistance data IEs. Details are for FFS at this moment.

Proposal 9.	Study whether periodic reporting of measurements from UE to LMF needs enabled (if not already supported).

- References

[1]	RP-210903, “Revised WID on NR Positioning Enhancements,” Intel Corporation, CATT

[2]	3GPP TR 38.857 Study on NR Positioning Enhancements (Release 17)

[3]	ISO-26262 Road Vehicles – Functional Safety

[4]	R2-2007647, “Discussion on GNSS position integrity error sources”, ESA

[5]	EGNOS Open Service (OS) Service Definition Document, v2.3

[6]	R2-2006541, TP for Study on Positioning Integrity and Reliability, Swift Navigation, Deutsche Telekom, u-blox, Ericsson, Mitsubishi Electric, Intel Corporation, CATT, UIC

[7]	R2-2006674, Discussion on error sources, threat models, occurrence rates and failure modes, CATT

[8]	Elliott D. Kaplan, Christopher J. Hegarty, “Understanding GPS/GNSS Principles and Applications” Third Edition. Artech House

[9]	R2-2007656, Discussion on methodologies for position integrity, ESA

- Annex A. Overbounding the ranging errors

The Gaussian distribution is by far the most widely used distribution in engineering. There are many reasons that explain its ubiquitous use and success, among which is the application of the central limit which tells that the mean of a large number of independent, identically distributed random variables tends to a Gaussian distribution, regardless of their original distribution. As a result, many sources of random noise indeed (approximately) follow Gaussian distributions. In terms of distributions, the central limit theory tells that when a probability density function is convoluted many times with itself, it will look more and more Gaussian with every convolution.

As a consequence, it has been a long tradition in the design of, research on, and analysis of navigation systems that error sources are assumed to have a zero mean Gaussian probability density function (in the absence of failures). This assumption has been instrumental in the development of real-life systems as it allows for a relatively simple metric for trade-off purposes in system designs, and has been the basis for the design of almost all algorithms in GNSS-based systems

But, despite the promises of the central limit theorem, most of the error sources in GNSS do not follow a Gaussian distribution (e.g. multipath). Moreover, error sources not always have zero means, especially not when observed over a relatively short period of time. Because requirements are often specified per operation, and averaging over multiple operations is not allowed, this implies that even in the error-free situation the measurement deviations might not be free from biases.

If the integrity results (the computation of the PLs) are built upon the assumption of zero mean Gaussian range error distributions, then, when the actual distribution is not (or cannot proven to be) zero mean Gaussian, the question is whether the integrity results can still be employed. The answer is obvious: the integrity equations can be used, as long as it is guaranteed not to give an overly optimistic assessment of system integrity. Thus, representing the range error distributions with overbounding Gaussian distributions will provide a conservative result that will overbound the actual position error distribution.

There are several ways of overbouding an error distribution with a Gaussian distribution (e.g. overbounding can be defined in the PDF or in the CDF, or can assume a zero-mean error distribution or not). Some of these methods are described in [4], [5] and [6].

PDF-overbounding

CDF Overbounding (left) and its Relationship to PDFs (right)

Paired-Gaussian Overbounding

The characteristics of the error distribution shall be taken into account (e.g. mass of the tails, if it is a non-zero mean distribution, if it is multimodal, etc.). The more degrees of freedom the overbounding method has to adjust to the actual error distribution will make possible to build tighter error bounds and, therefore, the overbounding will be less conservative and the performances will be improved (lower PLs will lead to higher availability).

As an example of overbounding non-zero mean error distributions, ARAIM overbounds the uncertainty of the SIS error with two parameters ([7]), one to overbound the SIS error (variance and short-term bias) and a second one to overbound the SIS long-term bias error.

- Annex B. Uncertainty of ranging measurement

- B.1. Uncertainty of range measurement in UE-based

Scenario: UE-based - Ranging, UE location, and position integrity is computed at the UE. LMF provides assistance data in support of the three functions.

GNSS receiver, present in the UE, aided by the information provided by the network, performs ranging on GNSS signals, compute its position, and estimates the trustworthiness of the location estimate (integrity).

Use Case:  Railway, Automotive, IIoT (outdoor only)

LPP Procedure: Periodic Assistance Data Transfer

| LMF sends to UE | UE computes | Observation |
| --- | --- | --- |
| σI2, σT2, σURE2 | σUERE,i2 Total uncertainty for satellite i σENV+Rx2 Uncertainty of the measurements in the given environment and due to receiver characteristics (function of multipath, thermal noise, etc.) | Of course, these quality indicators/variance needs to be provided together with the associated SSR IEs (see specification impact item further below) |

Specification impact:

- Possible extension of GNSS-SSR IE with additional fields, representative to the quality of each GNSS error here modelled as SSR: GNSS-SSR-OrbitCorrections, GNSS-SSR-ClockCorrections, GNSS-SSR-CodeBias, GNSS-SSR-PhaseBias, etc. Alternatively, a new IE collecting quality indicators flags for all GNSS SSR IEs could be defined.

- B.2. Uncertainty of range measurement in UE-assisted

Scenario: UE-assisted - Ranging is performed at the UE, which then forwards all measurements to LMF for location, and position integrity estimation.

Use Case: Industrial IoT applications, Pay-as-you-drive,

LPP Procedure: Location Information Transfer (A-GNSS-RequestLocationInformation and A-GNSS-ProvideLocationInformation) (we think periodicity feature, i.e. like for Periodic Assistance Data Transfer procedure, would be required).

| Target replies to a solicitation form Server | Target provides unsolicited location information to the server |
| --- | --- |

In this scenario, the UE can send additional information to the LMF besides the ranging measurements. The content of GNSS-MeasurementList IE can be used to send to LMF information about the quality of the measurements performed by the UE. With this additional information, the LMF can estimate the σENV+Rx2. Note, in any foreseeable scenario, LMF, as the source of A-GNSS assistance data, can be aware of the quality of GNSS corrections. In other words, LMF knows σI2, σT2, 
σURE2. To compute the total uncertainity of a measurement provided by the UE, the LMF would need to receive the σENV+Rx2 from UE or enough information (e.g., multipath quality) to be able to compute it itself.

At a first glance, the GNSS-MeasurementList IE seems to contain enough information on measurement quality that can be shared by UE to LMF and mpathDet field (Multipath indicator value) is of particular interest as multipath is the dominant term in σENV+Rx2.

From TS 37.355:

mpathDet This field contains the multipath indicator value, defined in the table Value of mpathDet to Multipath Indication relation below.

Value of mpathDet to Multipath Indication relation

| Value of mpathDet | Multipath Indication |
| --- | --- |
| 00 | Not measured |
| 01 | Low, MP error < 5m |
| 10 | Medium, 5m < MP error < 43m |
| 11 | High, MP error > 43m |

Therefore

| UE sends to LMF | LMF knows | LMF computes |
| --- | --- | --- |
| Option 1: σENV+Rx2 (if estimated by UE) Option 2: or information that may help the LMF estimate σENV+Rx2 (e.g. GNSS-MeasurementList IE and in particular mpathDet field) | σI2, σT2, σURE2 | σUERE, i2 Total uncertainty for satellite i [and] σENV+Rx2 (in case not provided by UE and only if all required information is sent over by the UE) |

Specification impact:

- Possible extension of GNSS-Measurement IE with additional fields, if any identified, or improvements to existing fields (e.g., improve resolution of mpathDet to achieve better granularity).

- The 37.355 includes period reporting of Assistance Data with direction from LMF to UE. It is not clear whether periodic reporting of measurements from UE to LMF is also supported. According to our interpretation of existing LPP, UE-assisted positioning seems to be snapshot based. This may not be enough for all possible applications in IIoT, Railway, and Road where the LCS client is outside the UE.