3GPP TSG-RAN WG2 #114-e                                                                                      R2-2106353

eMeeting, 19th – 27th May, 2021                                                              Revision of R2-2104158

Agenda Item:	    8.3.4

Source: 	    MediaTek Inc.

Title:		    Paging with service indication

Document for:	    Discussion and decision

# 1 Introduction

In this contribution, we focus on the following objective from MUSIM WID [1].

- Unless SA2 find an alternative solution or decides otherwise, specify mechanism for an incoming page to indicate to the UE whether the service is VoLTE/VoNR[ RAN2].

  - RAT Concurrency: Network A is either LTE or NR. Network B is either LTE or NR.

  - Applicable UE architecture: Single-Rx/Dual-Rx/Single-Tx

In RAN2#113bis-e, the following agreements are made

- 1: RAN2 works to support the MUSIM paging cause feature that SA2 is working on and also addresses the paging cause issue raised by SA2 LS.

- 2: RAN2 attempts to reply LS to SA2 once we progress on solution and agree on CR(s) that support/address the above feature/issue.

- 5: If RAN2 agrees to add a paging cause value (or any other information that could lead to a specific paging cause) in Uu paging message, RAN2 specifies the relevant UE behavior (i.e. inform or passing to the upper layer) upon its reception in both LTE and NR specifications.

- RAN2 does not intend to introduce alternative paging IDs for MUSIM paging (unless requested by SA2).

In this paper, we discuss the service indication in paging message according to the WID and SA2 LS S2-2102039 [2]. It is proposed to have new paging causes “voice” and “other”.

# 2 Discussion

For MUSIM UEs, a RRC_CONNECTED UE in Network A also monitors paging from Network B. If paged in Network B, UE needs to decide whether to switch to Network B. UE would prefer to do so, for example, if the incoming traffic is for voice services. Our proposal for such indication is to introduce a new Paging Cause for voice service, as described in Solution#1 of TR 23.761 [2]. The new paging cause allows the UE to identify the reason for paging, without requiring a paging response and RRC connection establishment.

Furthermore, based on the LS S2-2102039 [2] from SA2, the UE should be able to distinguish “non-voice” and “any service”, where the “any service” is from the legacy network that does not support new paging cause.

SA2 has concluded that the UE shall be able to discriminate the case where it is being paged for non-voice service from the case where it is being paged (for any service) by a RAN node not supporting the Paging Cause, either because the RAN node does not support the Paging Cause feature, or because in case of RAN sharing it is configured to operate without the Paging Cause feature for some of the connected CNs.

Therefore, a single new cause value may not be enough. The new RAN node should be able to indicate two different service type, e.g. “Voice” and “Other”.

Based on offline discussion [3] in RAN2#113bis-e, there are basically two approaches to indicate the service type to the UE.

- Direction A - Use UU paging message to identify service type

- Direction B - Use UU paging message + NAS indication to identify service type

We think the most simple and reasonable design is to have two new cause values in the UU paging message (i.e. Direction A). Some companies suggest to use “paging cause enable indicator” in NAS as an indicator of RAN’s supporting of MUSIM (i.e. no need to have “non-voice” paging cause.) To make the NAS information useful, paging cause support should be homogeneous in the cells within a registration area. We are not sure this is going to be case. In addition, combining of NAS message and UU paging message to determinate this bring unnecessary complexity to the UE. Therefore, we propose to add two new paging cause in both LTE and NR paging message.

Proposal 1: Introduce new paging causes "Voice" and "Other” (or “non-Voice”) in LTE and NR paging message.

With this design, a MUSIM UE understand the service from network B as following

- If paging cause is “Voice”, the incoming MT call is for voice service (VoLTE or VoNR)

- If paging cause is “Other”, the incoming MT call is for non-voce service

- If the new paging cause is absent, the incoming MT call is for any service

  - Network B is a legacy network or it does not intent to support the new paging cause.

# 3 Conclusions

Base on the discussion in section 2, we propose the following proposals:

Proposal 1: Introduce new paging causes "Voice" and "Other” (or “non-Voice”) in LTE and NR.

# 4 References

[1] RP-210316, “Revised WID: Core part: Support for Multi-SIM devices for LTE/NR”, Vivo

[2] R2-2102664/S2-2102039, “LS on System support for Multi-USIM devices”, SA2

[3] R2-2104331, “Summary of [AT113bis-e][230][MUSIM] Reply LS to SA2 on paging cause (Intel)”, Intel