3GPP TSG-RAN WG2 #114-e	R2-2106545

Electronic meeting, 19th – 27th May, 2021

Title:	LS on Group IDs for Network selection (GINs)

Response to:	-

Release:	Release 17

Work Item:	NG_RAN_PRN_enh-Core

Source:	RAN2

To:	SA2

Cc:

Contact Person:

#### Name:	Felipe Arraño Scharager

E-mail Address:	felipe.arrano.scharager@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN2 discussed relevant broadcasting details concerning Group IDs for Network selection (GINs) and identified some ambiguities as further described below.

1.1 GINs for onboarding and accessing SNPNs with external credentials:

The following note is captured in the conclusion of TR 23.700-07 (see clause 8.4.1):

NOTE 3:	The Group ID(s) in the SIB that UE can use for selecting an O-SNPN are the same as the Group ID(s) in the SIB that the UE uses for SNPN selection as part of KI#1.

However, within RAN2 there are two different interpretations regarding this note:

- that the same GIN-List is used for both SNPN access with external credentials (i.e., SA2’s KI#1) and for onboarding (i.e. SA2’s KI#4); or

- that only the GIN encoding/format is the same for both features, but different lists should (potentially) be used for each.

Thus, RAN2 would like to ask the following.

Question 1a: Can a common list of GINs be used for both onboarding and SNPN access using external credentials?

Question 1b: Can RAN2 assume that higher layers may use different GIN values for each feature?

1.2 GIN encoding:

RAN2 observes that as per the current versions of TR 23.700-07 and TS 23.501, there is a discrepancy regarding the encoding used by GINs. On the one hand, TR 23.700-07 concludes in clause 8.1.4 that:

-	Group ID as a specific case of SNPN ID reusing SNPN ID encoding in TS 23.003 [15], where:

-	Assignment mode 1 indicates self-managed Home SP Group ID values as the NID Value is chosen independently at deployment time.

-	Assignment mode 0 indicates Home SP Group ID is globally unique as the NID Value is globally unique. One possibility for ensuring uniqueness is to use IANA PEN as in TS 23.003 [15].

While on the other hand, TS 23.501, clause 5.30.2.2, states:

#### 5.30.2.2	Broadcast system information

NG-RAN nodes which provide access to SNPNs broadcast the following information:

[…]

-	List of supported Group IDs for Network Selection (GINs) per SNPN. GIN reuses the NID encoding in TS 23.003 [15] and can be self-managed or globally unique;

[…]

On this matter, RAN2 would therefore like to clarify the discrepancy between the above specifications:

Question 2: Can SA2 clarify whether a GIN is encoded:

- as an SNPN ID (i.e., a PLMN ID plus a NID); or

- as a NID.

2. Actions:

To SA2 group.

ACTION: 	RAN2 respectfully asks SA2 to answer the questions above.

3. Date of Next TSG-RAN WG2 Meetings:

3GPP RAN2#115-e	from 2021-08-16	to 2021-08-27		Electronic Meeting

3GPP RAN2#116-e	from 2021-11-01	to 2021-11-12		Electronic Meeting