3GPP TSG RAN WG2 Meeting #114-e		R2-2106622

Online, May 19 – May 27 2021

Title:	[Draft] LS on resource reselection trigger sl-reselectAfter

Response to:

Release:	Release 16

Work Item:	5G_V2X_NRSL-Core

Source:	Apple [To be RAN2]

To:	RAN1

Cc:

Contact person:

#### Name:	Zhibin Wu

E-mail Address:	zhibin_wu@apple.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# Overall description

In NR V2X, one of the resource reselection triggers for mode 2 UE is specified in TS 38.331 as below and has a configured value range of integer 1 to 9:

sl-ReselectAfter

Indicates the number of consecutive skipped transmissions before triggering resource reselection for sidelink communication (see TS 38.321 [3]).

In TS 38.321, the TX UE behavior is described as follows:

If the TX resource (re-)selection check procedure is triggered on the selected pool of resources for a Sidelink process according to clause 5.22.1.1, the MAC entity shall for the Sidelink process: <unrelated text omitted> 1> if sl-ReselectAfter is configured and the number of consecutive unused transmission opportunities on resources indicated in the selected sidelink grant is equal to sl-ReselectAfter; or <unrelated text omitted> 2> clear the selected sidelink grant associated to the Sidelink process, if available; 2> trigger the TX resource (re-)selection.

1> if a selected sidelink grant is available for retransmission(s) of a MAC PDU which has been positively acknowledged as specified in clause 5.22.1.3.3: 2> clear the PSCCH duration(s) and PSSCH duration(s) corresponding to retransmission(s) of the MAC PDU from the selected sidelink grant.

RAN2 understands that this sl-reselectAfter parameter and related MAC procedure are reused from LTE-V2X, in which HARQ feedback does not exist.  In NR V2X, when HARQ feedback has indicated that retransmission of the MAC PDU is not needed,  it is unclear to RAN2 whether the remaining retransmission resources for a MAC PDU should be counted towards “consecutive unused transmission opportunities” on resources indicated in the selected SL grant. Also, in the legacy case of UE skipping due to no data to transmit, the number of unused retransmission opportunities (besides the skipped initial transmission opportunity) associated with the selected SL grant could be much larger, compared to only up to one retransmission allowed in LTE V2X.

Thus, in lieu of the above differences between NR V2X and LTE V2X, RAN2 has the following questions:

Question 1: In NR V2X, according to RAN1 agreement, how “unused retransmission opportunities” shall be counted towards “consecutive unused transmission opportunities” to trigger resource reselection, in case of HARQ feedback indicating that retransmission is not needed, and in case of there is no new data to transmit?

Question 2: In NR V2X, according to RAN1 agreement, is it the correct understanding that only the resources already indicated in SCI shall be counted towards “consecutive unused transmission opportunities” to trigger resource reselection?

# 2	Actions

To 3GPP RAN1:

ACTION: RAN2 respectfully asks RAN1 to provide answer to the above questions.

# 3	Dates of next TSG RAN WG2 meetings

TSG RAN WG2 Meeting #115-e	16 – 27  August 2021

TSG RAN WG2 Meeting #116-e	1 – 12    November 2021