3GPP TSG-RAN WG2 #114-e electronic	R2-2106645

Electronic Meeting, 19th – 27th May, 2021

Agenda Item:	8.17.1

Source:	Intel Corporation

Title:	[AT114-e][035][feMIMO] TCI states indication for PDCCH (Intel)

Document for:	Discussion, Decision

# 1	Introduction

This contribution summarizes the following discussion:

[AT114-e][035][feMIMO] TCI states indication for PDCCH (Intel)

Scope: Treat R2-2104712 and the related submitted tdocs.

Discuss the topic, attempt to make some basic agreements, e.g. agree to have the requested MAC CE, and potentially identify FFS.

Intended outcome: Report

Deadline: Monday May 24 for on-line CB

Contact person(s) for each participating company:

| Company | Name | Email Address |
| --- | --- | --- |
| Intel | Youn Heo | Youn.hyoung.heo@intel.com |
| ZTE | Fei Dong | dong.fei@zte.com.cn |
| LG | Hanul Lee | hanul.lee@lge.com |
| Xiaomi | Yumin Wu | wuyumin@xiaomi.com |
| Lenovo | Lianhai | Wulh5@lenovo.com |
| OPPO | Xin You | youxin@oppo.com |
| CATT | Erlin Zeng | erlin.zeng@catt.cn |
| Huawei, HiSilicon | David Lecompte | david.lecompte@huawei.com |
| Qualcomm | Ruiming Zheng | rzheng@qti.qualcomm.com |
| Samsung | Seungri Jin | seungri.jin@samsung.com |
| vivo | Chenli | Chenli5g@vivo.com |
| Ericsson | Helka-Liina Määttänen | Helka-liina.maattanen@ericsson.com |
| Nokia | Tero Henttonen | tero.henttonen@nokia.com |

# 2	Discussion

RAN1 sent an LS [1] to inform RAN1 agreement on PDCCH enhancement. In this LS, RAN1 agreed to introduce enhanced MAC CE ignalling for PDCCH activating two TCI states for SFN-based PDCCH transmission.

Agreement - Introduce enhanced MAC CE signaling for PDCCH activating two TCI states for SFN-based PDCCH transmission - The corresponding MAC CE includes at least the following fields - Serving cell ID - CORESET ID - Two TCI state IDs - FFS whether for CA scenario additionally support RRC configured set of the serving cells which can be addressed by a single MAC CE - FFS whether or not enhanced MAC CE signaling is applicable to a CORESET configured with CORESETPoolindex

All contributions [2-5] seem to agree that the existing TCI State Indication for UE-specific PDCCH MAC CE is not enough.

Q1: Do you agree that the exsiting MAC CE is not sufficient to support Rel-17 PDCCH enhancement and therefore we should introduce enhanced MAC CE?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Intel | Yes |  |
| ZTE | Yes | The current MAC CE carry only one TCI state information for the PDCCH. |
| LG | Yes |  |
| Xiaomi | Yes |  |
| Lenovo&MM | Yes |  |
| OPPO | Yes |  |
| CATT | Yes |  |
| Huawei, HiSilicon | Yes |  |
| Qualcomm | Yes |  |
| Samsung | Yes |  |
| vivo | Yes |  |
| Ericsson | Yes |  |
| Nokia, Nokia Shanghai Bell | Yes (see comments) | Current MAC CE allows to indicate serving cell ID, CORESET ID and TCI state ID - see below. Two TCI state IDs could be activated via two MAC CEs (sent in the same TB), which would require 2*2=4 octets. Using a new MAC CE would likely require 3 octets (5+4+2*7 = 23 bits --> 3 octets), so the overhead would be 1 octet but would not require new MAC CEs to be defined. So there is some benefit from the new MAC CE anyway. However, in case additional information is needed in the MAC CE, a new MAC CE is anyway required (as the current MAC CE for PDCCH TCI state indication is not extendible), so we might need to wait for RAN1 to provide all the information before proceeding with CR. |

RAN1 requested to include the following fields 1) serving cell ID, 2) CORESET ID, 3) Two TCI state IDs.

Q2: Do you agree that the enhanced MAC CE should include the following fields 1) serving cell ID, 2) CORESET ID and 3) Two TCI state IDs?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Intel | Yes |  |
| ZTE | Yes |  |
| LG | Yes |  |
| Xiaomi | Yes |  |
| Lenovo&MM | Yes |  |
| OPPO | Yes |  |
| CATT | Yes |  |
| Huawei, HiSilicon | Yes |  |
| Qualcomm | Yes |  |
| Samsung | Yes |  |
| vivo | Yes |  |
| Ericsson | Yes | At least |
| Nokia, Nokia Shanghai Bell | Yes | This is the minimum information needed. However, before rushing to create CRs for this, we could wait for further RAN1 feedback on other features (e.g. the "unified TCI state" concept) |

Three companies provide the possible enhanced MAC CE structure. Two of them ([2,4]) are the same format, while the other [5] is slightly different. However, there seems no big difference. Is there any preference between two formats?

Q3: Which MAC CE strcture is preferred?

Option 1 [2,4]						Option 2 [5]

| Company | Preference | Comments |
| --- | --- | --- |
| Intel | Either one is ok. |  |
| ZTE | Either one is Okay |  |
| LG | Option 1 | We think both options work, but we prefer to simply add 1byte of new field after the existing format. |
| Xiaomi | Either one is ok | Although we are the proponent of Option 2, Option 1 is also acceptable to us. With the R bit at the beginning of the MAC CE, the MAC CE would be more future extentable. |
| Lenovo&MM | Either one is fine. | If more TCI state should be added e.g. 3 TCI state, option 1 is easy to extend. |
| OPPO | Either one is Ok. |  |
| CATT | Either one is ok. |  |
| Huawei, HiSilicon | Either one is ok. | Perhaps we don't need to rush to make a choice |
| Qualcomm | Option 1 | Slightly prefer to option 1. It is simpler. Just add 1-byte right after the original format. |
| Samsung | Either one is ok. | This would be the baseline, we don’t need to make a choice now. |
| vivo | Either one is OK | But we think it is not urgent for RAN2 to make the decision on the choice. RAN1 will have more requirements on the design of multiple new MAC CEs. We could consider them together. |
| Ericsson | either | Prefer not to make such selection now. If there is strong push to start running MAC CR now, we can just have header for this MAC CE and editors note. |
| Nokia, Nokia Shanghai Bell | No strong view | Agree with LGE than option 1 is more similar to legacy but option 2 is more aligned with the usual way to do new MAC CEs. Either could still work so no strong view. |

There are some views that RAN2 should ask RAN1 on some questions to clarify [4,5].

- Whether the enhanced TCI state indication for UE specific PDCCH MAC CE can be applied to a set of serving cells configured in simultaneousTCI-UpdateList1 or simultaneousTCI-UpdateList2? [4]

- Which CORESET can be indicated with two TCI states ? [5]

- How many TCI states (i.e. maximum number) can be configured for the CORESET indicated with two TCI states? [5]

- Anything else?

Q4: Do you agree to send LS to aks RAN1 some questions? If yes, are the above question A-C reasonable to ask? Companies are also invited to provide more questions if deemed useful.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Intel | No | Question A: This issue is currently captured as FFS bullet in RAN1 LS [1]. Question B: we understand that it should not be applicable to CORESET 0 similar to Rel-15 MAC CE design. Question C: There is no proposal in RAN1 to increase the number of TCI states to larger value comparing to Rel-15. |
| ZTE | No | Question A: Agree with Intel Question B: Regarding the comments from Intel, it seems CORESET0 is supported by this MAC CE as shown below - CORESET ID: This field indicates a Control Resource Set identified with ControlResourceSetId as specified in TS 38.331 [5], for which the TCI State is being indicated. In case the value of the field is 0, the field refers to the Control Resource Set configured by controlResourceSetZero as specified in TS 38.331 [5]. The length of the field is 4 bits; In our understanding, all the CORESET including CORESET0 can be indicated by this MAC CE as Rel15, no need to ask the question. Question C: Agree with Intel In addition to above three questions, we think there is one question D is supposed to be asked to RAN1: In rel-15, the first 64 entries of the tci-States-ToAddModList can be applied to CORESET 0 while up to 128 entries of the tci-States-ToAddModList can be applied to the CORESET other than CORESET 0. we would like to ask whether this rule is still available for the newly introduced MAC CE? |
| LG | No | We have same understanding with Intel on Question A. Regarding Question B and C, if there is no additional request from RAN1, we think that RAN2 can go with same way as legacy. |
| Xiaomi | B. C. | Question A is already part of the RAN1 feMIMO discussion. For Question B and C, we think that which CORESET can be configured should be up to RAN1 to decided. Even though companies consider that we should allow CORESET#0 as legacy, the RRC configuration of TCI state for CORESET#0 still needs to be provided by RAN1. We just hope companies can keep this in mind that the CORESET configuration details should be up to RAN1 to decided, RAN2 should not make any assumption based on the old MAC CE. |
| Lenovo&MM | No | Question A: RAN1 is discussins this. Question B: RAN1 will discuss whether two TCI states are applied to CORESET 0. Question C: Ageee with Intel. |
| OPPO | No | Question A: The discussion on whether to support group-based TCI state indication for UE specific PDCCH is ongoing in RAN1, we can wait for their agreement on that. Question B: Agree with ZTE. TCI states indication for coreset 0 by MAC CE is supported in legacy spec, that is, all of the coresets can be indicated with two TCI states. Question C: Agree with Intel, the number of TCI states shall keep as legacy. |
| CATT | No |  |
| Huawei, HiSilicon | No | RAN1 will provide such information anyway |
| Qualcomm | No | RAN1 is still under discussing about this MAC CE on some of questions above. If RAN2 has different understanding on the detailed field interpretation, RAN2 should check with RAN1 and let RAN1 make final decision. |
| Samsung | No | For now, we think RAN2 just wait for RAN1. |
| vivo | No | The corresponding discussion is on-going in RAN1. We understand that RAN1 will provide such information later. We could just wait for more information from RAN1. |
| Ericsson | No | Most of these if not all will get clarified when RRC parameters come. Companies can also remind their RAN1 delegates to provide this as input when they next time send LS |
| Nokia, Nokia Shanghai Bell | No (see comments) | A: Agree that this need not be asked. RAN1 will tell RAN2 when they decide. B: As ZTE pointed out, CORESET#0 is supported now and should be supported also with the new mechanism. That's because it's possible that cell only uses CORESET#0 for everything. C: The question is a bit unclear but we assume RAN1 will tell if the amount of bits needed for TCI states changes. Since RAN1 is currently discussing the "unified TCI state" anyway, we expect later information possibly affecting RAN2 MAC CE design anyway. Hence, we think RAN2 can wait for further RAN1 feedback first and then decide whether LS is needed. |

Q5: Is there any aspect that RAN2 needs to discuss?

| Company | Comments |
| --- | --- |
| Nokia, Nokia Shanghai Bell | Once RAN1 progresses on the "unified TCI state", we may need some modifications to MAC on that. But it's best to wait for RAN1 progress on that before trying to take it into account. |

# 3	Conclusion

All companies agree the following points:

Proposal 1: An enhanced MAC CE is introduced for PDCCH activating two TCI states.

Proposal 2: The enhanced MAC CE includes 1) serving cell ID, 2) CORESET ID and 3) Two TCI state IDs.

Most of companies don’t have preference on MAC CE structure but two companies showed preference on option 1 to have a common structure as the legacy MAC CE. One company mention that the enhanced MAC CE structure may be affected based on RAN1’s on-going discussion on “unified TCI state”.

Proposal 3: The enhanced MAC CE is extended based on the legacy MAC CE for PDCCH (i.e. option 1) but the CRs will be discussed after RAN1 send further details.

Regarding the need of asking further questions to RAN1, most of companies think there is no need to ask in this meeting, but we could wait until RAN1 provide more details.

Proposal 4: no LS is required in this meeting.

# 4	References

- R2-2104712	LS on TCI states indication for PDCCH (R1-2104064; contact: Intel)	RAN1	LS in	Rel-17	NR_feMIMO-Core	To:RAN2

- R2-2105027	Enhanced MAC CE for PDCCH in multi-TRP deployment	Intel Corporation	discussion	Rel-17	NR_feMIMO

- R2-2105907	On the LS about Activating two TCI states with a MAC CE	Ericsson	discussion	NR_feMIMO-Core

- R2-2105858	Consideration on the enhanced TCI state indication MAC CE for PDCCH	ZTE, Sanechips	discussion	Rel-17	NR_feMIMO-Core

- R2-2105731	Enhanced TCI State Indication for UE-specific PDCCH MAC CE	Xiaomi Communications	discussion	Rel-17	NR_feMIMO-Core