3GPP TSG-RAN WG2 Meeting #114-e	R2-2106712

# Electronic meeting, May 19 – 27, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 2699 | rev | - | Current version: | 16.4.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to ca-ParametersNR-ForDC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2021-05-26 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The current definition of ca-ParametersNR-forDC implies that: - If, and only if, the UE supports all features that it reports for CA also for NR-DC, the UE should include ca-ParametersNRDC but omit all versions of ca-ParametersNR-ForDC(-vXXXX) therein. The NW should then inherit the content of the corresponding ca-ParametersNR(-vXXXX) and assume that the UE supports them for NR-DC as well; - If the UE supports one or more features for CA but not for NR-DC (or a feature for NR-DC but not for CA), the UE has to include all the ca-ParametersNR-ForDC(-vXXXX) fields applicable to NR-DC within CA-ParametersNRDC and its extensions; However, 1) creates compatibility problems between new UEs and legacy gNBs. For instance, when reporting its capabilities to a legacy gNB, a Rel-16 UE may not include in CA-ParametersNRDC any of the ca-ParametersNR-ForDC from Rel-15 (since it does not support them for NR-DC). But it includes the Rel-16 extension, ca-ParametersNRDC-v1610, since it supports those features for NR-DC. From the perspective of the legacy gNB, which does not understand the Rel-16 extension, it will find the CA-ParametersNRDC without any Rel-15 fields for ca-ParametersNR-ForDC, and will thus consider that the UE indicated that all its parameters reported for CA in Rel-15 in ca-ParametersNR are also applicable for NR-DC. Hence, this behaviour is corrected so that the inheritance of ca-ParametersNR(-vXXX) upon absence of ca-ParametersNR-forDC(-vXXX) for NR-DC is handled independently for each extension of ca-ParametersNR-forDC(-vXXX). For instance, if the UE included ca-ParametersNR-v1540 branch and it also supports those features for NR-DC, it should omit ca-ParametersNR-ForDC-v1540, since the UE capabilities reported in ca-ParametersNR-v1540 shall be inherited to NR-DC as well. If the UE, however, included ca-ParametersNR containing parallelTxSRS-PUCCH-PUSCH, but it does not support parallelTxSRS-PUCCH-PUSCH for NR-DC, it would still have to report ca-ParametersNR-ForDC, but without including parallelTxSRS-PUCCH-PUSCH therein. In other words, the NR-DC capability is either coming fully from ca-ParametersNR-ForDC(-vXXXX) or fully from ca-ParametersNR(-vXXXX), and not from both these IE “groups”. If ca-ParametersNR-ForDC(-vXXXX) is absent, ca-ParametersNR(-vXXXX) is valid also for NR-DC. If the UE includes ca-ParametersNR-ForDC(-vXXXX), and an IE under ca-ParametersNR-ForDC(-vXXXX) is absent, the capability IE shall be considered absent for NR-DC even if it is present in ca-ParametersNR(-vXXXX). If ca-ParametersNR-ForDC(-vXXXX) is present but empty, none of the capabilities under CA-ParametersNR(-vXXXX) are supported for NR-DC. Impact analysis Impacted 5G architecture options: NR-DC Impacted functionality: CA parameters supported for NR-DC Inter-operability: If the network implements the CR and the UE does not, the UE may omit an NR-DC extension ca-ParametersNR-ForDC(-vXXXX) for the sake of indicating that it does not support the fields therein for NR-DC. However, the network will interpret as the UE inherits the parameters of the omitted extension (ca-ParametersNR-ForDC(-vXXXX)) from the corresponding extension indicated for CA (ca-ParametersNR(-vXXXX)). Hence, the network may attempt to configure the UE with features it does not support for NR-DC. If the UE implements the CR and the network does not, the UE may omit an NR-DC extension ca-ParametersNR-ForDC(-vXXXX) for the sake of indicating that the UE inherits the parameters of the omitted extension (ca-ParametersNR-ForDC(-vXXXX)) from the corresponding extension indicated for CA (ca-ParametersNR(-vXXXX)). However, the network will interpret as the UE does not support the parameters of the omitted extension (ca-ParametersNR-ForDC(-vXXXX)). Hence, the network may not configure features for NR-DC that the UE may have actually supported. This CR is considered mandatory to support the impacted functionality. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 6.3.3 UE capability information elements - CA-ParametersNRDC - correct so that so that the inheritance of ca-ParametersNR(-vXXX) upon absence of ca-ParametersNR-forDC(-vXXX) for NR-DC is handled independently for each extension of ca-ParametersNR-forDC(-vXXX). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A compatibility problem will be present for NR-DC configuration between new UEs and legacy gNBs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

#### –	CA-ParametersNRDC

The IE CA-ParametersNRDC contains dual connectivity related capabilities that are defined per band combination.

CA-ParametersNRDC information element

-- ASN1START

-- TAG-CA-PARAMETERS-NRDC-START

CA-ParametersNRDC ::=                         SEQUENCE {

ca-ParametersNR-ForDC                       CA-ParametersNR                              OPTIONAL,

ca-ParametersNR-ForDC-v1540                 CA-ParametersNR-v1540                        OPTIONAL,

ca-ParametersNR-ForDC-v1550                 CA-ParametersNR-v1550                        OPTIONAL,

ca-ParametersNR-ForDC-v1560                 CA-ParametersNR-v1560                        OPTIONAL,

featureSetCombinationDC                     FeatureSetCombinationId                      OPTIONAL

}

CA-ParametersNRDC-v1610 ::= SEQUENCE {

-- R1 18-1: Semi-static power sharing mode1 between MCG and SCG cells of same FR for NR dual connectivity

intraFR-NR-DC-PwrSharingMode1-r16        ENUMERATED {supported}         OPTIONAL,

-- R1 18-1a: Semi-static power sharing mode 2 between MCG and SCG cells of same FR for NR dual connectivity

intraFR-NR-DC-PwrSharingMode2-r16        ENUMERATED {supported}         OPTIONAL,

-- R1 18-1b: Dynamic power sharing between MCG and SCG cells of same FR for NR dual connectivity

intraFR-NR-DC-DynamicPwrSharing-r16      ENUMERATED {short, long}       OPTIONAL,

asyncNRDC-r16                            ENUMERATED {supported}         OPTIONAL

}

CA-ParametersNRDC-v1630 ::=                         SEQUENCE {

ca-ParametersNR-ForDC-v1610                 CA-ParametersNR-v1610                        OPTIONAL,

ca-ParametersNR-ForDC-v1630                 CA-ParametersNR-v1630                        OPTIONAL

}

CA-ParametersNRDC-v1640 ::=                  SEQUENCE {

ca-ParametersNR-ForDC-v1640                  CA-ParametersNR-v1640                        OPTIONAL

}

-- TAG-CA-PARAMETERS-NRDC-STOP

-- ASN1STOP

CA-ParametersNRDC field descriptions

ca-ParametersNR-forDC (with and without suffix) If this field is present for a band combination, it reports the UE capabilities when NR-DC is configured with the band combination. If a version of this field (i.e., with or without suffix) is absent for a band combination, the corresponding ca-ParametersNR field version in BandCombination is applicable to the UE configured with NR-DC for the band combination. If a version of this field (i.e., with or without suffix) is present for a band combination but does not contain any parameters, the UE does not support the corresponding field version when configured with NR-DC for the band combination.

featureSetCombinationDC If this field is present for a band combination, it reports the feature set combination supported for the band combination when NR-DC is configured. If this field is absent for a band combination, the featureSetCombination in BandCombination (without suffix) is applicable to the UE configured with NR-DC for the band combination.

END OF CHANGE