3GPP TSG-RAN WG2 Meeting #115 electronic			     R2-2107397

Online, August, 2021

Agenda Item:	8.14.2.2

Source: 	OPPO

Title:  	Discussion on QoE measurement pausing and resuming

Document for:	Discussion and decision

- Introduction

At reception of QoE release, the UE shall discard any unsent QoE reports corresponding to the released QoE configuration

FFS whether pause resume will affect all configurations or whether pause resume can act selectively per configuration.

On whether to store reports in the AS or the application layer at Pause, send LS to SA4/SA5/SA3 to inform them about the options and their pros/cons (if possible) and ask them for feedback. RAN2 will continue work on this topic based on the feedback received.

In the last RAN2#114-e meeting, agreements regarding QoE measurement pausing and resuming have been made, indicated as follows:

In this contribution, we would like to further address our views on the FFS and other aspects

2 Discussion

In the RAN2 #114e meeting, it was FFS whether pause/resume operation will have impact on all configurationS or only some or one configuration. Generally, the pause/resume operation is commended to do when the air-interface is overloaded temporarily. According to the R2-113bise meeting, SRB4 is defined for transmission of the QoE measurement report. Compared with other SRB such as SRB1, SRB2 and SRB3, the scheduling priority of the SRB4 is the lowest. As a result, in our opinion, whether or not to pause QoE collection job for all or some/one of the QoE configuration has little difference since when the air-interface is overloaded, QoE measurement reports can share only little resources. Bearing this in mind, we should not bring much complexity to the RAN specification and therefore we propose to only have one category of pause/resume operation that have impact on all QoE configurations.

Proposal 1: RAN2 to agree that the granularity of pause/resume action will affect all configurations

Subsequently, regarding how to execute the QoE pause/resume when RAN air-interface is overloaded, in our opinion, CN/OAM shall be at least indicated by the RAN of the decision of the QoE pause/resume, and thus not expecting the feedback of the QoE measurement report from RAN. Although the details of the specification of the signalling over NG interface is out of RAN2 scope, but we still think that maybe a LS needed to be sent out towards SA2 querying whether or not such indication is needed.

Proposal 2: RAN2 to agree that a LS needed to be sent out towards SA2 querying whether or not indication of the RAN decision of the QoE measurement pause/resume towards CN/OAM is needed.

3. Conclusion

Based on the discussion above, following observations and proposals are made:

Proposal 1: RAN2 to agree that only one category of pause/resume operation that pause/resume will affect all configurations should be specified.

Proposal 2: RAN2 to agree that a LS needed to be sent out towards RAN3 for the need of indication of the RAN decision of the QoE pause/resume.