3GPP TSG-RAN WG2 Meeting #115-e	R2-2108187

# Electronic Meeting, 16 – 27 August, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 2756 | rev | - | Current version: | 15.14.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on release of RadioBearerConfig during MR-DC release |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2021-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | When the MR-DC release procedure is triggered, the UE releases all the lower layers of the SCG but is not clear how the PDCP entity and bearer configured within the RadioBearerConfig IE are handled. The understanding is that when the MR-DC release is triggered, the UE should not release the RadioBearerConfig, unless the network instruct the UE to do so. This is not enterely clear from the NOTE present in TS 38.331, section 5.3.5.4. NOTE: Release of cell group means only release of the lower layer configuration of the cell group but the RadioBearerConfig may not be released. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Section 5.3.5.4 - The part “but the RadioBearerConfig may not be released” is deleted as this is not a UE implementation whether to keep the RadioBearerConfig or not but rather a network choice. Impact Analysis Impacted 5G architecture options: (NG)EN-DC, NE-DC,NR-DC Impacted functionality: MR-DC release Inter-operability: 1. If the network is implemented according to the CR and the UE is not, the UE may release the RadioBearerConfig even if the network does not wish to do it and this may create a misalignment between the UE and the network. 2. If the UE is implemented according to the CR and the network is not, the UE may release the RadioBearerConfig even if the network does not wish to do it and this may create a misalignment between the UE and the network. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the CR is not agreed, the UE may release the RadioBearerConfig even if the network does not wish to do it and this may create a misalignment between the UE and the network. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

#### 5.3.5.4	Secondary cell group release

The UE shall:

1>	as a result of SCG release triggered by E-UTRA (i.e. (NG)EN-DC case) or NR (i.e. NR-DC case):

2>	reset SCG MAC, if configured;

2>	for each RLC bearer that is part of the SCG configuration:

3>	perform RLC bearer release procedure as specified in 5.3.5.5.3;

2>	release the SCG configuration;

2>	stop timer T310 for the corresponding SpCell, if running;

2>	stop timer T304 for the corresponding SpCell, if running.

NOTE:	Release of cell group means only release of the lower layer configuration of the cell group.

END OF CHANGES