3GPP TSG-RAN WG2 Meeting #115-e                                        R2-2108338

Online, Aug 9th – Aug 27th, 2021

Agenda Item:	9.2.4.1

Souce:	MediaTek Inc.

Title:	On Cell Re-selection in IoT-NTN

Document for:	Discussion and decision

# 1 Introduction

A satellite forms multiple beams projecting on Earth. Each beam covers a certain area on earth. As the LEO satellite is moving, these beams are also actually moving with time. This means, if a UE is static on earth and receives the signal from a satellite, the serving beam of a UE will be changing over time. A beam is actually the coverage through a set of antenna elements. For different set of antenna elements, the formed beam on Earth can be adjusted to reduce overlapping between beams. This has been discussed during the IoT-NTN Study Item (SI) [1] and NR-NTN SI [2]. Table 1 below highlights the major RAN2 recommendations for Idle Mode operations in NR-NTN [1].

Table 1: Major Recommendations from RAN2 on Idle mode Operations in NR-NTN

- Definition of additional assistance information for cell selection/reselection (e.g. using UE location information, satellite Ephemeris information) - NTN cell specific information in SIB

In this contribution, we discuss cell re-selection in IoT-NTN, with minimal impact on the specifications and point out that UE’s location information processing is not required.

# 2 Discussion

For initial cell selection IoT-NTN can use legacy cell selection criteria, i.e., Srxlev > 0 AND Squal > 0, mentioned in Section 5.2.3.2/5.2.3.2a of 3GPP TS 36.304 [2], where Srxlev and Squal are defined as:

For eMTC:
Srxlev = Qrxlevmeas – (Qrxlevmin + Qrxlevminoffset )– Pcompensation - Qoffsettemp  and

Squal = Qqualmeas – (Qqualmin + Qqualminoffset) - Qoffsettemp.

For NB-IoT:

Srxlev = Qrxlevmeas – Qrxlevmin – Pcompensation - Qoffsettemp

Squal = Qqualmeas – Qqualmin – Qoffsettemp

For intra-frequency cell re-selection, as mentioned in Section 5.2.4.2/5.2.4.2a of 3GPP TS 36.304 [2], if the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, the UE may choose not to perform intra-frequency measurements, otherwise, the UE shall perform intra-frequency measurements. Hence, by suitably adjusting the values of SIntraSearchP and SIntraSearchQ the network can control when the individual UE’s will start searching the neighbour cells.

Figure 1(a): RSRP variation with mobility in TN cell    Figure 1(b): RSRP variation with mobility in NTN cell

Figure 1(a) shows simulation results in terrestrial networks (TN), with a UE moving along the x axis and cells -1, 0 and 1 are located along the x-axis at positions -2.5km, 0km and 2.5km respectively. Figure 1(b), on the other hand, demonstrates simulation results in an NTN network, with the UE located in the centre of the Satellite trajectory and the cells (beam-spots) -1, 0 and 1 located at angles -5, 0 and 5 degrees respectively. The relative difference of RSRP between the strongest cell (beam) and the next strongest for each of TN/NTN cases is compared below in Figure 2.

Figure 2: Comparative RSRP Dynamics with mobility in TN and NTN

A close look into Figure 2 reveals that although the RSRP difference changes faster in TN, for most of the cell areas (almost ~ 64%), the difference between TN and NTN is only around 1.8dB. The deviation is primarily evident at the cell centre where re-selection is not expected to take place. Thus, R16-based initial cell selection and intra frequency measurements could be reused in IoT-NTN.

Observation 1: For most of the cell areas the difference in RSRP changes between TN and NTN is not much.

Proposal 1: Legacy eMTC and NB-IoT cell selection procedures and intra frequency measurements could be reused in IoT-NTN.

It is expected that TN and NTN systems will operate at different frequencies. Hence, priority based inter-frequency cell reselection, as mentioned in Section 5.2.4.5 of 3GPP 36.304 [2] could be used for cell reselection between TN and NTN systems for eMTC. The eNB of NTN can configure suitable values of ThreshX, HighQ, ThreshX, High, ThreshServing, LowQ, and ThreshServing, LowP to assist the UEs in efficient cell re-selection. A reasonable network implementation would provide higher priorities to TN cells. Similarly for NB-IoT, different frequency specific offsets Qoffset as specified in Section 5.2.4.6 of 3GPP 36.304 [2] can configured by the eNB for TN and NTN cells to promote reselections to TN systems. However, the ultimate decision is left to network deployment.

Observation 2: UE’s Location information does not provide any additional gain over existing re-selection mechanisms and also results in increased power consumption.

Proposal 2: Legacy eMTC and NB-IoT priorities and frequency specific offsets can be reused to control TN-NTN cell re-selection.

Similarly, for cells with the same priority, existing ranking schemes could be used. As mentioned in Section 5.2.4.6 of 3GPP 36.304 [2], existing ranking mechanisms estimate ranking of serving and neighbour cells as: Rs = Qmeas,s + Qhyst – Qoffsettemp + QoffsetSCPTM and Rn = Qmeas,n – Qoffset – Qoffsettemp+ QoffsetSCPTM. Network can configure and adjust the values of Qoffset and Qoffsettemp to prioritize the particular cells (beam-spots) along the trajectory of the LEO satellites.

As an example, let’s assume the UE is in beam 6, as shown in Figure 3. The network knows that beam 2 and beam 7 are the possible candidates of next beam on 1st tier. It also knows that beam 3 and beam 11 are the candidates of the beam after next beam on 2nd tier. Hence, for UEs in the coverage of beam 6, the network can reduce the Qoffset and Qoffsettemp for beam 2 and beam 7, so that the rankings of beam 2 and beam 7 are increased and UE re-selects beam 2 and beam 7. This approach could be extended across 2nd tier, consisting of beam 3 and beam 11.

Figure 3: Beam trajectory and next cell information

Based, on the discussions above, UE’s location is clearly unnecessary as it does not provide any additional gain. Moreover, constant checking and processing of UE’s location will drain UE’s battery in the idle mode.

Proposal 3: Legacy eMTC and NB-IoT cell ranking schemes could be reused to trigger fast cell re-selection of upcoming neighbour cells in IoT-NTN.

LEO NTNs with moving beams might not have continuous cell coverage, i.e. there could be intermittent coverage holes between consecutive cells (beams). Hence, UEs might face out-of-coverage problems during the cell movements. Such information about coverage holes could be provided to the UEs by the network by considering satellite’s long-term (coarse-grained) ephemeris. During initial access, the assistance information corresponding to the serving beam can indicate the following information of the beams around the potential trajectory due to the satellite movement:

- Physical cell IDs of the beam candidates as the next beam (first tier)

- Physical cell IDs of the beam candidates as the beam after next beam (second tier)

- SSB information of the beam candidates as the next beam (first tier)

- The starting frequency domain position

- Periodicity and symbol offset

- SSB information of the beam candidates as the beam after next beam (second tier)

- The starting frequency domain position

- Periodicity and symbol offset

- The difference of the frequency pre-compensation value between the serving beam and the beam candidates as the next beam (first tier)

- The difference of the frequency pre-compensation value between the serving beam and the beam candidates as the beam after next beam (second tier).

Proposal 4: In NTN coverage holes associated with cell re-selection could be informed to the UEs by using satellite’s long-term (coarse-grained) ephemeris. UEs can use this information for acquiring knowledge about coverage holes (out-of-coverage) and cell re-selection.

# 3 Conclusion

In this contribution, we have the following observations and proposals regarding the idle mode operations, involving cell re-selection with moving cells in IoT-NTN:

Observation 1: For most of the cell areas the difference in RSRP changes between TN and NTN is not much.

Proposal 1: Legacy eMTC and NB-IoT cell selection procedures and intra frequency measurements could be reused in IoT-NTN.

Observation 2: UE’s Location information does not provide any additional gain over existing re-selection mechanisms and also results in increased power consumption.

Proposal 2: Legacy eMTC and NB-IoT priorities and frequency specific offsets can be reused to control TN-NTN cell re-selection.

Proposal 3: Legacy eMTC and NB-IoT cell ranking schemes could be reused to trigger fast cell re-selection of upcoming neighbour cells in IoT-NTN.

Proposal 4: In NTN coverage holes associated with cell re-selection could be informed to the UEs by using satellite’s long-term (coarse-grained) ephemeris. UEs can use this information for acquiring knowledge about coverage holes (out-of-coverage) and cell re-selection.

# 4 References

- RP-211456, TR 36.763 V1.0.0 Study on Narrow-Band Internet of Things (NB-IoT) / enhanced Machine Type Communication (eMTC) support for Non-Terrestrial Networks (NTN)

- 3GPP TR 38.821 Solutions for NR to support Non-Terrestrial Networks (NTN)

- 3GPP TS 36.304 User Equipment (UE) procedures in Idle mode and RRC Inactive state.